3GPP TSG-RAN5 Meeting #95-e	R5-222122

# Online, , 9th May 2022 - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2294 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for Procedure for UE-requested PDU session modification after the first S1 to N1 mode change |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The current definition of the procedure paramter ExpectedNumberOfPDUSessionModifications is: Usage: Parameter determining procedure sequence. A value for this parameter shall be provided when the procedure is called. Unless the test requires specific number of PDU modifications to take place the value should be set to pc_noOf_PDNsSameConnection + pc_noOf_PDNsNewConnection ([19]). Meaning: The number of PDU sessions which are expected to happen. Depends on the UE configuration and/or the context in which the procedure is used. The parameter pc_noOf_PDNsSameConnection is defined as : “Number of UE requested PDN connection establishments after ATTACH in a new signaling connection”. This implies that the PDN connection which is made during the initial EPS attach procedure is not counted in these PICs parameters. So the total count of the PDN connection established in S1 mode (and hence the value for p_ExpectedNumberOfNewPDUSessions) should actually be “pc_noOf_PDNsSameConnection + pc_noOf_PDNsNewConnection + 1” |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add +1 to definition of parameter ExpectedNumberOfPduSessionModification |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE operating in multi-PDU/PDN mode may fail test cases using this generic prcoedreu |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5A.2C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Assocaited with TTCN CR R5s220xxx |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | A conformant UE may fail test cases using this generic procedure |  |  |  |  |  |  |  |  |

### 4.5A.2C	Procedure for UE-requested PDU session modification after the first S1 to N1 mode change / Single-registration mode with N26

4.5A.2C.1	Scope

The purpose of this procedure is to handle UE-requested PDU session modification after the first S1 to N1 mode change in the scenario of Single-registration mode, 'Interworking without N26 interface not supported'.

Table 4.5A.2C.1-1: Conditions and other parameters

| Condition | Explanation |
| --- | --- |
| ExpectedNumberOfPDUSessionModifications | Usage: Parameter determining procedure sequence. A value for this parameter shall be provided when the procedure is called. Unless the test requires specific number of PDU modifications to take place the value should be set to pc_noOf_PDNsSameConnection + pc_noOf_PDNsNewConnection +1 ([19]). Meaning: The number of PDU sessions which are expected to happen. Depends on the UE configuration and/or the context in which the procedure is used. |
| K | Usage: Parameter determining procedure sequence. Meaning: The number of PDN SESSION MODIFICATION REQUEST messages already processed including the one that is currently being processed. |
| L | Usage: Parameter determining procedure sequence. Meaning: The number of PDN SESSION MODIFICATION REQUEST being received so far. |
| N | Usage: Parameter determining procedure sequence. The parameter is initialised with the value of ExpectedNumberOfPDUSessionModifications Meaning: Loop (step sequence repetition) control. |

4.5A.2C.2	Procedure description

4.5A.2C.2.1	Initial conditions

The UE is in RRC_CONNECTED state.

4.5A.2C.2.2	Procedure sequence

Table 4.5A.2C.2.2-1: Procedure for UE-requested PDU session modification after the first S1 to N1 mode change

| St | Procedure | Message Sequence |  | Verdict |
| --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |
| 0 | Set K = 0, L = 0, N = (ExpectedNumberOfPDUSessionModifications). | - | - | - |
| 1 | The procedure specified in Table 4.5A.2C.2.2-2 takes place. | - | - | - |
| 2 | Set K = K +1. | - | - | - |
| - | EXCEPTION: In parallel to the events described in steps 3-5a1 below the events specified in Table 4.5A.2C.2.2-2 may take place. | - | - | - |
| 3 | The SS transmits a RRCReconfiguration message and a PDU SESSION MODIFICATION COMMAND. | <-- | NR RRC: RRCReconfiguration 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMMAND | - |
| - | EXCEPTION: Depending upon UE implementation, step 4 and 4A can occur in any order | - | - | - |
| 4 | The UE transmits a RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - |
| 4A | The UE transmits a PDU SESSION MODIFICATION COMPLETE message. | --> | 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMPLETE |  |
| - | EXCEPTION: Steps 5a1 to 5b1 describe behaviour depending UE implementation; the "lower case letter" identifies a step sequence that take place if the UE performs a specific action. | - | - | - |
| 5a1 | IF L > K (NOTE 1) THEN repeat from step 2. | - | - | - |
| 5b1 | ELSE IF K < N (NOTE 2) THEN repeat from step 1. | - | - | - |
| NOTE 1: One (or more) further PDU SESSION MODIFICATION REQUEST message has been received in parallel. NOTE 2: Less PDU SESSION MODIFICATION REQUEST messages than expected have been received and processed so far -> Further request are expected from the UE. |  |  |  |  |

Table 4.5A.2C.2.2-2: Reception of PDU SESSION MODIFICATION REQUEST message

| St | Procedure | Message Sequence |  | Verdict |
| --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |
| 1 | Start Wait_Timer = 8 sec. NOTE: 8s were chosen to cater for T3540 being set to 10s. | - | - | - |
| - | EXCEPTION: Steps 2a1 to 2b1 describe behaviour depending UE implementation; the "lower case letter" identifies a step sequence that take place if the UE performs a specific action. | - | - | - |
| 2a1 | The UE transmits an ULInformationTransfer message and a PDU SESSION MODIFICATION REQUEST with PDU session ID which has been associated with a default EPS bearer set up during the UE operation in S1. | --> | NR RRC: ULInformationTransfer 5GSM: PDU SESSION MODIFICATION REQUEST | - |
| 2a2 | Stop Wait_Timer. | - | - | - |
| 2a3 | Set L = L +1. | - | - | - |
| 2a4 | Check: Does the L>N? (NOTE 1) | - | - | F |
| 2b1 | Check: Does Wait_Timer expire? (NOTE 2) | - | - | F |
| NOTE 1: The SS shall raise a fail verdict when there are more PDU SESSION MODIFICATION REQUEST messages received than expected and terminate the test. The reason for such a behaviour can be e.g. wrongly set PICS with which the ExpectedNumberOfNewPDNConnections was initiated. NOTE 2: Unless this is specified explicitly otherwise e.g. in a test procedure which calls the procedure specified in the present table, the SS shall raise a fail verdict when there are less PDU SESSION MODIFICATION REQUEST messages than expected received at this point and terminate the test. The reason for such a behaviour can be e.g. wrongly set PICS with which the ExpectedNumberOfNewPDUSessions was initiated, |  |  |  |  |

4.5A.2C.2.3	Specific message contents

All specific message contents shall be according clause 4.6 and 4.7 with the below exceptions:

Table 4.5A.2C.2.3-1: UL NAS TRANSPORT (Step 1, Table 4.5A.2C.2.2-2)

| Derivation Path: Table 4.7.1-10. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Request type |  | '101'B | "modification request" |  |

Table 4.5A.2C.2.3-2: PDU SESSION MODIFICATION REQUEST (Step 1, Table 4.5A.2C.2.2-2)

| Derivation Path: Table 4.7.2-7. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PDU session ID |  | The PDU session ID associated with the default EPS bearer of the PDN connection which is being transferred into PDU session and for which the UE needs to indicate its capabilities via the modification procedure. (NOTE 1) |  |  |
| 5GSM capability |  | Present. Contents not checked. |  |  |
| Integrity protection maximum data rate |  | Present. Contents not checked. |  |  |
| NOTE 1: The PDU session ID was provided in the ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT when the UE attached in S1. If the UE provides here an unknown PDU session ID then the SS shall set an Inconclusive verdict. |  |  |  |  |

Table 4.5A.2C.2.3-3: RRCReconfiguration (step 3, Table 4.5A.2C.2.2-1)

| Derivation Path: Table 4.6.1-13 and condition NR if SRB2 is not yet established |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | RadioBearerConfig with conditions SRB2 and DRB2 |  | NOT IMS_PDU |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig with condition SRB2_DRB2 |  | NOT IMS_PDU |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 4.5A.2C.2.3-4: RRCReconfiguration (step 3, 4.5A.2C.2.2-1)

| Derivation Path: Table 4.6.1-13 and condition NR if SRB2 is already established |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | RadioBearerConfig with condition DRBn | n is chosen as the next available number higher or equal to 2 |  |
|  |  | RadioBearerConfig with condition DRB1 |  | IMS_PDU |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig with condition DRBn | n is set to the same value as for the radioBearerConfig IE above |  |
|  |  | CellGroupConfig with condition DRB1 |  | IMS_PDU |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |