3GPP TSG-RAN5 Meeting #95-e	R5-222148

# Online, , 9th May 2022 - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-7 | CR | 0027 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New MCData Test Case 7.2.2 Off-network Enhanced Status CT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCenhUEConTest |  |  |  |  |  | Date: |  |  | 2022-04-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | No test case existed for an off-network Enhanced Status CT call |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test case 7.2.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An off-network Enhanced Status CT call would not be tested |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Specific message contents include changes proposed in CR R5-222141 - New MCData off-network signalling messages in 5.5.3.8 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<START OF CHANGES>

### 7.2.2	Off-network / Enhanced Status (ES) / Client Terminated (CT)

7.2.2.1	Test Purpose (TP)

(1)

with { UE (MCData Client) registered and authorized for MCData Service, including authorized for MCData Service in off-network environment, and, the UE is in an off-network environment }

ensure that {

when { the UE (MCData Client) receives an enhanced status via the SDS OFF-NETWORK MESSAGE message with a disposition of DELIVERY }

then { UE (MCDATA Client) sends SDS OFF-NETWORK NOTIFICATION message with a disposition notification type of DELIVERED and, initiates counter CFS2 (SDS notification retransmission) to 1 and starts timer TFS2 (SDS notification retransmission), and, renders the operational value of the received Enhanced Status ID as enhanced status to the MCDATA User }

}

(2)

with { UE (MCData Client) having sent a SDS OFF-NETWORK NOTIFICATION message and started timer TFS2 (SDS notification retransmission) }

ensure that {

when { TFS2 (SDS notification retransmission) expires }

then { UE (MCData Client) retransmits the SDS OFF-NETWORK NOTIFICATION message and, stops re-transmitting if the counter CFS2 (SDS notification retransmission) has reached its maximum value and TFS2 (SDS notification retransmission) }

}

7.2.2.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in:

TS 24.282 clauses 14.3.2, 9.3.2.4, 12.3.2, 12.3.5. Unless otherwise stated these are Rel-15 requirements.

[TS 24.282, clause 14.3.2]

Upon receipt of a SDS OFF-NETWORK MESSAGE message, the MCData client:

1)	shall follow the procedure defined in clause 9.3.2.4;

2)	shall attempt to match the received value with a "/<x>/<x>/Common/MCData/EnhSvcOpValues/<x>/EnhSvcOpID" leaf node present in the group configuration as specified in 3GPP TS 24.483 [4]; and

3)	if a match is found, shall render the associated operational value from "/<x>/<x>/Common/MCData/EnhSvcOpValues/<x>/EnhSvcOpValue" leaf node as enhanced status to the MCData user.

[TS 24.282, clause 9.3.2.4]

Upon receiving an SDS OFF-NETWORK MESSAGE message with a SDS disposition request type IE, the MCData client:

1)	shall store the value of Sender MCData user ID IE as the stored notification target MCData user ID;

2)	shall store the value of Conversation ID IE as the stored conversation ID;

3)	shall store the value of Message ID IE as the stored SDS message ID;

4)	shall store the current UTC time as the stored SDS notification time;

5)	if present, shall store the value of Application ID IE as the stored SDS application ID;

6)	if present, shall store the value of the Extended application ID IE as the stored SDS extended application ID;

7)	if present, shall store the value of MCData group ID IE to the stored target MCData group ID; and

8)	if the SDS disposition request type IE is set to:

a)	"DELIVERY" then, shall send a SDS OFF-NETWORK NOTIFICATION message as described in clause 12.3.2;

b)	"READ" then, shall send a SDS OFF-NETWORK NOTIFICATION message as described in clause 12.3.3; or

c)	"DELIVERY AND READ" then, shall start timer TFS3 (delivery and read).

NOTE: Duplicate messages (re-transmissions) that are received by the MCData client should not be processed again.

[TS 24.282, clause 12.3.2]

To send an off-network SDS delivery notification, the MCData client:

1)	shall store "DELIVERED" as the disposition type;

2)	shall generate a SDS OFF-NETWORK NOTIFICATION message as specified in clause 15.1.8. In the SDS OFF-NETWORK NOTIFICATION message, the MCData client:

a)	shall set the Sender MCData user ID IE to its own MCData user ID as specified in clause 15.2.15;

b)	shall set the Conversation ID IE as the stored conversation ID as specified in clause 15.2.9;

c)	shall set the Message ID IE as the stored SDS message ID as specified in clause 15.2.10;

d)	shall set the Date and time IE as the stored SDS notification time as specified in clause 15.2.8;

e)	shall set the SDS disposition notification type IE to the stored disposition type as specified in clause 15.2.5; and

f)	may set:

i)	the Application ID IE to the stored SDS application ID as specified in clause 15.2.7; or

ii)	the Extended application ID IE to the stored extended SDS application ID as specified in clause 15.2.24;

3)	shall send the SDS OFF-NETWORK NOTIFICATION message to the stored notification target MCData user ID as specified in clause 9.3.1.1;

4)	shall initialise the counter CFS2 (SDS notification retransmission) with the value set to 1; and

5)	shall start timer TFS2 (SDS notification retransmission).

[TS 24.282, clause 12.3.5]

Upon expiry of timer TFS2 (SDS notification retransmission), the MCData client:

1)	shall generate a SDS OFF-NETWORK NOTIFICATION message as specified in clause 15.1.8. In the SDS OFF-NETWORK NOTIFICATION message, the MCData client:

a)	shall set the Sender MCData user ID IE to its own MCData user ID as specified in clause 15.2.15;

b)	shall set the Conversation ID IE as the stored conversation ID as specified in clause 15.2.9;

c)	shall set the Message ID IE as the stored SDS message ID as specified in clause 15.2.10;

d)	shall set the Date and time IE as the stored SDS notification time as specified in clause 15.2.8;

e)	shall set the SDS disposition type IE to the stored disposition type as specified in clause 15.2.5; and

f)	may set:

i)	the Application ID IE to the stored SDS application ID as specified in clause 15.2.7; or

ii)	the Extended application ID IE to the stored extended SDS application ID as specified in clause 15.2.24;

2)	shall send the SDS OFF-NETWORK NOTIFICATION message to the stored sender MCData user ID as specified in clause 9.3.1.1;

3)	shall increment the counter CFS2 (SDS notification retransmission) by 1; and

4)	shall start timer TFS2 (SDS notification retransmission) if the associated counter CFS2 (SDS notification retransmission) has not reached its upper limit.

7.2.2.3	Test description

7.2.2.3.1	Pre-test conditions

System Simulator:

-	SS-UE1 (MCData Client)

-	For the underlying "transport bearer" over which the SS and the UE will communicate, the SS is behaving as SS-UE1 as defined in TS 36.508 [24], configured for and operating as ProSe Direct Communication transmitting and receiving device.

-	GNSS simulator to simulate a location and provide a timing reference for the assistance of E-UTRAN off-network testing.

NOTE 1:	For operation in off-network environment, it needs be ensured that after the UE is powered up it considers the Geographical area #1 that is simulated by the GNSS simulator as being one of the geographical areas set in the USIM for operation when UE is "not served by E-UTRAN".

-	SS-NW (MCData server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCVideo operation in the MCVideo configuration document).

NOTE 2:	The SS operation as NW (MCVideo server) is needed only for the Preamble if the UE has to perform the Generic Test Procedure for MCVideo Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2.

IUT:

-	UE (MCData Client)

-	The test USIM set as defined in TS 36.579-1 [2], subclause 5.5.10 is inserted.

-	For the underlying "transport bearer" over which the SS and the UE will communicate, the UE is behaving as a ProSe enabled UE as defined in TS 36.508 [24], configured for and operating as ProSe Direct Communication transmitting and receiving device.

-	CFS2 (SDS notification retransmission)is set to the default value of 5.

-	TFS2 (SDS notification retransmission)is set to the default value of 40 ms.

Preamble:

--	The UE has performed the Generic Test Procedure for MCData UE registration as specified in TS 36.579-1 [2], subclause 5.4.2B.

-	The MCData User performs the Generic Test Procedure for MCData Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2.

-	The GNSS simulator is configured to simulate a location in the centre of Geographical area #1 and provide a timing reference, as defined in TS 36.508 [24] Table 4.11.2-2 scenario #1.

-	The UE is switched-off.

-	UE States at the end of the preamble

-	The UE is in state 'switched-off'.

7.2.2.3.2	Test procedure sequence

Table 7.2.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Power up the UE. | - | - | - | - |
| 2 | Trigger the UE to reset UTC time and location. NOTE: The UTC time and location reset may be performed by MMI or AT command (+CUTCR). | - | - | - | - |
| 3 | Activate the MCData Client Application and register User A as the MCData User (TS 36.579-5 [5], px_MCX_User_A_username, px_MCX_User_A_password). (NOTE 1) | - | - | - | - |
| - | EXCEPTION: The E-UTRA/EPC actions which are related to the MCData call establishment are described in TS 36.579-1 [2], subclause 5.4.10 'Generic Test Procedure for MCPTT CT communication over ProSe direct one-to-many communication out of E-UTRA coverage / Announcing/Discoveree procedure for group member discovery'. The test sequence below shows only the MCData relevant messages exchanged. | - | - | - | - |
| - | EXCEPTION: Steps 4-6 are repeated 5 times. | - | - | - | - |
| 4 | SS-UE1 (MCData Client) sends an enhanced status via the SDS OFF-NETWORK NOTIFICATION message with disposition request type of DELIVERY. | <-- | SDS OFF-NETWORK MESSAGE | - | - |
| 5 | Start 40 millisecond timer. | - | - | - | - |
| 6 | 40 millisecond timer expires | - | - | - | - |
| - | EXCEPTION: Steps 7-9 are repeated CFS1=5 times (CFS1 defined in 24.282 [31] Table G.3.1-1) | - | - | - | - |
| 7 | Check: Does the UE (MCData Client) send a SDS OFF-NETWORK MESSAGE message with disposition notification type of DELIVERED? NOTE: It is expected that the UE - shall initialize the counter CFS2 (SDS notification retransmission) with the value set to 1 on the first transmission, and, increase it by 1 with each re-transmission. - shall start timer TFS2 (SDS notification retransmission) | --> | SDS OFF-NETWORK NOTIFICATION | 1,2 | P |
| 8 | Start TFS2 (SDS notification retransmission)=40 milliseconds as defined in 24.282 [31] Table F.3.1-1. | - | - | - | - |
| 9 | TFS2 expires. | - | - | - | - |
| - | EXCEPTION: UE releases the E-UTRA connection. The E-UTRA/EPC actions which are related to the MCData call release are described in TS 36.579-1 [2], subclause 5.4.8, 'Generic Test Procedure for MCPTT communication over ProSe direct one-to-one communication out of E-UTRA coverage - release by the UE'. | - | - | - | - |
| 10 | Check: Does the UE (MCDATA Client) render the operational value of the received Enhanced Status ID as enhanced status to the MCDATA User? (NOTE 1) | - | - | 1 | P |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

7.2.2.3.3	Specific message contents

Table 7.2.2.3.3-1: SDS OFF-NETWORK MESSAGE (step 4, Table 7.2.2.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.10-1, condition DELIVERED, MCD_grp |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |  |
| Payload | Payload as described in Table 7.2.2.3.3-2 |  |  |  |  |

Table 7.2.2.3.3-2: Payload in the SDS OFF-NETWORK MESSAGE (step 4, Table 7.2.2.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.10-3, condition DELIVERED, MCD_grp |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |  |
| Data payload message identity | '00000011'B | Data payload | TS 24.282 [87] clause 15.2.2 |  |  |
| Number of payloads | 1 | 1 payload | TS 24.282 [87] clause 15.2.12 |  |  |
| Payload |  |  | TS 24.282 [87] clause 15.2.13 |  |  |
| Payload IEI | '78'O |  |  |  |  |
| Length of Payload | length of the content |  |  |  |  |
| Payload content type | '00000110'B | ENHANCED STATUS |  |  |  |
| Payload data | "0" | The id as defined in the MCData Group Configuration Document | TS 36.579-1 [2], Table 5.5.7.3-1 |  |  |

Table 7.2.2.3.3-3: SDS OFF-NETWORK NOTIFICATION (steps 7, Table 7.2.2.3.2-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.11-1, condition DELIVERED

<END OF CHANGES>