3GPP TSG-RAN5 Meeting #95-e	R5-222155

# Online, , 9th May 2022 - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-6 | CR | 0053 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New MCVIDEO OFF-NETWORK TC 7.1.2.1 Off-network Broadcast Call CO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCenhUEConTest |  |  |  |  |  | Date: |  |  | 2022-04-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | No test case existed for an off-network CO broadcast group call |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new test case 7.1.2.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An off-network CO broadcast group call would not be tested |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2, 7.1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Specific message contents include changes proposed in CR R5-222142 - New MCVideo Off-network Message Defaults 5.5.14 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<START OF CHANGES>

### 7.1.2	Broadcast Group Calls

#### 7.1.2.1	Off-network / Group Call / Broadcast Group Call / Broadcast Group Call Retransmitting / Broadcast Group Call Release / Client Originated (CO)

7.1.2.1.1	Test Purpose (TP)

(1)

with { UE (MCVideo Client) registered and authorised for MCVideo Service, including authorised to initiate broadcast calls in off-network environment, and the UE (MCVideo Client) is in an off-network environment }

ensure that {

when { the MCVideo User requests to initiate an MCVideo broadcast call }

then { UE (MCVideo Client) sends a GROUP CALL BROADCAST message and enters the "B2: in-progress broadcast group call" state }

}

(2)

with { UE (MCVideo Client) in the "B2: in-progress broadcast group call" state, and the UE (MCVideo Client) is in an off-network environment }

ensure that {

when { the timer TFB2 (broadcast retransmission) expires }

then { UE (MCVideo Client) retransmits the GROUP CALL BROADCAST message }

}

(3)

with { UE (MCVideo Client) in the "B2: in-progress broadcast group call" state, and the UE (MCVideo Client) is in an off-network environment }

ensure that {

when { the MCVideo User requests to end the MCVideo broadcast call }

then { UE (MCVideo Client) sends a GROUP CALL BROADCAST END message and enters the "B1: start-stop" state }

}

7.1.2.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.281 clauses 9.4.2.4.1, 9.4.2.4.9, 9.4.2.4.7, 9.4.2.4.11, TS 24.581 clause 7.2.3.2.2. Unless otherwise stated these are Rel-15 requirements.

[TS 24.281, clause 9.4.2.4.1]

When in the "B1: start-stop" state, upon the indication from MCVideo user to initiate the broadcast group call, the MCVideo client:

1)	shall generate an SDP body as specified in clause 9.3.1.1.2 and store it as the SDP body of the call;

2)	shall generate a random number with uniform distribution between 0 and 65535 and store it as the call identifier of the call;

3)	shall store own MCVideo user ID as the originating MCVideo user ID of the call;

4)	shall store "BROADCAST GROUP CALL" as the current call type;

5)	shall generate a GROUP CALL BROADCAST message as specified in clause 17.1.18. In the GROUP CALL BROADCAST message, the MCVideo client:

a)	shall set the Call identifier IE to the stored call identifier of the call;

b)	shall set the Call type IE to the stored current call type;

c)	shall set the Originating MCVideo user ID IE to the stored originating MCVideo user ID of the call;

d)	shall set the MCVideo group ID IE to the stored MCVideo group ID of the call; and

e)	shall set the SDP IE to the stored SDP body of the call;

6)	shall set the ProSe per-packet priority to the value corresponding to MCVideo off-network broadcast callas described in 3GPP TS 24.483 [4];

7)	shall start transmission control as originating transmission participant as described specified in clause a.b in 3GPP TS 24.581 [5];

8)	shall send the GROUP CALL BROADCAST message as specified in clause 9.3.1.1.1;

9)	shall establish a media session based on the stored SDP body of the call;

10)	shall start timer TFB2 (broadcast retransmission); and

11)	shall enter the "B2: in-progress broadcast group call" state.

[TS 24.281, clause 9.4.2.4.9]

When in the "B2: in-progress broadcast group call" state, upon expiry of timer TFB2 (broadcast retransmission), the MCVideo client:

1)	shall generate a GROUP CALL BROADCAST message as specified in clause 17.1.18. In the GROUP CALL BROADCAST message, the MCVideo client:

a)	shall set the Call identifier IE to the stored call identifier of the call;

b)	shall set the Call type IE to the stored current call type;

c)	shall set the Originating MCVideo user ID IE to the stored originating MCVideo user ID of the call;

d)	shall set the MCVideo group ID IE to the stored MCVideo group ID of the call; and

e)	shall set the SDP IE to the stored SDP body of the call;

2)	shall send the GROUP CALL BROADCAST message as specified in clause 9.3.1.1.1;

3)	shall restart timer TFB2 (broadcast retransmission); and

4)	shall remain in the "B2: in-progress broadcast group call" state.

[TS 24.281, clause 9.4.2.4.7]

When in the "B2: in-progress broadcast group call" state, upon an indication from the originating MCVideo user to release the in-progress broadcast group call, the MCVideo client:

1)	shall release the media session;

2)	shall generate a GROUP CALL BROADCAST END message as specified in clause 17.1.19. In the GROUP CALL BROADCAST END message, the MCVideo client:

a)	shall set the Call identifier IE to the stored call identifier of the call;

b)	shall set the Originating MCVideo user ID IE to the stored originating MCVideo user ID of the call; and

c)	shall set the MCVideo group ID IE to the stored MCVideo group ID of the call;

3)	shall send the GROUP CALL BROADCAST END message as specified in clause 9.3.1.1.1;

4)	shall stop timer TFB2 (broadcast retransmission);

5)	shall stop transmission control; and

6)	shall enter the "B1: start-stop" state.

[TS 24.281, clause 9.4.2.4.7]

When in the "B2: in-progress broadcast group call" state or "B4: ignoring same call ID" state, upon expiry of timer TFB1 (max duration) the MCVideo client:

1)	shall release the media session;

2)	shall clear the stored call identifier;

3)	shall stop transmission control, if running; and

4)	shall enter the "B1: start-stop" state.

[TS 24.581, clause 7.2.3.2.2]

When an MCVideo call is established with session announcement including an explicit transmission request, the originating transmission participant:

1.	shall create an instance of a transmission participant state transition diagram for basic operation state machine;

2.	shall send Transmission Granted message towards other transmission participants. The Transmission Granted message:

a.	shall include the granted priority in the Transmission priority field;

b.	shall include the MCVideo user's own MCVideo ID in the User ID field; and

c.	if the transmission request is a broadcast group call, system call, emergency call or an imminent peril call, shall include a Transmission Indicator field indicating the relevant call types; and

3.	shall enter 'O: transmission arbitration' state.

7.1.2.1.3	Test description

7.1.2.1.3.1	Pre-test conditions

System Simulator:

-	SS-UE1 (MCVideo client)

-	For the underlying "transport bearer" over which the SS and the UE will communicate, the SS is behaving as SS-UE1 as defined in TS 36.508 [24], configured for and operating as ProSe Direct Communication transmitting and receiving device.

-	GNSS simulator to simulate a location and provide a timing reference for the assistance of E-UTRAN off-network testing.

NOTE 1:	For operation in off-network environment, it needs be ensured that after the UE is powered up it considers the Geographical area #1 that is simulated by the GNSS simulator as being one of the geographical areas set in the USIM for operation when UE is "not served by E-UTRAN".

-	SS-NW (MCVideo server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCVideo operation in the MCVideo configuration document).

NOTE 2:	The SS operation as NW (MCVideo server) is needed only for the Preamble if the UE has to perform the MCVideo User performs the Generic Test Procedure for MCVideo Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2.

IUT:

-	UE (MCVideo client)

-	The test USIM set as defined in TS 36.579-1 [2], subclause 5.5.10 is inserted.

-	For the underlying "transport bearer" over which the SS and the UE will communicate, the UE is behaving as a ProSe enabled UE as defined in TS 36.508 [24], configured for and operating as ProSe Direct Communication transmitting and receiving device.

Preamble:

-	The UE has performed the Generic Test Procedure for MCVideo UE registration as specified in TS 36.579-1 [2], subclause 5.4.2A.

-	The MCVideo User performs the Generic Test Procedure for MCVideo Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2.

-	The GNSS simulator is configured to simulate a location in the centre of Geographical area #1 and provide a timing reference, as defined in TS 36.508 [24] Table 4.11.2-2 scenario #1.

-	The UE is switched-off.

-	UE States at the end of the preamble

-	The UE is in state 'switched-off'.

7.1.2.1.3.2	Test procedure sequence

Table 7.1.2.1.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Power up the UE | - | - | - | - |
| 2 | Trigger the UE to reset UTC time and location. NOTE: The UTC time and location reset may be performed by MMI or AT command (+CUTCR). | - | - | - | - |
| 3 | Activate the MCVideo Client Application and register User A as the MCVideo User (TS 36.579-5 [5], px_MCX_User_A_username, px_MCX_User_A_password). (NOTE 1) | - | - | - | - |
| 4 | Make the UE (MCVideo Client) initiate an off-network broadcast call (NOTE 1) | - | - | - | - |
| - | EXCEPTION: The E-UTRA/EPC actions which are related to the MCVideo off-network call establishment are described in TS 36.579-1 [2], subclause 5.4.11 'Generic Test Procedure for MCPTT CO communication over ProSe direct one-to-many communication out of E-UTRA coverage / Monitoring/Discoverer procedure for group member discovery / One-to-many communication'. The test sequence below shows only the MCVideo relevant messages exchanged. | - | - | - | - |
| 5 | Check: Does the UE (MCVideo client) send a GROUP CALL BROADCAST message? (NOTE 2) | --> | GROUP CALL BROADCAST | 1 | P |
| 6 | SS starts Timer=10s (Timer TFB2 (broadcast retransmission)) |  |  |  |  |
| 7 | Check: Does the UE (MCVideo Client) send a Transmission Granted message towards the other transmission participants? | --> | Transmission Granted | 1 | P |
| 8 | Timer=10s expires | - | - | - | - |
| 9 | Check: Does the UE (MCVideo client) send a GROUP CALL BROADCAST message using the same Call identifier IE as used in step 4? | --> | GROUP CALL BROADCAST | 2 | P |
| 10 | Make the UE (MCVideo Client) cancel the broadcast call (NOTE 1), (NOTE 3) | - | - | - | - |
| 11 | Check: Does the UE (MCVideo client) send a GROUP CALL BROADCAST END message and enter the "B1: start-stop" state? (NOTE 4) | --> | GROUP CALL BROADCAST END | 3 | P |
| - | EXCEPTION: UE releases the E-UTRA connection. The E-UTRA/EPC actions which are related to the MCVideo call release are described in TS 36.579-1 [2], subclause 5.4.8, 'Generic Test Procedure for MCPTT communication over ProSe direct one-to-one communication out of E-UTRA coverage - release by the UE'. | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. NOTE 2: Timer TFB2 (broadcast retransmission) is started by the MCVideo Client upon the sending of the GROUP CALL BROADCAST message. Timer TFB2 (broadcast retransmission)=10s, as defined in TS 36.579-1 [2], Table 5.5.8.1-1. At the expire of timer TFB2 (broadcast retransmission), the MCVideo Client resends the GROUP CALL BROADCAST message. NOTE 3: If the MCVideo User does not cancel the broadcast call before timer TFB2 (broadcast retransmission) expires, then the MCVideo Client will resend the GROUP CALL BROADCAST message per TS 24.379 [9] and the remaining steps will not be valid. If during test execution it is found that the specified timer value is not large enough, then a new value needs to be specified. NOTE 4: Timer TFB2 (broadcast retransmission) is stopped by the MCVideo Client upon the sending of the GROUP CALL BROADCAST END message. |  |  |  |  |  |

7.1.2.1.3.3	Specific message contents

Table 7.1.2.1.3.3-1: Transmission Granted (step 7, Table 7.1.2.1.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.11.2.1-1, condition OFF-NETWORK |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Transmission Indicator |  |  |  |
| Transmission Indicator | "0100000000000000" | Broadcast group call |  |

Table 7.1.2.1.3.3-2: GROUP CALL BROADCAST (step 9, Table 7.1.2.1.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.5.6.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Call identifier | Use the same Call identifier as used in the GROUP CALL BROADCAST message sent in step 5 |  |  |

Table 7.1.2.1.3.3-3: GROUP CALL BROADCAST END (step 11, Table 7.1.2.1.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.5.6.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Call identifier | Use the same Call identifier as used in the GROUP CALL BROADCAST message sent in step 5 |  |  |

<END OF CHANGES>