3GPP TSG-RAN5 Meeting #95-e	R5-222158

# Online, , 9th May 2022 - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-6 | CR | 0056 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New MCVIDEO OFF-NETWORK TC 7.2.2 Off-network Auto Private Call CT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCenhUEConTest |  |  |  |  |  | Date: |  |  | 2022-04-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | No test case existed for an off-network basic private CT call |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new test case 7.2.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An off-network basic private CT call would not be tested |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Specific message contents include changes proposed in CR R5-222142 - New MCVideo Off-network Message Defaults 5.5.14 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<START OF CHANGES>

### 7.2.2	Off-network / Private Call / Automatic Commencement Mode / Client Terminated (CT)

7.2.2.1	Test Purpose (TP)

(1)

with { UE (MCVideo Client) registered and authorized for MCVideo Service, including authorized to initiate/cancel private calls with automatic commencement in off-network environment, and, the UE is in an off-network environment }

ensure that {

when { the UE (MCVideo Client) receives a request for establishment of an MCVideo private call with Automatic Commencement Mode via a PRIVATE CALL SETUP REQUEST message }

then { UE (MCVideo Client) sends a PRIVATE CALL ACCEPT message accepting the establishment of a private call on-demand Automatic Commencement Mode, and, respects the transmission control imposed by the transmission control entity/arbitrator }

}

(2)

with { UE (MCVideo Client) registered and authorized for MCVideo Service, including authorized to initiate/cancel private calls with automatic commencement in off-network environment, and, the UE is in an off-network environment, and, UE (MCVideo Client) having sent a PRIVATE CALL SETUP REQUEST message requesting establishment of a private call }

ensure that {

when { the UE (MCVideo Client) does not receive response to the request until the timer TFP1 (private call request retransmission) expires }

then { UE (MCVideo Client) retransmits the PRIVATE CALL SETUP REQUEST message requesting private call if the counter CFP1 (private call request retransmission) has not reached its max value and increments the counter CFP1 with one, and, stops re-transmitting if the counter CFP1 (private call request retransmission) has reached its max value and TFP1 (private call request retransmission) has expired, and, ignores a PRIVATE CALL ACCEPT message if recevied after the counter CFP1 (private call request retransmission) has reached its max value and TFP1 (private call request retransmission) has expired }

}

(3)

with { UE (MCVideo Client) registered and authorized for MCVideo Service, including authorized to initiate/cancel private calls with automatic commencement in off-network environment, and, the UE is in an off-network environment, and, UE (MCVideo Client) having sent a PRIVATE CALL SETUP REQUEST message requesting establishment of a private call }

ensure that {

when { the UE (MCVideo Client) receives a PRIVATE CALL ACCEPT message }

then { UE (MCVideo Client) responds with a PRIVATE CALL ACCEPT ACK message and, respects the transmission control imposed by the transmission control entity/arbitrator }

}

(4)

with { UE (MCVideo Client) having an ongoing private call }

ensure that {

when { the MCVideo User receives a PRIVATE CALL RELEASE message to end the private call }

then { UE (MCVideo Client) sends a PRIVATE CALL RELEASE ACK message }

}

7.2.2.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in:

TS 24.281 clauses 10.3.2.4.3.2, 10.3.2.4.3.4, 10.3.2.4.5.4, 10.3.2.4.5.7, TS 24.581 clauses 7.2.3.2.4, 7.2.3.4.2, 7.2.3.7.8, 7.2.3.6.4. Unless otherwise stated these are Rel-15 requirements.

[TS 24.281, clause 10.3.2.4.3.2]

When in the "P0: start-stop" or "P1: ignoring same call id" state, upon receiving a PRIVATE CALL SETUP REQUEST message with Commencement mode IE set to "AUTOMATIC COMMENCEMENT MODE" and Call identifier IE different than stored call identifier and media session declared in SDP body of PRIVATE CALL SETUP REQUEST message can be established, the MCVideo client:

1)	shall store the Call identifier IE in the received message as call identifier;

2)	shall set the stored current call type to "PRIVATE CALL";

3)	shall set the stored current ProSe per-packet priority to value corresponding to MCVideo off-network private call as described in 3GPP TS 24.483 [4].

4)	shall store the MCVideo user ID of the caller IE in the received PRIVATE CALL SETUP REQUEST message as caller ID;

5)	shall store own MCVideo user ID as callee ID;

6)	if the SDP offer contains an "a=key-mgmt" attribute field with a "mikey" attribute value containing a MIKEY-SAKKE I_MESSAGE:

a)	shall extract the MCVideo ID of the originating MCVideo user from the initiator field (IDRi) of the I_MESSAGE as described in 3GPP TS 33.180 [8];

b)	shall convert the MCVideo ID to a UID as described in 3GPP TS 33.180 [8];

c)	shall use the UID to validate the signature of the MIKEY-SAKKE I_MESSAGE as described in 3GPP TS 33.180 [8];

d)	if the validation of the signature failed, shall generate a PRIVATE CALL REJECT message as specified in clause 17.1.8. In the PRIVATE CALL REJECT message, the MCVideo client:

i)	shall set the call identifier IE to the stored call identifier;

ii)	shall set the MCVideo user ID of the caller IE with the stored caller ID;

iii)	shall set the MCVideo user ID of the callee IE with the stored callee ID;

iv)	shall set the Reason IE as "FAILED", if requested to restrict notification of call failure and the value of "/<x>/<x>/Common/PrivateCall/FailRestrict" leaf node present in the user profile as specified in 3GPP TS 24.483 [4] is set to "true". Otherwise, shall set the reason IE as "E2E SECURITY CONTEXT FAILURE";

v)	shall send the PRIVATE CALL REJECT message in response to the request message according to rules and procedures as specified in clause 10.3.1.1.1; and

vi)	shall remain in the current state;

e)	if the validation of the signature was successful:

i)	shall extract and decrypt the encapsulated PCK using the terminating user's (KMS provisioned) UID key as described in 3GPP TS 33.180 [8];

ii)	shall extract the PCK-ID, from the payload as specified in 3GPP TS 33.180 [8];

iii)	shall generate and store answer SDP based on received SDP offer IE in PRIVATE CALL SETUP REQUEST message, as defined in clause 10.3.1.1.2;

iv)	shall generate a PRIVATE CALL ACCEPT message as specified in clause 17.1.7. In the PRIVATE CALL ACCEPT message, the MCVideo client:

A)	shall set the Call identifier IE to the stored call identifier; and

B)	shall set the MCVideo user ID of the caller IE with stored caller ID.

C)	shall set the MCVideo user ID of the callee IE with stored callee ID; and

D)	shall set the SDP answer IE with the stored answer SDP;

v)	shall send PRIVATE CALL ACCEPT message in response to the request message according to rules and procedures as specified in clause 10.3.1.1.1;

vi)	shall establish a media session based on the SDP body of the stored answer SDP;

vii)	shall initialize the counter CFP4 with value set to 1;

viii)	shall start timer TFP4 (private call accept retransmission); and

ix)	shall enter the "P5: pending" state; and

NOTE:	With the PCK successfully shared between the originating MCVideo client and the terminating MCVideo client, both clients are able to use SRTP/SRTCP to create an end-to-end secure session.

7)	if the SDP offer does not contain an "a=key-mgmt" attribute, the MCVideo client:

a)	shall generate and store answer SDP based on received SDP offer IE in PRIVATE CALL SETUP REQUEST message, as defined in clause 10.3.1.1.2;

b)	shall generate a PRIVATE CALL ACCEPT message as specified in clause 17.1.7:

i)	shall set the Call identifier IE to the stored call identifier;

ii)	shall set the MCVideo user ID of the caller IE with stored caller ID.

iii)	shall set the MCVideo user ID of the callee IE with stored callee ID; and

iv)	shall set the SDP answer IE with the stored answer SDP;

c)	shall send PRIVATE CALL ACCEPT message in response to the request message according to rules and procedures as specified in clause 10.3.1.1.1;

d)	shall establish a media session based on the SDP body of the stored answer SDP;

e)	shall initialize the counter CFP4 with value set to 1;

f)	shall start timer TFP4 (private call accept retransmission); and

g)	shall enter the "P5: pending" state.

[TS 24.281, clause 10.3.2.4.3.4]

When in the "P5: pending" state, upon receiving a PRIVATE CALL ACCEPT ACK message or RTP media from originating user, the MCVideo client:

1)	shall stop timer TFP4 (private call accept retransmission);

2)	shall start transmission control as terminating MCVideo client as specified in clause a.b in 3GPP TS 24.581 [5];

3)	shall start timer TFP5 (max duration); and

4)	shall enter the "P4: part of ongoing call" state.

[TS 24.281, clause 10.3.2.4.5.4]

When in the "P4: part of ongoing call" state, upon receiving a PRIVATE CALL RELEASE message, the MCVideo client:

1)	shall generate a PRIVATE CALL RELEASE ACK message as specified in clause 17.1.10;

a)	shall set the Call identifier IE to the stored call identifier;

b)	shall set the MCVideo user ID of the caller IE the stored caller ID; and

c)	shall set the MCVideo user ID of the callee IE with the stored callee ID.

2)	shall send the PRIVATE CALL RELEASE ACK message in response to the request message according to rules and procedures as specified in clause 10.3.1.1.1;

3)	shall terminate the media session for private call;

4)	shall start timer TFP7 (waiting for any message with same call identifier); and

5)	shall enter the "P1: ignoring same call id" state.

[TS 24.281, clause 10.3.2.4.5.7]

When in the "P1: ignoring same call id" state, upon expiry of timer TFP7 (waiting for any message with same call identifier) the MCVideo client:

1)	shall clear the stored call identifier; and

2)	shall enter the "P0: start-stop" state.

[TS 24.581, clause 7.2.3.2.4]

When an MCVideo private call is established the terminating transmission participant:

1.	shall create an instance of a transmission participant state transition diagram for basic operation state machine;

2.	shall start timer T203(End of RTP media); and

3.	shall enter 'O: has no permission' state.

[TS 24.581, clause 7.2.3.4.2]

If the transmission participant receives an indication from the MCVideo user that the MCVideo user wants to send media, the transmission participant:

1.	shall send the Transmission Request message to other clients. The Transmission Request message:

a.	if a priority different than the default transmission priority is required, shall include the Transmission Priority field with the requested priority in the <Transmission Priority> element;

b.	shall include the MCVideo ID of the MCVideo user in the User ID field; and

c.	if the transmission request is a broadcast group call, system call, emergency call or an imminent peril call, shall include a Transmission Indicator field indicating the relevant call types;

2.	shall initialise the counter C201 (Transmission request) with value set to 1;

3.	shall start timer T201 (Transmission Request); and

4.	shall enter 'O: pending request' state.

[TS 24.581, clause 7.2.3.7.8]

Upon receiving Transmission Granted message and if the <User ID> value in the User ID field matches its own MCVideo ID and User ID of transmission participant sending the Transmission Granted message matches the stored User ID of current transmission arbitrator, the transmission participant:

1.	shall stop timer T201 (Transmission Request);

2.	clear the stored current transmission arbitrator;

3.	shall store the transmitter list of the Transmission Granted message;

4.	may provide a transmission granted notification to the MCVideo user; and

5.	shall enter 'O: has permission' state.

[TS 24.581, clause 7.2.3.6.4]

Upon receiving an indication from the MCVideo user to release permission to send RTP media, the transmission participant:

1.	shall stop timer T206 (Stop talking warning), if running;

2.	shall stop timer T207 (Stop talking), if running;

3.	shall send a Transmission Release message towards other transmission participants. The Transmission Release message:

a.	shall include the MCVideo ID of the MCVideo user in the User ID field; and

b.	if the session is not initiated as a broadcast group call with the B-bit set to '1' (Broadcast group call), shall include a Transmission Indicator field set to '0' (normal call); and

4.	shall enter 'O: has no permission' state.

7.2.2.3	Test description

7.2.2.3.1	Pre-test conditions

System Simulator:

-	SS-UE1 (MCVideo Client)

-	For the underlying "transport bearer" over which the SS and the UE will communicate, the SS is behaving as SS-UE1 as defined in TS 36.508 [24], configured for and operating as ProSe Direct Communication transmitting and receiving device.

-	GNSS simulator to simulate a location and provide a timing reference for the assistance of E-UTRAN off-network testing.

NOTE 1:	For operation in off-network environment, it needs be ensured that after the UE is powered up it considers the Geographical area #1 that is simulated by the GNSS simulator as being one of the geographical areas set in the USIM for operation when UE is "not served by E-UTRAN".

-	SS-NW (MCVideo server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCVideo operation in the MCVideo configuration document).

NOTE 2:	The SS operation as NW (MCVideo server) is needed only for the Preamble if the UE has to perform the Generic Test Procedure for MCVideo Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2.

IUT:

-	UE (MCVideo Client)

-	The test USIM set as defined in TS 36.579-1 [2], subclause 5.5.10 is inserted.

-	For the underlying "transport bearer" over which the SS and the UE will communicate, the UE is behaving as a ProSe enabled UE as defined in TS 36.508 [24], configured for and operating as ProSe Direct Communication transmitting and receiving device.

Preamble:

--	The UE has performed the Generic Test Procedure for MCVideo UE registration as specified in TS 36.579-1 [2], subclause 5.4.2A.

-	The MCVideo User performs the Generic Test Procedure for MCVideo Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2.

-	The GNSS simulator is configured to simulate a location in the centre of Geographical area #1 and provide a timing reference, as defined in TS 36.508 [24] Table 4.11.2-2 scenario #1.

-	The UE is switched-off.

-	UE States at the end of the preamble

-	The UE is in state 'switched-off'.

7.2.2.3.2	Test procedure sequence

Table 7.2.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Power up the UE. | - | - | - | - |
| 2 | Trigger the UE to reset UTC time and location. NOTE: The UTC time and location reset may be performed by MMI or AT command (+CUTCR). | - | - | - | - |
| 3 | Activate the MCVideo Client Application and register User A as the MCVideo User (TS 36.579-5 [5], px_MCX_User_A_username, px_MCX_User_A_password). (NOTE 1) | - | - | - | - |
| - | EXCEPTION: The E-UTRA/EPC actions which are related to the MCVideo call establishment are described in TS 36.579-1 [2], subclause 5.4.6 'Generic Test Procedure for MCPTT CT communication over ProSe direct one-to-one communication out of E-UTRA coverage-establishment'. The test sequence below shows only the MCVideo relevant messages exchanged. | - | - | - | - |
| 4 | SS-UE1 (MCVideo Client) sends a PRIVATE CALL SETUP REQUEST. | <-- | PRIVATE CALL SETUP REQUEST | - | - |
| 5 | Check: Does the UE (MCVideo Client) send a PRIVATE CALL ACCEPT? | --> | PRIVATE CALL ACCEPT | 1 | P |
| 6 | SS-UE1 (MCVideo Client) sends a PRIVATE CALL ACCEPT ACK. | <-- | PRIVATE CALL ACCEPT ACK | - | - |
| 7 | SS-UE1 (MCVideo Client) sends Transmission Granted message. | <-- | Transmission Granted | - | - |
| 8 | Make the MCVideo User press the PTT button requesting permission to talk. NOTE: The MCVideo User shall keep the button pressed until otherwise written. | - | - | - | - |
| 9 | Check: Does the UE (MCVideo Client) send a Transmission Request message? | --> | Transmission Request | 1 | P |
| 10 | SS-UE1 (MCVideo Client) sends Transmission Granted message. | <-- | Transmission Granted | - | - |
| 11 | Make the MCVideo User release transmission control. (NOTE 1) | - | - | - | - |
| 12 | Check: Does the UE (MCVideo Client) send a Transmission Release message? | --> | Transmission Release | 1 | P |
| 14 | SS-UE1 (MCVideo Client) sends PRIVATE CALL RELEASE message. | <-- | PRIVATE CALL RELEASE | - | - |
| 15 | Check: Does the UE (MCVideo Client) send a PRIVATE CALL RELEASE ACK message. | --> | PRIVATE CALL RELEASE ACK | 2 | P |
| - | EXCEPTION: SS releases the E-UTRA connection. The E-UTRA/EPC actions which are related to the MCVideo call release are described in TS 36.579-1 [2], subclause 5.4.7, 'Generic Test Procedure for MCPTT communication over ProSe direct one-to-one communication out of E-UTRA coverage - release by the SS'. | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

7.2.2.3.3	Specific message contents

Table 7.2.2.3.3-1: PRIVATE CALL SETUP REQUEST (step 4, Table 7.2.2.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.14.8-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MCVideo user ID of the caller | px_MCVideo_ID_User_B |  |  |
| MCVideo user ID of the callee | px_MCVideo_ID_User_A |  |  |
| SDP offer | As described in TS 36.579-1 [2], Table 5.5.3.1.4-2 with condition PRIVATE_CALL |  |  |

Table 7.2.2.3.3-2: PRIVATE CALL ACCEPT (step 5, Table 7.2.2.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.14.10-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MCVideo user ID of the caller | px_MCVideo_ID_User_B |  |  |
| MCVideo user ID of the callee | px_MCVideo_ID_User_A |  |  |
| SDP answer | As described in TS 36.579-1 [2], Table 5.5.3.1.3-2 with condition PRIVATE_CALL |  |  |

Table 7.2.2.3.3-3: PRIVATE CALL ACCEPT ACK (step 6, Table 7.2.2.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.14.14-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MCVideo user ID of the caller | px_MCVideo_ID_User_B |  |  |
| MCVideo user ID of the callee | px_MCVideo_ID_User_A |  |  |

Table 7.2.2.3.3-4: Transmission Granted (step 7, Table 7.2.2.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.11.2.1-1, condition OFF-NETWORK |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| User ID |  |  |  |
| User ID | px_MCVideo_ID_User_B |  |  |

Table 7.2.2.3.3-5: Transmission Request (step 9, Table 7.2.2.3.2-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.11.1.1-1, condition OFF-NETWORK

Table 7.2.2.3.3-6: Transmission Granted (step 10, Table 7.2.2.3.2-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.11.2.1-1, condition OFF-NETWORK

Table 7.2.2.3.3-7: PRIVATE CALL RELEASE (step 4, Table 7.2.2.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.14.12-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MCVideo user ID of the caller | px_MCVideo_ID_User_B |  |  |
| MCVideo user ID of the callee | px_MCVideo_ID_User_A |  |  |

Table 7.2.2.3.3-7: PRIVATE CALL RELEASE ACK (step 4, Table 7.2.2.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.14.13-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MCVideo user ID of the caller | px_MCVideo_ID_User_B |  |  |
| MCVideo user ID of the callee | px_MCVideo_ID_User_A |  |  |

<END OF CHANGES>