3GPP TSG-RAN5 Meeting #95-e	R5-222164

# Online, , 9th May 2022 - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-2 | CR | 1369 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Applicability Additions for TCs 13.1.23, 13.1.24, and 13.1.1.25 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test |  |  |  |  |  | Date: |  |  | 2022-04-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | New test cases have been proposed in clause 13 of 36.523-1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new test cases to Table 4-1 and conditions for the new test cases in Table 4-1a |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A device may fail a test because the improper applicability would be applied |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The new test cases are: R5-222166 - New TC 13.1.23 MCVideo with Dedicated Bearer of QCI 67-Attach-Call setup CO R5-222167 - New TC 13.1.24 MCVideo with Dedicated Bearer of QCI 2-Attach-Call setup CO R5-222168 - New TC 13.1.25 MCData-Attach-Call setup CO |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<START OF CHANGES>

<START OF CHANGES for TABLE 4-1 >

Table 4-1: Applicability of tests and additional information for testing

| Clause | TC Title | Release | Applicability |  | Additional Information |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment | Specific ICS | Specific IXIT | Number of TC Executions | Release other RAT |
| 6 | Idle mode operations |  |  |  |  |  |  |  |
| 6.1.1.1 | PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  | Either TC 6.1.1.1 or TC 6.1.1.1b shall be executed. (Note 4) |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.1.1a | PLMN selection / Automatic mode / between FDD and TDD | Rel-8 | C142 | UEs supporting E-UTRA FDD and E-UTRA TDD |  |  |  |  |

…

| 13 |  | Multi layer Procedures |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13.1.1 |  | Activation and deactivation of additional data radio bearer in E-UTRA |  | Rel-8 |  | R |  | UEs supporting E-UTRA |  | pc_eFDD |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |  |
| 13.1.2 |  | Call setup from E-UTRAN RRC_IDLE / CS fallback to UTRAN with redirection / MO call |  | Rel-8 |  | C48 |  | UEs supporting E-UTRA and UTRA and CS fallback and speech and NOT Category M1 |  | pc_eFDD |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  | Rel-9 UTRA TDD |  |
| 13.1.2a |  | Call setup from E-UTRAN RRC_IDLE / CS fallback to UTRAN with redirection including System Information / MO call |  | Rel-9 (Note 3) |  | C104 |  | UEs supporting E-UTRA and UTRA and CS fallback and use of the UTRA system information provided by RRCConnectionRelease upon redirection and speech and NOT Category M1 |  | pc_eFDD |  |  |  |  |  | Rel-8 UTRA FDD |  |
|  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  | Rel-9 UTRA TDD |  |
| 13.1.3 |  | Call setup from E-UTRAN RRC_CONNECTED / CS fallback to UTRAN with redirection / MT call |  | Rel-8 |  | C84 |  | UEs supporting E-UTRA and UTRA and CS fallback and speech and PS domain services and CS domain services simultaneously and NOT Category M1 |  | pc_eFDD |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  | Rel-9 UTRA TDD |  |
| 13.1.4 |  | Call setup from E-UTRAN RRC_IDLE / CS fallback to UTRAN with handover / MT call |  | Rel-8 |  | C81F |  | UEs supporting E-UTRA and UTRA and CS fallback and Feature Group Indicator 8 and speech and PS domain services and CS domain services simultaneously and NOT Category M1 |  | pc_eFDD |  |  |  |  |  |  |  |
|  |  |  |  |  |  | C81T |  |  |  | pc_eTDD |  |  |  |  |  | Rel-9 UTRA TDD |  |
| 13.1.5 |  | Call setup from E-UTRAN RRC_CONNECTED / CS fallback to UTRAN with handover / MO call |  | Rel-8 |  | C81F |  | UEs supporting E-UTRA, UTRA, CS fallback and Feature Group Indicator 8 and speech and PS domain services and CS domain services simultaneously and NOT Category M1 |  | pc_eFDD |  |  |  |  |  |  |  |
|  |  |  |  |  |  | C81T |  |  |  | pc_eTDD |  |  |  |  |  | Rel-9 UTRA TDD |  |
| 13.1.6 |  | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13.1.7 |  | Call setup from E-UTRA RRC_IDLE / CS fallback to GSM with redirection / MT call |  | Rel-8 |  | C57 |  | UEs supporting E-UTRA and GERAN and CS fallback and speech and NOT Category M1 |  | pc_eFDD |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |  |
| 13.1.8 |  | Call setup from E-UTRA RRC_CONNECTED / CS fallback to GSM with redirection / MO call |  | Rel-8 |  | C60 |  | UEs supporting E-UTRA and GERAN and CS fallback and speech and NOT Category M1 |  | pc_eFDD |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |  |
| 13.1.9 |  | Call setup from E-UTRA RRC_IDLE / CS fallback to GSM with CCO without NACC / MO call |  | Rel-8 |  | C96F |  | UEs supporting E-UTRA and GERAN and CS fallback and Feature Group Indicator 10 and speech and NOT Category M1 |  | pc_eFDD |  |  |  |  |  |  |  |
|  |  |  |  |  |  | C96T |  |  |  | pc_eTDD |  |  |  |  |  |  |  |
| 13.1.10 |  | Call setup from E-UTRA RRC_CONNECTED / CS fallback to GSM with CCO without NACC / MT call |  | Rel-8 |  | C96F |  | UEs supporting E-UTRA and GERAN and CS fallback and Feature Group Indicator 10 and speech and NOT Category M1 |  | pc_eFDD |  |  |  |  |  |  |  |
|  |  |  |  |  |  | C96T |  |  |  | pc_eTDD |  |  |  |  |  |  |  |
| 13.1.11 |  | Call setup from E-UTRA RRC_IDLE / CS fallback to GSM with PSHO / EDTM not supported / MT call |  | Rel-8 |  | C110F |  | UEs supporting E-UTRA and GERAN and CS fallback and PS handover from E-UTRAN to GERAN and Feature Group Indicator 23 and speech and NOT Category M1 |  | pc_eFDD |  |  |  |  |  |  |  |
|  |  |  |  |  |  | C110T |  |  |  | pc_eTDD |  |  |  |  |  |  |  |
| 13.1.12 |  | Call setup from E-UTRA RRC_CONNECTED / CS fallback to GSM with PSHO / EDTM not supported / MO call |  | Rel-8 |  | C110F |  | UEs supporting E-UTRA and GERAN and CS fallback and PS handover from E-UTRAN to GERAN and Feature Group Indicator 23 and speech and NOT Category M1 |  | pc_eFDD |  |  |  |  |  |  |  |
|  |  |  |  |  |  | C110T |  |  |  | pc_eTDD |  |  |  |  |  |  |  |
| 13.1.13 |  | Call setup from E-UTRA RRC_IDLE / CS fallback to GSM with PSHO / EDTM supported / MT call |  | Rel-8 |  | C111F |  | UEs supporting E-UTRA and GERAN and EDTM and CS fallback and PS handover from E-UTRAN to GERAN and Feature Group Indicator 23 and speech and NOT Category M1 |  | pc_eFDD |  |  |  |  |  |  |  |
|  |  |  |  |  |  | C111T |  |  |  | pc_eTDD |  |  |  |  |  |  |  |
| 13.1.14 |  | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13.1.15 |  | Call setup from E-UTRAN RRC_IDLE / CS fallback to UTRAN with redirection / MT call / UTRAN cell is barred |  | Rel-8 |  | C48 |  | UEs supporting E-UTRA and UTRA and CS fallback and speech and NOT Category M1 |  | pc_eFDD |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  | Rel-9 UTRA TDD |  |
| 13.1.16 |  | Emergency call setup from E-UTRAN RRC_IDLE / CS fallback to UTRAN with handover |  | Rel-8 |  | C105F |  | UEs supporting E-UTRA and UTRA and CS fallback and Feature Group Indicator 8 and speech and NOT Category M1 |  | pc_eFDD |  |  |  |  |  |  |  |
|  |  |  |  |  |  | C105T |  |  |  | pc_eTDD |  |  |  |  |  | Rel-9 UTRA TDD |  |
| 13.1.17 |  | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13.1.18 |  | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13.1.19 |  | Emergency call setup from E-UTRAN RRC_IDLE / IMS VoPS supported / EMC BS not supported / CS fallback to UTRAN or GERAN with redirection |  | Rel-9 |  | C249 |  | UEs supporting E-UTRA and (UTRA or GERAN) and combined EPS/IMSI attach and CS fallback and CS speech and VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS" and NOT Category M1 |  | pc_eFDD |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |  |
| 13.1.20 |  | Emergency call setup from E-UTRAN RRC_IDLE / IMS VoPS not supported / EMC BS supported / CS fallback to UTRAN or GERAN with redirection |  | Rel-9 |  | C249 |  | UEs supporting E-UTRA and (UTRA or GERAN) and combined EPS/IMSI attach and CS fallback and CS speech and VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS" and NOT Category M1 |  | pc_eFDD |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |  |
| 13.1.21 |  | Emergency Call setup from E-UTRA RRC_IDLE but IMS voice not available / IMS VoPS supported / EMC BS supported / UE performs emergency call via CS domain |  | Rel-9 |  | C249 |  | UEs supporting E-UTRA and (UTRA or GERAN) and combined EPS/IMSI attach and CS fallback and CS speech and VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS" and NOT Category M1 |  | pc_eFDD |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |  |
| 13.1.22 |  | MCPTT / Attach / Call setup CO |  | Rel-14 |  | C397 |  | UEs supporting E-UTRA and MCPTT Client |  | pc_eFDD |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |  |
| 13.1.23 |  | MCVideo with Dedicated Bearer of QCI 67 / Attach / Call setup CO |  |  |  | C409 |  | UEs supporting E-UTRA and MCVideo Client |  | pc_eFDD |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |  |
| 13.1.24 |  | MCVideo with Dedicated Bearer of QCI 2 / Attach / Call setup CO |  |  |  | C409 |  | UEs supporting E-UTRA and MCVideo Client |  | pc_eFDD |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |  |
| 13.1.25 |  | MCData / Attach / Call setup CO |  |  |  | C410 |  | UEs supporting E-UTRA and MCData Client |  | pc_eFDD |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |  |
| 13.2.1 |  | RRC connection reconfiguration / E-UTRA to E-UTRA |  | Rel-8 |  | C12 |  | UEs supporting E-UTRA or (CE Mode A and “eventA3 for intra-frequency neighbouring cells in normal coverage CE Mode A” and “intra-frequency handover to target cell in normal coverage and CE Mode A”) |  | pc_eFDD |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |  |

<END OF CHANGES for Table 4-1>

<START OF CHANGES for TABLE 4-1a >

Table 4-1a: Applicability of tests Conditions

C01 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C01a IF (A.4.1-1/1 OR A.4.1-1/2) AND [8]A.1/1 AND (A.4.5-2/3 OR A.4.5-2/4) AND NOT (A.4.3.2-2A/1) THEN R ELSE N/A

…

C397 IF (A.4.1-1/1 OR A.4.1-1/2) AND [56] A.4.1-1/1 THEN R ELSE N/A

C398 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/214 THEN R ELSE N/A

C399 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/217 THEN R ELSE N/A

C400 IF A.4.1-1/8 AND A.4.4-1/218 THEN R ELSE N/A

C401 Void

C402 IF A.4.1-1/8 AND A.4.4-1/219THEN R ELSE N/A

C403 IF A.4.1-1/9 AND A.4.4-1A/17 THEN R ELSE N/A

C404 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/220 THEN R ELSE N/A

C405 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/216 AND A.4.4-1/221

C406 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/122 AND A.4.4-1/222 THEN R ELSE N/A

C407 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/122 AND A.4.4-1/223 THEN R ELSE N/A

C408 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.3.4-1/215 THEN R ELSE N/A

C409 IF (A.4.1-1/1 OR A.4.1-1/2) AND [56] A.4.1-1/3 THEN R ELSE N/A

C410 IF (A.4.1-1/1 OR A.4.1-1/2) AND [56] A.4.1-1/4 THEN R ELSE N/A

<END OF CHANGES for TABLE 4-1a >

<END OF CHANGES>