3GPP TSG-RAN5 Meeting #95-e	R5-222165

# Online, , 9th May 2022 - 20th May 2022

Agenda Item:	6.5.6

Source:	NIST, FirstNet, AT&T

Title:	Specifying LTE multi-layer test cases for MCVideo and MCData enabled devices

Document for:	Endorsement

# 1	Introduction

RAN5 has specified testing of Mission Critical Services application clients in the TS 36.579-x series with MCPTT Client conformance test cases defined in 36.579-2, MCVideo conformance test cases defined in 36.579-6, and MCData conformance test cases defined in 36.579-7.

RAN5 has considered different test models in response to market interest and currently is not focusing on the option of so called "traditional" testing of an Application communication over a 3GPP RAT (in this case LTE) where the MCVideo or MCData enabled device's behaviour in the RAT is implicitly tested during the testing of the Application.

For the strictly LTE behaviour of a device, RAN5 has specified testing in the TS 36.523-1. Due to test design historical reasons, a MCPTT enabled Device could not be tested for its LTE behaviour unless the MCPTT Client was disabled (the LTE specification cannot handle the MCPTT specific APN/PDN/Bearer establishments). The consequence was that RAN5 test specs did not provide means for verifying the LTE behaviour of a MCPTT enabled UE. However in the recent past, a test case was introduced in TS 36.523-1 that aimed at verifying the LTE relevant behaviour when the MCPTT Client (TC 13.1.22):

-	Registers for MCPTT Services

-	Initiates an MCPTT Call

Now MCPTT enabled devices can be tested for LTE registration and Mission Critical Services bearer establishment. However, there are no equivalent test cases for MCVideo enabled devices and MCData enabled devices. There is a commercial need for these test cases with the commercial availability of MCVideo and MCData services.

# 2	Proposal

It is proposed that RAN5 agrees on the specification of 3 new LTE test cases aimed at verifying the LTE relevant behaviour when the MCVideo Client or MCData Client:

-	Registers for MCPTT Services

-	Initiates an MCPTT Call

To facilitate development of such a test on the existing LTE test platforms, the MCVideo Client  and MCData Client application relevant exchange throughout the test cases should be kept to a minimum and should not be strictly verified for compliance with the MCPTT requirements. The 3 new test cases will be very similar to the existing TS 36.523-1 test case 13.1.22. Proposed new test cases:

- 13.1.23 MCVideo with Dedicated Bearer of QCI 67 / Attach / Call setup CO

- 13.1.24 MCVideo with Dedicated Bearer of QCI 2 / Attach / Call setup CO

- 13.1.25 MCData / Attach / Call setup CO

# 4	References

[1]	R5-222166 - New TC 13.1.23 MCVideo with Dedicated Bearer of QCI 67-Attach-Call setup CO

[2]	R5-222167 - New TC 13.1.24 MCVideo with Dedicated Bearer of QCI 2-Attach-Call setup CO

[3]	R5-222168 - New TC 13.1.25 MCData-Attach-Call setup CO

[4]	R5-222169 - Adding specs to References for MCData and MCVideo

[5]	R5-222164 - Applicability Additions for TCs 13.1.23, 13.1.24, and 13.1.1.25