3GPP TSG-RAN5 Meeting #95-e	R5-222260

# Electronic Meeting, 9th May 2022 - 20th May 2022

# Source:	Qualcomm Incorporated

# Title:	Call Flow for addition of video to a voice call

# Document for:	Discussion

## 1.	Introduction

TS 34.229-5, TC 7.21, MTSI MO Voice Call / add video and remove video / without preconditions at both originating UE and terminating UE /5GS, tests voice call without precondition upgrade to video call by the SS. The test expects UE to send 183 Session in progress  with SDP answer when requesting for the video call upgrade (Step 3).

However, it is possible that UE can directly send 200OK with SDP answer and completely skip the 183 session in progress and acknowledgement procedure (step 3-5) for the mid-call upgrade. The current TC does not allow such behavior.

## 2.	Discussion

TS 24.229 is not clear if it is okay for UE to send 200OK with SDP answer to upgrade the call but  it’s been observed that operators, in the field deployments for same scenario, allow the UE to upgrade the call with 200OK with SDP answer directly and then setup the bearer accordingly.

Given the lack of clarity, one way to address this ambiguity would be to allow above option as well in the TC to handle both UE behavior, as detailed below.

At step 1, SS sends re-invite to upgrade the current audio call to video call. Next step can be

If (UE sends 183 session in progress with SDP answer)

(SS will respond with PRACK, UE sends 200 OK then SS will set up the bearer for video call) OR;

If (UE sends 200OK with SDP answer, 183 session in progress is not sent)

(SS will acknowledge 200OK then SS will set up the bearer for video call)

Else

(fail)

## 3.	Proposals and way forward

1.   Update TC 7.21 prose to allow both UE behavior as described in 2 above

- Send an LS to 3GPP CT1 WG and seek spec clarification