3GPP TSG-RAN5 Meeting #95-e	R5-222265

# Electronic Meeting, 9th May 2022 - 20th May 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2299 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving test frequency for n53 10 Mhz CBW |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm CDMA Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Even though n53 supports 10 Mhz CBW with 30 Khz SCS, the default test frequencies in Cl 6.2.3 of TS 38.508-1 defines n53 with 5 MHz CBW with 15 Khz SCS for maximum conformance coverage. Hence a note is required in Table 6.2.3.1-3 to state the reason why 5 MHz is chosen instead on 10 MHz. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a Note in Table 6.2.3.1-3 stating “For n53 the 5MHz CBW is defined instead of 10 Mhz CBW considering maximum conformance coverage” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Clarity for choosing n53 5 Mhz CBW over 10 MHz CBW is now specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.2.3	Default test frequencies

Editor’s note:	For FR2 test frequencies using 100 MHz default channel bandwidth it is FFS if 100MHz channel bandwidth can be used for FR2 multicell protocol testing.

#### 6.2.3.1	Test frequencies for NR standalone signalling testing

The default channel bandwidth for signalling test is specified per NR band. The test frequencies are defined so that no frequency overlapping takes place, in order to avoid unnecessary inter-frequency interference.

For signalling test cases, the mapping of frequency ranges to NR test frequencies are as follows:

-	for band with only one test frequency (e.g. n51): Low Range (NRf1);

-	for band with up to two test frequencies: Low Range (NRf1) and High Range (NRf2);

-	for band with up to three test frequencies: Low Range (NRf1), Mid Range (NRf2) and High Range (NRf3);

-	for band with up to four test frequencies: Low Range (NRf1), Mid Low Range (NRf2), Mid High Range (NRf3) and High Range (NRf4);

The signalling test frequencies NRf5, NRf6, NRf7 are mapped respectively as NRf1, NRf2, NRf3 on the operating band for inter-band.

The test frequencies, subcarrier spacing, default channel bandwidth, SS/PBCH block and CORESET#0 parameters for signalling is specified in Table 6.2.3.1-1 (FDD FR1 BW 5MHz), Table 6.2.3.1-2 (FDD FR1 BW 10MHz), Table 6.2.3.1-3 (TDD FR1 BW 5MHz), Table 6.2.3.1-4 (TDD FR1 BW 10MHz), Table 6.2.3.1-4A (TDD FR1 BW 60MHz), Table 6.2.3.1-5 (TDD FR1 BW 100MHz), Table 6.2.3.1-6 (TDD FR2 BW 100MHz) and Table 6.2.3.1-7 (NR FDD FR1 SUL bands).

<<Contents Skipped>>

<<Start of changes>>

Table 6.2.3.1-3: Test frequencies for NR TDD FR1 bands using 5 MHz channel bandwidth

| NR Band | SCS [kHz] | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA[ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n34 | 15 | 5 | 25 | Downlink & Uplink | Low, Mid, High | Same values as for Low, Mid and High range in clause 4.3.1.1.1.34 for bandwidth=5 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |
| n51 | 15 | 5 | 25 | Downlink & Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.51 for bandwidth=5 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |
| n53 | 15 | 5 | 25 | Downlink & Uplink | Low, High | Same values as for Low and High range in clause 4.3.1.1.1.53 for bandwidth=5 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: For n53 the 5MHz CBW is defined instead of 10 Mhz CBW considering maximum conformance coverage, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<<End of changes>>