3GPP TSG-RAN5 Meeting #95-e	R5-222272

# Electronic Meeting, 9th May 2022 - 20th May 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2901 | rev | - | Current version: | 16.11.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to EN-DC RRC TC 8.2.3.17.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm CDMA Technologies, Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | At step 1, SS has configured SFTD measurement (NCell – ReportConfig ID 1) with “reportQuantityCellNR” having “ss-rsrp”= TRUE and UE sends the Measurement report with measResultCellListSFTD-r15 including rsrpResult-r15. At step 8, TC is not expecting the UE to include ss-rsrp as reportQuantityCellNR is omitted in the Measurement configuration at Step 6 (ReportConfig ID(1) is used here as well) As per TS38.331, “reportQuantityCellNR” is “Needs On” field and thus UE is supposed to retain existing config and sending the ss-rsrp in SFTD meas report at step 8 which results in TC failure as Table 8.2.3.17.1.3.3-4 expects rsrpResult-r15 to be omitted. |  |  |  |  |  |  |  |  |
| Summary of change: |  | At step 6, SS shall configure SFTD measurement (NCell – ReportConfig ID 1) with “reportQuantityCellNR ss-rsrp= FALSE” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TTCN implementation will be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.3.17.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <Start of modified section>

##### 8.2.3.17.1	Measurement configuration control and reporting / SFTD / EN-DC

8.2.3.17.1.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state with reportSFTD-Meas configured }

ensure that {

when { The sfn-OffsetResult and frameBoundaryOffsetResult results become available }

then { UE transmits a MeasurementReport message including the MeasResultCellSFTD IE }

}

(2)

with { UE in RRC_CONNECTED state in EN-DC mode with reportSFTD-Meas configured for Serving NR PSCell }

ensure that {

when { The sfn-OffsetResult and frameBoundaryOffsetResult results for the Serving NR PSCell becomes available }

then { UE transmits a MeasurementReport message including the MeasResultCellSFTD IE }

}

8.2.3.17.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.331, clauses 5.3.5.3, 5.5.2.1, 5.5.3.1, 5.5.4.1, and 5.5.5.1. Unless otherwise stated these are Rel-15 requirements.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

...

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

...

1>	else:

2>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration, upon which the procedure ends; [TS 38.331, clause 5.5.2.1]

[TS 36.331, clause 5.5.2.1]

E-UTRAN applies the procedure as follows:

-	to ensure that, whenever the UE has a measConfig, it includes a measObject for each LTE serving frequency;

...

-	to configure at most one measurement identity using a reporting configuration with reportSFTD-Meas;

...

The UE shall:

...

1>	if the received measConfig includes the measObjectToAddModList:

2>	perform the measurement object addition/ modification procedure as specified in 5.5.2.5;

...

1>	if the received measConfig includes the reportConfigToAddModList:

2>	perform the reporting configuration addition/ modification procedure as specified in 5.5.2.7;

...

1>	if the received measConfig includes the measIdToAddModList:

2>	perform the measurement identity addition/ modification procedure as specified in 5.5.2.3;

...

[TS 36.331, clause 5.5.3.1]

The UE shall:

1>	whenever the UE has a measConfig, perform RSRP and RSRQ measurements for each serving cell as follows:

...

1>	for each measId included in the measIdList within VarMeasConfig:

...

2>	else:

3>	if a measurement gap configuration is setup; or

3>	if the UE does not require measurement gaps to perform the concerned measurements:

...

4>	if the reportSFTD-Meas is set to pSCell in the associated reportConfig:

5>	perform SFTD measurements between the PCell and the NR PSCell;

4>	if the reportSFTD-Meas is set to neighborCells in the associated reportConfig:

5>	perform SFTD measurements between the PCell and NR cell(s) on the frequency indicated in the associated measObject;

...

2>	perform the evaluation of reporting criteria as specified in 5.5.4;

...

[TS 36.331, clause 5.5.4.1]

If security has been activated successfully, the UE shall:

1>	for each measId included in the measIdList within VarMeasConfig:

...

2>	else:

...

3>	else if the corresponding measObject concerns NR:

4>	if the reportSFTD-Meas is set to pSCell in the corresponding reportConfigInterRAT:

5>	consider the PSCell to be applicable;

4>	else if the reportSFTD-Meas is set to neighborCells in the corresponding reportConfigInterRAT:

5>	if cellsForWhichToReportSFTD is configured in the corresponding measObjectNR:

6>	consider any neighbouring NR cell on the associated frequency that is included in cellsForWhichToReportSFTD to be applicable;

5>	else:

6>	consider up to 3 strongest neighbouring NR cells detected on the associated frequency to be applicable when the concerned cells are not included in the blackCellsToAddModList defined within the VarMeasConfig for this measId;

...

2>	else if the purpose is included and set to reportStrongestCells, reportStrongestCellsForSON, reportLocation sidelink or sensing and if a (first) measurement result is available:

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	if the purpose is set to reportStrongestCells and reportStrongestCSI-RSs is set to FALSE:

...

4>	else (i.e. the reportAmount is equal to 1):

5>	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after the quantity to be reported becomes available for the PCell and for the strongest cell among the applicable cells, or becomes available for the pair of PCell and the PSCell in case of SSTD measurements, or becomes available for each requested pair of PCell and NR cell or the maximal measurement reporting delay as specified in TS 36.133 [16], clause 8.17.2.3 in case of SFTD measurements;

...

[TS 36.331, clause 5.5.5.1]

Figure 5.5.5.1-1: Measurement reporting

For the measId for which the measurement reporting procedure was triggered, the UE shall set the measResults within the MeasurementReport message as follows:

1>	set the measId to the measurement identity that triggered the measurement reporting;

1>	set the measResultPCell to include the quantities of the PCell;

...

1>	if there is at least one applicable neighbouring cell to report:

2>	set the measResultNeighCells to include the best neighbouring cells up to maxReportCells in accordance with the following:

...

3>	else:

4>	include the applicable cells for which the new measurement results became available since the last periodical reporting or since the measurement was initiated or reset;

NOTE 1:	The reliability of the report (i.e. the certainty it contains the strongest cells on the concerned frequency) depends on the measurement configuration i.e. the reportInterval. The related performance requirements are specified in TS 36.133 [16].

3>	for each cell that is included in the measResultNeighCells, include the physCellId;

3>	if the triggerType is set to event; or the purpose is set to reportStrongestCells or to reportStrongestCellsForSON:

4>	for each included cell, include the layer 3 filtered measured results in accordance with the reportConfig for this measId, ordered as follows:

...

5>	if the measObject associated with this measId concerns NR:

6>	set the measResultCell to include the quantity(ies) indicated in the reportQuantityCellNR within the concerned reportConfig;

...

6>	sort the included cells in order of decreasing sorting quantity determined as specified in 5.5.5.3;

...

1>	if the reportSFTD-Meas is set to neighborCells or pSCell within the corresponding reportConfigInterRAT for this measId, for each applicable cell for which results are available:

2>	set sfn-OffsetResult and frameBoundaryOffsetResult to the measurement results provided by lower layers;

2>	if the ss-rsrp in the reportQuantityCellNR is set to TRUE within the corresponding reportConfigInterRAT for this measId:

3>	include rsrpResult set to the RSRP of the concerned cell;

1>	increment the numberOfReportsSent as defined within the VarMeasReportList for this measId by 1;

1>	stop the periodical reporting timer, if running;

1>	if the numberOfReportsSent as defined within the VarMeasReportList for this measId is less than the reportAmount as defined within the corresponding reportConfig for this measId:

2>	start the periodical reporting timer with the value of reportInterval as defined within the corresponding reportConfig for this measId;

1>	else:

2>	if the triggerType is set to periodical:

3>	remove the entry within the VarMeasReportList for this measId;

3>	remove this measId from the measIdList within VarMeasConfig;

...

1>	else:

2>	submit the MeasurementReport message to lower layers for transmission, upon which the procedure ends;

8.2.3.17.1.3	Test description

8.2.3.17.1.3.1	Pre-test conditions

System Simulator:

-	E-UTRA Cell 1 is the PCell, NR Cell 1 is the inter-RAT neighbour cell.

-	System information blocks combination 31 as defined in TS 36.508 [7] clause 4.4.3.1.1 is used in E-UTRA PCell.

UE:

-	None.

Preamble:

-	The UE is in state RRC_CONNECTED using generic procedure parameter Connectivity (E-UTRA/EPC) established according to TS 38.508-1 [4], clause 4.5.4..

8.2.3.17.1.3.2	Test procedure sequence

Table 8.2.3.17.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message including MeasConfig to setup inter-RAT SFTD measurement on NR neighbour cell (NR Cell 1) | <-- | E-UTRA RRC: RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message. | --> | E-UTRA RRC: RRCConnectionReconfigurationComplete | - | - |
| 3 | Check: Does the UE transmit a MeasurementReport message including measResultCellListSFTD within the next 5s? NOTE 1 | --> | E-UTRA RRC: MeasurementReport | 1 | P |
| 4 | The SS transmits an RRCConnectionReconfiguration including nr-SecondaryCellGroupConfig messages to add NR Cell 1 as PSCell. | <-- | E-UTRA RRC: RRCConnectionReconfiguration (RRCReconfiguration) | - | - |
| 5 | The UE transmits an RRCConnectionReconfigurationComplete message including scg-ConfigResponseNR-r15 | --> | E-UTRA RRC: RRCConnectionReconfigurationComplete (RRCReconfigurationComplete) | - | - |
| 6 | The SS transmits an RRCConnectionReconfiguration message including MeasConfig to setup intra-frequency SFTD measurement on NR PSCell (NR Cell 1) | <-- | E-UTRA RRC: RRCConnectionReconfiguration | - | - |
| 7 | The UE transmits an RRCConnectionReconfigurationComplete message. | --> | E-UTRA RRC: RRCConnectionReconfigurationComplete | - | - |
| 8 | Check: Does the UE transmit a MeasurementReport message including measResultCellListSFTD within the next 3s? NOTE 2 | --> | E-UTRA RRC: MeasurementReport | 2 | P |
| NOTE 1: Time limit is chosen based on the inter-RAT SFTD measurement reporting delay requirements specified in TS 36.133 [10] clause 8.1.2.4.25 and 8.1.2.4.26. NOTE 2: Time limit is chosen based on the intra-frequency SFTD measurement reporting delay requirements specified in TS 36.133 [10] clause 8.17.2 |  |  |  |  |  |

8.2.3.17.1.3.3	Specific message contentsTable 8.2.3.17.1.3.3-1: RRCConnectionReconfiguration (step 1 and 6, Table 8.2.3.17.1.3.2-1)

Derivation Path: TS 36.508 [7] Table 4.6.1-8 with condition MEAS

Table 8.2.3.17.1.3.3-2: MeasConfig (Table 8.2.3.17.1.3.3-1)

| Derivation Path: TS 36.508 [7] Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF MeasObjectToAddMod { | 2 entries |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectEUTRA | MeasObjectEUTRA-GENERIC(Freq) specified in 36.508 [7] Table 4.6.6-2 | Freq is the carrier frequency for E-UTRA Cell 1 |  |
|  | MeasObjectEUTRA-GENERIC(Freq) with condition Band > 64 specified in 36.508 [7] Table 4.6.6-2 | Freq is the carrier frequency for E-UTRA Cell 1 | Band > 64 |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod[2] SEQUENCE { |  | entry 2 |  |
| measObjectId | 2 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR-r15 | MeasObjectNR-GENERIC (Freq) specified in 36.508 [7] Table 4.6.6-2B | Freq is the carrier frequency for NR Cell 1 |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigInterRAT | ReportConfigInterRAT | Table 8.2.3.17.1.3.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 2 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| measGapConfig | MeasGapConfig-GP1 specified in 36.508 [7] Table 4.6.6-1A |  |  |
| measObjectToAddModList-v9e0 | Not present |  |  |
| measObjectToAddModList-v9e0 SEQUENCE (SIZE (1..maxObjectId)) OF MeasObjectToAddMod-v9e0 { | 1 entry |  | Band > 64 |
| MeasObjectToAddMod-v9e0[1] SEQUENCE { |  | entry 1 |  |
| measObjectEUTRA-v9e0 SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Downlink EARFCN for the carrier frequency of E-UTRA Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.3.17.1.3.3-3: ReportConfigInterRAT (Table 8.2.3.17.1.3.3-2)

| Derivation Path: TS 36.508 [7] Table 4.6.6-8B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigInterRAT-PERIODICAL ::= SEQUENCE { |  |  |  |
| reportQuantityCellNR-r15 SEQUENCE { |  |  | step 6 |
| ss-rsrp | false |  |  |
| ss-rsrq | false |  |  |
| ss-sinr | false |  |  |
| } |  |  |  |
| reportQuantityCellNR-r15 SEQUENCE { |  |  | step 1 |
| ss-rsrp | true |  |  |
| ss-rsrq | false |  |  |
| ss-sinr | false |  |  |
| } |  |  |  |
| reportSFTD-Meas-r15 | neighborCells |  | step 1 |
|  | pSCell |  | step 6 |
| } |  |  |  |

Table 8.2.3.17.1.3.3-4: MeasurementReport (step 3 and 8, Table 8.2.3.17.1.3.2-1)

| Derivation Path: TS 36.508 [7] Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |  |
| measResults SEQUENCE { |  |  |  |  |
| measId |  | 1 |  |  |
| measResultPCell SEQUENCE { |  |  |  |  |
| rsrpResult |  | (0..97) |  |  |
| rsrqResult |  | (0..34) |  |  |
| } |  |  |  |  |
| measResultNeighCells |  | Not checked |  |  |
| } |  |  |  |  |
| measResultForECID-r9 |  | Not checked |  |  |
| locationInfo-r10 |  | Not checked |  |  |
| measResultServFreqList-r10 |  | Not checked |  |  |
| measId-v1250 |  | Not checked |  |  |
| measResultPCell-v1250 |  | Not checked |  |  |
| measResultCSI-RS-List-r12 |  | Not checked |  |  |
| measResultForRSSI-r13 |  | Not checked |  |  |
| measResultServFreqListExt-r13 |  | Not checked |  |  |
| measResultSSTD-r13 |  | Not checked |  |  |
| measResultPCell-v1310 |  | Not checked |  |  |
| ul-PDCP-DelayResultList-r13 |  | Not checked |  |  |
| measResultListWLAN-r13 |  | Not checked |  |  |
| measResultPCell-v1360 |  | Not checked |  |  |
| measResultListCBR-r14 |  | Not checked |  |  |
| measResultListWLAN-r14 |  | Not checked |  |  |
| measResultServFreqListNR-r15 |  | Not checked |  |  |
| measResultCellListSFTD-r15 SEQUENCE (SIZE (1..maxCellSFTD)) OF MeasResultCellSFTD-r15 { |  |  |  |  |
| MeasResultCellSFTD-r15[1] SEQUENCE { |  |  |  |  |
| physCellId-r15 |  | PhysCellIdNR-r15 of NR Cell 1 |  |  |
| sfn-OffsetResult-r15 |  | (0..1023) |  |  |
| frameBoundaryOffsetResult-r15 |  | (-30720..30719) |  |  |
| rsrpResult-r15 |  | Not present |  | step 8 |
|  |  | (0..127) |  | step 3 |
| } |  |  |  |  |
| } |  |  |  |  |
| logMeasResultListBT-r15 |  | Not checked |  |  |
| logMeasResultListWLAN-r15 |  | Not checked |  |  |
| measResultSensing-r15 |  | Not checked |  |  |
| heightUE-r15 |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.2.3.17.1.3.3-5: RRCConnectionReconfiguration (step 4, Table 8.2.3.17.1.3.2-1)

Derivation Path: TS 36.508 [7] Table 4.6.1-8 with condition MCG_and_SCG

Table 8.2.3.17.1.3.3-6: RRCConnectionReconfigurationComplete (step 5, Table 8.2.3.17.1.3.2-1)

Derivation Path: TS 36.508 [7] Table 4.6.1-9 with condition MCG_and_SCG

## <End of modifed section>