3GPP TSG-RAN5 Meeting #95-e	R5-222315

# Electronic meeting, 9 – 20 May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1636 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Transmit OFF power for NR-DC in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of Transmit OFF power for NR-DC in FR1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add NR-DC for clause 6.3A.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Transmit OFF power for NR-DC in FR1 will not be specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------- Start of changes --------

### 6.3A.2	Transmit OFF power for CA and NR-DC

#### 6.3A.2.0	Minimum conformance requirements

For inter-band carrier aggregation and dual connectivity with uplink assigned to two NR bands, the transmit OFF power specified in subclause 6.3.2 is applicable for each component carrier when the transmitter is OFF on all component carriers. The transmitter is considered to be OFF when the UE is not allowed to transmit on any of its ports.

For intra-band contiguous carrier aggregation, the transmit OFF power specified in clause 6.3.2 is applicable for each component carrier when the transmitter is OFF on all component carriers. The transmitter is considered to be OFF when the UE is not allowed to transmit on any of its ports.

For intra-band non-contiguous carrier aggregation, the transmit OFF power specified in clause 6.3.2 is applicable for each component carrier when the transmitter is OFF on all component carriers. The transmitter is considered to be OFF when the UE is not allowed to transmit on any of its ports.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3A.2

#### 6.3A.2.1	Transmit OFF power for CA (2UL CA) and NR-DC

6.3A.2.1.1	Test purpose

To verify that the UE transmit OFF power for 2UL CA or NR-DC is lower than the value specified in the test requirement.

6.3A.2.1.2	Test applicability

The requirements of 6.3A.2.1 apply in test cases 6.3A.3.1 Transmit ON/OFF time mask for CA (2UL CA) and NR-DC to all types of NR UE release 15 and forward that support 2UL CA or NR UE release 16 and forward that support NR-DC. Therefore, no test case description and requirements are specified.

6.3A.2.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.3A.2.0.

6.3A.2.1.4	Test description

This test is covered by clause 6.3A.3.1 Transmit ON/OFF time mask for 2UL CA and NR-DC.

6.3A.2.1.5	Test requirement

The requirement for the transmit OFF power of each component carrier shall not exceed the values specified in Table 6.3A.2.1.5-1.

Table 6.3A.2.1.5-1: Transmit OFF power

| Channel bandwidth (MHz) | Transmit OFF power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 5 | -50+TT | 4.515 |
| 10 | -50+TT | 9.375 |
| 15 | -50+TT | 14.235 |
| 20 | -50+TT | 19.095 |
| 25 | -50+TT | 23.955 |
| 30 | -50+TT | 28.815 |
| 40 | -50+TT | 38.895 |
| 50 | -50+TT | 48.615 |
| 60 | -50+TT | 58.35 |
| 80 | -50+TT | 78.15 |
| 100 | -50+TT | 88.23 |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.3A.2.1.5-2 |  |  |

Table 6.3A.2.1.5-2: Test Tolerance (Transmit OFF power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | FFS | FFS |
| 40MHz < BW ≤ 100MHz | FFS | FFS |

-------- End of changes -------