3GPP TSG-RAN5 Meeting #95-e	R5-222380

# Electronic Meeting, 9th – 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2308 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating RRCReconfiguration and RadioBearerConfig for NR-DC and NE-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. The condition DRBn for RadioBearerConfig defined in table 4.6.3-132 is specifed to configure a non-default DRB, however this table is used for both: - PDU session establishment (referenced from table 4.5A.2.2.3-2) when a default DRB shall be configured - Configuration of an SCG bearer for both NR-DC and NE-DC (reference tables 4.8.1-1CA and 4.8.1-1CB) when a non-default DRB shall be configured. 2. When a split SCG bearer is configured, in both NR-DC and NE-DC, the cell group configuration shall be provided for both the MCG and SCG cells. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Introduce a new condition in RadioBearerConfig to configure the SCG DRB as a non-default DRB. 2. Add the cell group configuration on the MCG cell for NR-DC and NE-DC split bearers. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The configuration for NR-DC and NE-DC will remain incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3, 4.8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This is to resolve RAN5 SIG AP#94.02 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 4.6.3	Radio resource control information elements

<Lines Skipped>

#### –	RadioBearerConfig

Table 4.6.3-132: RadioBearerConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| srb-ToAddModList | Not present |  |  |
| srb-ToAddModList SEQUENCE (SIZE (1..2)) OF SRB-ToAddMod { | 1 entry |  | SRB1 |
| SRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| SRB-Identity | SRB-Identity with condition SRB1 |  |  |
| reestablishPDCP | Not present |  |  |
| discardOnPDCP | Not present |  |  |
| pdcp-Config | Not present | Default |  |
| } |  |  |  |
| } |  |  |  |
| srb-ToAddModList SEQUENCE (SIZE (1..2)) OF SRB-ToAddMod { | 1 entry |  | SRB2, RESUME |
| SRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| SRB-Identity | SRB-Identity with condition SRB2 |  |  |
| reestablishPDCP | Not present |  |  |
|  | true |  | RESUME |
| discardOnPDCP | Not present |  |  |
| pdcp-Config | Not present | Default |  |
| } |  |  |  |
| } |  |  |  |
| srb-ToAddModList SEQUENCE (SIZE (1..2)) OF SRB-ToAddMod { | 1 entry |  | SRB3 |
| SRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| srb-Identity | SRB-Identity with condition SRB3 |  |  |
| reestablishPDCP | Not present |  |  |
| discardOnPDCP | Not present |  |  |
| pdcp-Config | Not present | Default |  |
| } |  |  |  |
| } |  |  |  |
| srb-ToAddModList SEQUENCE (SIZE (1..2)) OF SRB-ToAddMod { | 2 entries |  | SRB_NR_PDCP REEST |
| SRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| SRB-Identity | SRB-Identity with condition SRB1 |  |  |
| reestablishPDCP | Not present |  |  |
|  | true |  | SRB_NR_PDCP AND Re-establish_PDCP |
| discardOnPDCP | Not present |  |  |
| pdcp-Config | Not present | Default |  |
| } |  |  |  |
| SRB-ToAddMod[2] SEQUENCE { |  | entry 2 |  |
| SRB-Identity | SRB-Identity with condition SRB2 |  |  |
| reestablishPDCP | Not present |  |  |
|  | true |  | SRB_NR_PDCP AND Re-establish_PDCP, REEST |
| discardOnPDCP | Not present |  |  |
| pdcp-Config | Not present | Default |  |
| } |  |  |  |
| } |  |  |  |
| srb3-ToRelease | Not present |  |  |
| drb-ToAddModList | Not present |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  | EN-DC_DRB |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| cnAssociation CHOICE { |  |  |  |
| eps-BearerIdentity | 6 |  |  |
| } |  |  |  |
| drb-Identity | DRB-Identity using condition DRB2 |  |  |
| reestablishPDCP | Not present |  |  |
|  | true |  | EN-DC_DRB AND Re-establish_PDCP |
| recoverPDCP | Not present |  |  |
|  | true |  | EN-DC_DRB AND Recover_PDCP |
| pdcp-Config | PDCP-Config |  |  |
| } |  |  |  |
| } |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  | MCG_NR_PDCP |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| cnAssociation CHOICE { |  |  |  |
| eps-BearerIdentity | 12 | EPS Bearer Id of default MCG DRB |  |
| } |  |  |  |
| drb-Identity | 8 | DRB Id of default MCG DRB |  |
| reestablishPDCP | Not present |  |  |
| recoverPDCP | Not present |  |  |
| pdcp-Config | PDCP-Config |  |  |
| } |  |  |  |
| } |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  | DRB1 |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config | SDAP-Config |  |  |
| } |  |  |  |
| drb-Identity | DRB-Identity using condition DRB1 |  |  |
| reestablishPDCP | Not present |  |  |
|  | true |  | DRB1 AND Re-establish_PDCP |
| recoverPDCP | Not present |  |  |
|  | true |  | DRB1 AND Recover_PDCP |
| pdcp-Config | PDCP-Config |  |  |
| daps-Config-r16 | Not present |  |  |
|  | true |  | DRB1 AND DAPS_PDCP |
| } |  |  |  |
| } |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  | DRB2 |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config | SDAP-Config |  |  |
| } |  |  |  |
| drb-Identity | DRB-Identity using condition DRB2 |  |  |
| reestablishPDCP | Not present |  |  |
|  | true |  | DRB2 AND Re-establish_PDCP |
| recoverPDCP | Not present |  |  |
|  | true |  | DRB2 AND Recover_PDCP |
| pdcp-Config | PDCP-Config |  |  |
| daps-Config-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | n entries | n is the number of DRBs established before RRC resume or RRC re-establishment | RESUME, REEST |
| DRB-ToAddMod[k, k=1..n] SEQUENCE { |  | entry [k, k=1..n] |  |
| cnAssociation | Not present |  |  |
| drb-Identity | DRB-Identity with condition DRBk |  |  |
| reestablishPDCP | true |  |  |
| recoverPDCP | Not present |  |  |
| pdcp-Config | Not present |  |  |
| daps-Config-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  | DRBn |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config | SDAP-Config |  |  |
|  | SDAP-Config with conditionNo-defaultDRB |  | SCG_DRB |
| } |  |  |  |
| drb-Identity | DRB-Identity with condition DRBn |  |  |
| reestablishPDCP | Not present |  |  |
| recoverPDCP | Not present |  |  |
| pdcp-Config | PDCP-Config |  |  |
|  | PDCP-Config with condition Split |  | Split |
| } |  |  |  |
| } |  |  |  |
| drb-ToReleaseList | Not present |  |  |
| securityConfig | Not present |  | SRB1 |
| securityConfig SEQUENCE { |  |  |  |
| securityAlgorithmConfig | SecurityAlgorithmConfig |  |  |
| keyToUse | master |  |  |
|  | secondary |  | SRB3, EN-DC_DRB, SecondaryKeys |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SRB3 | Establishment of SRB3 |
| MCG_NR_PDCP | EN-DC MCG DRB configured or reconfigured with NR PDCP |
| SRB_NR_PDCP | EN-DC SRB1 and SRB2 configured with NR PDCP |
| SRB1 | Establishment of SRB1 |
| SRB2 | Establishment of SRB2 |
| DRB1 | Establishment of DRB1 |
| DRB2 | Establishment of DRB2 |
| DRBn | Establishment of DRBn |
| EN-DC_DRB | EN-DC DRB configured on SCG |
| Re-establish_PDCP | Re-establishment of PDCP |
| Recover_PDCP | Recovery of PDCP |
| RESUME | Used in RRCResume Message |
| REEST | The first RRCReconfiguration message after successful completion of the RRC re-establishment procedure. |
| SecondaryKeys | NR-DC SCG or MCG DRB configured or reconfigured with secondary security keys |
| Split | Split PDCP: more than one RLC |
| DAPS_PDCP | Used when the bearer is configured as DAPS bearer |
| SCG_DRB | Used when the bearer is configured as an SCG bearer in either NR-DC or NE-DC |

<End of Modified Section>

<Next Modified Section>

## 4.8	Reference configurations

### 4.8.1	Radio configurations

–	RRCReconfiguration-DRB(n, m)

Table 4.8.1-1: RRCReconfiguration-DRB (n, m)

| Derivation Path: Table 4.6.1-13 with condition NR. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig-DRB-NR(n,m) |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig-DRB(n, m) | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

–	RRCReconfiguration-HO

Table 4.8.1-1A: RRCReconfiguration-HO

| Derivation Path: Table 4.6.1-13. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig with conditions SRB_NR_PDCP and DRBn and Re-establish_PDCP |  | RBConfig_KeyChange |
|  | RadioBearerConfig with conditions DRBn and Recover_PDCP |  | RBConfig_NoKeyChange |
| secondaryCellGroup | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig with conditions PCell_change | OCTET STRING (CONTAINING CellGroupConfig) |  |
| masterKeyUpdate | Not present |  |  |
| masterKeyUpdate SEQUENCE { |  |  | RBConfig_KeyChange |
| keySetChangeIndicator | false | Horizontal key derivation |  |
| nextHopChainingCount | NextHopChainingCount |  |  |
| nas-Container | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| RBConfig_KeyChange | RadioBearerConfig to perform Intra-NR handover with security key change |
| RBConfig_NoKeyChange | RadioBearerConfig to perform Intra-NR handover without security key change |

–	RRCReconfiguration-SRB2-DRB(n, m)

Table 4.8.1-1B: RRCReconfiguration-SRB2-DRB(n, m)

| Derivation Path: Table 4.6.1-13 with condition NR. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig-SRB2-DRB(n,m) |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig-SRB2-DRB(n, m) | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

–	RRCReconfiguration-NR-DC-DRB

Table 4.8.1-1CA: RRCReconfiguration-NR-DC-DRB

| Derivation Path: Table 4.6.1-13 with condition NR-DC. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | Not present |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | Not present |  | MCG(s) and SCG |
|  | CellGroupConfig with condition DRB(n+m+1) | OCTET STRING (CONTAINING CellGroupConfig) | MCG(s) and split |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| radioBearerConfig2 | RadioBearerConfig with conditions DRB(n+m+1) and SCG_DRB and SecondaryKeys | OCTET STRING (CONTAINING RadioBearerConfig) DRB(n,m) already configured on MCG | MCG(s) and SCG |
|  | RadioBearerConfig with conditions DRB(n+m+1) and SCG_DRB and Split and SecondaryKeys | OCTET STRING (CONTAINING RadioBearerConfig) DRB(n,m) already configured on MCG | MCG(s) and split |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

–	RRCReconfiguration-NE-DC-DRB

Table 4.8.1-1CB: RRCReconfiguration-NE-DC-DRB

| Derivation Path: Table 4.6.1-13 with condition NE-DC. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | Not present |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | Not present |  | MCG(s) and SCG |
|  | CellGroupConfig with condition DRB(n+m+1) | OCTET STRING (CONTAINING CellGroupConfig) | MCG(s) and split |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| radioBearerConfig2 | RadioBearerConfig with conditions DRB(n+m+1) and SCG_DRB and SecondaryKeys | OCTET STRING (CONTAINING RadioBearerConfig) DRB(n,m) already configured on MCG | MCG(s) and SCG |
|  | RadioBearerConfig with conditions DRB(n+m+1) and SCG_DRB and Split and SecondaryKeys | OCTET STRING (CONTAINING RadioBearerConfig) DRB(n,m) already configured on MCG | MCG(s) and split |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of Modified Section>