3GPP TSG-RAN5 Meeting #95-e	R5-222383

# Online, , 9th May 2022 - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2909 | rev | - | Current version: | 16.11.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR IIoT PDCP test cases 7.1.3.5.6.x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_IIoT-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Adding further clarification on the test case procedure and correcting typos. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Corrected typos in test purpose and test case procedure - Moved step 3C before the exception - Added clarifications in step 15 - Assigned verdict in step 19. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification will be misleading |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.5.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 7.1.3.5.6	PDCP Duplication / 3 RLC entities

##### 7.1.3.5.6.1	PDCP Duplication / 3 RLC entities / Intra-band Contiguous CA

7.1.3.5.6.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and pdcpDuplication is configured and activated}

ensure that {

when { UE has PDCP SDUs available for transmission}

then { the UE transmits the PDCP SDUs on all the associated RLC entities}

}

(2)

Void

(3)

with { UE in RRC_CONNECTED state and pdcpDuplication is configured}

ensure that {

when { UE has PDCP SDUs available for transmission}

then { the UE transmits the PDCP SDUs on the primary RLC entity}

}

(4)

with { UE in RRC_CONNECTED state and pdcpDuplication is configured and activated}

ensure that {

when { the UE had transmitted the PDCP SDUs on all the associated RLC entities and successful delivery of a PDCP Data PDU is confirmed by one of the associated AM RLC entities}

then { the other AM RLC entities discard the duplicated PDCP Data PDU}

}

(5)

with { UE in RRC_CONNECTED state and pdcpDuplication is configured and activated}

ensure that {

when { UE receives MAC Control Element to deactivate PDCP Duplication on one of the associated RLC entity configured with PDCP duplication}

then { the UE deactivates the PDCP duplication for the indicated associated RLC entity}

7.1.3.5.6.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in:  TS 38.321:5.10, 6.1.3.11, 6.1.3.32, TS 38.323:5.2.1, 5.11.1, 5.11.2 and TS 38.331: 5.3.5.6.4. Unless otherwise stated these are Rel-16 requirements.

[TS 38.323, clause 5.2.1]

When submitting a PDCP PDU to lower layer, the transmitting PDCP entity shall:

-	if the transmitting PDCP entity is associated with one RLC entity:

-	submit the PDCP PDU to the associated RLC entity;

-	else, if the transmitting PDCP entity is associated with at least two RLC entities:

-	if the PDCP duplication is activated for the RB:

-	if the PDCP PDU is a PDCP Data PDU:

-	duplicate the PDCP Data PDU and submit the PDCP Data PDU to the associated RLC entities activated for PDCP duplication;

-	else:

-	submit the PDCP Control PDU to the primary RLC entity;

-	else (i.e. the PDCP duplication is deactivated for the RB):

-	if the split secondary RLC entity is configured; and

-	if the total amount of PDCP data volume and RLC data volume pending for initial transmission (as specified in TS 38.322 [5]) in the primary RLC entity and the split secondary RLC entity is equal to or larger than ul-DataSplitThreshold:

-	submit the PDCP PDU to either the primary RLC entity or the split secondary RLC entity;

-	else, if the transmitting PDCP entity is associated with the DAPS bearer:

-	if the uplink data switching has not been requested:

-	submit the PDCP PDU to the RLC entity associated with the source cell;

-	else:

-	if the PDCP PDU is a PDCP Data PDU:

-	submit the PDCP Data PDU to the RLC entity associated with the target cell;

-	else:

-	if the PDCP Control PDU is associated with source cell:

-	submit the PDCP Control PDU to the RLC entity associated with the source cell;

-	else:

-	submit the PDCP Control PDU to the RLC entity associated with the target cell;

-	else:

-	submit the PDCP PDU to the primary RLC entity.

[TS 38.323, clause 5.11.1]

For the PDCP entity configured with pdcp-Duplication, the transmitting PDCP entity shall:

-	for SRBs:

-	activate the PDCP duplication;

-	for DRBs:

-	if the activation of PDCP duplication is indicated for the DRB:

-	activate the PDCP duplication for the DRB;

-	if the activation of PDCP duplication is indicated for at least one associated RLC entities:

-	activate the PDCP duplication for the indicated associated RLC entities;

-	activate the PDCP duplication for the DRB;

-	if the deactivation of PDCP duplication is indicated for the DRB:

-	deactivate the PDCP duplication for the DRB;

-	if the deactivation of PDCP duplication is indicated for at least one associated RLC entities:

-	deactivate the PDCP duplication for the indicated associated RLC entities;

-	if all associated RLC entities other than the primary RLC entity are deactivated for PDCP duplication:

-	deactivate the PDCP duplication for the DRB.

[TS 38.323, clause 5.11.2]

For the PDCP entity configured with pdcp-Duplication, the transmitting PDCP entity shall:

-	if the successful delivery of a PDCP Data PDU is confirmed by one of the associated AM RLC entities:

-	indicate to the other AM RLC entities to discard the duplicated PDCP Data PDU;

-	if the deactivation of PDCP duplication is indicated for the DRB:

-	indicate to the RLC entities other than the primary RLC entity to discard all duplicated PDCP Data PDUs;

-	if the deactivation of PDCP duplication is indicated for at least one associated RLC entities:

-	indicate to the RLC entities deactivated for PDCP duplication to discard all duplicated PDCP Data PDUs.

[TS 38.321, clause 5.10]

If one or more DRBs are configured with PDCP duplication, the network may activate and deactivate the PDCP duplication for all or a subset of associated RLC entities for the configured DRB(s).

The PDCP duplication for the configured DRB(s) is activated and deactivated by:

-	receiving the Duplication Activation/Deactivation MAC CE described in clause 6.1.3.11;

-	receiving the Duplication RLC Activation/Deactivation MAC CE described in clause 6.1.3.32;

-	indication by RRC.

The PDCP duplication for all or a subset of associated RLC entities for the configured DRB(s) is activated and deactivated by:

-	receiving the Duplication RLC Activation/Deactivation MAC CE described in clause 6.1.3.32;

-	indication by RRC.

The MAC entity shall for each DRB configured with PDCP duplication:

1>	if a Duplication Activation/Deactivation MAC CE is received activating the PDCP duplication of the DRB:

2>	indicate the activation of PDCP duplication of the DRB to upper layers.

1>	if a Duplication Activation/Deactivation MAC CE is received deactivating the PDCP duplication of the DRB:

2>	indicate the deactivation of PDCP duplication of the DRB to upper layers.

1>	if a Duplication RLC Activation/Deactivation MAC CE is received activating PDCP duplication for associated RLC entities of a DRB configured with PDCP duplication:

2>	indicate the activation of PDCP duplication for the indicated secondary RLC entity(ies) of the DRB to upper layers.

1>	if a Duplication RLC Activation/Deactivation MAC CE is received deactivating PDCP duplication for associated RLC entities of a DRB configured with PDCP duplication:

2>	indicate the deactivation of PDCP duplication for the indicated secondary RLC entity(ies) of the DRB to upper layers.

[TS 38.331, clause 5.3.5.6.4]

The UE shall:

1>	for each drb-Identity value included in the drb-ToReleaseList that is part of the current UE configuration; or

1>	for each drb-Identity value that is to be released as the result of full configuration according to 5.3.5.11:

2>	release the PDCP entity and the drb-Identity;

2>	if SDAP entity associated with this DRB is configured:

3>	indicate the release of the DRB to SDAP entity associated with this DRB (TS 37.324 [24], clause 5.3.3);

2>	if the UE is operating in EN-DC:

3>	if a new bearer is not added either with NR or E-UTRA with same eps-BearerIdentity:

4>	indicate the release of the DRB and the eps-BearerIdentity of the released DRB to upper layers.

NOTE 1:	The UE does not consider the message as erroneous if the drb-ToReleaseList includes any drb-Identity value that is not part of the current UE configuration.

NOTE 2:	Whether or not the RLC and MAC entities associated with this PDCP entity are reset or released is determined by the CellGroupConfig.

7.1.3.5.6.1.3	Test description

7.1.3.5.6.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the PCell, NR Cell 3 and NR Cell 6 are the SCells

UE:

-	None

Preamble:

-	Same Pre-test conditions as in clause 7.1.3.0.

7.1.3.5.6.1.3.2	Test procedure sequence

Table 7.1.3.5.6.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an NR RRCReconfiguration message to add 2 SCells and to configure parameters for PdcpDuplication. | <-- | RRCReconfiguration | - | - |
| 1A | UE transmits NR RRCReconfigurationComplete message. | --> | RRCReconfigurationComplete | - | - |
| 1B | The SS transmits a SCell Activation MAC-CE on PCell (NR Cell 1) to activate NR SCells (NR Cell 3 and NR Cell 6). | <-- | MAC PDU (SCell Activation/Deactivation MAC CE of one octet (C1=1)) | - | - |
| 2 | The SS sends a PDCP Data PDU on the DRB on the AM RLC primary entity. | <-- | PDCP DATA PDU | - | - |
| 3 | Check: Does UE transmit a PDCP Data PDU on the AM RLC primary entity? | --> | PDCP DATA PDU | 3 | P |
| 3A | SS transmits a MAC CE to activate PDCP Duplication for all AM RLC entities associated to the DRB. | <-- | MAC PDU (Duplication RLC Activation MAC Control Element) | - | - |
| 3B | The SS is configured not to send RLC ACK on all 3 RLC entities. (Note 2) | - | - | - | - |
| 3C | The SS sends a PDCP Data PDU on the DRB on the AM RLC primary entity. | <-- | PDCP DATA PDU | - | - |
| - | EXCEPTION: Steps 3D, 4 & 4A below can occur in any order. | - | - | - | - |
| 3D | Check: Does UE transmit a PDCP Data PDU on the AM RLC primary entity? | --> | PDCP DATA PDU | 1 | P |
| 4 | Check: Does UE transmit a PDCP Data PDU on the associated AM RLC entity on the SCell (NR Cell3)? | --> | PDCP DATA PDU | 1 | P |
| 4A | Check: Does UE transmit a PDCP Data PDU on the associated AM RLC entity on the SCell (NR Cell6)? | --> | PDCP DATA PDU | 1 | P |
| 5 | SS transmits a MAC CE to deactivate PDCP Duplication for the AM RLC entity associated with the DRB on SCell (NR Cell 6). | <-- | MAC PDU (Duplication RLC Deactivation MAC Control Element) | - | - |
| 5A | The SS is configured not to send RLC ACK on all 3 RLC entities. | - | - | - | - |
| 6 | The SS sends a PDCP Data PDU on the DRB on the AM RLC primary entity. | <-- | PDCP DATA PDU | - | - |
| - | EXCEPTION: Steps 6A-6B below can occur in any order. | - | - | - | - |
| 6A | UE transmits a PDCP Data PDU on the activated primary AM RLC entity. | --> | PDCP DATA PDU | - | - |
| 6B | UE transmits a PDCP Data PDU on the activated secondary AM RLC entity. | --> | PDCP DATA PDU | - | - |
| 7 | Check: Does UE transmit a PDCP Data PDU on the deactivated AM RLC entity by step 5? | --> | PDCP DATA PDU | 5 | F |
| 8-14 | Void | - | - | - | - |
| 15 | The SS stops allocating any UL grant on the SCells .(Note 1) | - | - | - | - |
| 16 | The SS sends a PDCP Data PDU on the DRB on the AM RLC primary entity. | <-- | PDCP DATA PDU | - | - |
| 17 | UE transmits a PDCP Data PDU on the AM RLC primary entity | --> | PDCP DATA PDU | - | - |
| 18 | The SS resumes normal UL grant allocation on the SCells. | - | - | - | - |
| - | EXCEPTION: Steps 19-21 below can occur in any order. | - | - | - | - |
| 19 | Check: Does UE transmit a PDCP Data PDU on the AM RLC primary? | --> | PDCP DATA PDU | 4 | F |
| 20 | Check: Does UE transmit a PDCP Data PDU on the AM RLC primary or the associated AM RLC entity on the SCell (NR Cell 3) in next five seconds? | --> | PDCP DATA PDU | 4 | F |
| 21 | Check: Does UE transmit a PDCP Data PDU on the AM RLC primary or the associated AM RLC entity on the SCell (NR Cell 6) in next five seconds? | --> | PDCP DATA PDU | 4 | F |
| Note 1: Discard of RLC SDU is not possible if submitted to lower layers. Therefore, Grant is not provided so that RLC SDU is not submitted to lower layers. Note2: SS is configured not to send RLC ACK for the next PDU on all RLC entities to ensure that UE side primary or secondary entities do not discard the PDU before PDSCH transmission upon receipt of RLC ACK on one of the entities. This may cause UE side entities to retransmit the PDU but it shall be ignored by SS side RLC entity as it has already been received though not acknowledged due to SS side RLC being configured in test mode. |  |  |  |  |  |

7.1.3.5.6.1.3.3	Specific message contents

Table 7.1.3.5.6.1.3.3-1: Void

Table 7.1.3.5.6.1.3.3-1A: RRCReconfiguration (Step 1, Table 7.1.3.5.6.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | RadioBearerConfig-DRB |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig-SCells |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.3.5.6.1.3.3-1B: RadioBearerConfig-DRB (Table 7.1.3.5.6.1.3.3-1A)

| Derivation Path: TS 38.508-1 [7], Table 4.6.3-132 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| srb3-ToRelease | Not present |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| drb-Identity | DRBj |  |  |
| pdcp-Config | PDCP-Config |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.5.6.1.3.3-1C: CellGroupConfig-SCells (Table 7.1.3.5.6.1.3.3-1A)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..max LC-ID)) OF RLC-BearerConfig { | 2 entries |  |  |
| RLC-BearerConfig[1] | RLC-BearerConfig-1 | entry 1 Table 7.1.3.5.6.3.3-3 Secondary RLC 1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig-2 | entry 2 Table 7.1.3.5.6.3.3-4 Secondary RLC 2 |  |
| RLC-BearerConfig[3] | RLC-BearerConfig-3 | entry 3 Table 7.1.3.5.6.3.3-5 Secondary RLC 3 |  |
| } |  |  |  |
| mac-CellGroupConfig | Not present |  |  |
| physicalCellGroupConfig | Not present |  |  |
| spCellConfig | Not present |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig { | 2 entries |  |  |
| SCellConfig[1] SEQUENCE { |  | entry 1 |  |
| sCellIndex | SCellIndex for NR Cell 3 |  |  |
| sCellConfigCommon | ServingCellConfigCommon as per TS 38.508-1 [4] Table 4.6.3-168 |  |  |
| sCellConfigDedicated | ServingCellConfig as per TS 38.508-1 [4] Table 4.6.3-167 |  |  |
| } |  |  |  |
| SCellConfig[2] SEQUENCE { |  | entry 2 |  |
| sCellIndex | SCellIndex for NR Cell 6 |  |  |
| sCellConfigCommon | ServingCellConfigCommon as per TS 38.508-1 [4] Table 4.6.3-168 |  |  |
| sCellConfigDedicated | ServingCellConfig as per TS 38.508-1 [4] Table 4.6.3-167 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.5.6.1.3.3-2: PDCP-Config (Table 7.1.3.5.6.1.3.3-1B)

| Derivation Path: 38.508-1 [4], Table 4.6.3-99 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCP-Config ::= SEQUENCE { |  |  |  |
| t-Reordering | Not present |  |  |
| moreThanTwoRLC-DRB-r16 SEQUENCE { |  |  |  |
| duplicationState-r16 SEQUENCE (SIZE (3)) | 3 entries |  |  |
| duplicationState-r16[1] | true |  |  |
| duplicationState-r16[2] | true |  |  |
| duplicationState-r16[3] | true |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.5.6.1.3.3-3: RLC-BearerConfig-1 (Table 7.1.3.5.6.1.3.3-1C)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-148 with Condition AM |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLC-BearerConfig ::= SEQUENCE { |  |  |  |
| logicalChannelIdentity | LogicalChannelIdentity with condition DRB j | ID of primary logical channel |  |
| servedRadioBearer CHOICE { |  |  |  |
| drb-Identity | DRB-Identity with condition DRB j | DRB |  |
| } |  |  |  |
| rlc-Config CHOICE { |  |  |  |
| am SEQUENCE { |  |  |  |
| ul-AM-RLC SEQUENCE { |  |  |  |
| maxRetxThreshold | t32 | To ensure RLC failure happens before RLF |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| mac-LogicalChannelConfig SEQUENCE { |  |  |  |
| ul-SpecificParameters SEQUENCE { |  |  |  |
| allowedServingCells SEQUENCE (SIZE (1..maxNrofServingCells-1)) OF ServCellIndex { | 1 entry |  |  |
| ServCellIndex[1] | ServCellIndex of NR Cell 1 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.5.6.1.3.3-4: RLC-BearerConfig-2 (Table 7.1.3.5.6.1.3.3-1C)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-148 with Condition AM |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLC-BearerConfig ::= SEQUENCE { |  |  |  |
| logicalChannelIdentity | LogicalChannelIdentity with condition DRB j+1 | To ensure ID of secondary logical channel ID is different with existing logical channel and the primary logical channel |  |
| servedRadioBearer CHOICE { |  |  |  |
| drb-Identity | DRB-Identity with condition DRB j | DRB |  |
| } |  |  |  |
| rlc-Config CHOICE { |  |  |  |
| am SEQUENCE { |  |  |  |
| ul-AM-RLC SEQUENCE { |  |  |  |
| maxRetxThreshold | t1 | To ensure RLC failure happens before RLF |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| mac-LogicalChannelConfig SEQUENCE { |  |  |  |
| ul-SpecificParameters SEQUENCE { |  |  |  |
| allowedServingCells SEQUENCE (SIZE (1..maxNrofServingCells-1)) OF ServCellIndex { | 1 entry |  |  |
| ServCellIndex[1] | ServCellIndex of NR Cell 3 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.5.6.1.3.3-5: RLC-BearerConfig-3 (Table 7.1.3.5.6.1.3.3-1C)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-148 with Condition AM |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLC-BearerConfig ::= SEQUENCE { |  |  |  |
| logicalChannelIdentity | LogicalChannelIdentity with condition DRB j+2 | To ensure ID of secondary logical channel ID is different with existing logical channel and the primary logical channel |  |
| servedRadioBearer CHOICE { |  |  |  |
| drb-Identity | DRB-Identity with condition DRB j | DRB |  |
| } |  |  |  |
| rlc-Config CHOICE { |  |  |  |
| am SEQUENCE { |  |  |  |
| ul-AM-RLC SEQUENCE { |  |  |  |
| maxRetxThreshold | t1 | To ensure RLC failure happens before RLF |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| mac-LogicalChannelConfig SEQUENCE { |  |  |  |
| ul-SpecificParameters SEQUENCE { |  |  |  |
| allowedServingCells SEQUENCE (SIZE (1..maxNrofServingCells-1)) OF ServCellIndex { | 1 entry |  |  |
| ServCellIndex[1] | ServCellIndex of NR Cell 6 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 7.1.3.5.6.2	PDCP Duplication / 3 RLC entities / Intra-band non-Contiguous CA

The scope and description of the present TC is the same as test case 7.1.3.5.6.1 with the following differences:

##### -	CA configuration: Intra-band non-Contiguous CA replaces Intra-band Contiguous CA