3GPP TSG-RAN5 Meeting #95-e	R5-222554
Electronic Meeting, 9th May – 20th May 2022

Agenda item:	5.4.17

Source:	Keysight Technologies

Title:	On Permitted Methodologies and Applicability

Document for:	Discussion and Endorsement

# Introduction

This contribution addresses the need to define and clarify the permitted methodologies.

# Overview of Permitted Methodologies in TR38.810

Test methodologies for FR2 conformance testing were initially discussed in [1], specifically for UE RF, demodulation, and RRM.

In [1], the following methodologies were defined with the following applicability:

Table 1: Applicability of permitted methodologies

| Permitted Methodology | UE RF Applicability | RRM Applicability | Demod Applicability |
| --- | --- | --- | --- |
| DFF | EIRP, TRP, EIS, EVM, spurious emissions and blocking metrics | 2 AoA | Yes |
| Simplified DFF | EIRP, TRP, EIS, EVM, spurious emissions and blocking metrics | 1 AoA | Yes |
| IFF | EIRP, TRP, EIS, EVM, spurious emissions and blocking metrics | 1 AoA | Yes |
| NFTF | EIRP, TRP, and spurious emissions metrics | N/A | N/A |
| DNF | N/A | N/A | Yes (“No final conclusions on the feasibility of DNF setup were made”) |

Most of these methodologies were subsequently defined in TS 38.508-1 [2], see B.2 with the exception of the DNF methodology, i.e.,

B.2 Permitted Test Methods 1139 B.2.1 General 1139 B.2.2 Direct far field (DFF) 1139 B.2.3 Direct far field (DFF) setup simplification for centre of beam measurements 1144 B.2.4 Indirect far field (IFF): Compact Antenna Test Range (CATR) 1145 B.2.5 Near field to far field transform (NFTF) 1146

# Permitted UE RF Methodologies

For UE RF testing, the permitted methodologies are defined in Clause B.2 of Error! Reference source not found. and in the editor’s note in Clause 5.1.1.3 Error! Reference source not found..

#### 5.1.1.3 Requirements for OTA tests Editor’s Note: - The UE pre-configuration mentioned below to disable UL Tx diversity schemes shall be voided once a test methodology solution to minimize spectral flatness artefacts between TE and UE over all test points is defined. - The permitted test methods for transmission and reception test are DFF, DFF with simplification for centre of beam measurements, IFF and NFTF and are described in TR 38.810[24]. The minimum requirements for each test setup are described in the following clauses.

The applicability to UE RF test metrics, highlighted in Table 1, was incorporated in the descriptions of the respective methodologies in Clauses B.2.x.1 of Error! Reference source not found.. On the other hand, the applicability per permitted test method was captured in Annex J of [5] but rather incomplete for select test cases only, i.e.,

Table J-1: Test case applicability per permitted test method Clause; No DUT antenna configuration declaration; DUT antenna configuration declaration. Configuration 1 (one antenna panel with D ≤ 5 cm active at any one time); Configuration 2 (More than one antenna panel D ≤ 5 cm without phase coherency between panels active at any one time); Configuration 3 (Any phase coherent antenna panel of any size). 6.5.1 Occupied bandwidth; IFF; DFF, DFF simplification, IFF, NFTF; DFF, DFF simplification, IFF, NFTF; IFF. 6.5.2.1 Spectrum Emission Mask; IFF; DFF, DFF simplification, IFF, NFTF; DFF, DFF simplification, IFF, NFTF; IFF. 6.5.2.3 Adjacent leakage ratio; IFF; DFF, DFF simplification, IFF, NFTF; DFF, DFF simplification, IFF, NFTF; IFF. 7.5 Adjacent Channel Selectivity; IFF; DFF, DFF simplification, IFF, NFTF; DFF, DFF simplification, IFF, NFTF; IFF. 7.6.2 In-band Blocking; IFF; DFF, DFF simplification, IFF, NFTF; DFF, DFF simplification, IFF, NFTF; IFF. NOTE: D = DUT radiating aperture declared by UE vendor..

It is proposed to update the applicability in [5], e.g., instead of listing the applicability for each test case clause, it is suggested to list the applicability for each test metric defined in [5].

Observation 1: The definition and applicability of permitted methodologies for UE RF testing has been properly captured in TS 38.508-1 but needs to be updated in TS 38.521-2.

Proposal 1: Update the applicability of permitted methodologies for UE RF testing in TS 38.521-2.

It should be noted that currently only the IFF methodology has completed all the requirements in RAN5, i.e., the TTs have been defined only for this one methodology in TS 38.521-2 [5].

# Permitted RRM Methodologies

In RAN5, additional permitted methodologies were introduced for 2 AoA RRM testing, specifically the ‘Enhanced IFF’ and the ‘IFF+DFF’ methodologies; these methodologies were captured in the descriptions of the RRM baseline setup in Clause 7.1.3.2 of [2]. Unfortunately, the description of these additional methodologies is rather incomplete and thus hard to comprehend for interested parties that have not following these discussions over the last few years. It is therefore proposed to add clarification of these methodologies in Clause B.2 of [2] similar to the methodologies defined in this clause already.

Proposal 2: Add high-level descriptions of the Enhanced IFF and the IFF+DFF methodologies in TS 38.508-1

The applicability of the permitted methodologies has been properly defined in both TS 38.508-1 [2], e.g., Clause 7.1.3.2.1

The NMAX_AoAs for the different permitted test methods is: - For UE RRM baseline measurement setup based on DFF, the supported NMAX_AoAs = 2. - For UE RRM baseline measurement setup based on simplified DFF, the supported NMAX_AoAs = 1. - For UE RRM baseline measurement setup based on IFF, the supported NMAX_AoAs = 1. - For UE RRM baseline measurement setup based on enhanced IFF, the supported NMAX_AoAs = 2. - For UE RRM baseline measurement setup based on IFF+DFF, the supported NMAX_AoAs = 2.

and TS 38.533 [4], e.g., Annex I

Table I.0-1 AoA Test Setup applicability per permitted test method AoA Test Setup; D > 5cm or No declaration; D ≤ 5cm. Setup 1; IFF, Enhanced IFF; DFF, IFF, Enhanced IFF, IFF+DFF. Setup 2a; IFF, Enhanced IFF; DFF, IFF, Enhanced IFF, IFF+DFF. Setup 2b; IFF, Enhanced IFF; DFF, IFF, Enhanced IFF, IFF+DFF. Setup 3; Enhanced IFF; DFF, Enhanced IFF, IFF+DFF. Setup 4a; Enhanced IFF; DFF, Enhanced IFF, IFF+DFF. Setup 4b; Enhanced IFF; DFF, Enhanced IFF, IFF+DFF. NOTE1: D =The diameter of the smallest sphere that encloses the radiating parts of the phase coherent array antenna(s) active at any one time during the test., declared by UE vendor as per Table 4.3.9-9 in TS 38.508-2 NOTE2: DFF indicates both DFF and DFF simplification in TR 38.810. NOTE3: For DFF and DFF part of IFF+DFF, minimum range length needs to meet the requirement as specified in 38.508-1 Annex B.2.2-4 with the declared D.

Observation 2: The applicability of the permitted RRM methodologies was properly captured in TS 38.508-1 and TS 38.533.

# Permitted Demodulation Methodologies

While permitted test methodologies for UE demodulation testing were defined in [1], the definitions have not been properly captured in any Technical Specification yet. The only references of permitted methodologies in TS 38.521-4 [6] is in Clauses F.2.1 and F.2.2, e.g.,

If the Test System using one of the permitted test methods defined in TR38.903 [20] for a test is known to have a measurement uncertainty greater than that specified in clause F.1, it is still permitted to use this apparatus provided that an adjustment is made value as follows:

A reference to a Technical Report for a definition of permitted methodologies should be avoided; more importantly, the TR38.903 [7] does not even properly define the permitted methodologies for demodulation testing. It should also be highlighted that TS 38.508-1 does not define the permitted methodologies for demodulation testing or any applicability of existing UE RF methodologies to demodulation testing.

It is therefore proposed to incorporate the permitted methodologies for demodulation testing, originally defined in [1], in TS 38.508-1 [2]. Given the testability concerns for UE demodulation testing related to marginal/insufficient SNR with increasing frequency, especially for FR2-2 as outlined in Clause 7.2 of the Enhanced Testability TR 38.884 [3], it is important to include the DNF methodology as permitted methodology as well and address the feasibility concerns based on many of the findings from the Enhanced Test Method SI [3]. The MU/MTSU for this new methodology are pending definition in RAN5.

Proposal 3: Properly define the permitted methodologies for demodulation testing in TS 38.508-1

Proposal 4: Add high-level description of the DNF methodology for demodulation testing in TS 38.508-1

# Conclusion

The following observations and conclusions were made in this contribution.

Observation 1: The definition and applicability of permitted methodologies for UE RF testing has been properly captured in TS 38.508-1 but needs to be updated in TS 38.521-2.

Observation 2: The applicability of the permitted RRM methodologies was properly captured in TS 38.508-1 and TS 38.533.

Proposal 1: Update the applicability of permitted methodologies for UE RF testing in TS 38.521-2.

Proposal 2: Add high-level descriptions of the Enhanced IFF and the IFF+DFF methodologies in TS 38.508-1

Proposal 3: Properly define the permitted methodologies for demodulation testing in TS 38.508-1

Proposal 4: Add high-level description of the DNF methodology for demodulation testing in TS 38.508-1

# References

- TR38.810, Study on test methods, V16.6.1 (2020-09)

- TS 38.508-1, User Equipment (UE) conformance specification; Part 1: Common test environment, V17.4.0 (2022-03)

- TR 38.884, Study on enhanced test methods for FR2 NR UEs, V18.0.0 (2022-03)

- TS 38.533, User Equipment (UE) conformance specification; Radio Resource Management (RRM), V17.2.0 (2022-03)

- TS 38.521-2, User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone, V16.11.0 (2022-03)

- TS 38.521-4: User Equipment (UE) conformance specification; Radio transmission and reception; Part 4: Performance requirements, V16.11.0 (2022-03)

- TR 38.903, Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases, V16.11.0 (2022-03)