3GPP TSG-RAN5 Meeting #95-e	R5-222557
Electronic Meeting, 9th May – 20th May 2022

Agenda item:	5.3.39.1

Source:	Keysight Technologies

Title:	On NF Methodologies

Document for:	Discussion and Endorsement

# Introduction

This contribution addresses the applicability of the NF methodologies for conformance tests with relaxations, specifically the low UL/high DL power test cases [1].

# Discussion

The efforts of the NF methodologies made in RAN4 as part of the Enhanced Test Methods SI for select FR2 conformance test cases were summarized in [2]. The total amount of relaxation improvement corresponds to the improvements in the OTA portion, i.e., reduction of free-space path losses, and the conducted portion of the conformance test system, i.e., reduction of system losses due to simplification of switching, signal conditioning, etc.

Observation 1: The total amount of relaxation improvement corresponds to the improvements in the OTA portion and the conducted portion of the conformance test system

It was found and documented [1][2] that the best-case improvement of the relaxations specifically related to the OTA portion of the test system is ~14dB due to the reduction of path losses when testing in the NF.

Observation 2: The best-case improvement of the relaxations related to the OTA portion of the conformance test system is ~14dB.

While this level of improvement alone is not sufficient to overcome the relaxations for all low-UL/high-DL test cases [1], those improvements far outweigh the improvements of state-of-the-art equipment to date.

Observation 3: The improvements related to the OTA portion/free-space path loss reductions far outweigh the improvements of state-of-the-art equipment to date.

The discussion of free-space path loss reductions in RAN4#94-e lead to a creation of an action point whether these efforts should be continued if it was determined that the worst-case relaxations cannot completely be eliminated, i.e.,

| Action ID | sWG | Action | Responsible | Relevant Tdoc | Deadline | Status |
| --- | --- | --- | --- | --- | --- | --- |
| AP#94e.21 | RF | Determine whether the new NF methodologies is to be considered if the applicable FR2 test requirement relaxations cannot completely be eliminated. | KEYS, R&S, Apple | R5-221260r1 | RAN5#96 | Open |

The evaluation of different options for the conducted portion of the conformance test system with the goal to optimize the relaxations while balancing the complexity of the system is on-going, including for existing permitted methodologies [3]. Early indications suggest further reductions of the relaxation are possible for the NF portion but more time is needed to quantify the improvement while considering the upgradeability and complexity of test systems.

Observation 4: The assessment of improvements in relaxations related to the conducted portion of the conformance test system while balancing the upgradeability and complexity of test systems is on-going and more time is needed

It is therefore proposed to keep this action point open until more detailed analyses are available and to potentially consider NF methodologies even if they fall short of completely eliminating the relaxations by a few dB.

Proposal 1: Keep action point AP#94e.21 open until more detailed analyses of the improvements in relaxations related to the conducted portion of the conformance test system is available.

# Conclusion

The following observations and conclusions were made in this contribution.

Observation 1: The total amount of relaxation improvement corresponds to the improvements in the OTA portion and the conducted portion of the conformance test system

Observation 2: The best-case improvement of the relaxations related to the OTA portion of the conformance test system is ~14dB.

Observation 3: The improvements related to the OTA portion/free-space path loss reductions far outweigh the improvements of state-of-the-art equipment to date.

Observation 4: The assessment of improvements in relaxations related to the conducted portion of the conformance test system while balancing the upgradeability and complexity of test systems is on-going and more time is needed

Proposal 1: Keep action point AP#94e.21 open until more detailed analyses of the improvements in relaxations related to the conducted portion of the conformance test system is available.

# References

- TR 38.884, Study on enhanced test methods for FR2 NR UEs, V18.0.0 (2022-03)

- R5-221640, On NF Methodologies for Low UL/High DL Power Test Cases, Keysight Technologies, 3GPP TSG-RAN5 Meeting #94-e, March 2022

- R5-223217, Discussion on improvements of permitted test methods, Keysight Technologies, 3GPP TSG-RAN5 Meeting #95-e, May 2022