3GPP TSG-RAN5 Meeting #95-e	R5-222617

# Electronic Meeting, 09th – 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2331 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of default message contents for NR SL Demod |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - To Add default message contents for NR SL demod tests |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Default message contents for NR SL demod tests are added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

5.4.3	Sidelink information elements for Demodulation Performance tests

As defined in clause 4.6.3 with the following exceptions:

SL-ResourcePool

Table 5.4.3-1: SL-ResourcePool

| Derivation Path: Table 4.6.6-25 with condition SL_HARQ |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-ResourcePool-r16 ::= SEQUENCE { |  |  |  |  |
| sl-PSFCH-Config-r16 CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| sl-PSFCH-RB-Set-r16 |  | ones(1,50) | All "1" sequence with a length of 50 bits | BW = 20MHz |
|  |  | ones(1,100) | All "1" sequence with a length of 100 bits | BW = 40MHz |
| sl-NumMuxCS-Pair-r16 |  | n1 |  |  |
| sl-MinTimeGapPSFCH-r16 |  | sl3 |  |  |
| sl-PSFCH-CandidateResourceType-r16 |  | allocSubCH |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-NumSubchannel-r16 |  | 5 |  | BW = 20MHz |
|  |  | 10 |  | BW = 40MHz |
| sl-RB-Number-r16 |  | 50 |  | BW = 20MHz |
|  |  | 100 |  | BW = 40MHz |
| sl-TimeResource-r16 |  | one(1,160) | All "1" sequence with a length of 160 bits |  |
| } |  |  |  |  |

SL-Thres-RSRP-List

Table 5.4.3-2: SL-Thres-RSRP-List

| Derivation Path: Table 4.6.6-32 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-Thres-RSRP-List-r16 ::= SEQUENCE (SIZE (64)) OF INTEGER { |  | 64 entries |  |  |
| INTEGER[k, k=1..64] |  | 66 | entry k corresponds to infinity dBm |  |
| } |  |  |  |  |

5.5	Common procedures for RF testing

<End of modified section 1>