3GPP TSG-RAN5 Meeting #95-e	R5-222623

# Electronic Meeting, 09th – 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0529 | rev | - | Current version: | 16.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR SL Demod TC 11.1.6 - imbalance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - To Add NR SL Demod TC 11.1.6 - Power imbalance performance with two links |  |  |  |  |  |  |  |  |
| Summary of change: |  | - NR SL Demod TC 11.1.6 - Power imbalance performance with two links is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.6 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 CR 0535 TS 38.522 CR 0164 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

11.1.6	Power imbalance performance with two links

11.1.6.1	2Rx requirements

11.1.6.1.0	Minimum requirements

The minimum requirements are specified in Table 11.1.6.1.0-2 with the test parameters specified in Table 11.1.6.1.0-1. In this test scenario, The Sidelink UE 1 and 2 are synchronized to GNSS or GNSS-equivalent synchronization reference.

Table 11.1.6.1.0-1: Test parameters

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Active cell(s) |  |  | None |
| Active Sidelink UE(s) |  |  | Sidelink UE 1, Sidelink UE 2 |
| Sidelink UE 1 | Sidelink Transmissions |  | PSCCH + PSSCH |
|  | PSSCH DMRS pattern(Note 1) |  | {2,3} |
|  | Sub-channel allocation |  | Sub-channel 0 |
|  | Time offset (Note 2) | s | 0 |
|  | Frequency offset (Note 3) | Hz | 0 |
|  | Antenna configuration |  | 1x2 Low |
|  | PSFCH periodicity | Slots | 4 |
|  | MinTimeGapPSFCH | Slots | 3 |
| Sidelink UE 2 | Sidelink Transmissions |  | PSCCH + PSSCH |
|  | PSSCH DMRS pattern(Note 1) |  | {2,3} |
|  | Sub-channel allocation |  | Sub-channnel 3 |
|  | Time offset (Note 2) | s | 0 |
|  | Frequency offset (Note 3) | Hz | 0 |
|  | Antenna configuration |  | 1x2 Low |
|  | PSFCH periodicity | Slots | 4 |
|  | MinTimeGapPSFCH | Slots | 3 |
| Note 1: {x, y}: x and y means the number of DMRS symbols for slot with PSFCH transmission and without PSFCH transmission, respectively. Note 2: Time offset of transmitted Sidelink UE signal with respect to GNSS reference timing. Note 3: Frequency offset of transmitted Sidelink UE signal with respect to GNSS reference frequency. |  |  |  |

Table 11.1.6.1.0-2: Minimum performance

| Test number | Bandwidth (MHz)/ Subcarrier spacing(kHz) | Sidelink UE | PSSCH Reference channel | Modulation format and code rate | Propagation condition | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | PSSCH BLER (%) | SNR (dB) of PSSCH |
| 1 | 20 / 30 | 1 | R.PSSCH.2-1.4 | QPSK, 0.30 | AWGN | (Note 1) | 30.35 |
|  |  | 2 | R.PSSCH.2-1.4 | QPSK, 0.30 | AWGN | 10 | 4.8 |
| Note 1: There is no BLER requirement for Sidelink UE 1. |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-4 [5], clause 11.1.6.1.1

11.1.6.1.1	Test purpose

The purpose is to verify the demodulation performance when receiving PSSCH transmissions from two Sidelink UEs with power imbalance in one slot.

11.1.6.1.2	Test applicability

This test case applies to all types of NR UE release 16 and forward which support NR sidelink communication, as specified in Table 11.1.1.1.2-1.

11.1.6.1.3	Test description

11.1.6.1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in 38.521-1 [7] subclause 5.3E.

PSCCH and PSSCH reference measurement channels are set according to Table 11.1.1.2-1, Table 11.1.6.1.0-1 and Annex A.6 as appropriate.

Configurations of the GNSS simulator are specified in TS 38.508-1 [6] Table 4.11.2-2 and the GNSS simulator is configured for Scenario #1: static in Geographical area #1. Geographical area #1 is also pre-configured in the UE.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Low Range, as defined in TS 38.508-1 [6] clause 4.3.1.8.

Channel Bandwidths to be tested: As specified per test number in Table 11.1.6.1.0-2 as defined in TS 38.508-1 [6] clause 4.3.1.8.

1.	Connect the SS, the faders, and the AWGN noise source to the UE antenna connectors, connect the SS COM port to the UE COM port, and connect the GNSS simulator to the UE GNSS RX antenna connector as shown in TS 38.508-1 [6] Annex A, Figure A.3.1.9.1(TE part) and Figure A.3.2.7.1 (UE part).

2.	The parameter settings for the V2X sidelink transmission over PC5 interface are pre-configured according to TS 38.508-1 [6] clause 4.10.1. Message content exceptions are defined in clause 11.1.6.1.3.3.

3.	Sidelink physical channels and signals are initially set up according to Table 11.1.1.2-1, Table 11.1.6.1.0-1 and Annex A.6 as appropriate.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state 4-A with generic procedure parameters Test Loop Function = On according to TS 38.508-1 [6] clause 4.4A.2. The UE is configured as the receiving UE. Message contents are defined in clause 11.1.6.1.3.3.

6.	The GNSS simulator is triggered to start step 1 of Scenario #1 to simulate a location in the centre of Geographical area #1. Wait for the UE to acquire the GNSS signal.

11.1.6.1.3.2	Test procedure

1.	Sidelink UE1 and Sidelink UE2 transmit PSCCH/PSSCH RMC according to SL-PreconfigurationNR and Table 11.1.6.1.0-1. Both sidelink UE1 and sidelink UE2 transmit MAC padding bits on their sidelink RMC respectively.

2.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Table 11.1.6.1.4-1 as appropriate.

3.	Measure the average PSSCH BLER of sidelink UE2 according to the test method described in Annex G.5 for a duration sufficient to achieve statistical significance. If the measured average PSSCH BLER of sidelink UE2 is less than the reference value in Table 11.1.6.1.4-1 pass the UE. Otherwise fail the UE.

4.	Repeat steps from 1 to 3 for each subtest in Table 11.1.6.1.4-1 as appropriate.

11.1.6.1.3.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 5.4 with the following exceptions.

Table 11.1.6.1.3.3-1: Physical layer parameters for SCI format 1-A

| Derivation Path: TS 38.508-1 [6] Table 4.3.6.2.1.1-1 |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| DMRS pattern | indicates the 1st entry in sl-PSSCH-DMRS-TimePatternList-r16 if mod (i, 4), else indicates the 2nd entry in sl-PSSCH-DMRS-TimePatternList-r16. where i is the logical slot index belong to resource pool per 1024 radio frame as specified in 38.214 [12] clause 8 | "0"B if mod (i, 4) = 0 "1"B if mod (i, 4) ≠ 0 |  |

Table 11.1.6.1.3.3-2: SL-ResourcePool

| Derivation Path: TS 38.508-1 [6] Table 5.4.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-ResourcePool-r16 ::= SEQUENCE { |  |  |  |  |
| sl-PSSCH-Config-r16 CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| sl-PSSCH-DMRS-TimePatternList-r16 SEQUENCE (SIZE (1..3)) OF INTEGER (2..4) { |  | 2 entries |  |  |
| INTEGER[1] |  | 2 | entry 1 |  |
| INTEGER[2] |  | 3 | entry 2 |  |
| } |  |  |  |  |
| } |  |  |  |  |

11.1.6.1.4	Test Requirements

For the parameters specified in Table 11.1.6.1.0-1 and SNR specified in Table 11.1.6.1.4-1, the average PSSCH BLER of sidelink UE2 obtained in step 3 shall be below the reference value specified in Table 11.1.6.1.4-1. The PSSCH sidelink reference channels are defined in Annex A Table A.6.2.2-1.

Table 11.1.6.1.4-1: Test performance

| Test number | Bandwidth (MHz)/ Subcarrier spacing(kHz) | Sidelink UE | PSSCH Reference channel | Modulation format and code rate | Propagation condition | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | PSSCH BLER (%) | SNR (dB) of PSSCH |
| 1 | 20 / 30 | 1 | R.PSSCH.2-1.4 | QPSK, 0.30 | AWGN | (Note 1) | 30.35 |
|  |  | 2 | R.PSSCH.2-1.4 | QPSK, 0.30 | AWGN | 10 | 5.4 |
| Note 1: There is no BLER requirement for Sidelink UE 1. |  |  |  |  |  |  |  |

<End of modified section 1>