3GPP TSG-RAN5 Meeting #95-e	R5-222780

# Electronic Meeting, 09th – 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0413 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR IMS TC 8.3-Originating Identification Restriction Signalling 5GS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The description that steps 10-12 of generic procedure specified in Table 4.9.15.2.2-1 of TS 38.508-1 are performed, is missing between step 12 and 13. But the corresponding behavior has been implemented in TTCN code. - RRCReconfigurationComplete message is missing after steps 2-7. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Split steps 9-19. Add steps 12A-12C corresponding to steps 10-12 of generic procedure specified in Table 4.9.15.2.2-1 of TS 38.508-1. - Add Table 8.3.3.2-2 for RRCReconfigurationComplete message and Exception description in test body. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description of test body is not aligned with the implemetation of TTCN code. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

8.3	Originating Identification Restriction / Signalling / 5GS

8.3.1	Test Purpose (TP)

(1)

with { UE is registered to IMS and OIR in temporary mode is activated }

ensure that {

when { UE is made to initiate a voice call }

then { UE sends INVITE with "Anonymous" in the From header }

}

8.3.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-15 requirements.

[TS 24.607 clause 4.5.2.1]:

If the originating user wishes to override the default setting of "presentation not restricted" of the OIR service in temporary mode:

-	The originating UE shall include an "anonymous" From header field. The convention for configuring an anonymous From header field described in IETF RFC 3323 [6] should be followed; i.e. From: "Anonymous" <sip:anonymous@anonymous.invalid>;tag= xxxxxxx.

-	If only the P-Asserted-Identity needs to be restricted the originating UE shall include a Privacy header field [6] set to "id" in accordance with IETF RFC 3325 [7].

-	If all headers containing private information that the UE cannot anonymize itself need to be restricted, the originating UE shall include a Privacy header field set to "header" in accordance with IETF RFC 3323 [6].

NOTE 2:	The Privacy header field value "header" does not apply to the identity in the From header field.

8.3.3	Test description

8.3.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	The UE contains either ISIM and USIM applications or only USIM application on UICC.

-	The UE is configured to register for IMS after switch on.

-	The UE is configured to use preconditions.

-	The UE is configured to use Telephony Originating Identification Restriction in temporary mode

Preamble:

-	UE is in state 1N-A (TS 38.508-1 [21]) and registered to IMS

8.3.3.2	Test procedure sequence

Table 8.3.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE attempt an IMS speech call with originating identification restriction. | - | - | - | - |
| 2-7 | Steps 2-7 of generic procedure specified in Table 4.9.15.2.2-1 of TS 38.508-1 [21] are performed. | - | - | - | - |
| - | EXCEPTION: In parallel with Step 8, parallel behaviour defined in table 8.3.3.2-2 takes place | - | - | - | - |
| 8 | Check: Does the UE sends INVITE with first SDP offer with including an "Anonymous" From header field? | --> | INVITE | 1 | P |
| 9-12 | Steps 2-5 of Annex A.4.1 are perfomed | - | - | - | - |
| 12A-12C | SS triggers resource reservation: Steps 10-12 of generic procedure specified in Table 4.9.15.2.2-1 of TS 38.508-1 [21] are performed. | - | - | - | - |
| 13-19 | Steps 6-12 of Annex A.4.1 happen. | - | - | - | - |
| 20 | Make the UE release the IMS speech call. | - | - | - | - |
| 21-22 | Steps 1-2 defined in annex A.7 are performed to MO release the IMS speech call. | - | - | - | - |

Table 8.3.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |

8.3.3.3	Specific message contents

Table 8.3.3.3-1: INVITE (step 8, Table 8.3.3.2-1)

| Derivation Path: Step 1 of A.4.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| From addr-spec |  |  |  |  |
|  |  | “Anonymous" <sip:anonymous@anonymous.invalid> |  |  |
| Privacy |  | id or header (mutually exclusive) |  | RFC 3323[50] RFC 3325[51] |

<End of modified section 1>