3GPP TSG-RAN5 Meeting #95-e	R5-222804

# Electronic Meeting, May 9 - May 20 2022

Source:	Huawei, HiSilicon

Title:	Discussion paper on legacy test cases extension for Redcap

Agenda item:	6.3.28.6

Document for:	Information and Discussion

Background

The RAN5 Work Item of supporting reduced capability NR devices were introduced at the last meeting. In this WI, we will add several RedCap dedicated test cases and need to review all the R15 legacy test cases. Similar to SNPN only UEs, now we need to discuss about R15 legacy test cases adjustment for Redcap only UEs. Fortunately, we can get some experience from the discussion paper R5-220837, in which the proposal#2 seems acceptable in RAN5.

Discussion

The WI Redcap mainly introduce the following changes on the default configuration and generic procedures.in TS 38.508-1

- SIBs: SIB1 is to check if Redcap is barred, SIB2 is to indicate measurement relax and SIB4 is to provide Redcap supporting information of neighbor cells.

- RACH procedure: Msg1 and Msg 3 for UE identification.

- RRCReconfiguration: RadioBearerConfig for PDCP and higher layer parameters and CellGroupConfig for RLC and lower layer parameters.

In fact, almost of the changes above could be fixed in TS 38.508-1 as default configurations for Redcap.

Proposal

Referring to the discussion on R5-220837 at the last meeting, a new chapter is needed to capture legacy test cases adjustment for verticals. The proposed chapter title was “Applicable R-15 tests for verticals”.

Considering Redcap + Feature X beyond R15 might need to be captured in the future, I think we’d better to be careful with the new chapter title.

Proposal 1: we propose RAN5 uses a more inclusive chapter title for vertical test, such as “Applicable legacy tests for verticals”. and then the sub-clause could be assigned to any vertical feature, such as SNPN Only UE, Redcap only UE and so on. In this case, vertical feature + Feature X beyond R15 could be captured in the same sub-clause X.1, X.2,…

Please see the following proposed:

X 	 	Applicable legacy tests for verticals

X.1		SNPN Only UE

…

X.2		Redcap Only UE

X.2.1		Idle Mode Tests for Redcap

X.2.1.1 	Cell selection / Qrxlevmin & Cell reselection (Intra NR) – Redcap Only

X.2.1.1	Cell selection / Qrxlevmin & Cell reselection (Intra NR) – Redcap Only

X.2.1.1.1	Test Purpose (TP)

Same as TC 6.1.2.1 but applied to Redcap Only UE

X.2.1.1.2	Conformance requirements

Same as TC 6.1.2.1 but applied to Redcap Only UE

X.2.1.1.3	Test description

X.2.1.1.3.1	Pre-test conditions

Same as TC 6.1.2.1 with the following differences:

-	Carrier bandwidth of NR Cell shall be 20M.

12.1.1.1.3.2	Test procedure sequence

Same as TC 6.1.2.1

X.2.2		Layer 2

…

X.2.n		Power saving for Redcap (Redcap + R16 Feature PowSav)

Open questions

The following is kindly asked:

Question 1: Any other proposal for the prose structure?