3GPP TSG-RAN5 Meeting #95-e	R5-222805

# Electronic Meeting, May 9 - May 20 2022

Source:	Huawei, HiSilicon

Title:	Discussion On IMS Data Channel test cases

Agenda item:	4.2.1

Document for:	Information and Discussion

Background

The IMS data channel uses the data channel media type as defined in 3GPP Release 16 TS 26.114 and can be used in parallel with other media types such as voice and video in the Multimedia Telephony (MMTel) service. This data channel is highly flexible and can be used to carry any type of information between the User Equipment (UE) and the network or end-to-end between UEs. It is adapted to be used in the 3GPP MMTel context by new procedures in 3GPP Release 16 TS 26.114 and through minor extensions to existing call handling procedures in 3GPP Release 17 TS 24.229, TS 29.165 TS 24.173.

Discussion

IMS Data Channel applies to VoLTE and VoNR. So for each Radio access, the objective includes at least the following areas:

- Initial Registration to check if UE can include the sip.app-subtype media feature tag with a value "webrtc-datachannel" if the UE supports IMS data channel.

- Bootstrap data channel to check if UE and network can establish bootstrap data channel.

- Application data channel to check if application data channel can be established between UE and network or end-to-end between UEs.

- IMS data channel associated with an IMS call (Voice, Video) to check if UE can support using data channel simultaneously with voice or video media.

- MT Data Channel Call coexists with other scenario such as Forking to check if a terminating UE that supports data channel can be selected.

- MO Data Channel Call fallback to voice call to check if the data channel call can fallback to voice call when originating UE requests data channel but terminating UE doesn't support data channel.

In fact, the core Spec of IMS data channel has already been included and completed in SA4 R16 WI 5G_MEDIA_MTSI_ext, which also includes several other sub-features, such as ANBR and so on. IMS data channel looks relatively independent of other sub-features. Please see the WID SP-190640.

Huawei, Hisilicon would like to work on IMS data channel test, however it's just part of the SA4 WI.

Proposal

The following is proposed:

Proposal 1: If there is any company interested in the other parts of this SA4 WI, we could start the work together based on a new proposed WID which contains all sub-features we interested.

Proposal 2: If there is no company interested in the other parts of this SA4 WI, the IMS data channel test could be done in a dedicated WI. Please find the test details in the draft WID R5-222806.

Proposal 3: RAN5 will work on the IMS data channel test cases within TEI_16 for NR part and TEI_9 for LTE part.

If we can’t find a collaborator finally, Huawei, Hisilicon would prefer Proposal 2. A dedicated WI would be easier to manage and maintain since plenty of test cases need to be added according to the draft WID R5-222806.