3GPP TSG-RAN5 Meeting #95-e	R5-222812

# Electronic Meeting, 09th – 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2942 | rev | - | Current version: | 16.11.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR TC 7.1.1.7.1.1-Activation and Deactivation of Scells |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - According to current TTCN implementation, UE may receive the UL grant at step 6 before the lastest transmission occasion of SR. Then UE may cancel pending SR and the test case will fail at step 5. Same issue may take place at step 14. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add Exception description after step 5 and 14, and add time delay for the transmission of UL grant. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the TC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.7.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

7.1.1.7.1.1	Activation/Deactivation of SCells / Activation/Deactivation MAC control element reception / sCellDeactivationTimer / Intra-band Contiguous CA

7.1.1.7.1.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with SCell configured }

ensure that {

when { UE receives an SCell Activation/Deactivation MAC CE activating the Scell }

then { UE starts monitoring PDCCH on activated SCell }

}

(2)

with(UE in RRC_CONNECTED state with SCell activated )

ensure that {

when{ UE receives a DL assignment on SCell PDCCH }

then {  UE restarts the sCellDeactivationTimer }

}

(3)

with ( UE in RRC_CONNECTED state with SCell activated )

ensure that {

when{ UE sCellDeactivationTimer expires }

then { UE deactivates the SCell and stops monitoring PDCCH on SCell }

}

(4)

with (UE in RRC_CONNECTED state with SCell activated )

ensure that {

when{ UE receives a SCell Activation/Deactivation MAC CE deactivating the SCell }

then { UE deactivates the SCell and stops monitoring PDCCH on SCell }

}

(5)

with( UE in RRC_CONNECTED state with SCell activated and UL CA is supported)

ensure that {

when{ UE receives a UL assignment on SCell and has data available for transmission }

then { UE transmits the UL MAC PDU }

}

7.1.1.7.1.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.321, clauses 5.9 and TS 38.331 clause 5.3.5.5.2. Unless otherwise stated these are Rel-15 requirements.

[TS 38.321, clause 5.9]

If the MAC entity is configured with one or more SCells, the network may activate and deactivate the configured SCells. Upon configuration of an SCell, the SCell is deactivated.

The configured SCell(s) is activated and deactivated by:

-	receiving the SCell Activation/Deactivation MAC CE described in subclause 6.1.3.10;

-	configuring sCellDeactivationTimer timer per configured SCell (except the SCell configured with PUCCH, if any): the associated SCell is deactivated upon its expiry.

The MAC entity shall for each configured SCell:

1>	if an SCell Activation/Deactivation MAC CE is received activating the SCell:

2>	activate the SCell according to the timing defined in TS 38.213 [6]; i.e. apply normal SCell operation including:

3>	SRS transmissions on the SCell;

3>	CSI reporting for the SCell;

3>	PDCCH monitoring on the SCell;

3>	PDCCH monitoring for the SCell;

3>	PUCCH transmissions on the SCell, if configured.

2>	start or restart the sCellDeactivationTimer associated with the SCell in the slot when the SCell Activation/Deactivation MAC CE was received;

2>	(re-)initialize any suspended configured uplink grants of configured grant Type 1 associated with this SCell according to the stored configuration, if any, and to start in the symbol according to rules in subclause 5.8.2;

2>	trigger PHR according to subclause 5.4.6.

1>	else if an SCell Activation/Deactivation MAC CE is received deactivating the SCell; or

1>	if the sCellDeactivationTimer associated with the activated SCell expires:

2>	deactivate the SCell according to the timing defined in TS 38.213 [6];

2>	stop the sCellDeactivationTimer associated with the SCell;

2>	stop the bwp-InactivityTimer associated with the SCell;

2>	clear any configured downlink assignment and any configured uplink grant Type 2 associated with the SCell respectively;

2>	suspend any configured uplink grant Type 1 associated with the SCell;

2>	flush all HARQ buffers associated with the SCell.

1>	if PDCCH on the activated SCell indicates an uplink grant or downlink assignment; or

1>	if PDCCH on the Serving Cell scheduling the activated SCell indicates an uplink grant or a downlink assignment for the activated SCell; or

1>	if a MAC PDU is transmitted in a configured uplink grant or received in a configured downlink assignment:

2>	restart the sCellDeactivationTimer associated with the SCell.

1>	if the SCell is deactivated:

2>	not transmit SRS on the SCell;

2>	not report CSI for the SCell;

2>	not transmit on UL-SCH on the SCell;

2>	not transmit on RACH on the SCell;

2>	not monitor the PDCCH on the SCell;

2>	not monitor the PDCCH for the SCell;

2>	not transmit PUCCH on the SCell.

HARQ feedback for the MAC PDU containing SCell Activation/Deactivation MAC CE shall not be impacted by PCell, PSCell and PUCCH SCell interruptions due to SCell activation/deactivation in TS 38.133 [11].

When SCell is deactivated, the ongoing Random Access procedure on the SCell, if any, is aborted.

[TS 38.321, clause 6.1.3.10]

The SCell Activation/Deactivation MAC CE of one octet is identified by a MAC PDU subheader with LCID as specified in Table 6.2.1-1. It has a fixed size and consists of a single octet containing seven C-fields and one R-field. The SCell Activation/Deactivation MAC CE with one octet is defined as follows (Figure 6.1.3.10-1).

The SCell Activation/Deactivation MAC CE of four octets is identified by a MAC PDU subheader with LCID as specified in Table 6.2.1-1. It has a fixed size and consists of four octets containing 31 C-fields and one R-field. The SCell Activation/Deactivation MAC CE of four octets is defined as follows (Figure 6.1.3.10-2).

For the case with no Serving Cell with a ServCellIndex as specified in TS 38.331 [8] larger than 7, SCell Activation/Deactivation MAC CE of one octet is applied, otherwise SCell Activation/Deactivation MAC CE of four octets is applied.

-	Ci: If there is an SCell configured for the MAC entity with SCellIndex i as specified in TS 38.331 [8], this field indicates the activation/deactivation status of the SCell with SCellIndex i, else the MAC entity shall ignore the Ci field. The Ci field is set to "1" to indicate that the SCell with SCellIndex i shall be activated. The Ci field is set to "0" to indicate that the SCell with SCellIndex i shall be deactivated;

-	R: Reserved bit, set to "0".

Figure 6.1.3.10-1: SCell Activation/Deactivation MAC CE of one octet

Figure 6.1.3.10-2: SCell Activation/Deactivation MAC CE of four octets

7.1.1.7.1.1.3	Test description

7.1.1.7.1.1.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that DRB is configured in RLC AM mode according to Table 7.1.1. 7.1.1.3.1-1 and in additionNR Cell 3 is configured as NR Active SCell.

Table 7.1.1. 7.1.1.3.1-1: RLC parameters

| t-PollRetransmit | ms80 |
| --- | --- |

7.1.1.7.1.1.3.2	Test procedure sequence

Table 7.1.1.7.1.1.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 |
| --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -85 | -85 |

Table 7.1.1.7.1.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits an RRCReconfiguration message to configure NR SCell(NR Cell 3). Note 1 | <-- | (RRCReconfiguration) | - | - |
| 2 | The UE transmits RRCReconfigurationComplete message. Note 2 | --> | (RRCReconfigurationComplete) | - | - |
| 3 | The SS transmits Activation MAC control element to activate NR SCell on NR SpCell. | <-- | MAC PDU (SCell Activation/Deactivation MAC CE of one octet (C1=1)) | - | - |
| 4 | 200 ms after step 3, the SS indicates a new transmission on PDCCH of SCell and transmits a MAC PDU (containing an RLC PDU) | <-- | MAC PDU | - | - |
| 5 | Check: Does the UE transmit a Scheduling Request on PUCCH? | --> | (SR) | 1 | P |
| - | EXCEPTION: The SS ignores following scheduling requests before step 6. | - | - | - | - |
| 6 | 50ms after step 4, the SS sends an UL grant suitable for transmitting loop back PDU on NR SpCell. | <-- | (UL Grant) | - | - |
| 7 | The UE transmits a MAC PDU containing the loop back PDU corresponding to step 4. | --> | MAC PDU | - | - |
| 8 | The SS transmits a MAC PDU containing RLC status PDU acknowledging reception of RLC PDU in step 7 on NR SpCell | <-- | MAC PDU | - | - |
| 9 | 400 ms(sCellDeactivationTimer = 320 ms) after step 4, the SS indicates a new transmission on PDCCH of NR SCell and transmits a MAC PDU (containing an RLC PDU) | <-- | MAC PDU | - | - |
| 10 | Check: Does the UE transmit a Scheduling Request on PUCCH in next 1 second? | --> | (SR) | 2 | F |
| 11 | The SS transmits Activation MAC control element to activate SCell on NR SpCell. | <-- | MAC PDU ((SCell Activation/Deactivation MAC CE of one octet (C1=1)) | - | - |
| 12 | 200 ms after step 11 The SS indicates a new transmission on PDCCH of NR SCell and transmits a MAC PDU (containing just padding or RLC status PDU, but no RLC data PDU) | <-- | MAC PDU | - | - |
| 13 | 400 ms after step 11 the SS indicates a new transmission on PDCCH of NR SCell and transmits a MAC PDU (containing an RLC PDU) | <-- | MAC PDU | - | - |
| 14 | Check: Does the UE transmit a Scheduling Request on PUCCH? | --> | (SR) | 1,3 | P |
| - | EXCEPTION: The SS ignores following scheduling requests before step 15. | - | - | - | - |
| 15 | 50ms after step 13, the SS sends an UL grant suitable for transmitting loop back PDU IF pc_UL_NR_CA_2CC on NR SCell ELSE on NR SpCell. | <-- | (UL Grant) | - | - |
| 16 | The UE transmits a MAC PDU containing the loop back PDU corresponding to step 14 | --> | MAC PDU | 5 | P |
| 17 | The SS transmits a MAC PDU containing RLC status PDU acknowledging reception of RLC PDU in step 16 | <-- | MAC PDU | - | - |
| 18 | The SS transmits Deactivation MAC control element to de-activate SCell on NR SpCell. | <-- | MAC PDU (SCell Activation/Deactivation MAC CE of one octet (C1=0)) | - | - |
| 19 | The SS indicates a new transmission on PDCCH of NR SCell and transmits a MAC PDU (containing an RLC PDU) | <-- | MAC PDU | - | - |
| 20 | Check: Does the UE transmit a Scheduling Request on PUCCH in the next 1 second? | --> | (SR) | 4 | F |
| Note 1: For EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration. Note 2: For EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. |  |  |  |  |  |

7.1.1.7.1.1.3.3	Specific message contents

Table 7.1.1.7.1.1.3.3-1: RRCReconfiguration (step 1, Table 7.1.1.7.1.1.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| secondaryCellGroup |  | CellGroupConfig |  | EN-DC |
| nonCriticalExtension SEQUENCE { |  |  |  | NR |
| masterCellGroup |  | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.7.1.1.3.3-2: CellGroupConfig (Table 7.1.1.7.1.1.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SCellConfig { |  | 1 entry |  |  |
| SCellConfig[1] SEQUENCE { |  |  | entry 1 |  |
| sCellIndex |  | SCellIndex as per TS 38.508-1 [4] table 4.6.3-154 |  |  |
| sCellConfigCommon |  | ServingCellConfigCommon |  |  |
| sCellConfigDedicated |  | ServingCellConfig |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.7.1.1.3.3-3: ServingCellConfigCommon (Table 7.1.1.7.1.1.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| physCellId | Physical Cell Identity of NR Cell 3 |  |  |
| uplinkConfigCommon | Not present |  | Not pc_UL_NR_CA_2CC |
| } |  |  |  |

Table 7.1.1.7.1.1.3.3-4: ServingCellConfig (Table 7.1.1.7.1.1.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| uplinkConfig | Not present |  | Not pc_UL_NR_CA_2CC |
| uplinkConfig SEQUENCE { |  |  | pc_UL_NR_CA_2CC |
| initialUplinkBWP | BWP-UplinkDedicated |  |  |
| } |  |  |  |
| sCellDeactivationTimer | ms320 |  |  |
| } |  |  |  |

Table 7.1.1.7.1.1.3.3-5: BWP-UplinkDedicated (Table 7.1.1.7.1.1.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkDedicated ::= SEQUENCE { |  |  |  |
| pucch-Config | Not present |  |  |
| } |  |  |  |

<End of modified section 1>