Title: 	Discussion on test point selection for Configured transmitted power for Inter-band EN-DC within FR1 (2 E-UTRA CC, 1 NR CC)

Source: 	Huawei, HiSilicon

Introduction

The purpose of this paper is to propose test point selection of Configured transmitted power testing in inter-band EN-DC with 2 E-UTRA CCs and 1 NR CC within FR1.

There are exceptional requirements in TS 38.101-3, therefore the test points selection have to take both LTE and NR carriers into consideration.

Discussion

As a reference, below is the general initial conditions used for LTE configured transmitted power testing for intra-band contiguous CA:

Table 6.2.5A.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  | Mid range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 25 | 25 | N/A for this test | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 25 | 50 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 50 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 50 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 72 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 75 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 100 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 100 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 100 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 100 | 100 |  | QPSK | 18 | P_18@0 | S_0@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1 Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |

And general initial conditions used for NR FR1 Configured transmitted power testing:

Table 6.2.4.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Mid range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest |  |
| Test Parameters for Channel Bandwidths |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for minimum output power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | test case | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 |  | DFT-s-OFDM QPSK | Inner Full |
| 33 |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: UE operating in TDD mode with PI/2 PBSK modulation and UE indicates support for UE capability [powerBoosting-pi2BPSK] and the IE powerBoostPi2BPSK is set to 1 for bands n40, n41, n77, n78 and n79. |  |  |  |

Test Environment

Based on the baseline test environment proposed in [1], test environment for Configured transmitted power for FR1 NR shall follow what is defined as in LTE. Same rule shall be used for EN-DC RF testing.

Proposal 1: Propose Test Environment as Normal, TL/VL, TL/VH, TH/VL, TH/VH for Configured transmitted power for inter band EN-DC with 2 E-UTRA CCs and 1 NR CC within FR1.

Test Frequencies selection for inter-band EN-DC

In LTE inter-band CA testing, the test frequencies are selected as Mid range for each band. The same approach can be adopted in EN-DC.

Proposal 2: It is proposed to test Mid range for both LTE and NR for inter-band EN-DC with 2 E-UTRA CCs and 1 NR CC within FR1 testing.

Test Channel Bandwidth selection

There is no need to test all the combinations of different LTE and NR bandwidths. The approach from LTE inter-band CA testing can be followed that the maximum and minimum aggregated bandwidth are tested for inter-band EN-DC.

Proposal 3: Select lowest NRB_agg and highest NRB_agg transmission bandwidth as the selected channel bandwidths for E-UTRA CG; select Lowest and Highest as the selected channel bandwidths NR CG transmission.

Modulation and RB allocation selections

The purpose of this test case is to verify that the UE can interpret the P-max parameter correctly. Therefore in LTE and NR FR1 the Configured transmitted power is tested only for the combinations of modulation and RB allocation for which no MPR/AMPR applies.

The same principle shall be followed for inter-band EN-DC testing. The MPR and A-MPR requirement for inter-band EN-DC configuration are specified in TS 36.101 and TS 38.101-1 respectively. In order to achieve the minimum MPR/A-MPR value, the test configurations in LTE intra-band contiguous CA and NR FR1 testing shall be selected for inter-band EN-DC with 2 E-UTRA CCs and 1 NR CC testing.

Proposal 4: The test configurations in LTE intra-band contiguous CA and NR FR1 testing shall be selected for inter-band EN-DC with 2 E-UTRA CCs and 1 NR CC testing. For NR CG, only DFT-s-OFDM QPSK modulation is selected to ensure 0 dB MPR/A-MPR for saving test points.

NR and LTE transmission

In TS 38.101-3, requirements vary for multiple scenarios including:

- For non-overlapping transmission, the requirement for transmitted power is specified for each CG.

If the transmissions from NR and E-UTRA do not overlap, then the complete clauses for configured transmitted power for E-UTRA and NR respectively from their own specifications apply with the modifications specified above.

- For overlapping transmission of UE not supporting dynamic power sharing, the requirement for transmitted power is specified for each CG.

If the EN-DC UE does not support dynamic power sharing, then the complete clauses for configured transmitted power for E-UTRA and NR respectively from their own specifications TS 36.101 [4] and TS 38.101-1 [2] respectively apply with the modifications specified above and PTotalEN-DC applies.

- For overlapping transmission of UE supporting dynamic power sharing, the requirement for transmitted power is specified for the total power on all carriers. Scaling or dropping of power on NR carrier might happen depending on parameter configurations.

When a UE supporting dynamic sharing is configured for overlapping E-UTRA uplink and NR uplink transmissions, the UE can set its configured maximum output power PCMAX_ E-UTRA,c and PCMAX_ NR,c for the configured E-UTRA and NR uplink carriers, respectively, and its configured maximum transmission power for EN-DC operation, PTotalEN-DC, as specified above.

The measured total maximum output power PUMAX over both CGs/RATs, measured over the transmission reference time duration is

PUMAX = 10 log10 [pUMAX,c,E-UTRA + pUMAX,c,NR],

where pUMAX,c,E-UTRA and pUMAX,c,NR denotes the measured output power of serving cell c for E-UTRA and NR respectively, expressed in linear scale.

The measured total configured maximum output power PUMAX shall be within the following bounds:

PCMAX_L -TLOW (PCMAX_L)  ≤  PUMAX  ≤  PCMAX_H + THIGH (PCMAX_H)

- For UE supporting dynamic power sharing, if the power of NR carrier is scaled, additional requirement for NR carrier is specified.

When E-UTRA and NR transmissions overlap and the condition (If (a=TRUE) AND (b=FALSE)) is met, SCG shall be transmitted and the following supplementary minimum requirement apply for the measured SCG power, PUMAX,f,c,NR (q), under nominal conditions.

10log(pCMAX L,f,c,,NR c(q)/X_scale)  –  TLOW (10log(pCMAX L,f,c,,NR c(q)/X_scale) )}  ≤  PUMAX,f,c,NR (q) ≤  10log(pCMAX H, f,c,,NR c (q)) + THIGH (10log(pCMAX H, f,c,,NR c (q))).

All of above scenarios need to be considered in the test cases.

Proposal 5: The test case shall cover non-overlapping transmission and overlapping transmission for UE not supporting DPS, non-overlapping transmission and overlapping transmission for UE supporting DPS (including scaling and dropping of NR carrier).

Selection of test points of transmitted power level

The test points of configured output power relate to various power level, or P-max configuration. The selecting of test points shall ensure full test coverage of Pcmax tolerance, or TLOW (PCMAX_L) and THIGH (PCMAX_H). Since the Pcmax tolerance is already partially verified in MPR/A-MPR, only a subset of PCMAX level need to be considered as supplementary in Configured transmitted power.

Test points are selected for each scenario analysed in 2.5

- For non-overlapping transmission, the requirement for transmitted power is specified for LTE CG and NR CG, respectively.

In this scenario the requirements follows SA requirement, therefore test points can follow SA as well. In TS 36.521-1 6.2.5A, P-max = -10dBm/10dBm/15dBm. In TS 38.521-1 6.2.4, P-max = -10dBm/10dBm/15dBm/20dBm, but the Pcmax tolerance corresponding to 20dBm is also covered in MPR, so 20dBm can be skipped here.

As a result, -10dBm/10dBm/15dBm are selected for this scenario.

- For overlapping transmission of UE not supporting dynamic power sharing, the requirement for transmitted power is specified for LTE CG and NR CG, respectively.

In this scenario the requirements on each CG follows SA requirement. The maximum possible transmitted power is already verified in MOP with PLTE=20dBm, and PNR =20dBm. The test points in scenario 1 can be reused. Besides, in order to verify UE enter TDM pattern when PLTE+PNR <= PENDC, E-UTRA CG and NR CG are both configured to 23dBm, so that UE not supporting DPS transmits on E-UTRA subframe and NR slot respectively as per reference TDD configuration.

P-max = -10dBm/10dBm/15dBm/23dBm are selected for this scenario.

- For overlapping transmission of UE supporting dynamic power sharing, the requirement for transmitted power is specified for the total power on all carriers. Scaling or dropping of power on NR carrier might happen depending on parameter configurations.

In this scenario the PCMAX tolerance is specified for total power. Considering in MPR test case only NR carrier is verified, it is proposed to verify all the PCMAX ranges except the maximum output power.

P-max = -10dBm/15dBm/17dBm/20dBm/21dBm/22dBm are selected for this scenario, and PLTE and PNR would be 3dB lower than P-Max.

Table 6.2B.4.1.3-2: PCMAX tolerance for Dual Connectivity E-UTRA-NR

| PCMAX(dBm) | Tolerance TLOW (PCMAX_L) (dB) | Tolerance THIGH (PCMAX_H) (dB) |
| --- | --- | --- |
| 23 ≤ PCMAX ≤ 33 | 3.0 | 2.0 |
| 22 ≤ PCMAX < 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX< 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX < 21 | 6.0 | 4.0 |
| 16 ≤ PCMAX < 20 | 5.0 |  |
| 11 ≤ PCMAX < 16 | 6.0 |  |
| -40 ≤ PCMAX < 11 | 7.0 |  |
| NOTE 1: For UEs not indicating support of dynamic power sharing, the upper tolerance Thigh shall be reduced by 0.3 dB for P ≥ 20 dBm. |  |  |

- For UE supporting dynamic power sharing, if the power of NR carrier is scaled, additional requirement for NR carrier is specified.

In order to enforce UE enter NR scaling scenario, for power class 3 E-UTRA CG is configured to 20dBm, NR CG is configured to 23dBm, so that 20+23> 23dBm and 20+17<23dBm. The condition of NR scaling can be met. Similar for power class 2, E-UTRA CG is configured to 23dBm and NR CG is configured to 26dBm

In order to enforce UE enter NR dropping scenario, E-UTRA CG and NR CG are both configured to 23dBm for power class 3 or 26dBm for power class 2, so that even with NR scaling the Pcmax can’t be less than 23dBm for power class 3 or 26dBm for power class 2.

Number of Test Points for each EN-DC combo

The total test points for Configured transmitted power testing is tabulated for inter-band EN-DC with 2 E-UTRA CCs and 1 NR CC in Table 2.7-1.

Table 2.7-1 Maximum Number of Test Steps for inter-band EN-DC Configured transmitted power

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) | Power settings | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- | --- |
| 5 | 1 | 2 | 1 | -UE not supporting DPS: Non-overlapping transmission: 2 Overlapping transmission: 1 -UE supporting DPS: Non-overlapping transmission: 2 Overlapping transmission: 1 | -UE not supporting DPS: Non-overlapping transmission: 3 Overlapping transmission: 4 -UE supporting DPS: Non-overlapping transmission: 3 Overlapping transmission: 8 | -UE not supporting DPS: 100 -UE supporting DPS: 140 |

Conclusion

In conclusion, the following selection of test points are proposed for inter band EN-DC with 2 E-UTRA CCs and 1 NR CC Configured transmitted power measurement:

Proposal 1: Propose Test Environment as Normal, TL/VL, TL/VH, TH/VL, TH/VH for Configured transmitted power for inter band EN-DC with 2 E-UTRA CCs and 1 NR CC within FR1.

Proposal 2: It is proposed to test Mid range for both LTE and NR for inter-band EN-DC with 2 E-UTRA CCs and 1 NR CC within FR1 testing.

Proposal 3: Select lowest NRB_agg and highest NRB_agg transmission bandwidth as the selected channel bandwidths for E-UTRA CG; select Lowest and Highest as the selected channel bandwidths NR CG transmission.

Proposal 4: The test configurations in LTE intra-band contiguous CA and NR FR1 testing shall be selected for inter-band EN-DC with 2 E-UTRA CCs and 1 NR CC testing. For NR CG, only DFT-s-OFDM QPSK modulation is selected to ensure 0 dB MPR/A-MPR for saving test points.

Proposal 5: The test case shall cover non-overlapping transmission and overlapping transmission for UE not supporting DPS, non-overlapping transmission and overlapping transmission for UE supporting DPS (including scaling and dropping of NR carrier).

References

[1] R5-180065, “Discussion about Test parameters for NR TRx TCs“, NTT DOCOMO, RAN5 Ad hoc, Munich, Jan. 2018.

[2] 3GPP TS 38.101-1 v15.0.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 15)

[3] 3GPP TS 38.521-1 v0.4.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone (Release 15)