3GPP TSG-RAN5 Meeting #95-e	R5-222838

# Online, , 9th May 2022 - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2949 | rev | - | Current version: | 16.11.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 8.1.1.2.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current Prose for 8.1.1.2.4 , Test purpose is to verify Rel-15 UE by sending the extended field in system information that it does not comprehend . for this test purpose SIB11-r16 is sent along with SIB2 as per the table 8.1.1.2.4.3.3-2: System Information. For SS to schedule SIB11-r16, it is also mandatory that SIB1 si_SchedulingInfo schedulingInfoList sib_MappingInfo should contain the SIB11-r16 entry. At present, the Note 1 in Table 8.1.1.2.4.3.2-1 specifies that NR-15 in Table 4.4.3.1.2-1 Combinations of system information blocks of TS 38.508-1 [4] be used, but this does not include SIB1, SIB2 and SIB11. This needs to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the Not 1 to refer to condition NR-18 in Table 4.4.3.1.2-1 Combinations of system information blocks of TS 38.508-1. R5-222835 is raised to introduce this condition in 38.508 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.508-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 8.1.1.2.4	RRC connection establishment / Extended and spare fields in SI

8.1.1.2.4.1	Test Purpose (TP)

(1)

with { UE is powered on and receives system information }

ensure that {

when { UE receives an optional spare or extended field in system information that it does not comprehend }

then { UE treats system information as if the spare or extended field were absent and system information is not ignored, and UE establishes an RRC connection }

}

8.1.1.2.4.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.331, clauses 5.2.2.3.1, 10.1 and 10.3. Unless otherwise stated, these are Rel-15 requirements.

[TS 38.331, clause 5.2.2.3.1]

The UE shall:

1>	apply the specified BCCH configuration defined in 9.1.1.1;

1>	if the UE is in RRC_IDLE or in RRC_INACTIVE; or

1>	if the UE is in RRC_CONNECTED while T311 is running:

2>	acquire the MIB, which is scheduled as specified in TS 38.213 [13];

2>	if the UE is unable to acquire the MIB;

3>	perform the actions as specified in clause 5.2.2.5;

2>	else:

3>	perform the actions specified in clause 5.2.2.4.1.

1>	if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1 and pagingSearchSpace and has received an indication about change of system information; or

1>	if the UE is in RRC_IDLE or in RRC_INACTIVE; or

1>	if the UE is in RRC_CONNECTED while T311 is running:

2>	if ssb-SubcarrierOffset indicates SIB1 is transmitted in the cell (TS 38.213 [13]) and if SIB1 acquisition is required for the UE:

3>	acquire the SIB1, which is scheduled as specified in TS 38.213 [13];

3>	if the UE is unable to acquire the SIB1:

4>	perform the actions as specified in clause 5.2.2.5;

3>	else:

4>	upon acquiring SIB1, perform the actions specified in clause 5.2.2.4.2.

2>	else if SIB1 acquisition is required for the UE and ssb-SubcarrierOffset indicates that SIB1 is not scheduled in the cell:

3>	perform the actions as specified in clause 5.2.2.5.

NOTE:	The UE in RRC_CONNECTED is only required to acquire broadcasted SIB1 if the UE can acquire it without disrupting unicast data reception, i.e. the broadcast and unicast beams are quasi co-located.

[TS 38.331, clause 10.1]

The generic error handling defined in the subsequent clauses applies unless explicitly specified otherwise e.g. within the procedure specific error handling.

The UE shall consider a value as not comprehended when it is set:

-	to an extended value that is not defined in the version of the transfer syntax supported by the UE;

-	to a spare or reserved value unless the specification defines specific behaviour that the UE shall apply upon receiving the concerned spare/reserved value.

The UE shall consider a field as not comprehended when it is defined:

-	as spare or reserved unless the specification defines specific behaviour that the UE shall apply upon receiving the concerned spare/reserved field.

[TS 38.331 clause 10.3]

The UE shall, when receiving an RRC message on any logical channel:

1>	if the message includes a field that has a value that the UE does not comprehend:

2>	if a default value is defined for this field:

3>	treat the message while using the default value defined for this field;

2>	else if the concerned field is optional:

3>	treat the message as if the field were absent and in accordance with the need code for absence of the concerned field;

2>	else:

3>	treat the message as if the field were absent and in accordance with sub-clause 10.4.

8.1.1.2.4.3	Test description

8.1.1.2.4.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1.

UE:

None.

Preamble:

-	The UE is switched off

8.1.1.2.4.3.2	Test procedure sequence

Table 8.1.1.2.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | System information that includes extended and spare fields that the UE does not comprehend is broadcasted on BCCH. (Note 1) | <-- | RRC: SYSTEM INFORMATION (BCCH) | - | - |
| 2 | The UE is switched on. | - | - | - | - |
| 3 | Does the UE transmit an RRCSetupRequest message on NR Cell 1? | --> | RRCSetupRequest | 1 | P |
| 4 | The SS transmits an RRCSetup message on Cell 1. | <-- | RRCSetup | - | - |
| 5 | Check: Does the UE transmit an RRCSetupComplete message to confirm the successful completion of the connection establishment and initiates the registration procedure by including the REGISTRATION REQUEST message? | --> | RRCSetupComplete 5GMM: REGISTRATION REQUEST | 1 | P |
| 6-21a1 | Steps 5 to 20a1 of the registration procedure described in TS 38.508 Table 4.5.2.2-2 are performed on NR Cell 1. | - | - | - | - |
| Note 1 The NR-18 in Table 4.4.3.1.2-1 Combinations of system information blocks of TS 38.508-1 [4] is used |  |  |  |  |  |

8.1.1.2.4.3.3	Specific message contents

Table 8.1.1.2.4.3.3-1: MIB (step 1, Table 8.1.1.2.4.3.2-1)

| Derivation Path: TS 38.508-1 Table 4.6.1-6 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| MIB ::= SEQUENCE { |  |  |  |  |
| spare | ‘1’B | In Rel-15, the lowest bit is considered a spare according to TS 38.331 [12]. | For Rel-15 |
| } |  |  |  |

Table 8.1.1.2.4.3.3-2: SystemInformation (step 1, Table 8.1.1.2.4.3.2-1)

| Derivation Path: TS 38.508-1, Table 4.6.1-29 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SystemInformation ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| systemInformation SEQUENCE { |  |  |  |  |
| sib-TypeAndInfo SEQUENCE (SIZE (1..maxSIB)) OF CHOICE { |  |  |  |  |
| SIB2 |  |  |  |  |
| SIB11-r16 |  |  | In Rel-15, SIB11 is considered an extended field according to TS 38.331 [12]. |  |
| } |  |  |  |  |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.1.1.2.4.3.3-3: SIB11 (Table 8.1.1.2.4.3.3-2)

| Derivation Path: TS 38.508-1, Table 4.6.2-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB11-r16 ::= SEQUENCE { |  |  |  |  |
| measIdleConfigSIB-r16 |  | Not Present |  |  |
| lateNonCriticalExtension |  | Not Present |  |  |
| } |  |  |  |  |