3GPP TSG-RAN5 Meeting #95-e	R5-222844

# Electronic Meeting, 09th – 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2357 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add test procedures for MBS Multicast test |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS_5MBS_5MBUSA-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1, There is no common test procedures for UE to join in MBS Multicast Session. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, Add test procedures for MBS Multicast test. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No common test procedures for UE to join in MBS Multicast Session. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

4.9.X	Test procedure for MBS Multicast session join and session establishment

4.9.X.1	Scope

The purpose of this procedure is to establish a multicast MBS session.

4.9.X.2	Procedure description

4.9.X.2.1	Initial conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

User Equipment:

-	The UE is in state 1N-A.

4.9.X.2.2	Procedure sequence

Table 4.9.X.2.2-1: Procedure for MBS Multicast session join and session establishment

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| - | EXCEPTION: Step 1a1 to 1b12a1 describe behaviour that depends on the UE capability the “lower case letter” identifies a step sequence that take place. | - | - | - | - |
| 1a1 | IF pc_Join_MBS_by_PDU_Modification THEN cause the UE to request establishment of associated PDU Session to the MBS DNN.(Note 1) | - | - | - | - |
| 1a2-1a7 | Steps 2-7 of expected sequence from Table 4.5.4.2-3 are performed. | - | - | - | - |
| - | EXCEPTION: Depending upon UE implementation, steps 1a8 and 1a9 can occur in any order | - | - | - | - |
| 1a8 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 1a9 | The UE transmits an UL NAS TRANSPORT message and a PDU SESSION ESTABLISHMENT REQUEST without MBS session ID and join request. | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT REQUEST | - | P |
| 1a10 | The SS transmits an RRCReconfiguration message and an PDU SESSION ESTABLISHMENT ACCEPT | <-- | NR RRC: RRCReconfiguration 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT ACCEPT | - | - |
| 1a11 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| - | EXCEPTION: Step 1a12a1 describes behaviour depending UE implementation; the "lower case letter" identifies a step sequence that take place if the UE performs a specific action. | - | - | - |  |
| 1a12a1 | IF the 'IP address allocation' for the DNN for which the PDU session is established is set to "Yes" in Table 4.8.4-1 THEN, the generic procedure for IP address allocation in the user plane, specified in subclause 4.5A.3, takes place performing IP address allocation in the user plane. | - | - | - | - |
| 1a13 | Trigger UE to join MBS Multicast session. | - | - | - | - |
| 1a14 | The UE transmits an ULInformationTransfer message and a PDU SESSION MODIFICATION REQUEST message including MBS session ID and join request. | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION REQUEST | - | P |
| 1a15 | The SS transmits an RRCReconfiguration message and a PDU SESSION MODIFICATION COMMAND. | <-- | NR RRC: RRCReconfiguration 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMMAND | - | - |
| - | EXCEPTION: Depending upon UE implementation, steps 1a16 and 1a17can occur in any order. | - | - | - | - |
| 1a16 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 1a17 | The UE transmits an ULInformationTransfer message and a PDU SESSION MODIFICATION COMPLETE message. | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMPLETE | - | P |
| 1b1 | ELSE trigger UE to join MBS Multicast session. | - | - | - | - |
| 1b2-1b7 | Steps 2-7 of expected sequence from Table 4.5.4.2-3 are performed. | - | - | - | - |
| - | EXCEPTION: Depending upon UE implementation, steps 1b8 and 1b9 can occur in any order | - | - | - | - |
| 1b8 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 1b9 | The UE transmits an UL NAS TRANSPORT message and a PDU SESSION ESTABLISHMENT REQUEST including MBS session ID and join request. | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT REQUEST | - | P |
| 1b10 | The SS transmits an RRCReconfiguration message and a PDU SESSION ESTABLISHMENT ACCEPT | <-- | NR RRC: RRCReconfiguration 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT ACCEPT | - | - |
| 1b11 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| - | EXCEPTION: Step 1b12a1 describes behaviour depending UE implementation; the "lower case letter" identifies a step sequence that take place if the UE performs a specific action. | - | - | - | - |
| 1b12a1 | IF the 'IP address allocation' for the DNN for which the PDU session is established is set to "Yes" in Table 4.8.4-1 THEN, the generic procedure for IP address allocation in the user plane, specified in subclause 4.5A.3, takes place performing IP address allocation in the user plane. | - | - | - | - |
| Note 1: This could be done by e.g. MMI or AT command. |  |  |  |  |  |

4.9.X.3	Specific message contents

Table 4.9.X.3-1: PDU SESSION ESTABLISHMENT REQUEST (step 1a9, Table 4.9.X.2.2-1)

| Derivation Path: TS 38.508-1 [21], Table 4.7.2-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Requested MBS container |  | Not present |  |  |

Table 4.9.X.3-2: PDU SESSION MODIFICATION REQUEST (step 1a14, Table 4.9.X.2.2-1)

| Derivation Path: TS 38.508-1 [21], Table 4.7.2-7. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Requested MBS container |  |  |  |  |
| MBS session information |  |  |  |  |
| MBS operation |  | ‘01’B | Join MBS session |  |
| Type of MBS session ID |  | ‘01’B | Temporary Mobile Group Identity (TMGI) |  |
| MBS session ID |  |  |  |  |
| MBMS Service ID |  | ‘000001’B |  |  |
| MCC |  | See table 4.4.2-3 |  |  |
| MNC |  | See table 4.4.2-3 |  |  |

Table 4.9.X.3-3: PDU SESSION MODIFICATION COMMAND (step 1a15, Table 4.9.X.2.2-1)

Derivation Path: TS 38.508-1 [21], Table 4.7.2-9 with condition MBS_Multicast.

Table 4.9.X.3-4: PDU SESSION ESTABLISHMENT REQUEST (step 1b9, Table 4.9.X.2.2-1)

| Derivation Path: TS 38.508-1 [21], Table 4.7.2-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Requested MBS container |  |  |  |  |
| MBS session information |  |  |  |  |
| MBS operation |  | ‘01’B | Join MBS session |  |
| Type of MBS session ID |  | ‘01’B | Temporary Mobile Group Identity (TMGI) |  |
| MBS session ID |  |  |  |  |
| MBMS Service ID |  | ‘000001’B |  |  |
| MCC |  | See table 4.4.2-3 |  |  |
| MNC |  | See table 4.4.2-3 |  |  |

Table 4.9.X.3-5: PDU SESSION ESTABLISHMENT ACCEPT (step 1b10, Table 4.9.X.2.2-1)

Derivation Path: TS 38.508-1 [21], Table 4.7.2-2 with condition MBS_Multicast.

<End of modified section 1>