3GPP TSG-RAN5 Meeting #95-e	R5-222860

# Online, ,9th May 2022 - 20th May 2022

| Text Proposal |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38.918 | CR | n/a | rev | - | Current version: | 0.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to Test Configurations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_Slice_Test |  |  |  |  |  | Date: |  |  | 2022-04-24 |
| Category: | n/a |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The test configurations for network slicing is incomplete |  |  |  |  |  |  |  |  |
| Summary of change: |  | - 1.Clarify the function of test equipment in 5.2.1 - 2.Add the different type of Traffic Descriptor of second application in 5.2.3 and 5.2.5 - 3.Remove Editor’s Note in 5.2.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TR 38.918 will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1, 5.2.3, 5.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

5.2	Test Configurations

5.2.1	UE Full Stack Network Slicing Test Equipment

The test equipment utilized for 5G NR UE Full Stack Testing for Network Slicing should consist of the following items:

- System Simulator(s) is used to simulate the 5G network for network slicing testing. System simulator(s) could receive network slice configuration scheme from main controller and send network slicing related configuration to UE via signalling. System simulator(s) could receive application data from UE and send application data to application server simulator(s).

- Application Server Simulator(s) is used to simulate the service in application layer for receiving application data from UE via system simulator(s) and handling the received application data. Application Server Simulator(s) could be either implemented in Main Controller or implemented as an independent unit.

- Main Controller is used to get and configure the  type of traffic descriptor to be tested to Application Client Simulator, generate network slice configuration scheme and configure the network slice configuration scheme to System Simulator(s).

- Application Client Simulator(s) suitable for generating Traffic Descriptor of application(s) and corresponding application data on UE side.

.......

<<SKIP UNCHANGED PART >>>

.......

5.2.3	Network Slicing Configurations

Unless otherwise stated, following application information and corresponding S-NSSAI will be used as default network slicing configurations.

Table 5.2.3-1: Network Slicing Configurations

| Application | OS App id | DNN | FQDN | IP 3 Tuples | S-NSSAI |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | SST | SD |
| APP A | pc_OS_App_ID | pc_APN_ID_Specific | pc_Des_FQDN | pc_IP_Address, pc_Protocol_ID, pc_Sinlge_Remote_Port | ‘00000001'B | 0x000001 |
| APP B | pc_OS_App_ID_2nd | pc_APN_ID_Specific_2nd | Void | Void | ‘00000010'B | 0x000001 |

.......

<<SKIP UNCHANGED PART >>>

.......

5.2.5	UE Specific Items

The following parameters are recommended to be provided by UE or application supplier to indicate the information of application using network slicing during test.

| Parameter Name | Parameter Type | Description |
| --- | --- | --- |
| pc_APN_ID_Specific | charstring | APN/DNN value of the simulation application client defined in clause 5.3.1 The APN/DNN Network Identifier portion of the Access Point / Data Network Name, as defined in TS 23.003 [11], subclause 9.1 |
| pc_OS_App_ID | charstring | OS App Id value of the simulation application client defined in clause 5.3.1 The coding of OS App Id is up to UE implementation |
| pc_Des_FQDN | charstring | The destination FQDN value of Application Server Simulator(s) defined in clause 5.2.1 The destination FQDN as defined in subclause 28.3.2.1 in 3GPP TS 23.003 [11] |
| pc_IP_Address | charstring | The IP address of Application Server Simulator(s) defined in clause 5.2.1 The coding of IP address is four octet IPv4 address and 4 octet IPv4 address mask for IPv4 or sixteen octet IPv6 address and one octet prefix length for IPv6 |
| pc_Protocol_ID | charstring | The IPv4 protocol identifier or IPv6 next header of Application Server Simulator(s) defined in clause 5.2.1 |
| pc_Sinlge_Remote_Port | charstring | The port number of Application Server Simulator(s) defined in clause 5.2.1 |
| pc_APN_ID_Specific_2nd | charstring | APN/DNN value of the second application simulated by simulation application client in the scenario of two applications running in parallel The APN/DNN Network Identifier portion of the Access Point / Data Network Name, as defined in TS 23.003 [11], subclause 9.1 |
| pc_OS_App_ID_2nd | charstring | OS App Id value of the second application simulated by simulation application client in the scenario of two applications running in parallel The coding of OS App Id is up to UE implementation |

<<< END OF CHANGES >>>