3GPP TSG-RAN5 Meeting #95-e	R5-222877

# Electronic Meeting, May 9 -20, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0329 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Limiting MBR relaxation reporting to Rel-15 only |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Bureau Veritas |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The UE is allowed to declare the per-band MBR relaxation only in Rel-15. For Rel-16 and forward, the MPR relaxation is fixed per band. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding limitation to the MBR relaxation declaration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PICS might be misused by UEs Rel-16 and forward. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

A.4.3.9	Additional capabilities for UE declared capability

Table A.4.3.9-1: UE declared capabilities

| Item | UE declared capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Enhanced Type 1 Receiver for NR | 38.101-4, 5 | Rel-15 | pc_nr_enh_type1_receiver | Support for Enhanced Type 1 Receiver (SU-MIMO Interference Mitigation advanced receiver) |
| 2 | Vehicular UE | 38.101-1, 3 | Rel-15 | pc_nr_vehicular_ue |  |

Table A.4.3.9-2: UE declared multi-band peak EIRP relaxation factors for Rel-15 FR2 power class 3 UE

| Item |  | Supported FR2 bands set |  | Ref. |  | Release |  | peak EIRP relaxation factor per band, MBp (dB) (Note 1) |  |  |  |  |  |  |  | Maximum sum of MBp, ∑MBP (dB) (Note 2) |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | n257 |  | n258 |  | n260 |  | n261 |  |  |  |  |  |
| 1 |  | n257, n258 |  | 38.101-2, 6.2.1.3 |  | Rel-15 |  |  |  |  |  | N/A |  | N/A |  | 1.3 |  |  |  |
| 2 |  | n257, n260 |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  | 1.0 |  |  |  |
| 3 |  | n258, n260 |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  | 1.0 |  |  |  |
| 4 |  | n258, n261 |  |  |  |  |  | N/A |  |  |  | N/A |  |  |  | 1.0 |  |  |  |
| 5 |  | n260, n261 |  |  |  |  |  | N/A |  | N/A |  | N/A |  | N/A |  | 0.0 |  | No relaxation factor allowed |  |
| 6 |  | n257, n258, n260 |  |  |  |  |  |  |  |  |  |  |  | N/A |  | 1.7 |  |  |  |
| 7 |  | n257, n258, n261 |  |  |  |  |  |  |  |  |  | N/A |  |  |  | 1.7 |  |  |  |
| 8 |  | n257, n260, n261 |  |  |  |  |  |  |  | N/A |  |  |  |  |  | 0.5 |  |  |  |
| 9 |  | n258, n260, n261 |  |  |  |  |  | N/A |  |  |  |  |  |  |  | 1.5 |  |  |  |
| 10 |  | n257, n258, n260, n261 |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.7 |  |  |  |
| 11 |  | n257, n261 |  |  |  |  |  |  |  | N/A |  | N/A |  |  |  | 0.0 |  | No relaxation factor allowed |  |
| Note 1: UE vendor to fill in the needed relaxation factor per band that is ≥0 for Rel-15 UE supporting only Rel-15 FR2 bands. One row to be filled in, the one matching the supported FR2 bands of the UE as declared in Table A.4.3.1-3. Note 2: Max allowed sum of MBp over all supported FR2 bands as defined in TS 38.521-2 clause 6.2.1.1.3.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.4.3.9-3: UE declared multi-band peak EIRP Spherical coverage relaxation factors for Rel-15 FR2 power class 3 UE

| Item |  | Supported FR2 bands set |  | Ref. |  | Release |  | EIRP Spherical coverage relaxation factor per band, MBs (dB) (Note 1) |  |  |  |  |  |  |  | Maximum sum of MBs, ∑MBs (dB) (Note 2) |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | n257 |  | n258 |  | n260 |  | n261 |  |  |  |  |  |
| 1 |  | n257, n258 |  | 38.101-2, 6.2.1.3 |  | Rel-15 |  |  |  |  |  | N/A |  | N/A |  | 1.25 |  |  |  |
| 2 |  | n257, n260 |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  | 0.75 |  | Maximum 0.4 dB relaxation allowed for n260 |  |
| 3 |  | n258, n260 |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  | 0.75 |  | Maximum 0.4 dB relaxation allowed for n260 |  |
| 4 |  | n258, n261 |  |  |  |  |  | N/A |  |  |  | N/A |  |  |  | 1.25 |  |  |  |
| 5 |  | n260, n261 |  |  |  |  |  | N/A |  | N/A |  |  |  |  |  | 0.75 |  | No relaxation allowed for n260 |  |
| 6 |  | n257, n258, n260 |  |  |  |  |  |  |  |  |  |  |  | N/A |  | 1.75 |  | Maximum 0.4 dB relaxation allowed for n260 |  |
| 7 |  | n257, n258, n261 |  |  |  |  |  |  |  |  |  | N/A |  |  |  | 1.75 |  |  |  |
| 8 |  | n257, n260, n261 |  |  |  |  |  |  |  | N/A |  |  |  |  |  | 1.25 |  | Maximum 0.4 dB relaxation allowed for n260 |  |
| 9 |  | n258, n260, n261 |  |  |  |  |  | N/A |  |  |  |  |  |  |  | 1.25 |  | Maximum 0.4 dB relaxation allowed for n260 |  |
| 10 |  | n257, n258, n260, n261 |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.75 |  | Maximum 0.4 dB relaxation allowed for n260 |  |
| 11 |  | n257, n261 |  |  |  |  |  |  |  | N/A |  | N/A |  |  |  | 0.0 |  | No relaxation factor allowed |  |
| Note 1: UE vendor to fill in the needed relaxation factor per band that is ≥0 for Rel-15 UE supporting only Rel-15 FR2 bands. One row to be filled in, the one matching the supported FR2 bands of the UE as declared in Table A.4.3.1-3 Note 2: Max allowed sum of MBs over all supported FR2 bands as defined in TS 38.521-2 clause 6.2.1.1.3.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.4.3.9-4a: FDD 4 Rx antenna ports Capabilities

| Item |  | Band | Ref. | Release | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 |  | FDD Band n1 | 38.101-1, 7.3.2 | Rel-15 |  |
| 2 |  | FDD Band n2 | 38.101-1, 7.3.2 | Rel-15 |  |
| 3 |  | FDD Band n3 | 38.101-1, 7.3.2 | Rel-15 |  |
| … |  |  |  |  |  |
| 7 |  | FDD Band n7 | 38.101-1, 7.3.2 | Rel-15 | NOTE 2 |
| ... |  |  |  |  |  |
| 28 |  | FDD Band n28 | 38.101-1, 7.3.2 | Rel-16 | 4 Rx operation is targeted for FWA form factor |
| ... |  |  |  |  |  |
| 30 |  | FDD Band n30 | 38.101-1, 7.3.2 | Rel-16 |  |
| ... |  |  |  |  |  |
| 66 |  | FDD Band n66 | 38.101-1, 7.3.2 | Rel-15 |  |
| … |  |  |  |  |  |
| 70 |  | FDD Band n70 | 38.101-1, 7.3.2 | Rel-15 |  |
| 71 |  | FDD Band n71 | 38.101-1, 7.3.2 | Rel-16 | 4 Rx operation is targeted for FWA form factor |
| NOTE 1: At least one band from those listed in the present table needs to be supported if UE has indicated support of the capability defined in Table A.4.3.1-7a/2. NOTE 2: Support of 4 Rx for this band is mandatory for non-vehicular UEs i.e. if support has NOT been indicated to the capability specified in Table A.4.3.9-1/2. |  |  |  |  |  |

Table A.4.3.9-4b: TDD 4 Rx antenna ports Capabilities

| Item |  | Band | Ref. | Release | Comments |
| --- | --- | --- | --- | --- | --- |
| 34 |  | TDD Band n34 | 38.101-1, 7.3.2 | Rel-15 |  |
| ... |  |  |  |  |  |
| 38 |  | TDD Band n38 | 38.101-1, 7.3.2 | Rel-15 | NOTE 2 |
| 39 |  | TDD Band n39 | 38.101-1, 7.3.2 | Rel-15 |  |
| ... |  |  |  |  |  |
| 40 |  | TDD Band n40 | 38.101-1, 7.3.2 | Rel-15 |  |
| 41 |  | TDD Band n41 | 38.101-1, 7.3.2 | Rel-15 | NOTE 2 |
| ... |  |  |  |  |  |
| 48 |  | TDD Band n48 | 38.101-1, 7.3.2 | Rel-16 | NOTE 2 |
| … |  |  |  |  |  |
| 77 |  | TDD Band n77 | 38.101-1, 7.3.2 | Rel-15 | NOTE 2 |
| 78 |  | TDD Band n78 | 38.101-1, 7.3.2 | Rel-15 | NOTE 2 |
| 79 |  | TDD Band n79 | 38.101-1, 7.3.2 | Rel-15 | NOTE 2 |
| NOTE 1: At least one band from those listed in the present table needs to be supported if UE has indicated support of the capability defined in Table A.4.3.1-7a/3. NOTE 2: Support of 4 Rx for this band is mandatory for non-vehicular UEs i.e. if support has NOT been indicated to the capability specified in Table A.4.3.9-1/2. |  |  |  |  |  |

<End of Changes >