3GPP TSG-RAN5 Meeting #95-e	R5-222986

# Online, , 9th May 2022 - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-3 | CR | 2631 | rev | - | Current version: | 16.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of feMob test case 5.1.58 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_feMob-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Message contents are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added message contents. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.58 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

5.1.58	 E-UTRAN TDD - FDD inter-band inter-frequency async DAPS handover

5.1.58.1	Test purpose

To verify the UE’s ability to perform E-UTRAN TDD - FDD inter-band inter-frequency async DAPS handover in RRC_CONNECTED state when an E-UTRAN TDD - FDD inter-band inter-frequency async DAPS handover is commanded by meeting the UE maximum RRC procedure delay and interruption time requirements.

5.1.58.2	Test applicability

This test applies to all types of E-UTRA UE supporting FDD and TDD UE release 16 and forward. Applicability requires support for Inter-frequency async DAPS handover and Feature Group Indicators 5, 25 and 30.

5.1.58.3	Minimum conformance requirements

When the UE receive a RRC message implying E-UTRAN FDD – FDD Intra-band Inter-frequency async DAPS handover the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover1 seconds from the end of the last TTI containing the RRC command.

Dhandover1 = TRRC_procedure + TIU + 20 ms

Where:

RRC procedure delay = 15 ms and is specified in clause 11.2 in TS 36.331 [2].

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to 30 ms.

During Dhandover1 the UE shall be continuously scheduled on Cell 1 and the interruption on Cell 1 shall not exceed 2ms (Tinterrupt1).

After successful RACH procedure of the target cell, when the UE receives an RRC message implying source cell release command, the UE shall accomplish the release actions specified in TS 38.331 [2] within Dhandover2.

Dhandover2 = TRRC_procedure+ Tinterrupt2

Where:

TRRC_procedure is the maximum RRC procedure delay to be defined in clause 11.2 in TS 36.331 [2].

Tinterrupt2 is defined in clause 5.7.2.1.2.

During Dhandover2 the UE is allowed an interruption of up to Tinterrupt2 on target cell.

The normative reference for this requirement is TS 36.133 [4] clause 5.7.2.1and A.5.1.58.

5.1.58.4	Test description

5.1.58.4.1	Initial conditions

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: According to Annex E table E-1 and TS 36.508 [7] clauses 4.4.2 and 4.3.1.

Channel Bandwidth to be tested: 10 MHz as defined in TS 36.508 [7] clause 4.3.1.

1.	Connect the SS (node B emulator) and AWGN noise source to the UE antenna connectors as shown in TS 36.508 [7] Annex A figure A.20 for UE with 2Rx RF band and Annex A, Figure A.78a for 4Rx capable UE without any 2Rx RF bands.

2.	The general test parameter settings are set up according to Table 5.1.58.4.1-1.

3.	Propagation conditions are set according to Annex B clause B.0.

4.	Message contents are defined in clause 5.1.58.4.3.

5.	There are two E-UTRA carriers and one E-UTRA TDD Cell 1 and one E-UTRA FDD Cell 2 on each carrier specified in the test. Cell 1 is the cell used for connection setup with the power level set according to Annex C.0 and C.1 for this test.

Table 5.1.58.4.1-1: General test parameters for E-UTRAN TDD - FDD inter-band inter-frequency async DAPS handover test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
|  | Neighbouring cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| Final condition | Active cell |  | Cell 2 |  |
| E-UTRA RF channel number |  |  | 1, 2 | FDD and TDD carriers on the different band are used |
| Channel Bandwidth (BWchannel) |  | MHz | 10 |  |
| A3-Offset |  | dB | -4 |  |
| Hysteresis |  | dB | 0 |  |
| TimeToTrigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  | DRX_L | As specified in clause A.3.3 |
| PRACH configuration |  |  | 4 | As specified in table 5.7.1-2 in TS 36.211 |
| Access Barring Information |  | - | Not sent | No additional delays in random access procedure |
| Time offset between cells |  |  | 3ms | asynchronous cells |
| Gap pattern configuration Id |  |  | 0 | As specified in Table 8.1.2.1-1 started before T2 starts |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |
| T4 |  | ms | 100 |  |
| T5 |  | ms | 100 |  |

5.1.58.4.2	Test procedure

The test consists of one active cell and one neighbour cell. PDCCHs indicating new transmissions shall be sent continuously to ensure that the UE would not enter the DRX state. The test consists of five successive time periods, with time durations of T1, T2, T3, T4 and T5 respectively.

1.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 7.2A.3. Cell 1 is the active cell.

2.	Set the parameters according to T1 in Table 5.1.42.5-1. Propagation conditions are set according to Annex B clause B.1.1. T1 starts.

3.	SS shall transmit an RRCConnectionReconfiguration message.

4.	The UE shall transmit RRCConnectionReconfigurationComplete message.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 5.1.42.5-1.

6.	UE shall transmit a MeasurementReport message triggered by Event A3.

7.	SS shall transmit an RRCConnectionReconfiguration message defined in Table 5.1.42.4.3-6 implying DAPS handover to Cell 2.

8.	The start of T3 is the instant when the last TTI containing the RRC connection reconfiguration message implying handover is sent to the UE, at that instant the SS shall switch the power settings from T2 to T3 as specified in Table 5.1.42.5-1.

9.	The UE shall transmit RRCConnectionReconfigurationComplete message.

10. If the UE transmits the uplink PRACH channel to Cell 2 less than 50 ms from the beginning of time period T3, continue with step 11. Otherwise, the number of failure tests is increased by one, skip to step 18.

11.	SS shall transmit an RRCConnectionReconfiguration message defined in Table 5.1.42.4.3-12 implying source cell release.

12. The start of T4 is the instant when the last TTI containing the RRC message implying source cell release is sent to the UE. During T4, the UE shall perform source cell release.

13. The UE shall transmit RRCConnectionReconfigurationComplete message.

14.	If the UE transmits the RRCConnectionReconfigurationComplete message to Cell 2 less than 21ms from the beginning of time period T4 continues with step 15. Otherwise, the number of failure tests is increased by one and skip to step 17.

15. Starting T5 and the UE shall stops to send CSI report to the source cell.

16. If the UE stops to send CSI report to the source cell, the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

17.	Cause UE handover back to Cell 1 (if the DAPS handover fails, switch off the UE) or switch off the UE. Then ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 7.2A.3. Cell 1 is the active cell.

18.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

19.	Repeat step 2-18 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

5.1.58.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6 with the following exceptions:

Table 5.1.58.4.3-1: Common Exception messages for E-UTRAN TDD - FDD inter-band inter-frequency async DAPS handover test requirement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-3 Table H.3.1-7 Table H.3.6-2 |

Table 5.1.58.4.3-2: ReportConfigEUTRA-A3: Additional E-UTRAN TDD - FDD inter-band inter-frequency async DAPS handover test requirement

| Derivation Path: TS 36.508 [7] clause 4.6.6, Table 4.6.6-6 ReportConfigEUTRA-A3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigEUTRA-A3 ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA3 SEQUENCE { |  |  |  |
| a3-Offset | -8 (-4 dB) | -4 is actual value in dB (-8 * 0.5 dB) |  |
| reportOnLeave | FALSE |  |  |
| } |  |  |  |
| } |  |  |  |
| Hysteresis | 0 (0 dB) | 0 is actual value in dB (0 * 0.5 dB) |  |
| timeToTrigger | 0 (0 ms) |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.1.58.4.3-3: MeasResults: Additional E-UTRAN TDD - FDD inter-band inter-frequency async DAPS handover test requirement

| Derivation Path: 36.331 clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResults ::= SEQUENCE { |  |  |  |
| measId | 1 | Identifies the measurement id for the reporting being performed |  |
| measResultServCell SEQUENCE { |  |  |  |
| rsrpResult |  | Set according to specific test |  |
| rsrqResult |  | Set according to specific test |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA | MeasResultListEUTRA |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.1.58.4.3-4: MeasResultListEUTRA: Additional E-UTRAN TDD - FDD inter-band inter-frequency async DAPS handover test requirement

| Derivation Path: 36.331 clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResultListEUTRA::= SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { |  |  |  |
| physCellId | PhysCellId |  |  |
| measResult SEQUENCE { |  |  |  |
| rsrpResult |  | Set according to specific test |  |
| rsrqResult |  | Set according to specific test |  |
| } |  |  |  |
| } |  |  |  |

Table 5.1.58.4.3-5: RACH-ConfigCommon-DEFAULT: Additional E-UTRAN TDD - FDD inter-band inter-frequency async DAPS handover test requirement

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-12 RACH-ConfigCommon-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon-DEFAULT ::= SEQUENCE { |  |  |  |
| powerRampingParameters SEQUENCE { |  |  |  |
| preambleInitialReceivedTargetPower | dBm-90 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.1.58.4.3-6: RRCConnectionReconfiguration: Additional E-UTRAN TDD - FDD inter-band inter-frequency async DAPS handover test requirement

| Derivation Path: 36.508, Table 4.6.1-8, condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| measConfig | Not present |  |  |
| mobilityControlInfo | MobilityControlInfo |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.1.58.4.3-7: MobilityControlInfo: Additional E-UTRAN TDD - FDD inter-band inter-frequency async DAPS handover test requirement

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 2 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 2 |  |  |
| } |  |  |  |
| radioResourceConfigCommon | RadioResourceConfigCommon-DEFAULT |  |  |
| rach-ConfigDedicated | Rach-ConfigDedicated-DEFAULT |  |  |
| } |  |  |  |

Table 5.1.58.4.3-8: RadioResourceConfigCommon-DEFAULT: Additional E-UTRAN TDD - FDD inter-band inter-frequency async DAPS handover test requirement

| Derivation Path: 36.508, Table 4.6.3-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommon-DEFAULT ::= SEQUENCE { |  |  |  |
| rach-ConfigCommon | RACH-ConfigCommon-DEFAULT |  |  |
| } |  |  |  |

Table 5.1.58.4.3-9:  RACH-ConfigCommon-DEFAULT: Additional E-UTRAN TDD - FDD inter-band inter-frequency async DAPS handover test requirement

| Derivation Path: 36.508, Table 4.6.3-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon-DEFAULT ::= SEQUENCE { |  |  |  |
| ra-SupervisionInfo SEQUENCE { |  |  |  |
| preambleTransMax | n200 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.1.58.4.3-10: RadioResourceConfigDedicated: Additional E-UTRAN TDD - FDD inter-band inter-frequency async DAPS handover test requirement

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated::= SEQUENCE { |  |  |  |  |
| srb-ToAddModList |  | Not present |  |  |
| drb-ToAddModList SEQUENCE { |  | 1 entries |  |  |
| drb-ToAddMod |  | DRB-ToAddMod-DEFAULT |  |  |
| } |  |  |  |  |
| drb-ToReleaseList |  | Not present |  |  |
| mac-MainConfig |  | Not present |  |  |
| mac-MainConfig |  | Not Present |  |  |
| sps-Config |  | Not present |  |  |
| physicalConfigDedicated |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 5.1.58.4.3-11: DRB-ToAddMod-DEFAULT: Additional E-UTRAN TDD - FDD inter-band inter-frequency async DAPS handover test requirement

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DRB-ToAddMod-DEFAULT(bid) ::= SEQUENCE { |  |  | bid is the bearer identity (1..8) |  |
| eps-BearerIdentity |  | Not Present |  |  |
| drb-Identity |  | 1 |  |  |
| pdcp-Config |  | Not Present |  |  |
| rlc-Config |  | Not Present |  |  |
| logicalChannelIdentity |  | Not Present |  |  |
| logicalChannelConfig |  | Not Present |  |  |
| daps-HO-r16 |  | TRUE |  |  |
| } |  |  |  |  |

Table 5.1.58.4.3-12: RRCConnectionReconfiguration: Additional E-UTRAN TDD - FDD inter-band inter-frequency async DAPS handover test requirement

| Derivation Path: 36.508, Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| measConfig | Not present |  |  |
| mobilityControlInfo | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| daps-SourceRelease-r16 | TRUE |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

5.1.58.5	Test requirement

Tables 5.1.58.4.1-1 and 5.1.58.5-1 define the primary level settings including test tolerances for E-UTRAN TDD - FDD inter-band inter-frequency async DAPS handover test case.

Table 5.1.58.5-1: Cell specific test parameters for E-UTRAN FDD – TDD Inter-band Inter-frequency async DAPS handover test case

| Parameter | Unit | Cell 1 |  |  |  |  | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| E-UTRA RF Channel number |  | 1 |  |  |  |  | 2 |  |  |  |  |
| BWchannel | MHz | 10 |  |  |  |  | 10 |  |  |  |  |
| Special subframe configuration |  | NA |  |  |  |  | 6 |  |  |  |  |
| Uplink-downlink configuration |  | NA |  |  |  |  | 1 |  |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) A.3.2.1.2 (OP.2 FDD), A.3.2.2.1 (OP.1 TDD) and A.3.2.2.2 (OP.2 TDD) |  | OP.1 FDD | OP.1 FDD | OP.1 FDD | OP.1 FDD | OP.2 FDD | OP.2 TDD | OP.2 TDD | OP.1 TDD | OP.1 TDD | OP.1 TDD |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 FDD | DL Reference Measurement Channel R.0 FDD | DL Reference Measurement Channel R.0 FDD | DL Reference Measurement Channel R.0 FDD | NA | NA | NA | DL Reference Measurement Channel R.0 TDD | DL Reference Measurement Channel R.0 TDD | DL Reference Measurement Channel R.0 TDD |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 FDD | DL Reference Measurement Channel R.6 FDD | DL Reference Measurement Channel R.6 FDD | DL Reference Measurement Channel R.6 FDD | NA | NA | NA | DL Reference Measurement Channel R.6 TDD | DL Reference Measurement Channel R.6 TDD | DL Reference Measurement Channel R.6 TDD |
| PBCH_RA | dB | 0 |  |  |  |  | 0 |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |
|  | dB | 4 | 4 | 4 | 4 | 4 | -Infinity | 7 | 7 | 7 | 7 |
| Note 2 | dBm/15 kHz | -98 |  |  |  |  |  |  |  |  |  |
|  | dB | 4 | 4 | 4 | 4 | 4 | -Infinity | 7 | 7 | 7 | 7 |
| RSRP Note 3 | dBm/15 KHz | -94 | -94 | -94 | -91 | -91 | -infinity | -91 | -91 | -91 | -91 |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |

The handover delay Dhandover1 is defined as the time from the beginning of time period T3, to the moment the UE start to transmit the PRACH to Cell 2.

The handover delay Dhandover1 test requirement in this case is expressed as:

The handover delay Dhandover1 can be expressed as: Dhandover1 = TRRC_procedure + TIU + 20 ms.

The handover delay Dhandover1 shall be less than a total of 50 ms in this test case (note: this gives a total of 15 ms for maximum RRC procedure delay plus 15 ms for TIU).

During Dhandover1 the UE shall be continuously scheduled on Cell 1 and the interruption on Cell 1 shall not exceed 2ms (Tinterrupt1).

The handover delay Dhandover2 is defined as the time from the beginning of time period T4, to the moment the UE start to transmit RRCConnectionReconfigurationComplete message to Cell 2.

The handover delay Dhandover1 test requirement in this case is expressed as:

Handover delay Dhandover2=TRRC_procedure + Tinterrupt2, where:

RRC procedure delay = 15 ms and is specified in clause 11.2 in TS 36.331 [2].

Tinterrupt2 = 2 ms in the test; Tinterrupt2 is defined in clause 5.7.2.1.2.

The handover delay Dhandover2 shall be less than a total of 17ms in this test caseFor the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.