3GPP TSG-RAN5 Meeting #95-e	R5-223033

# Online, , 9th May 2022 - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2377 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add new messages and procedure for test function to limit Pcell Power |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Portugal |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduce the outline of necessary messages and procedures to allow limiting of PCell power via a conformance test function |  |  |  |  |  |  |  |  |
| Summary of change: |  | New test function messages added to limit PCell power in UL-CA test cases |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SCell drop in UL-CA tests due to expected power prioritization rules |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7A, 4.9.30, 4.9.31 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Resubmission of endorsed draftCR R5-221920 with Editor’s notes added |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged Sections skipped>

#### NSSAI DELETE RESPONSE

This message is only sent in the direction UE to SS, embedded in a RRC ULInformationTransfer message.

Table 4.7A.6-2: NSSAI DELETE RESPONSE

| Derivation Path: 38.509 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Protocol discriminator | 1 1 1 1 |  |  |
| Skip indicator | 0 0 0 0 |  |  |
| Message type | 1 0 1 0 0 1 1 1 |  |  |

### 4.7A.7	UE Power Limit Messages

## Editor’s note:	The UPLF test mode is currently applicable to equal PSD (equal channel bandwidths on all component carriers) only.

#### -	ACTIVATE POWER LIMIT REQUEST

This Message is only sent in the direction SS to UE, embedded in a RRC DLInformationTransfer message.

- Table 4.7A.7-1: ACTIVATE POWER LIMIT REQUEST

| Derivation Path: 38.509 clause 6.11.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Protocol discriminator | 1 1 1 1 |  |  |
| Skip indicator | 0 0 0 0 |  |  |
| Message type | 1 0 1 0 0 0 0 0 |  |  |
| POWER LIMIT REQUEST | 1 0 1 0 0 0 0 0 |  | NR FR2 2CA |
| POWER LIMIT REQUEST | 1 0 1 0 0 0 0 1 |  | NR FR2 3CA |
| POWER LIMIT REQUEST | 1 0 1 0 0 0 1 0 |  | NR FR2 4CA |
| POWER LIMIT REQUEST | 1 0 1 0 0 0 1 1 |  | NR FR2 5CA |
| POWER LIMIT REQUEST | 1 0 1 0 0 1 0 0 |  | NR FR2 6CA |
| POWER LIMIT REQUEST | 1 0 1 0 0 1 0 1 |  | NR FR2 7CA |
| POWER LIMIT REQUEST | 1 0 1 0 0 1 1 0 |  | NR FR2 8CA |

| Condition | Explanation |
| --- | --- |
| NR FR2 2CA | Power Limit Request corresponding to NR FR2 2CA only |
| NR FR2 3CA | Power Limit Request corresponding to NR FR2 3CA only |
| NR FR2 4CA | Power Limit Request corresponding to NR FR2 4CA only |
| NR FR2 5CA | Power Limit Request corresponding to NR FR2 5CA only |
| NR FR2 6CA | Power Limit Request corresponding to NR FR2 6CA only |
| NR FR2 7CA | Power Limit Request corresponding to NR FR2 7CA only |
| NR FR2 8CA | Power Limit Request corresponding to NR FR2 8CA only |

#### ACTIVATE POWER LIMIT RESPONSE

This message is only sent in the direction UE to SS, embedded in a RRC ULInformationTransfer message.

- Table 4.7A.7-2: ACTIVATE POWER LIMIT RESPONSE

| Derivation Path: 38.509 clause 6.11.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Protocol discriminator | 1 1 1 1 |  |  |
| Skip indicator | 0 0 0 0 |  |  |
| Message type | 1 0 1 0 0 0 0 1 |  |  |

#### DEACTIVATE POWER LIMIT REQUEST

This message is only sent in the direction SS to UE, embedded in a RRC DLInformationTransfer message.

- Table 4.7A.7-3: DEACTIVATE POWER LIMIT REQUEST

| Derivation Path: 38.509 clause 6.11.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Protocol discriminator | 1 1 1 1 |  |  |
| Skip indicator | 0 0 0 0 |  |  |
| Message type | 1 0 1 0 1 0 0 0 |  |  |

#### DEACTIVATE POWER LIMIT RESPONSE

This message is only sent in the direction UE to SS, embedded in a RRC ULInformationTransfer message.

- Table 4.7A.7-3: DEACTIVATE POWER LIMIT RESPONSE

| Derivation Path: 38.509 clause 6.11.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Protocol discriminator | 1 1 1 1 |  |  |
| Skip indicator | 0 0 0 0 |  |  |
| Message type | 1 0 1 0 1 1 0 0 |  |  |

<Unchanged sections skipped>

### 4.9.30	Test procedure to activate UE Power Limit Function (UPLF)

## Editor’s note:	The UPLF test mode is currently applicable to equal PSD (equal channel bandwidths on all component carriers) only.

4.9.30.1	Initiation

UE is operating in FR2 in RRC_CONNECTED state with UE power limit test function activated.

4.9.30.2	Procedure

Table 4.9.30.2-1: Test procedure Sequence

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| 1 | SS request UE to activate UE Power Limit Function | <-- | ACTIVATE POWER LIMIT REQUEST | - | - |
| 2 | UE confirms that UE Power Limit Function is activated | --> | ACTIVATE POWER LIMIT RESPONSE | - | - |

4.9.30.3	Specific Message contents

- Table 4.9.30.3-1: ACTIVATE POWER LIMIT REQUEST

| Derivation Path: 38.509 clause 6.11.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Protocol discriminator | 1 1 1 1 |  |  |
| Skip indicator | 0 0 0 0 |  |  |
| Message type | 1 0 1 0 0 0 0 0 |  |  |
| POWER LIMIT REQUEST | 1 0 1 0 0 0 0 0 |  | NR FR2 2CA |
| POWER LIMIT REQUEST | 1 0 1 0 0 0 0 1 |  | NR FR2 3CA |
| POWER LIMIT REQUEST | 1 0 1 0 0 0 1 0 |  | NR FR2 4CA |
| POWER LIMIT REQUEST | 1 0 1 0 0 0 1 1 |  | NR FR2 5CA |
| POWER LIMIT REQUEST | 1 0 1 0 0 1 0 0 |  | NR FR2 6CA |
| POWER LIMIT REQUEST | 1 0 1 0 0 1 0 1 |  | NR FR2 7CA |
| POWER LIMIT REQUEST | 1 0 1 0 0 1 1 0 |  | NR FR2 8CA |

| Condition | Explanation |
| --- | --- |
| NR FR2 2CA | Power Limit Request corresponding to NR FR2 2CA only |
| NR FR2 3CA | Power Limit Request corresponding to NR FR2 3CA only |
| NR FR2 4CA | Power Limit Request corresponding to NR FR2 4CA only |
| NR FR2 5CA | Power Limit Request corresponding to NR FR2 5CA only |
| NR FR2 6CA | Power Limit Request corresponding to NR FR2 6CA only |
| NR FR2 7CA | Power Limit Request corresponding to NR FR2 7CA only |
| NR FR2 8CA | Power Limit Request corresponding to NR FR2 8CA only |

- Table 4.9.3.30-2: ACTIVATE POWER LIMIT RESPONSE

| Derivation Path: 38.509 clause 6.11.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | 1 1 1 1 |  |  |
| Skip indicator |  | 0 0 0 0 |  |  |
| Message type |  | 1 0 1 0 0 0 0 1 |  |  |

### 4.9.31	Test procedure to deactivate UE Power Limit Function (UPLF)

## Editor’s note:	The UPLF test mode is currently applicable to equal PSD (equal channel bandwidths on all component carriers) only.

4.9.31.1	Initiation

UE is operating in FR2 in RRC_CONNECTED state.

4.9.31.2	Procedure

Table 4.9.2.2-1: Test procedure Sequence

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| 1 | SS request UE to deactivate UE Power Limit Function | <-- | DEACTIVATE POWER LIMIT REQUEST | - | - |
| 2 | UE confirms that UE Power Limit Function is deactivated | --> | DEACTIVATE POWER LIMIT RESPONSE | - | - |

4.9.31.3	Specific Message contents

Table 4.9.2.3-1: DEACTIVATE POWER LIMIT REQUEST

| Derivation Path: 38.509 clause 6.11.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Protocol discriminator | 1 1 1 1 |  |  |
| Skip indicator | 0 0 0 0 |  |  |
| Message type | 1 0 1 0 1 0 0 0 |  |  |

Table 4.9.2.3-2: DEACTIVATE POWER LIMIT RESPONSE

| Derivation Path: 38.509 clause 6.11.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Protocol discriminator | 1 1 1 1 |  |  |
| Skip indicator | 0 0 0 0 |  |  |
| Message type | 1 0 1 0 1 1 0 0 |  |  |

<End of changes>