3GPP TSG-RAN5 Meeting #95-e	R5-223047

# Online, , 9th May 2022 - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2969 | rev | - | Current version: | 16.11.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of Emergency Services TC 11.4.x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For emergency test cases, when the UE is in 5GMM-DEREGISTERED state, according to TS 24.501 Cl. 5.5.2 when upper layer ends the emergency call, the UE may perform the de-registration directly with local PDU session release. Therfore the referenced PDU release procedure in 4.9.12A in test procedure needs to be compatible with such kind of UE behaviours. When upper layers indicate that emergency services are no longer required, the UE if still registered for emergency services, may perform UE-initiated de-registration procedure followed by a re-registration to regain normal services, if the UE is in or moves to a suitable cell. If the de-registration procedure for 5GS services is performed, a local release of the PDU sessions, if any, for this particular UE is performed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | PDU release procedure was changed to optional in test steps with editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail this case unfairly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.2.3, 11.4.3.3, 11.4.5.3 and 11.4.9.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

11.4.2.3	Test description

11.4.2.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell

-	NR Cell 1, as defined in TS 38.508-1 [4], Table 4.4.2-3, with the exception that cell's PLMN is defined in Table 11.4.2.3.1-1 below.

Table 11.4.2.3.1-1: PLMN identifiers

| NR Cell | PLMN name |
| --- | --- |
| 1 | PLMN2 |

-	System information combination NR-1 as defined in TS 38.508-1 [4], subclause 4.4.3.1.2. SIB1 indicates ims-EmergencySupport.

UE:

-	The UE is equipped with a test USIM with USIM Configuration 15 as defined in TS 38.508-1 [4], Table 6.4.1-15 (PLMN2 is set in it as 'forbidden PLMN', PLMN1 is the HPLMN).

Preamble:

-	The UE is in test state 0N-B (Switched Off) as defined in TS 38.508-1 [4], subclause 4.4A.2. Prior to being switched off the UE is registered on PLMN1.

11.4.2.3.2	Test procedure sequence

Table 11.4.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Void | - | - | - | - |
| - | The SS configures: - NR Cell 1 as "Serving cell" | - | - | - | - |
| 2 | Switch the UE on. | - | - | - | - |
| 3 | Make the UE attempt an IMS emergency call dialling a number which is stored on the ME (e.g. 112 or 911). (NOTE 1) | - | - | - | - |
| 4 | Check: Does the UE performs Generic Test Procedure for IMS Emergency call establishment without IMS emergency registration as specified in TS 38.508-1 [4], subclause 4.9.12? | - | - | 1 | - |
| 5 | Make the UE release the emergency call. (NOTE 1) | - | - | - | - |
| 6 | Steps 1-3 of the Generic test procedure for IMS MO Emergency call release as specified in TS 38.508-1 [4], subclause 4.9.12A takes place. | - | - | - | - |
| 6A | Start Timer T0=5 seconds. NOTE: This is an arbitrary value to wait for UE to initiate emergency PDU session release. | - | - | - | - |
| - | EXCEPTION: Steps 6Aa1-6Ab1 describes optional behaviour that depends on the UE implementation. | - | - | - | - |
| 6Aa1 | The UE transmits a PDU SESSION RELEASE REQUEST message with PSI equal to the value assigned to the Emergency PDU session in step 4. | --> | PDU SESSION RELEASE REQUEST | - | - |
| 6Aa2 | Steps 4-5 of the Generic test procedure for IMS MO Emergency call release as specified in TS 38.508-1 [4], subclause 4.9.12A takes place. | - | - | - | - |
| 6Aa3 | Stop Timer T0. | - | - | - | - |
| 6Ab1 | Timer T0 expires. (NOTE 2) | - | - | - | - |
| 7 | Start Timer T1=10 seconds. NOTE: This is an arbitrary value to wait for UE initiated deregistration. | - | - | - | - |
| - | EXCEPTION: Steps 8a1-8b1 describes optional behaviour that depends on the UE implementation. | - | - | - | - |
| 8a1 | The UE transmits a DEREGISTRATION REQUEST message with De-registration type IE set to "Normal de-registration". | --> | NR RRC: ULInformationTransfer 5GMM: DEREGISTRATION REQUEST | - | - |
| 8a2 | The SS transmits a DEREGISTRATION ACCEPT message. | <-- | NR RRC: DLInformationTransfer 5GMM: DEREGISTRATION ACCEPT | - | - |
| 8a3 | Stop Timer T1. | - | - | - | - |
| 8b1 | Timer T1 expires. | - | - | - | - |
| 8b2 | Void | - | - | - | - |
| 8b3 | Void | - | - | - | - |
| 9 | SS releases the RRC connection. | <-- | NR RRC: RRCRelease | - | - |
| 9A | Make the UE attempt an IMS none-emergency call. (NOTE 1). | - | - | - | - |
| 10 | Check: Does the UE transmit an RRCSetupRequest message in the next 10 seconds? NOTE: This is an arbitrary value to wait for catching not allowed UE behaviour. | --> | NR RRC: RRCSetupRequest | 2 | F |
| NOTE 1: This could be done by e.g. MMI or AT command. NOTE 2: Before T0 expires, UE may send the DEREGISTRATION REQUEST message, then goes to step 8a1.. |  |  |  |  |  |

11.4.2.3.3	Specific message contents

[Skip unchanged text]

11.4.3.3	Test description

11.4.3.3.1	Pre-test conditions

System Simulator:

-	1 NR Cells

-	NR Cell 1, as defined in TS 38.508-1 [4] Table 4.4.2-3. System information combination NR-1 as defined in TS 38.508-1 [4], subclause 4.4.3.1.2. SIB1 indicates ims-EmergencySupport.

UE:

-	The UE is NOT equipped with USIM.

Preamble:

-	The UE is in test state 0-A (Switched Off) as defined in TS 38.508-1 [4], subclause 4.4A.2.

11.4.3.3.2	Test procedure sequence

Table 11.4.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Switch the UE on. | - | - | - | - |
| 2 | Make the UE attempt an IMS emergency call dialling a number which is stored on the ME (e.g. 112 or 911). (NOTE 1) | - | - | - | - |
| 3 | Check: Does the UE performs Generic Test Procedure for IMS Emergency call establishment without IMS emergency registration as specified in TS 38.508-1 [4], subclause 4.9.12? | - | - | 1 | P |
| 4 | Make the UE release the emergency call. (NOTE 1) | - | - | - | - |
| 5 | Steps 1-3 of the Generic test procedure for IMS MO Emergency call release as specified in TS 38.508-1 [4], subclause 4.9.12A takes place. | - | - | - | - |
| 5A | Start Timer T0=5 seconds. NOTE: This is an arbitrary value to wait for UE to initiate emergency PDU session release. | - | - | - | - |
| - | EXCEPTION: Steps 5Aa1-5Ab1 describes optional behaviour that depends on the UE implementation. | - | - | - | - |
| 5Aa1 | The UE transmits a PDU SESSION RELEASE REQUEST message with PSI equal to the value assigned to the Emergency PDU session in step 3. | --> | PDU SESSION RELEASE REQUEST | - | - |
| 5Aa2 | Steps 4-5 of the Generic test procedure for IMS MO Emergency call release as specified in TS 38.508-1 [4], subclause 4.9.12A takes place. | - | - | - | - |
| 5Aa3 | Stop Timer T0. | - | - | - | - |
| 5Ab1 | Timer T0 expires. (NOTE 2) | - | - | - | - |
| 6 | Start Timer T1=5 sec. NOTE: This is an arbitrary value to wait for UE initiated deregistration. | - | - | - | - |
| - | EXCEPTION: Steps 7a1-7b1 describes optional behaviour that depends on the UE implementation. | - | - | - | - |
| 7a1 | The UE transmits a DEREGISTRATION REQUEST message with De-registration type IE set to "Normal de-registration". | --> | NR RRC: ULInformationTransfer 5GMM: DEREGISTRATION REQUEST | - | - |
| 7a2 | The SS transmits a DEREGISTRATION ACCEPT message. | <-- | NR RRC: DLInformationTransfer 5GMM: DEREGISTRATION ACCEPT | - | - |
| 7a3 | Stop Timer T1. | - | - | - | - |
| 7b1 | Timer T1 expires | - | - | - | - |
| 8 | The Generic test procedure Switch/Power off procedure after EMERGENCY CALL RELEASED in RRC_CONNECTED of Table 10.3.7-1 in TS 38.523-3 [3] takes place. | - | - | - | - |
| NOTE 1: This could be done by e.g. MMI or AT command. NOTE 2: Before T0 expires, UE may send the DEREGISTRATION REQUEST message, then goes to step 7a1. |  |  |  |  |  |

11.4.3.3.3	Specific message contents

[Skip unchanged text]

11.4.5.3	Test description

11.4.5.3.1	Pre-test conditions

System Simulator:

-	2 NR Cells

-	NR Cell 1 and NR Cell 11 as defined in TS 38.508-1 [4], Table 4.4.2-3.

-	On all cells when active: System information combination NR-1 as defined in TS 38.508-1 [4], subclause 4.4.3.1.2. SIB1 indicates ims-EmergencySupport.

UE:

None.

Preamble:

-	Cell configuration " in accordance with TS 38.508-1 [4], Table 4.4.2-3:

-	NR Cell 1 "Serving cell"

-	NR Cell 11 "Non-Suitable "Off" cell"

-	The UE is in test state 1N-A as defined in TS 38.508-1 [4], subclause 4.4A.2 on NR Cell 1

-	During the initial registration:

-	In the list of tracking areas provided by the AMF (IE 'TAI list') contains only the TAI of NR Cell 1.

11.4.5.3.2	Test procedure sequence

Table 11.4.5.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: - NR Cell 11 as "Serving cell" - NR Cell 1 as "Non-Suitable "Off" cell". | - | - | - | - |
| - | EXCEPTION: The following messages are to be observed on NR Cell 11 unless explicitly stated otherwise. | - | - | - | - |
| 2 | The UE transmits an RRCSetupRequest message. | --> | NR RRC: RRCSetupRequest | - | - |
| 3 | SS transmit an RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| 4 | The UE transmits an RRCSetupComplete message to confirm the successful completion of the connection establishment and a REGISTRATION REQUEST message indicating "mobility registration updating" is sent to update the registration of the actual tracking area. | --> | NR RRC: RRCSetupComplete 5GMM: REGISTRATION REQUEST | - | - |
| 5 | SS sends a REGISTRATION REJECT message containing 5GMM cause value = #15 (No suitable cells in tracking area). | <-- | NR RRC: DLInformationTransfer 5GMM: REGISTRATION REJECT | - | - |
| 6 | The SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |
| 7 | Make the UE attempt an IMS emergency call dialling an emergency number e.g. 112 or 911). (NOTE 1) | - | - | - | - |
| 8 | Check: Does the UE performs Generic Test Procedure for IMS Emergency call establishment without IMS Emergency registration as specified in TS 38.508-1 [4], subclause 4.9.12? | - | - | 1 | P |
| 9 | Make the UE release the emergency call. (NOTE 1) | - | - | - | - |
| 10 | Step 1-3 of the Generic test procedure for IMS MO Emergency call release as specified in TS 38.508-1 [4], subclause 4.9.12A takes place. | - | - | - | - |
| 10A | Start Timer T0=5 seconds. NOTE: This is an arbitrary value to wait for UE to initiate emergency PDU session release. | - | - | - | - |
| - | EXCEPTION: Steps 10Aa1-10Ab1 describes optional behaviour that depends on the UE implementation. | - | - | - | - |
| 10Aa1 | The UE transmits a PDU SESSION RELEASE REQUEST message with PSI equal to the value assigned to the Emergency PDU session in step 8. | --> | PDU SESSION RELEASE REQUEST | - | - |
| 10Aa2 | Steps 4-5 of the Generic test procedure for IMS MO Emergency call release as specified in TS 38.508-1 [4], subclause 4.9.12A takes place. | - | - | - | - |
| 10Aa3 | Stop Timer T0. | - | - | - | - |
| 10Ab1 | Timer T0 expires. (NOTE 2) | - | - | - | - |
| 11 | Void. | - | - | - | - |
| 12 | Start Timer T1=5 seconds. NOTE: This is an arbitrary value to wait for UE to initiate deregistration. | - | - | - | - |
| - | EXCEPTION: Steps 12a1-12b1 describes optional behaviour that depends on the UE implementation. | - | - | - | - |
| 12a1 | The UE transmits a DEREGISTRATION REQUEST message with De-registration type IE set to “Normal de-registration”. | --> | NR RRC: ULInformationTransfer 5GMM: DEREGISTRATION REQUEST | - | - |
| 12a2 | The SS transmits a DEREGISTRATION ACCEPT message. | <-- | NR RRC: DLInformationTransfer 5GMM: DEREGISTRATION ACCEPT | - | - |
| 12a3 | Stop Timer T1. | - | - | - | - |
| 12b1 | Timer T1 expires | - | - | - | - |
| 12A | The SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |
| 12B | Make the UE attempt an IMS non-emergency call. (NOTE 1) | - | - | - | - |
| 12C | Check: Does the UE transmit an RRCSetupRequest message for initial registration procedure on NR Cell 11 within the next 10 seconds? | --> | NR RRC: RRCSetupRequest | 2 | F |
| 13 | The SS configures: - NR Cell 1 as "Suitable neighbour intra-frequency cell" - NR Cell 11 as "Serving cell". | - | - | - | - |
| 14 | Check: Does the UE perform on NR Cell 1 the Registration procedure for initial registration as specified in TS 38.508-1 [4] subclause 4.5.2.2-2, 'connected without release'? | - | - | 2 | P |
| NOTE 1: This could be done by e.g. MMI or AT command. NOTE 2: Before T0 expires, UE may send the DEREGISTRATION REQUEST message, then goes to step 12a1.. |  |  |  |  |  |

11.4.5.3.3	Specific message contents

[Skip unchanged text]

11.4.9.3	Test description

11.4.9.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell

-	NR Cell 1, as defined in TS 38.508-1 [4], Table 4.4.2-3.

-	System information combination NR-1 as defined in TS 38.508-1 [4], subclause 4.4.3.1.2. SIB1 indicates ims-EmergencySupport.

UE:

None.

Preamble:

-	The UE is in test state 0N-B (Switched Off) as defined in TS 38.508-1 [4], subclause 4.4A.2.

11.4.9.3.2	Test procedure sequence

Table 11.4.9.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Switch the UE on. | - | - | - | - |
| 2-4 | Steps 2-4 of Table 4.5.2.2-2 NR RRC_IDLE in TS38.508-1 [4] take place (the UE initiates an initial registration procedure). | - | - | - | - |
| 5 | SS sends a REGISTRATION REJECT message containing 5GMM cause value = #15 (No suitable cells in tracking area). | <-- | NR RRC: DLInformationTransfer 5GMM: REGISTRATION REJECT | - | - |
| 6 | The SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |
| 7 | Make the UE attempt an IMS emergency call dialling a number which is stored on the ME (e.g. 112 or 911). (NOTE 1) | - | - | - | - |
| 8 | Check: Does the UE performs Generic Test Procedure for IMS Emergency call establishment without IMS emergency registration as specified in TS 38.508-1 [4], subclause 4.9.12? | - | - | 1 | P |
| 9 | Make the UE release the emergency call. (NOTE 1) | - | - | - | - |
| 10 | Steps 1-3 of the Generic test procedure for IMS MO Emergency call release as specified in TS 38.508-1 [4], subclause 4.9.12A takes place. | - | - | - | - |
| 10A | Start Timer T0=5 seconds. NOTE: This is an arbitrary value to wait for UE to initiate emergency PDU session release. | - | - | - | - |
| - | EXCEPTION: Steps 10Aa1-10Ab1 describes optional behaviour that depends on the UE implementation. | - | - | - | - |
| 10Aa1 | The UE transmits a PDU SESSION RELEASE REQUEST message with PSI equal to the value assigned to the Emergency PDU session in step 8. | --> | PDU SESSION RELEASE REQUEST | - | - |
| 10Aa2 | Steps 4-5 of the Generic test procedure for IMS MO Emergency call release as specified in TS 38.508-1 [4], subclause 4.9.12A takes place. | - | - | - | - |
| 10Aa3 | Stop Timer T0. | - | - | - | - |
| 10Ab1 | Timer T0 expires. (NOTE 2) | - | - | - | - |
| 11 | Start Timer T1=5 seconds. NOTE: This is an arbitrary value to wait for UE initiated deregistration. | - | - | - | - |
| - | EXCEPTION: Steps 12a1-12b1 describes optional behaviour that depends on the UE implementation. | - | - | - | - |
| 12a1 | The UE transmits a DEREGISTRATION REQUEST message with De-registration type IE set to "Normal de-registration". | --> | NR RRC: ULInformationTransfer 5GMM: DEREGISTRATION REQUEST | - | - |
| 12a2 | The SS transmits a DEREGISTRATION ACCEPT message. | <-- | NR RRC: DLInformationTransfer 5GMM: DEREGISTRATION ACCEPT | - | - |
| 12a3 | Stop Timer T1. | - | - | - | - |
| 12b1 | Timer T1 expires | - | - | - | - |
| 12b2 | Void. | - | - | - | - |
| 12b3 | Void. | - | - | - | - |
| 13 | SS releases the RRC connection. | <-- | NR RRC: RRCRelease | - | - |
| 13A | Make the UE attempt an IMS none-emergency call. (NOTE 1) | - | - | - | - |
| 14 | Check: Does the UE transmit an RRCSetupRequest message in the next 10 seconds? NOTE: This is an arbitrary value to wait for catching not allowed UE behaviour. | --> | NR RRC: RRCSetupRequest | 2 | F |
| NOTE 1: This could be done by e.g. MMI or AT command. NOTE 2: Before T0 expires, UE may send the DEREGISTRATION REQUEST message, then goes to step 12a1. |  |  |  |  |  |

11.4.9.3.3	Specific message contents

[End of change]