3GPP TSG-RAN5 Meeting #95-e	R5-223069
Electronic Meeting, 9th May – 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2978 | rev | - | Current version: | 16.11.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Equivalent PLMN ID in the test procedure of 9.1.5.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO, ZEKU |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI-15, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the main behavior step 15, the REGISTRATION ACCEPT message is sent to Cell F (serving cell) whch PLMN ID is 003101, and the Cell A is included in Cell F EQPLMN list and it should be 00101F(cell A). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected the Equivalent PLMN ID in step 15. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Equivalent PLMNs ID will still be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

9.1.5.1.2	Initial registration / 5GS services / Equivalent PLMN list handling

9.1.5.1.2.1	Test Purpose (TP)

(1)

with { The UE in 5GMM-REGISTERED-INITIATED state }

ensure that {

when { UE receives a REGISTRATION ACCEPT with a new set of Equivalent PLMNs, then after Switch OFF and Switch ON }

then { The UE deletes the old equivalent PLMN list and uses the new equivalent PLMN list }

}

(2)

with { The UE in 5GMM-REGISTERED-INITIATED state }

ensure that {

when { UE receives a REGISTRATION ACCEPT that does not include Equivalent PLMNs, then after Switch OFF and Switch ON }

then { UE deletes the old equivalent PLMN list }

}

(3)

with { The UE in 5GMM-REGISTERED-INITIATED state }

ensure that {

when { the UE receives a REGISTRATION ACCEPT that includes equivalent PLMN list, and the UE detects that an entry in the equivalent PLMN list is also present in forbidden PLMN list }

then { UE deletes the entry in equivalent PLMN list that is also present in forbidden PLMN list }

}

9.1.5.1.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.501, clauses 5.5.1.2.4 and 5.5.1.3.4. Unless otherwise stated these are Rel-15 requirements.

[TS 24.501, clause 5.5.1.2.4]

…

The AMF may also include a list of equivalent PLMNs in the REGISTRATION ACCEPT message. Each entry in the list contains a PLMN code (MCC+MNC). The UE shall store the list as provided by the network, and if the initial registration procedure is not for emergency services, the UE shall remove from the list any PLMN code that is already in the list of "forbidden PLMNs". In addition, the UE shall add to the stored list the PLMN code of the registered PLMN that sent the list. The UE shall replace the stored list on each receipt of the REGISTRATION ACCEPT message. If the REGISTRATION ACCEPT message does not contain a list, then the UE shall delete the stored list.

…

[TS 24.501, clause 5.5.1.3.4]

…

The AMF may also include a list of equivalent PLMNs in the REGISTRATION ACCEPT message. Each entry in the list contains a PLMN code (MCC+MNC). The UE shall store the list as provided by the network, and if there is no emergency PDU session established, the UE shall remove from the list any PLMN code that is already in the list of "forbidden PLMNs". If the UE is not registered for emergency services and there is an emergency PDU session established, the UE shall remove from the list of equivalent PLMNs any PLMN code present in the "forbidden PLMNs list" when the emergency PDU session is released. In addition, the UE shall add to the stored list the PLMN code of the registered PLMN that sent the list. The UE shall replace the stored list on each receipt of the REGISTRATION ACCEPT message. If the REGISTRATION ACCEPT message does not contain a list, then the UE shall delete the stored list.

…

9.1.5.1.2.3	Test description

9.1.5.1.2.3.1	Pre-test conditions

System Simulator:

-	NGC Cell A and NGC Cell E and NGC Cell F are configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in TS 38.508-1 [4].

-	NGC Cell F is configured with NR frequency NRf3.

-	System information combination NR-4 as defined in TS 38.508 [4] clause 4.4.3.1.2 is used.

UE:

None.

Preamble:

-	The UE is in state Switched OFF (State 0-A) as per TS 38.508-1 [4] Table 4.4A.2-0.

9.1.5.1.2.3.2	Test procedure sequence

Table 9.1.5.1.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: - NGC Cell F as the "Serving cell". - NGC Cell E as the "Non-Suitable "off" cell". - NGC Cell A as the “Non-Suitable “off" cell". | - | - | - | - |
| 2 | The UE is Switched ON |  | - |  |  |
| - | The following messages are to be observed on NGC Cell F unless explicitly stated otherwise |  | - |  |  |
| 3-14 | Steps 2-13 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | - | - |
| 15 | SS transmits a REGISTRATION ACCEPT message with PLMN ID of NGC Cell A included in the Equivalent PLMNs IE | <-- | REGISTRATION ACCEPT | - | - |
| 16-20a1 | Steps 15-19a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | - | - |
| 21 | The UE is switched off by executing generic procedure in Table 4.9.6.3-1 in TS 38.508-1 [4] | - | - | - | - |
| 22 | The UE is Switched ON | - | - | - | - |
| 23-34 | Steps 2-13 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | - | - |
| 35 | SS transmits a REGISTRATION ACCEPT message with PLMN ID of NGC Cell E included in the Equivalent PLMNs IE | <-- | REGISTRATION ACCEPT | - | - |
| 36-40a1 | Steps 15-19a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | - | - |
| 41 | The UE is switched off by executing generic procedure in Table 4.9.6.3-1 in TS 38.508-1 [4]. | - | - | - | - |
| 42 | The SS configures: - NGC CelFl as the "Non-Suitable "off" cell". - NGC Cell E as the "Suitable neighbour cell". - NGC Cell A as the "Serving cell". | - | - | - | - |
| 43 | The UE is Switched ON. | - | - | - | - |
| - | The following messages are to be observed on NGC Cell E unless explicitly stated otherwise | - | - | - | - |
| 44-62a1 | Steps 2-20a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | 1 | P |
| 63 | The UE is switched off by executing generic procedure in Table 4.9.6.1-1 in TS 38.508-1 [4]. | - | - | - | - |
| 63A | The SS configures: - NGC Cell F as the "Serving cell". - NGC Cell E as the "Non-Suitable "off" cell". - NGC Cell A as the "Suitable neighbour cell". | - | - | - | - |
| 64 | The UE is Switched ON | - | - | - | - |
| - | The following messages are to be observed on NGC Cell A unless explicitly stated otherwise | - | - | - | - |
| 64A | Steps 2-19a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | 2 | P |
| 64B | The UE is switched off by executing generic procedure in Table 4.9.6.3-1 in TS 38.508-1 [4]. | - | - | - | - |
| 64C | The SS configures: - NGC Cell A as the "Non-Suitable "off" cell". | - | - | - | - |
| 64D | The UE is Switched ON. | - | - | - | - |
| 65-76 | Steps 2-13 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | 2 | P |
| 76A | Void. | - | - | - | - |
| 77 | SS transmits a REGISTRATION REJECT with cause #11 (PLMN not allowed) | <-- | REGISTRATION REJECT | - | - |
| 78 | The SS releases the RRC connection | - | - |  |  |
| 79 | The SS configures: - NGC Cell A as the "Serving cell". - NGC Cell E as the “Non-Suitable "off" cell ".. - NGC Cell F as the “Non-Suitable "off" cell ". | - | - | - | - |
| - | The following messages are to be observed on NGC Cell A unless explicitly stated otherwise | - | - | - | - |
| 80-93 | Steps 2-13 of Table 4.5.5.2.2- 2 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | - | - |
| 94 | SS transmits a REGISTRATION ACCEPT message with PLMN ID of NGC Cell E and NGC Cell F included in the Equivalent PLMNs IE | <-- | REGISTRATION ACCEPT | - | - |
| 95-99a1 | Steps 15-19a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | - | - |
| 100 | The UE is switched off by executing generic procedure in Table 4.9.6.3-1 in TS 38.508-1 [4]. | - | - | - | - |
| 101 | The SS configures: - NGC Cell A as the “Non-Suitable "off" cell ". - NGC Cell E as the "Suitable neighbour cell". - NGC Cell F as the “Serving cell". | - | - | - | - |
| - | The following messages are to be observed on NGC Cell E unless explicitly stated otherwise | - | - | - | - |
| 102 | The UE is Switched ON | - | - | - | - |
| 103-121a1 | Steps 2-20a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | 3 | P |
| 122 | The UE is switched off by executing generic procedure in Table 4.9.6.1-1 in TS 38.508-1 [4]. | - | - | - | - |
| 123 | The SS configures: - NGC Cell A as the “Non-Suitable "off" cell ". - NGC Cell E as the "Non-Suitable "off" cell ". - NGC Cell F as the “Serving cell". | - | - | - | - |
| 124 | The UE is Switched ON | - | - | - | - |
| - | The user is prompted to manually select PLMN of NGC Cell F | - | - | - | - |
| - | The following messages are to be observed on NGC Cell F unless explicitly stated otherwise | - | - | - | - |
| 125-143a1 | Steps 2-20a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | - | - |
| 144 | The user sets the UE in Automatic PLMN selection mode. | - | - | - | - |
| 145 | Wait for 300s to allow the UE to switch to automatic PLMN selection mode. | - | - | - | - |

9.1.5.1.2.3.3	Specific message contents

Table 9.1.5.1.2.3.3-1: REGISTRATION ACCEPT (step 15 Table 9.1.5.1.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Equivalent PLMNs |  | MCC=001, MNC=01F | PLMN ID of NGC Cell A |  |

Table 9.1.5.1.2.3.3-2: REGISTRATION ACCEPT (step 35 Table 9.1.5.1.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Equivalent PLMNs |  | MCC=002, MNC=101 | PLMN ID of NGC Cell E |  |

Table 9.1.5.1.2.3.3-3: REGISTRATION ACCEPT (step 94 Table 9.1.5.1.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Equivalent PLMNs |  | MCC=003, MNC=101 and MCC=002, MNC=101 | PLMN ID of NGC Cell F and PLMN ID of NGC Cell E |  |

Table 9.1.5.1.2.3.3-4: REGISTRATION REJECT (step 77 Table 9.1.5.1.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.1-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GMM Cause |  | ‘0000 1011’B | PLMN not allowed |  |

<<< END OF CHANGES >>>