3GPP TSG-RAN5 Meeting #95-e	R5-223129

# Electronic Meeting, 9th May – 20th May. 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1733 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating RB allocation for CBW 45MHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_lic_bands_BW_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 45MHz is introduced for NR band n1. The RB allocation for 45MHz is still absent in table 6.1-1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding RB allocation for 45MHz in table 6.1-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RB allocation for 45MHz CBW is absent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.1	General

Unless otherwise stated, the transmitter characteristics are specified at the antenna connector of the UE with a single or multiple transmit antenna(s). For UE with integral antenna only, a reference antenna with a gain of 0 dBi is assumed.

Transmitter requirements for UL MIMO operation apply when the UE transmits on 2 ports on the same CDM group. The UE may use higher MPR values outside this limitation.

Unless otherwise stated, Channel Bandwidth shall be prioritized in the selecting of test points. Subcarrier spacing shall be selected after Test Channel Bandwidth is selected.

Uplink RB allocations given in Table 6.1-1 are used throughout this section, unless otherwise stated by the test case.

Table 6.1-1: Common uplink configuration

| Channel Bandwidth | SCS(kHz) | OFDM | RB allocation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Edge_Full_Left | Edge_Full_Right | Edge_1RB_Left | Edge_1RB_Right | Outer_Full | Inner_Full | Inner_1RB_Left | Inner_1RB_Right |
| 5MHz | 15 | DFT-s | 2@0 | 2@23 | 1@0 | 1@24 | 25@0 | 12@6 | 1@1 | 1@23 |
|  |  | CP | 2@0 | 2@23 | 1@0 | 1@24 | 25@0 | 13@6 | 1@1 | 1@23 |
|  | 30 | DFT-s | 2@0 | 2@9 | 1@0 | 1@10 | 10@0 | 5@21 | 1@1 | 1@9 |
|  |  | CP | 2@0 | 2@9 | 1@0 | 1@10 | 11@0 | 5@21 | 1@1 | 1@9 |
|  | 60 | DFT-s | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | CP | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| 10MHz | 15 | DFT-s | 2@0 | 2@50 | 1@0 | 1@51 | 50@0 | 25@12 | 1@1 | 1@50 |
|  |  | CP | 2@0 | 2@50 | 1@0 | 1@51 | 52@0 | 26@13 | 1@1 | 1@50 |
|  | 30 | DFT-s | 2@0 | 2@22 | 1@0 | 1@23 | 24@0 | 12@6 | 1@1 | 1@22 |
|  |  | CP | 2@0 | 2@22 | 1@0 | 1@23 | 24@0 | 12@6 | 1@1 | 1@22 |
|  | 60 | DFT-s | 2@0 | 2@9 | 1@0 | 1@10 | 10@0 | 5@21 | 1@1 | 1@9 |
|  |  | CP | 2@0 | 2@9 | 1@0 | 1@10 | 11@0 | 5@21 | 1@1 | 1@9 |
| 15MHz | 15 | DFT-s | 2@0 | 2@77 | 1@0 | 1@78 | 75@0 | 36@18 | 1@1 | 1@77 |
|  |  | CP | 2@0 | 2@77 | 1@0 | 1@78 | 79@0 | 39@191 | 1@1 | 1@77 |
|  | 30 | DFT-s | 2@0 | 2@36 | 1@0 | 1@37 | 36@0 | 18@9 | 1@1 | 1@36 |
|  |  | CP | 2@0 | 2@36 | 1@0 | 1@37 | 38@0 | 19@9 | 1@1 | 1@36 |
|  | 60 | DFT-s | 2@0 | 2@16 | 1@0 | 1@17 | 18@0 | 9@4 | 1@1 | 1@16 |
|  |  | CP | 2@0 | 2@16 | 1@0 | 1@17 | 18@0 | 9@4 | 1@1 | 1@16 |
| 20MHz | 15 | DFT-s | 2@0 | 2@104 | 1@0 | 1@105 | 100@0 | 50@25 | 1@1 | 1@104 |
|  |  | CP | 2@0 | 2@104 | 1@0 | 1@105 | 106@0 | 53@26 | 1@1 | 1@104 |
|  | 30 | DFT-s | 2@0 | 2@49 | 1@0 | 1@50 | 50@0 | 25@12 | 1@1 | 1@49 |
|  |  | CP | 2@0 | 2@49 | 1@0 | 1@50 | 51@0 | 25@121 | 1@1 | 1@49 |
|  | 60 | DFT-s | 2@0 | 2@22 | 1@0 | 1@23 | 24@0 | 12@6 | 1@1 | 1@22 |
|  |  | CP | 2@0 | 2@22 | 1@0 | 1@23 | 24@0 | 12@6 | 1@1 | 1@22 |
| 25MHz | 15 | DFT-s | 2@0 | 2@131 | 1@0 | 1@132 | 128@0 | 64@32 | 1@1 | 1@131 |
|  |  | CP | 2@0 | 2@131 | 1@0 | 1@132 | 133@0 | 67@33 | 1@1 | 1@131 |
|  | 30 | DFT-s | 2@0 | 2@63 | 1@0 | 1@64 | 64@0 | 32@16 | 1@1 | 1@63 |
|  |  | CP | 2@0 | 2@63 | 1@0 | 1@64 | 65@0 | 33@16 | 1@1 | 1@63 |
|  | 60 | DFT-s | 2@0 | 2@29 | 1@0 | 1@30 | 30@0 | 15@71 | 1@1 | 1@29 |
|  |  | CP | 2@0 | 2@29 | 1@0 | 1@30 | 31@0 | 15@71 | 1@1 | 1@29 |
| 30MHz | 15 | DFT-s | 2@0 | 2@158 | 1@0 | 1@159 | 160@0 | 80@40 | 1@1 | 1@158 |
|  |  | CP | 2@0 | 2@158 | 1@0 | 1@159 | 160@0 | 80@40 | 1@1 | 1@158 |
|  | 30 | DFT-s | 2@0 | 2@76 | 1@0 | 1@77 | 75@0 | 36@18 | 1@1 | 1@76 |
|  |  | CP | 2@0 | 2@76 | 1@0 | 1@77 | 78@0 | 39@19 | 1@1 | 1@76 |
|  | 60 | DFT-s | 2@0 | 2@36 | 1@0 | 1@37 | 36@0 | 18@9 | 1@1 | 1@36 |
|  |  | CP | 2@0 | 2@36 | 1@0 | 1@37 | 38@0 | 19@9 | 1@1 | 1@36 |
| 40MHz | 15 | DFT-s | 2@0 | 2@214 | 1@0 | 1@215 | 216@0 | 108@54 | 1@1 | 1@214 |
|  |  | CP | 2@0 | 2@214 | 1@0 | 1@215 | 216@0 | 108@54 | 1@1 | 1@214 |
|  | 30 | DFT-s | 2@0 | 2@104 | 1@0 | 1@105 | 100@0 | 50@25 | 1@1 | 1@104 |
|  |  | CP | 2@0 | 2@104 | 1@0 | 1@105 | 106@0 | 53@26 | 1@1 | 1@104 |
|  | 60 | DFT-s | 2@0 | 2@49 | 1@0 | 1@50 | 50@0 | 25@12 | 1@1 | 1@49 |
|  |  | CP | 2@0 | 2@49 | 1@0 | 1@50 | 51@0 | 25@121 | 1@1 | 1@49 |
| 45MHz | 15 | DFT-s | 2@0 | 2@240 | 1@0 | 1@241 | 242@0 | 120@60 | 1@1 | 1@240 |
|  |  | CP | 2@0 | 2@240 | 1@0 | 1@241 | 242@0 | 121@60 | 1@1 | 1@240 |
|  | 30 | DFT-s | 2@0 | 2@117 | 1@0 | 1@118 | 119@0 | 60@30 | 1@1 | 1@117 |
|  |  | CP | 2@0 | 2@240 | 1@0 | 1@118 | 119@0 | 60@30 | 1@1 | 1@117 |
|  | 60 | DFT-s | 2@0 | 2@56 | 1@0 | 1@57 | 58@0 | 27@13 | 1@1 | 1@56 |
|  |  | CP | 2@0 | 2@240 | 1@0 | 1@57 | 58@0 | 29@14 | 1@1 | 1@56 |

| Channel Bandwidth | SCS(kHz) | OFDM | RB allocation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Edge_Full_Left | Edge_Full_Right | Edge_1RB_Left | Edge_1RB_Right | Outer_Full | Inner_Full | Inner_1RB_Left | Inner_1RB_Right |
| 50MHz | 15 | DFT-s | 2@0 | 2@268 | 1@0 | 1@269 | 270@0 | 135@67 | 1@1 | 1@268 |
|  |  | CP | 2@0 | 2@268 | 1@0 | 1@269 | 270@0 | 135@67 | 1@1 | 1@268 |
|  | 30 | DFT-s | 2@0 | 2@131 | 1@0 | 1@132 | 128@0 | 64@32 | 1@1 | 1@131 |
|  |  | CP | 2@0 | 2@131 | 1@0 | 1@132 | 133@0 | 67@33 | 1@1 | 1@131 |
|  | 60 | DFT-s | 2@0 | 2@63 | 1@0 | 1@64 | 64@0 | 32@16 | 1@1 | 1@63 |
|  |  | CP | 2@0 | 2@63 | 1@0 | 1@64 | 65@0 | 33@16 | 1@1 | 1@63 |
| 60MHz | 15 | DFT-s | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | CP | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 30 | DFT-s | 2@0 | 2@160 | 1@0 | 1@161 | 162@0 | 81@40 | 1@1 | 1@160 |
|  |  | CP | 2@0 | 2@160 | 1@0 | 1@161 | 162@0 | 81@40 | 1@1 | 1@160 |
|  | 60 | DFT-s | 2@0 | 2@77 | 1@0 | 1@78 | 75@0 | 36@18 | 1@1 | 1@77 |
|  |  | CP | 2@0 | 2@77 | 1@0 | 1@78 | 79@0 | 39@191 | 1@1 | 1@77 |
| 70MHz | 15 | DFT-s | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | CP | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 30 | DFT-s | 2@0 | 2@187 | 1@0 | 1@188 | 180@0 | 90@45 | 1@1 | 1@187 |
|  |  | CP | 2@0 | 2@187 | 1@0 | 1@188 | 189@0 | 95@47 | 1@1 | 1@187 |
|  | 60 | DFT-s | 2@0 | 2@91 | 1@0 | 1@92 | 90@0 | 45@22 | 1@1 | 1@91 |
|  |  | CP | 2@0 | 2@91 | 1@0 | 1@92 | 93@0 | 47@23 | 1@1 | 1@91 |
| 80MHz | 15 | DFT-s | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | CP | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 30 | DFT-s | 2@0 | 2@215 | 1@0 | 1@216 | 216@0 | 108@54 | 1@1 | 1@215 |
|  |  | CP | 2@0 | 2@215 | 1@0 | 1@216 | 217@0 | 109@54 | 1@1 | 1@215 |
|  | 60 | DFT-s | 2@0 | 2@105 | 1@0 | 1@106 | 100@0 | 50@25 | 1@1 | 1@105 |
|  |  | CP | 2@0 | 2@105 | 1@0 | 1@106 | 107@0 | 53@261 | 1@1 | 1@105 |
| 90MHz | 15 | DFT-s | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | CP | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 30 | DFT-s | 2@0 | 2@243 | 1@0 | 1@244 | 243@0 | 120@60 | 1@1 | 1@243 |
|  |  | CP | 2@0 | 2@243 | 1@0 | 1@244 | 245@0 | 123@61 | 1@1 | 1@243 |
|  | 60 | DFT-s | 2@0 | 2@119 | 1@0 | 1@120 | 120@0 | 60@30 | 1@1 | 1@119 |
|  |  | CP | 2@0 | 2@119 | 1@0 | 1@120 | 121@0 | 61@30 | 1@1 | 1@119 |
| 100MHz | 15 | DFT-s | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | CP | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 30 | DFT-s | 2@0 | 2@271 | 1@0 | 1@272 | 270@0 | 135@67 | 1@1 | 1@271 |
|  |  | CP | 2@0 | 2@271 | 1@0 | 1@272 | 273@0 | 137@68 | 1@1 | 1@271 |
|  | 60 | DFT-s | 2@0 | 2@133 | 1@0 | 1@134 | 135@0 | 64@32 | 1@1 | 1@133 |
|  |  | CP | 2@0 | 2@133 | 1@0 | 1@134 | 135@0 | 67@331 | 1@1 | 1@133 |
| Note 1: The allocated RB number LCRB is ceil(NRB/2) -1 in order to meet Inner RB allocation definition (RBStart,Low ≤ RBStart ≤ RBStart,High) described in subclause 6.2.2 of TS 38.101-1 [2]. |  |  |  |  |  |  |  |  |  |  |

<End of Changes>