3GPP TSG-RAN5 Meeting #95-e	R5-223202

# Online, , 9th May 2022 - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2984 | rev | - | Current version: | 16.11.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modification of testcase 8.1.5.11.5 idle/inactive measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Renumbering testcase due to removal of moving testcase 8.1.5.11.2 and 8.1.5.11.4 to MR-DC RRC chapter. Pre test conditions and test procedure have wrong system information and errors in the parametrization. Test specific messages are having unnecessary IEs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Renumbering testcase, fixed pre test conditions and test procedure for the system info and inaccurate parametrization. Removed unnecessary IEs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case not usable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 38.508-1, 38.523-2 CR 2295, 2345, 0228 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### ------------------------------------------------------- Start of changes -----------------------------------------------------

##### 8.1.5.11.3	Idle/Inactive measurements / Inactive mode / SIB11 configuration / Measurement of NR cells

8.1.5.11.3.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE receives a RRCRelease message including measIdleConfig and suspendConfig, SIB11 including measIdleCarrierListNR with ssb-MeasConfig }

then { UE enters RRC_INACTIVE state, performs cell measurements and reports measurement results in RRC_CONNECTED state }

}

(2)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE receives a RRCRelease message including measIdleConfig and suspendConfig, SIB11 including measIdleCarrierListNR without ssb-MeasConfig and ssb-MeasConfig parameters derived from SIB4 }

then { UE enters RRC_INACTIVE state, performs cell measurements and reports measurement results in RRC_CONNECTED state }

}

(3)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE receives a RRCRelease message including measIdleConfig and suspendConfig, SIB11 including measIdleCarrierListNR without ssb-MeasConfig, ssb-MeasConfig parameters derived from SIB4 and T331 expires }

then { UE enters RRC_INACTIVE state, UE does not report measurement results in RRC_CONNECTED state after T331 expiration }

}

(4)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE receives a RRCRelease message including measIdleConfig and suspendConfig, SIB11 does not include measIdleCarrierListNR }

then { UE enters RRC_INACTIVE state, UE does not report measurement results in RRC_CONNECTED state }

}

8.1.5.11.3.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 38.331 clause 5.3.8.3, 5.7.8 and 5.7.10.3. Unless otherwise stated, these are Rel-16 requirements.

[TS 38.331, clause 5.3.8.3]

1>	if the RRCRelease includes the measIdleConfig:

2>	if T331 is running:

3> stop timer T331;

3>	perform the actions as specified in 5.7.8.3;

2>	if the measIdleConfig is set to setup:

3>	store the received measIdleDuration in VarMeasIdleConfig;

3>	start timer T331 with the value set to measIdleDuration;

3>	if the measIdleConfig contains measIdleCarrierListNR:

4>	store the received measIdleCarrierListNR in VarMeasIdleConfig;

[TS 38.331, clause 5.7.8.1a]

While in RRC_IDLE or RRC_INACTIVE, and T331 is running, the UE shall:

1>	if VarMeasIdleConfig includes neither a measIdleCarrierListEUTRA nor a measIdleCarrierListNR received from the RRCRelease message:

2>	if the UE supports idleInactiveNR-MeasReport:

3>	if SIB11 includes the measIdleConfigSIB and contains measIdleCarrierListNR:

4>	store or replace the measIdleCarrierListNR of measIdleConfigSIB of SIB11 within VarMeasIdleConfig;

3>	else:

4>	remove the measIdleCarrierListNR in VarMeasIdleConfig, if stored;

1>	for each entry in the measIdleCarrierListNR within VarMeasIdleConfig that does not contain an ssb-MeasConfig received from the RRCRelease message:

2>	if there is an entry in measIdleCarrierListNR in measIdleConfigSIB of SIB11 that has the same carrier frequency and subcarrier spacing as the entry in the measIdleCarrierListNR within VarMeasIdleConfig and that contains ssb-MeasConfig:

3>	delete the ssb-MeasConfig of the corresponding entry in the measIdleCarrierListNR within VarMeasIdleConfig;

3>	store the SSB measurement configuration from SIB11 into nrofSS-BlocksToAverage, absThreshSS-BlocksConsolidation, smtc, ssb-ToMeasure, deriveSSB-IndexFromCell, and ss-RSSI-Measurement within ssb-MeasConfig of the corresponding entry in the measIdleCarrierListNR within VarMeasIdleConfig;

2>	else if there is an entry in interFreqCarrierFreqList of SIB4 with the same carrier frequency and subcarrier spacing as the entry in measIdleCarrierListNR within VarMeasIdleConfig:

3>	delete the ssb-MeasConfig of the corresponding entry in the measIdleCarrierListNR within VarMeasIdleConfig;

3>	store the SSB measurement configuration from SIB4 into nrofSS-BlocksToAverage, absThreshSS-BlocksConsolidation, smtc, ssb-ToMeasure, deriveSSB-IndexFromCell, and ss-RSSI-Measurement within ssb-MeasConfig of the corresponding entry in the measIdleCarrierListNR within VarMeasIdleConfig;

2>	else:

3>	remove the ssb-MeasConfig of the corresponding entry in the measIdleCarrierListNR within VarMeasIdleConfig, if stored;

1>	perform measurements according to 5.7.8.2a.

[TS 38.331, clause 5.7.8.2a]

While in RRC_IDLE or RRC_INACTIVE, and T331 is running, the UE shall:

1>	perform the measurements in accordance with the following:

2>	if the VarMeasIdleConfig includes the measIdleCarrierListNR and the SIB1 contains idleModeMeasurementsNR:

3>	for each entry in measIdleCarrierListNR within VarMeasIdleConfig that contains ssb-MeasConfig:

4>	if UE supports carrier aggregation or NR-DC between serving carrier and the carrier frequency and subcarrier spacing indicated by carrierFreq and ssbSubCarrierSpacing within the corresponding entry:

5>	perform measurements in the carrier frequency and subcarrier spacing indicated by carrierFreq and ssbSubCarrierSpacing within the corresponding entry;

5>	if the reportQuantities is set to rsrq:

6>	consider RSRQ as the cell sorting quantity;

5>	else:

6>	consider RSRP as the cell sorting quantity;

5>	if the measCellListNR is included:

6>	consider cells identified by each entry within the measCellListNR to be applicable for idle/inactive measurement reporting;

5>	else:

6>	consider up to maxCellMeasIdle strongest identified cells, according to the sorting quantity, to be applicable for idle/inactive measurement reporting;

5>	for all cells applicable for idle/inactive measurement reporting, derive cell measurement results for the measurement quantities indicated by reportQuantities;

5>	store the derived cell measurement results as indicated by reportQuantities for cells applicable for idle/inactive measurement reporting within measResultsPerCarrierListIdleNR in the measReportIdleNR in VarMeasIdleReport in decreasing order of the cell sorting quantity, i.e. the best cell is included first, as follows:

6>	if qualityThreshold is configured:

7>	include the measurement results from the cells applicable for idle/inactive measurement reporting whose RSRP/RSRQ measurement results are above the value(s) provided in qualityThreshold;

6>	else:

7>	include the measurement results from all cells applicable for idle/inactive measurement reporting;

2>	if, as a result of the procedure in this subclause, the UE performs measurements in one or more carrier frequency indicated by measIdleCarrierListNR or measIdleCarrierListEUTRA:

3>	store the cell measurement results for RSRP and RSRQ for the serving cell within measResultServingCell in the measReportIdleNR in VarMeasIdleReport.

NOTE 1:	How the UE performs idle/inactive measurements is up to UE implementation as long as the requirements in TS 38.133 [14] are met for measurement reporting.

NOTE 2:	The UE is not required to perform idle/inactive measurements on a given carrier if the SSB configuration of that carrier provided via dedicated signalling is different from the SSB configuration broadcasted in the serving cell, if any.

NOTE 3:	How the UE prioritizes which frequencies to measure or report (in case it is configured with more frequencies than it can measure or report) is left to UE implementation.

[TS 38.331, clause 5.7.8.3]

1>	if T331 expires or is stopped:

2>	release the VarMeasIdleConfig.

NOTE:	It is up to UE implementation whether to continue idle/inactive measurements according to SIB11 and SIB4 configurations or according to E-UTRA SIB5 and E-UTRA SIB24 configurations as specified in TS 36.331 [10] upon inter-RAT cell reselection to E-UTRA, after T331 has expired or stopped.

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

1>	if the idleModeMeasurementReq is included in the UEInformationRequest and the UE has stored VarMeasIdleReport that contains measurement information concerning cells other than the PCell:

2>	set the measResultIdleNR in the UEInformationResponse message to the value of measReportIdleNR in the VarMeasIdleReport, if available;

2>	discard the VarMeasIdleReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

8.1.5.11.3.3	Test Description

8.1.5.11.3.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1, serving cell. System information combination NR-18 defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used. Condition EMR_NR used for SIB1 and condition EMR_NR_SIB11 used for SIB11.

-	NR Cell 3, suitable for inter-frequency measurements. System information combination NR-4 defined in TS 38.508-1 [4] clause 4.4.3.1.2.

UE:

-	None.

Preamble:

-	The UE is in 5GS state 3N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-1.

8.1.5.11.3.3.2	Test procedure sequence

Table 8.1.5.11.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits RRCRelease message including measIdleConfig and suspendConfig to the UE. | <- | NR RRC: RRCRelease | - | - |
| 2 | Wait for 15 s for UE to enter RRC_INACTIVE mode and to perform measurements. | - | - | - | - |
| 3 | SS transmits Paging message to UE. | <-- | NR RRC: Paging | - | - |
| 4 | UE transmits RRCResumeRequest message to SS. | --> | NR RRC: RRCResumeRequest | - | - |
| 5 | SS transmits RRCResume message to UE. | <-- | NR RRC: RRCResume | - | - |
| 6 | UE transmits RRCResumeComplete message to SS. |  | NR RRC: RRCResumeComplete | - | - |
| 7 | The SS transmits UEInformationRequest message including idleModeMeasurementReq to the UE. | <- | NR RRC: UEInformationRequest | - | - |
| 8 | Check: Does the UE transmit UEInformationResponse including measResultIdleNR? | -> | NR RRC: UEInformationResponse | 1 | P |
| 9 | The SS changes system information combination to NR-4 defined in TS 38.508-1 [4] clause 4.4.3.1.2 with condition EMR_NR for SIB1 and EMR_NR_SIB4 for SIB11. | - | - | - | - |
| 10 | The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. | <- | PDCCH (DCI 1_0): Short Message | - | - |
| 11 | Wait for 2.1* modification period to allow the new system information to take effect. | - | - | - | - |
| 12 | The SS transmits RRCRelease message including measIdleConfig and suspendConfig to the UE. | <- | NR RRC: RRCRelease | - | - |
| 13 | Wait for 15 s for UE to enter RRC_INACTIVE mode and to perform measurements. | - | - | - | - |
| 14 | SS transmits Paging message to UE. | <-- | NR RRC: Paging | - | - |
| 15 | UE transmits RRCResumeRequest message to SS. | --> | NR RRC: RRCResumeRequest | - | - |
| 16 | SS transmits RRCResume message to UE. | <-- | NR RRC: RRCResume | - | - |
| 17 | UE transmits RRCResumeComplete message to SS. |  | NR RRC: RRCResumeComplete | - | - |
| 18 | The SS transmits UEInformationRequest message including idleModeMeasurementReq to the UE. | <- | NR RRC: UEInformationRequest | - | - |
| 19 | Check: Does the UE transmit UEInformationResponse including measResultIdleNR? | -> | NR RRC: UEInformationResponse | 2 | P |
| 20 | The SS transmits RRCRelease message including measIdleConfig and suspendConfig to the UE. | <- | NR RRC: RRCRelease | - | - |
| 21 | Wait for 15 s for UE to enter RRC_INACTIVE mode and to perform measurements. | - | - | - | - |
| 22 | SS transmits Paging message to UE. | <-- | NR RRC: Paging | - | - |
| 23 | UE transmits RRCResumeRequest message to SS. | --> | NR RRC: RRCResumeRequest | - | - |
| 24 | SS transmits RRCResume message to UE. | <-- | NR RRC: RRCResume | - | - |
| 25 | UE transmits RRCResumeComplete message to SS. |  | NR RRC: RRCResumeComplete | - | - |
| 26 | The SS transmits UEInformationRequest message including idleModeMeasurementReq to the UE. | <- | NR RRC: UEInformationRequest | - | - |
| 27 | Check: Does the UE transmit UEInformationResponse without measResultIdleNR? | -> | NR RRC: UEInformationResponse | 3 | P |
| 28 | The SS changes system information combination NR-4 to have no conditions for SIB11. | - | - | - | - |
| 29 | The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. | <- | PDCCH (DCI 1_0): Short Message | - | - |
| 30 | Wait for 2.1* modification period to allow the new system information to take effect. | - | - | - | - |
| 31 | The SS transmits RRCRelease message including measIdleConfig and suspendConfig to the UE. | <- | NR RRC: RRCRelease | - | - |
| 32 | Wait for 15 s for UE to enter RRC_INACTIVE mode and to perform measurements. | - | - | - | - |
| 33 | SS transmits Paging message to UE. | <-- | NR RRC: Paging | - | - |
| 34 | UE transmits RRCResumeRequest message to SS. | --> | NR RRC: RRCResumeRequest | - | - |
| 35 | SS transmits RRCResume message to UE. | <-- | NR RRC: RRCResume | - | - |
| 36 | UE transmits RRCResumeComplete message to SS. |  | NR RRC: RRCResumeComplete | - | - |
| 37 | The SS transmits UEInformationRequest message including idleModeMeasurementReq to the UE. | <- | NR RRC: UEInformationRequest | - | - |
| 38 | Check: Does the UE transmit UEInformationResponse without measResultIdleNR? | -> | NR RRC: UEInformationResponse | 4 | P |

8.1.5.11.3.3.3	Specific message contents

Table 8.1.5.11.3.3.3-1: RRCRelease (Table 8.1.5.11.3.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCRelease ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcRelease SEQUENCE { |  |  |  |
| suspendConfig | SuspendConfig |  |  |
| nonCriticalExtensions SEQUENCE { |  |  |  |
| nonCriticalExtensions SEQUENCE { |  |  |  |
| measIdleConfig-r16 SetupRelease { |  |  |  |
| setup | MeasIdleConfigDedicated-r16 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.11.3.3.3-2: SuspendConfig (Table 8.1.5.11.3.3.3-1)

| Derivation Path: TS 38.331 [12] Chapter 6.2.2 Message definitions - RRCRelease |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SuspendConfig ::= SEQUENCE { |  |  |  |
| fullI-RNTI | I-RNTI-Value | 38.508-1[4] Table 4.6.3-64 |  |
| shortI-RNTI | ShortI-RNTI-Value | 38.508-1[4] Table 4.6.3-170 |  |
| ran-PagingCycle | rf32 |  |  |
| ran-NotificationAreaInfo CHOICE { |  |  |  |
| cellList SEQUENCE (SIZE (1.. maxPLMNIdentities)) OF PLMN-RAN-AreaCell { | 2 entries |  |  |
| PLMN-RAN-AreaCellList[1] SEQUENCE { |  | entry 1 |  |
| plmn-Identity | PLMN1 |  |  |
| ran-AreaCells SEQUENCE (SIZE (1..32)) OF CellIdentity { | 1 entry |  |  |
| CellIdentity[1] | CellIdentity of NR Cell 1 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| PLMN-RAN-AreaCellList[2] SEQUENCE { |  | entry 2 |  |
| plmn-Identity | PLMN2 |  |  |
| ran-AreaCells SEQUENCE (SIZE (1..32)) OF CellIdentity { | 1 entry |  |  |
| CellIdentity[1] | CellIdentity of NR Cell 3 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| t380 | Not present |  |  |
| nextHopChainingCount | NextHopChainingCount | 38.508-1[4] Table 4.6.3-83 |  |
| } |  |  |  |

Table 8.1.5.11.3.3.3-3: MeasIdleConfigDedicated (Table 8.1.5.11.3.3.3-1)

| Derivation Path: TS 38.331 [12] Chapter 6.3.2 Radio resource control information elements - MeasIdleConfig |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasIdleConfigDedicated-r16 ::= SEQUENCE { |  |  |  |
| measIdleCarrierListNR-r16 | Not present |  |  |
| measIdleCarrierListEUTRA-r16 | Not present |  |  |
| measIdleDuration-r16 | sec120 |  |  |
| validityAreaList-r16 | Not present |  |  |
| } |  |  |  |

Table 8.1.5.11.3.3.3-4: RRCResumeRequest (Table 8.1.5.11.3.3.2-1)

Derivation Path: TS 38.508 [4], clause 4.6.1 table 4.6.1-19

Table 8.1.5.11.3.3.3-5: RRCResume (Table 8.1.5.11.3.3.2-1)

Derivation Path: TS 38.508 [4], clause 4.6.1 table 4.6.1-17

Table 8.1.5.11.3.3.3-6: RRCResumeComplete (Table 8.1.5.11.3.3.2-1)

Derivation Path: TS 38.508 [4], clause 4.6.1 table 4.6.1-18

Table 8.1.5.11.3.3.3-7: UEInformationRequest (Table 8.1.5.11.3.3.2-1)

Derivation Path: TS 38.508-1 [4], clause 4.6.1 table 4.6.1-32A with condition IDLE

Table 8.1.5.11.3.3.3-8: UEInformationResponse (Table 8.1.5.11.3.3.2-1)

| Derivation Path: TS 38.508-1 [4], clause 4.6.1 table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| measResultIdleNR-r16 SEQUENCE { |  |  |  |
| measResultServingCell-r16 SEQUENCE { |  |  |  |
| rsrp-Result-r16 | INTEGER (0..127) |  |  |
| rsrq-Result-r16 | INTEGER (0..127) |  |  |
| resultsSSB-Indexes-r16 SEQUENCE { |  |  |  |
| ssb-Index-r16 | INTEGER (0..maxNrofSSBs-1) |  |  |
| ssb-Results-r16 SEQUENCE { |  |  |  |
| ssb-RSRP-Result-r16 | INTEGER (0..127) |  |  |
| ssb-RSRQ-Result-r16 | INTEGER (0..127) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultsPerCarrierListIdleNR-r16 SEQUENCE (SIZE (1.. maxFreqIdle-r16)) OF MeasResultsPerCarrierIdleNR-r16 | 1 entry |  |  |
| MeasResultsPerCarrierIdleNR-r16[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r16 | ARFCN value corresponding to NR cell 3 |  |  |
| measResultsPerCellListIdleNR-r16 SEQUENCE (SIZE (1..maxCellMeasIdle-r16)) OF MeasResultsPerCellIdleNR-r16 { | 1 entry |  |  |
| MeasResultsPerCellIdleNR-r16[1] SEQUENCE { |  | entry 1 |  |
| physCellId-r16 | PhysCellId corresponding to NR cell 3 |  |  |
| measIdleResultNR-r16 SEQUENCE { |  |  |  |
| rsrp-Result-r16 | INTEGER (0..127) |  |  |
| rsrq-Result-r16 | INTEGER (0..127) |  |  |
| resultsSSB-Indexes-r16 SEQUENCE { |  |  |  |
| ssb-Index-r16 | INTEGER (0..maxNrofSSBs-1) |  |  |
| ssb-Results-r16 SEQUENCE { |  |  |  |
| ssb-RSRP-Result-r16 | INTEGER (0..127) |  |  |
| ssb-RSRQ-Result-r16 | INTEGER (0..127) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### ------------------------------------------------------- End of changes -----------------------------------------------------