3GPP TSG-RAN WG5 Meeting #95-e	R5-223213

# Online, , 9th May 2022 - 20th May 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.123-1 | CR | 3937 | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to eCall test cases 13.3.1.2, 13.3.1.3, 13.3.1.4, 13.3.1.5, 13.3.1.6, 13.3.1.7 and 13.3.1.10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, CETECOM GmbH |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2022-04-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The wording “In step xx, eCall UE shall release the correction” in 3G eCall tests under “Test Requirements” is incorrect because the UE is not allowed by HLAP protocol to release the connection of an eCall before CEN EN 16062 timer T2 = 1h expires or unless it is explicitly instructed by the AL-ACK message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Correct the wording to be precise, “In step xx UE shall send a RELEASE message.” and inline to requirements in other NAS test cases 2. Grammatical corrections at few places 3. Alignment of the requirements for Emergency Service Category bits. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect “Test Requirements” will remain in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.3.1.2, 13.3.1.3, 13.3.1.4, 13.3.1.5, 13.3.1.6, 13.3.1.7 and 13.3.1.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <Start of modified section>

#### 13.3.1.2	Test eCall using eCall capable UE with eCall only subscription

13.3.1.2.1	Definition and applicability

Applicable for all UEs supporting eCall with eCall only subscription and capable of triggering a Test eCall.

13.3.1.2.2	Conformance Requirement

The eCall inactivity procedure is applicable only to an eCall only mobile station (as determined by information configured in USIM).

[..]

While in eCALL INACTIVE state, the mobile station maintains awareness of a potential serving cell in a potential serving network but initiates no MM signalling with the network and ignores any paging requests.

The mobile station shall leave eCALL INACTIVE state only when one of the following events occur:

[..]

-	if there is a CM request for a call to an HPLMN designated non-emergency MSISDN for the purpose of accessing test and terminal reconfiguration services: the mobile station attempts normal location updating. Once this is complete, further MM and CM procedures are used to establish the non-emergency call.

Reference(s)

TS24.008 clause 4.4.7

13.3.1.2.3	Test purpose

1.	Verify that the eCall capable UE with an ‘eCall only’ enabled USIM is capable of making a test eCall.

13.3.1.2.4	Method of test

Initial conditions

The eCall capable UE is equipped with ‘eCall only’ enabled  USIM

UE is powered on and in MM IDLE eCALL INACTIVE state. In this state the UE with an ‘eCall only’ subscription shall not be registered on a network but may perform a background scan to identify available PLMNs.

System Simulator

1 cell, default parameters

The UE is equipped with a USIM containing default values except for those listed below.

| USIM field | Value/Remark |
| --- | --- |
| EFUST | Service n°2 Service Dialling Numbers (FDN) and Service n°89 eCall Data available |
| EFFDN | Display two FDNs, eCall Test Number (123456) and eCall reconfiguration number (345678) |
| EFEST | Enabled Services Table |

Related ICS/IXIT statement(s)

UE supporting ‘eCall only’ support on the USIM yes/no

UE providing a means to trigger a Test eCall yes/no

Test Procedure

a)	The UE is made to initiate a test eCall in accordance with manufacturer’s instructions. Upon successful completion of the test eCall, the call is cleared by the SS. (steps 1 – 27).

Expected Sequence

| Step | Direction |  | Message | Comments |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | UE |  |  | The UE contains test USIM with data specified under initial conditions. |
| 2 |  |  |  | A test eCall is initiated using the number located in the EFFDN portion of the test USIM. Establishment cause: registration UE performs RRC procedure for network registration, LOCATION UPDATING |
| 3 |  |  | LOCATION UPDATING REQUEST | “Location Updating Type” = Normal Location Updating |
| 4 |  |  | AUTHENTICATION REQUEST | IE Authentication Parameter AUTN shall be present in the message. |
| 5 |  |  | AUTHENTICATION RESPONSE | SRES specifies correct value. |
| 6 |  |  |  | SS starts security procedure. |
| 7 |  |  | LOCATION UPDATING ACCEPT |  |
| 8 |  |  | TMSI REALLOCATION COMPLETE |  |
| 9 |  |  |  | SS releases the RRC Connection. |
| 10 |  |  |  | UE establishes RRC procedure for conversational call. Establishment cause: originating conversational call. |
| 11 |  |  | CM SERVICE REQUEST | The CM service type IE indicates "originating conversational call". |
| 12 |  |  | AUTHENTICATION REQUEST | IE Authentication Parameter AUTN shall be present in the message. |
| 13 |  |  | AUTHENTICATION RESPONSE | SRES specifies correct value. |
| 14 |  |  |  | SS starts security procedure. |
| 15 |  |  | SETUP | If the Bearer capability IE is not included the default UMTS AMR speech version shall be assumed. SS verifies that the test number is sent in the SETUP message. |
| 16 |  |  | CALL PROCEEDING |  |
| 17 |  |  | ALERTING |  |
| 18 |  |  |  | SS sets up the radio bearer with the rate indicated by the SETUP message. |
| 19 |  |  | CONNECT |  |
| 20 |  |  | CONNECT ACKNOWLEDGE |  |
| 21 | UE |  |  | The DTCH is through connected in both directions. Traffic channel is kept active for at least 5 seconds. |
| 22 |  |  | Void |  |
| 23 |  |  | DISCONNECT |  |
| 24 |  |  | RELEASE |  |
| 25 |  |  | RELEASE COMPLETE |  |
| 26 | SS |  |  | The SS releases the RRC connection |
| 27 |  |  | CALL C.1 | If the test result of C.1 indicates that UE is in idle Mode state, the test passes, otherwise it fails. |

Specific Message Contents

13.3.1.2.5	Test requirements

a)	In step 2 UE shall make a test eCall to a number stored in the EFFDN portion of the test USIM. UE shall establish RRC procedure with establishment cause “Registration”.

b)	In step 3 UE shall send a LOCATION UPDATING REQUEST message.

c)	In step 15 UE shall send a SETUP message.

d)	In step 21 UE has through connected the DTCH in both directions.

e)	In step 24 UE shall send a RELEASE message.

f)	In step 27 UE returns to Idle Mode.

#### 13.3.1.3	Manually initiated eCall using eCall capable UE with “eCall only” subscription on USIM

13.3.1.3.1	Definition and applicability

Applicable for all UEs supporting eCall and support eCall only subscription and capable of initiating manual eCall.

13.3.1.3.2	Conformance Requirement

[..]

If eCall only calls are supported, then EFFDN shall only contain two entries. The first entry shall contain the eCall test number and the second entry shall contain the eCall reconfiguration number. A terminal in eCall only mode performs the FDN related procedures.

[…]

Reference(s)

TS31.102 clause 5.3.40.1

13.3.1.3.3	Test purpose

1 Verify that the eCall capable UE is able to make Emergency Call - USIM has capability for eCall only subscriptions

13.3.1.3.4	Method of test

Initial conditions

The eCall capable UE is equipped with ‘eCall only’ enabled USIM. If  an eCall is to be established using emergency short codes, as specified in 24.008, then EFecc shall contain emergency short codes and associated Emergency Service Category bit 6 (MIeC) and bit 7 (AIeC) set to 1. Alternatively, AT+CECALL, as specified in TS27.007, commands may be used for eCall initiation.

UE is Switched OFF.

System Simulator

1 cell, default parameters

The UE is equipped with a USIM containing default values except for those listed below.

| USIM field | Value/Remark |
| --- | --- |
| EFUST | Service n°2 Fixed Dialling Numbers (FDN) and Service n°89 eCall Data available |
| EFFDN | Display two FDNs, eCall Test Number (123456) and eCall reconfiguration number (345678) |
| EFEST | Enabled Services Table |

Related ICS/IXIT statement(s)

UE supporting ‘eCall only’ support on the USIM yes/no

UE providing a means to trigger a manual eCall yes/no

Test procedure

a)	The UE is made to initiate an eCall in accordance with manufacturer’s instructions. Upon successful completion of the eCall, the call is cleared by the SS. (steps 1 – 29)

Expected sequence

| Step | Direction |  | Message | Comments |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | UE |  |  | The UE contains test USIM with data specified under initial conditions. UE is Switched ON. |
| 2 | SS |  |  | The SS verifies that the UE does not send RRC CONNECTION REQUEST message with the IE "Establishment cause" set to "Registration" for a period of 60 seconds. |
| 3 | UE |  |  | UE is made to initiate a manual eCall in accordance with manufacturer’s instructions |
| 4 |  |  |  | UE performs RRC procedure for network registration and LOCATION UPDATING |
| 5 |  |  | LOCATION UPDATING REQUEST | “Location Updating Type” = Normal Location Updating |
| 6 |  |  | AUTHENTICATION REQUEST | IE Authentication Parameter AUTN shall be present in the message. |
| 7 |  |  | AUTHENTICATION RESPONSE | SRES specifies correct value. |
| 8 |  |  |  | SS starts security procedure. |
| 9 |  |  | LOCATION UPDATING ACCEPT |  |
| 10 |  |  | TMSI REALLOCATION COMPLETE |  |
| 11 |  |  |  | SS releases the RRC Connection. |
| 12 |  |  |  | UE establishes RRC procedure for emergency call. Establishment cause: emergency call. |
| 13 |  |  | CM SERVICE REQUEST | The CM service type IE indicates "emergency call". |
| 14 |  |  | AUTHENTICATION REQUEST | IE Authentication Parameter AUTN shall be present in the message. |
| 15 |  |  | AUTHENTICATION RESPONSE | SRES specifies correct value. |
| 16 |  |  |  | SS starts security procedure. |
| 17 |  |  | EMERGENCY SETUP | Emergency category set to Manually Initiated eCall. If the Bearer capability IE is not included the default UMTS AMR speech version shall be assumed. SS verifies that the Emergency Service Category IE bit 6 is set to 1 and all other bits are set to 0. |
| 18 |  |  | CALL PROCEEDING |  |
| 19 |  |  | ALERTING |  |
| 20 |  |  |  | SS sets up the radio bearer with the rate indicated by the EMERGENCY SETUP message. |
| 21 |  |  | CONNECT |  |
| 22 |  |  | CONNECT ACKNOWLEDGE |  |
| 23 | UE |  |  | The DTCH is through connected in both directions. Traffic channel is kept alive for at least 5 seconds. |
| 24 |  |  | Void |  |
| 25 |  |  | DISCONNECT |  |
| 26 |  |  | RELEASE |  |
| 27 |  |  | RELEASE COMPLETE |  |
| 28 | SS |  |  | The SS releases the RRC connection |
| 29 |  |  | CALL C.1 | If the test result of C.1 indicates that UE is in MM IDLE state, the test passes, otherwise it fails. |

Specific Message Contents

EMERGENCY SETUP (Step 17)

| Information Element | Value/remark |
| --- | --- |
| Emergency Category | Bit 1 Police Bit 2 Ambulance Bit 3 Fire Brigade Bit 4 Marine Guard Bit 5 Mountain Rescue Bit 6 manually initiated eCall Bit 7 automatically initiated eCall Bit 8 Spare and set to "0" Manually initiated eCall: Bit 6 = 1 and all other bits set to 0 Automatically initiated eCall: Bit 6 = 0 and all other bits set to 0 |

13.3.1.3.5	Test requirements

a. In step 1 UE is powered on.

b. In step 2 UE shall remain in the eCALL INACTIVE state and shall not perform RRC procedures for network registration until an eCall is initiated.

c. In step 4 UE shall perform RRC procedures for network registration and Location Updating.

d. In step 5 UE shall send a LOCATION UPDATING REQUEST message.

e. In step 17 UE shall send an EMERGENCY SETUP message with the Emergency Service Category IE bit 6 = 1 and all other bits set to 0.

f. In step 23 UE has through connected the DTCH in both directions.

g. In step 26 UE shall send a RELEASE message.

h. In step 29 UE returns to Idle Mode.

#### 13.3.1.4	Reconfiguration eCall using eCall capable UE with an ‘eCall only’ subscription on USIM

13.3.1.4.1	Definition and applicability

Applicable for all UEs supporting eCall and support ‘eCall only’ subscription and capable of triggering a reconfiguration eCall.

13.3.1.4.2	Conformance Requirement

The eCall inactivity procedure is applicable only to an eCall only mobile station (as determined by information configured in USIM).

[..]

While in eCALL INACTIVE state, the mobile station maintains awareness of a potential serving cell in a potential serving network but initiates no MM signalling with the network and ignores any paging requests.

The mobile station shall leave eCALL INACTIVE state only when one of the following events occur:

[..]

-	if there is a CM request for a call to an HPLMN designated non-emergency MSISDN for the purpose of accessing test and terminal reconfiguration services: the mobile station attempts normal location updating. Once this is complete, further MM and CM procedures are used to establish the non-emergency call.

Reference(s)

TS24.008 clause 4.4.7

13.3.1.4.3	Test purpose

1. Verify that the eCall capable UE with an ‘eCall only’ subscription is capable of making a reconfiguration eCall, to facilitate removal of the ‘eCall only’ restrictions on the USIM and permit access to additional subscription services.

13.3.1.4.4	Method of test

Initial conditions

The eCall capable UE is equipped with ‘eCall only’ USIM

UE is powered on and in MM IDLE eCALL INACTIVE state. In this state the UE with an 'eCall only’ subscription shall not be registered on a network but may perform a background scan to identify available PLMNs.

System Simulator

1 cell, default parameters,

The UE is equipped with a USIM containing default values except for those listed below.

| USIM field | Value/Remark |
| --- | --- |
| EFUST | Service n°2 Service Dialling Numbers (FDN) and Service n°89 eCall Data available |
| EFFDN | Display two FDNs, eCall Test Number (123456) and eCall reconfiguration number (345678) |
| EFEST | Enabled Services Table |

Related ICS/IXIT statement(s)

UE supporting ‘eCall only’ support on the USIM 	yes/no

UE providing a means to trigger a reconfiguration eCall    yes/no

Test Procedure

a) The UE is made to initiate a reconfiguration eCall in accordance with manufacturer’s instructions. Upon successful completion of the reconfiguration eCall, the call is cleared by the SS. (steps 1 – 20)

Expected Sequence

| Step | Direction |  | Message | Comments |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | UE |  |  | The UE contains test USIM with data specified under initial conditions. |
| 2 |  |  |  | UE establishes RRC procedures for network registration and a reconfiguration eCall using number located in the EFFDN portion of the test USIM. Establishment cause: registration |
| 3 |  |  | LOCATION UPDATING REQUEST | “Location Updating Type” = Normal Location Updating |
| 4 |  |  | AUTHENTICATION REQUEST | IE Authentication Parameter AUTN shall be present in the message. |
| 5 |  |  | AUTHENTICATION RESPONSE | SRES specifies correct value. |
| 6 |  |  |  | SS starts security procedure. |
| 7 |  |  | LOCATION UPDATING ACCEPT |  |
| 8 |  |  | TMSI REALLOCATION COMPLETE |  |
| 9 |  |  |  | SS releases the RRC Connection. |
| 10 |  |  |  | UE establishes RRC procedure for conversational call. Establishment cause: originating conversational call. |
| 11 |  |  | CM SERVICE REQUEST | The CM service type IE indicates "originating conversational call". |
| 12 |  |  | AUTHENTICATION REQUEST | IE Authentication Parameter AUTN shall be present in the message. |
| 13 |  |  | AUTHENTICATION RESPONSE | SRES specifies correct value. |
| 14 |  |  |  | SS starts security procedure. |
| 15 |  |  | SETUP | If the Bearer capability IE is not included the default UMTS AMR speech version shall be assumed. SS verifies that the reconfiguration number is sent in the SETUP message. |
| 16 |  |  | CALL PROCEEDING |  |
| 17 |  |  | ALERTING |  |
| 18 |  |  |  | SS sets up the radio bearer with the rate indicated by the SETUP message. |
| 19 |  |  | CONNECT |  |
| 20 |  |  | CONNECT ACKNOWLEDGE |  |
| 21 | UE |  |  | The DTCH is through connected in both directions. Traffic channel is kept alive for at least 5 seconds. |
| 22 | Void |  |  |  |
| 23 |  |  | DISCONNECT |  |
| 24 |  |  | RELEASE |  |
| 25 |  |  | RELEASE COMPLETE |  |
| 26 | SS |  |  | The SS releases the RRC connection |

Specific Message Contents

None.

13.3.1.4.5	Test requirements

a)	In step 2 UE shall make a reconfiguration eCall to a number stored in the EFFDN portion of the test USIM. The UE shall establish RRC procedures for network registration and with establishment cause “Registration”.

b)	In step 3 UE shall send a LOCATION UPDATING REQUEST message.

c)	In step 15 UE shall send a SETUP message.

d)	In step 21 UE has through connected the DTCH in both directions.

e)	In step 24 UE shall send a RELEASE message.

#### 13.3.1.5	Manually initiated eCall using eCall capable UE with eCall and non eCall subscriptions on USIM

13.3.1.5.1	Definition and applicability

Applicable for all UEs supporting eCall and support emergency speech call and eCall subscription and capable of initiating manual eCall.

13.3.1.5.2	Conformance Requirement

[..]

If eCall and normal calls are supported, then the last two entries of EFSDN shall contain the eCall test number and the eCall reconfiguration number respectively. A terminal in eCall and normal mode performs the SDN related procedures.

Reference(s)

TS31.102 clause 5.3.40.2

13.3.1.5.3	Test purpose

1.	Verify that the eCall capable UE with both eCall and non eCall subscriptions on the USIM is able to make a manually initiated eCall.

13.3.1.5.4	Method of test

Initial conditions

The eCall capable UE is equipped with eCall Capable USIM that permits access to eCall and non-eCall subscription services. If  an eCall is to be established using emergency short codes, as specified in 24.008,  then EFecc shall contain emergency short codes and  associated Emergency Service Category bit 6 (MIeC) and bit 7 (AIeC) set to 1. Alternatively, AT+CECALL, as specified in TS27.007, commands may be used for eCall initiation.

UE is switched on, is registered on a PLMN and is in MM IDLE state.

System Simulator

1 cell, default parameters

The UE is equipped with a USIM containing default values except for those listed below.

| USIM field | Value/Remark |
| --- | --- |
| EFUST | Service n°4 Service Dialling Numbers (SDN) and Service n°89 eCall Data available |
| EFSDN | Last two entries of SDNs, eCall Test Number (123456) and eCall reconfiguration number (345678) |
| EFEST | Enabled Services Table |

Related ICS/IXIT statement(s)

UE supporting eCall Capable support on the USIM yes/no

UE providing a means to trigger a manual eCall yes/no

Test Procedure

a)	The UE is made to initiate an eCall in accordance with manufacturer’s instructions. Upon successful completion of the eCall, the call is cleared by the SS. (steps 1 – 20).

Expected sequence

| Step | Direction |  | Message | Comments |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | UE |  |  | The UE contains test USIM with data specified under initial conditions. UE is made to initiate a manual eCall in accordance with manufacturer’s instructions. |
| 2 |  |  |  | UE performs RRC procedure with cause “Emergency call”. |
| 3 |  |  | CM SERVICE REQUEST | The CM service type IE indicates "emergency call establishment". |
| 4 |  |  | AUTHENTICATION REQUEST | IE Authentication Parameter AUTN shall be present in the message. |
| 5 |  |  | AUTHENTICATION RESPONSE | SRES specifies correct value. |
| 6 |  |  |  | SS starts security procedure. |
| 7 |  |  | Void |  |
| 8 |  |  | Void |  |
| 9 |  |  | EMERGENCY SETUP | Emergency category set to Manually Initiated eCall. If the Bearer capability IE is not included the default UMTS AMR speech version shall be assumed. SS verifies that the Emergency Service Category IE bit 6 is set to 1 and all other bits are set to 0. |
| 10 |  |  | CALL PROCEEDING |  |
| 11 |  |  | ALERTING |  |
| 12 |  |  |  | SS sets up the radio bearer with the rate indicated by the EMERGENCY SETUP message. |
| 13 |  |  | CONNECT |  |
| 14 |  |  | CONNECT ACKNOWLEDGE |  |
| 15 | UE |  |  | The DTCH is through connected in both directions. Traffic channel is kept active for at least 5 seconds. |
| 16 |  |  | Void |  |
| 17 |  |  | DISCONNECT |  |
| 18 |  |  | RELEASE |  |
| 19 |  |  | RELEASE COMPLETE |  |
| 20 | SS |  |  | The SS releases the RRC connection |

Specific Message Contents

EMERGENCY SETUP (Step 9)

| Information Element | Value/remark |
| --- | --- |
| Emergency Category | Bit 1 Police Bit 2 Ambulance Bit 3 Fire Brigade Bit 4 Marine Guard Bit 5 Mountain Rescue Bit 6 manually initiated eCall Bit 7 automatically initiated eCall Bit 8 Spare and set to "0" Manually initiated eCall: Bit 6 = 1 and all other bits set to 0 Automatically initiated eCall: Bit 6 = 0 and all other bits set to 0 |

13.3.1.5.5	Test requirements

a. In step 1 UE is made to initiate a manual eCall in accordance with manufacturer’s instructions.

b. In step 2 UE shall perform RRC procedure for Emergency Call.

c. In step 3 UE shall send a CM SERVICE REQUEST message.

d. In step 9 UE shall send an EMERGENCY SETUP message with the Emergency Service Category IE bit 6 = 1 and all other bits set to.

e. In step 15 UE has through connected the DTCH in both directions.

f. In step 18 UE shall send a RELEASE message.

#### 13.3.1.6	eCall Inactivity State after T3242 expires

13.3.1.6.1	Definition and applicability

Applicable for all UEs supporting eCall and ‘eCall only’ subscription and capable of initiating manual eCall.

13.3.1.6.2	Conformance Requirement

[..]

The eCall inactivity procedure is applicable only to an ‘eCall only’ mobile station (as determined by information configured in USIM). The procedure shall be started when timer T3242 or timer T3243 expires or is found to have already expired in any MM Idle state except NO IMSI, NO CELL AVAILABLE or PLMN SEARCH. The mobile station shall then stop other running timers (e.g. T3211, T3212, T3213) and shall perform the IMSI detach procedure if required by the serving network and if the update state is U1. The mobile station then enters MM Idle eCALL INACTIVE state and the mobile station shall delete any LAI, TMSI, ciphering key sequence number stored in the SIM/USIM and set the update state to U4 Updating Disabled.

While in eCALL INACTIVE state, the mobile station maintains awareness of a potential serving cell in a potential serving network but initiates no MM signalling with the network and ignores any paging requests.

The mobile station shall leave eCALL INACTIVE state only when one of the following events occur:

-	if the SIM or USIM is removed, the mobile station enters the NO IMSI state;

-	if coverage is lost, the mobile station enters PLMN SEARCH state;

-	if the mobile station is deactivated (e.g. powered off) by the user: the mobile station enters the NULL state;

-	if there is a CM request for an emergency services call: the MS uses the MM and CM procedures to establish the emergency call; or

-	if there is a CM request for a call to an HPLMN designated non-emergency MSISDN for the purpose of accessing test and terminal reconfiguration services: the mobile station attempts normal location updating. Once this is complete, further MM and CM procedures are used to establish the non-emergency call.

Reference(s)

TS24.008 clause 4.4.7

13.3.1.6.3	Test purpose

1 Verify that ‘eCall only’ capable UE enters eCall inactivity state after T3242 expires and leaves this state only when specified events occur.

13.3.1.6.4	Method of test

Initial conditions

The ‘eCall only’ capable UE is equipped with ‘eCall only’ enabled USIM. If  an eCall is to be established using emergency short codes, as specified in 24.008,  then EFecc shall contain emergency short codes and  associated Emergency Service Category bit 6 (MIeC) and bit 7 (AIeC) set to 1. Alternatively, AT+CECALL, as specified in TS27.007, commands may be used for eCall initiation.

UE is in eCall inactive eCALL INACTIVE state

System Simulator

- 1 cell, default parameters,

- the T3212 time-out value is 252 minutes.

The UE is equipped with a USIM containing default values except for those listed below.

| USIM field | Value/Remark |
| --- | --- |
| EFUST | Service n°2 Service Dialling Numbers (FDN) and Service n°89 eCall Data available |
| EFFDN | Last two entries of FDNs, eCall Test Number (123456) and eCall reconfiguration number (345678) |
| EFEST | Enabled Services Table |

Related ICS/IXIT statement(s)

UE supporting ‘eCall only’ enabled USIM yes/no

UE providing a means to trigger a manual eCall yes/no

Test Procedure

a)	The UE is made to initiate an eCall in accordance with manufacturer’s instructions. Upon successful completion of the eCall, the call is cleared by the SS. (steps 1 – 48).

Expected sequence

| Step | Direction |  | Message | Comments |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | UE |  |  | The UE contains test USIM with data specified under initial conditions. UE is made to initiate a manual eCall in accordance with manufacturer’s instructions. |
| 2 |  |  |  | UE performs RRC procedures for network registration and Location Updating. |
| 3 |  |  | LOCATION UPDATING REQUEST | “LOCATION UPDATING TYPE”= Normal Location Updating |
| 4 |  |  | AUTHENTICATION REQUEST | IE Authentication Parameter AUTN shall be present in the message. |
| 5 |  |  | AUTHENTICATION RESPONSE | SRES specifies correct value. |
| 6 |  |  |  | SS starts security procedure. |
| 7 |  |  | LOCATION UPDATING ACCEPT |  |
| 8 |  |  | TMSI REALLOCATION COMPLETE |  |
| 9 |  |  |  | SS releases the RRC Connection. |
| 10 |  |  |  | UE establishes RRC procedure for emergency call. Establishment cause: emergency call. |
| 11 |  |  | CM SERVICE REQUEST | The CM service type IE indicates " emergency call". |
| 12 |  |  | AUTHENTICATION REQUEST | IE Authentication Parameter AUTN shall be present in the message. |
| 13 |  |  | AUTHENTICATION RESPONSE | SRES specifies correct value. |
| 14 |  |  |  | SS starts security procedure. |
| 15 |  |  | EMERGENCY SETUP | Emergency category set to Manually Initiated eCall. If the Bearer capability IE is not included the default UMTS AMR speech version shall be assumed. SS verifies that the Emergency Service Category IE bit 6 is set to 1 and all other bits are set to 0. |
| 16 |  |  | CALL PROCEEDING |  |
| 17 |  |  | ALERTING |  |
| 18 |  |  |  | SS sets up the radio bearer with the rate indicated by the EMERGENCY SETUP message. |
| 19 |  |  | CONNECT |  |
| 20 |  |  | CONNECT ACKNOWLEDGE |  |
| 21 | UE |  |  | The DTCH is connected in both directions. Traffic channel is kept active for at least 5 seconds. |
| 22 | Void |  |  |  |
| 23 |  |  | DISCONNECT |  |
| 24 |  |  | RELEASE |  |
| 25 |  |  | RELEASE COMPLETE |  |
| 26 | SS |  |  | The SS releases the RRC Connection |
| 27 | SS |  |  | For next 12 hours SS verifies that UE sends Location Update Request periodically every 252 minutes and remains registered until T3242 expires |
| 27A |  |  | IMSI DETACH INDICATION | The UE performs IMSI Detach after timer T3242 expires |
| 28 | SS |  |  | The SS releases the RRC connection |
| 29 | UE |  |  | UE is made to initiate a manual eCall in accordance with manufacturer’s instructions. |
| 30 |  |  |  | UE performs RRC procedure for Location Updating. |
| 31 |  |  | LOCATION UPDATING REQUEST | “LOCATION UPDATING TYPE”= Normal Location Updating |
| 32 |  |  | AUTHENTICATION REQUEST | IE Authentication Parameter AUTN shall be present in the message. |
| 33 |  |  | AUTHENTICATION RESPONSE | SRES specifies correct value. |
| 34 |  |  |  | SS starts security procedure. |
| 35 |  |  | LOCATION UPDATING ACCEPT |  |
| 36 |  |  | TMSI REALLOCATION COMPLETE |  |
| 37 |  |  | EMERGENCY SETUP | Emergency category set to Manually Initiated eCall. If the Bearer capability IE is not included the default UMTS AMR speech version shall be assumed. SS verifies that the Emergency Service Category IE bit 6 is set to 1 and all other bits are set to 0. |
| 38 |  |  | CALL PROCEEDING |  |
| 39 |  |  | ALERTING |  |
| 40 |  |  |  | SS sets up the radio bearer with the rate indicated by the EMERGENCY SETUP message. |
| 41 |  |  | CONNECT |  |
| 42 |  |  | CONNECT ACKNOWLEDGE |  |
| 43 | UE |  |  | The DTCH is connected in both directions. Traffic channel is kept active for at least 5 seconds. |
| 44 | Void |  |  |  |
| 45 |  |  | DISCONNECT |  |
| 46 |  |  | RELEASE |  |
| 47 |  |  | RELEASE COMPLETE |  |
| 48 | SS |  |  | The SS releases the RRC Connection |

Specific Message Contents

EMERGENCY SETUP (Steps 15 and 37)

| Information Element | Value/remark |
| --- | --- |
| Emergency Category | Bit 1 Police Bit 2 Ambulance Bit 3 Fire Brigade Bit 4 Marine Guard Bit 5 Mountain Rescue Bit 6 manually initiated eCall Bit 7 automatically initiated eCall Bit 8 Spare and set to "0" Manually initiated eCall: Bit 6 = 1 and all other bits set to 0 Automatically initiated eCall: Bit 6 = 0 and all other bits set to 0 |

13.3.1.6.5	Test requirements

a. In step 1 UE is made to initiate a manual eCall in accordance with manufacturer’s instructions.

b. In step 2 UE shall perform RRC procedure for network registration and Location Updating.

c. In step 3 UE shall send a LOCATION UPDATING REQUEST message.

d. In step 15 UE shall send an EMERGENCY SETUP message with Emergency Service Category IE bit 6 = 1 and all other bits set to 0.

e. In step 21 UE has through connected the DTCH in both directions.

f. In step 24 UE shall send a RELEASE message.

g. In steps 27 UE shall remain registered on the network for a duration of T3242 following completion of eCall. UE shall detach from network upon expiry of T3242.

h. In step 29 UE is made to initiate a manually initiated eCall in accordance with manufacturer’s instructions.

i. In step 31 UE shall send a LOCATION UPDATING REQUEST message with “Location Updating Type” = Normal Location Updating.

j. In step 37 UE shall send an EMERGENCY SETUP message with Emergency Service Category IE bit 6 = 1 and all other bits set to 0.

k. In step 43 UE has through connected the DTCH in both directions.

l. In step 46 UE shall send a RELEASE message.

#### 13.3.1.7	Automatically initiated eCall

13.3.1.7.1	Definition and applicability

Applicable for all UEs supporting eCall and support emergency speech and eCall only  subscription and capable of initiating automatic eCall.

13.3.1.7.2	Conformance Requirement

[..]

If eCall only calls are supported, then EFFDN shall only contain two entries. The first entry shall contain the eCall test number and the second entry shall contain the eCall reconfiguration number. A terminal in eCall only mode performs the FDN related procedures

Reference(s)

TS31.102 clause 5.3.40.2

13.3.1.7.3	Test purpose

Verify that while making eCall automatically, eCall capable UE provides appropriate Service Category information to the network.( Bit 7 should be set).

13.3.1.7.4	Method of test

Initial conditions

The eCall capable UE is equipped with an eCall only enabled USIM. If  an eCall is to be established using emergency short codes, as specified in 24.008,  then EFecc shall contain emergency short codes and  associated Emergency Service Category bit 6 (MIeC) and bit 7 (AIeC) set to 1. Alternatively, AT+CECALL, as specified in TS27.007, commands may be used for eCall initiation.

UE is in eCall Inactive State.

System Simulator

1 cell, default parameters.

The UE is equipped with a USIM containing default values except for those listed below.

| USIM field | Value/Remark |
| --- | --- |
| EFUST | Service n°4 Service Dialling Numbers (FDN) and Service n°89 eCall Data available |
| EFFDN | Last two entries of FDNs, eCall Test Number (123456) and eCall reconfiguration number (345678) |
| EFEST | Enabled Services Table |

Related ICS/IXIT statement(s)

UE supporting eCall Only Capable support on the USIM yes/no

UE providing a means to trigger an automatic eCall yes/no

Test Procedure

a) The UE is made to initiate an eCall in accordance with manufacturer’s instructions. Upon successful completion of the eCall, the call is cleared by the SS. (steps 1 – 26)

Expected sequence

| Step | Direction |  | Message | Comments |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | UE |  |  | The UE contains test USIM with data specified under initial conditions. UE is made to initiate an automatic eCall in accordance with manufacturer’s instructions. |
| 2 |  |  |  | UE performs RRC procedure for Location Updating |
| 3 |  |  | LOCATION UPDATING REQUEST | “Location Updating Type” = Normal Location Updating |
| 4 |  |  | AUTHENTICATION REQUEST | IE Authentication Parameter AUTN shall be present in the message. |
| 5 |  |  | AUTHENTICATION RESPONSE | SRES specifies correct value. |
| 6 |  |  |  | SS starts security procedure. |
| 7 |  |  | LOCATION UPDATING ACCEPT |  |
| 8 |  |  | TMSI REALLOCATION COMPLETE |  |
| 9 |  |  |  | SS releases the RRC Connection. |
| 10 |  |  |  | UE establishes RRC procedure for emergency call. Establishment cause: emergency call. |
| 11 |  |  | CM SERVICE REQUEST | The CM service type IE indicates " emergency call". |
| 12 |  |  | AUTHENTICATION REQUEST | IE Authentication Parameter AUTN shall be present in the message. |
| 13 |  |  | AUTHENTICATION RESPONSE | SRES specifies correct value. |
| 14 |  |  |  | SS starts security procedure. |
| 15 |  |  | EMERGENCY SETUP | Emergency category set to Automatic Initiated eCall. If the Bearer capability IE is not included the default UMTS AMR speech version shall be assumed. SS verifies that the Emergency Service Category IE bit 7 is set to 1 and all other bits are set to 0. |
| 16 |  |  | CALL PROCEEDING |  |
| 17 |  |  | ALERTING |  |
| 18 |  |  |  | SS sets up the radio bearer with the rate indicated by the EMERGENCY SETUP message. |
| 19 |  |  | CONNECT |  |
| 20 |  |  | CONNECT ACKNOWLEDGE |  |
| 21 | UE |  |  | The DTCH is through connected in both directions. Traffic channel is kept active for at least 5 seconds. |
| 22 |  |  | Void |  |
| 23 |  |  | DISCONNECT |  |
| 24 |  |  | RELEASE |  |
| 25 |  |  | RELEASE COMPLETE |  |
| 26 | SS |  |  | The SS releases the RRC connection |

Specific Message Contents

EMERGENCY SETUP (Step 15)

| Information Element | Value/remark |
| --- | --- |
| Emergency Category | Bit 1 Police Bit 2 Ambulance Bit 3 Fire Brigade Bit 4 Marine Guard Bit 5 Mountain Rescue Bit 6 manually initiated eCall Bit 7 automatically initiated eCall Bit 8 Spare and set to "0" Manually initiated eCall: Bit 6 = 1 and all other bits set to 0 Automatically initiated eCall: Bit 6 = 0 and all other bits set to 0 |

13.3.1.7.5	Test requirements

a)	In step 1 UE is made to initiate an automatic eCall in accordance with manufacturer’s instructions.

b)	In step 2 UE shall perform RRC procedure for network registration and Location Updating.

c)	In step 3 UE shall send a LOCATION UPDATING REQUEST message.

d)	In step 15 UE shall send an EMERGENCY SETUP message with the Emergency Service Category IE bit 7 = 1 and all other bits set to.

e)	In step 21 UE has through connected the DTCH in both directions.

f)	In step 24 UE shall send a RELEASE message.

#### 13.3.1.8	Void

#### 13.3.1.9	Void

#### 13.3.1.10	eCall Inactivity State after T3243 expires

13.3.1.10.1	Definition and applicability

Applicable for all UEs supporting eCall and ‘eCall only’ subscription and capable of triggering a Test eCall.

13.3.1.10.2	Conformance Requirement

[..]

The eCall inactivity procedure is applicable only to an ‘eCall only’ mobile station (as determined by information configured in USIM). The procedure shall be started when timer T3242 or timer T3243 expires or is found to have already expired in any MM Idle state except NO IMSI, NO CELL AVAILABLE or PLMN SEARCH.

[..]

While in eCALL INACTIVE state, the mobile station maintains awareness of a potential serving cell in a potential serving network but initiates no MM signalling with the network and ignores any paging requests.

The mobile station shall leave eCALL INACTIVE state only when one of the following events occur:

[..]

--	if there is a CM request for a call to an HPLMN designated non-emergency MSISDN for the purpose of accessing test and terminal reconfiguration services: the mobile station attempts normal location updating. Once this is complete, further MM and CM procedures are used to establish the non-emergency call.

Reference(s)

3GPP TS 24.008 clause 4.4.7

13.3.1.10.3	Test purpose

1 Verify that ‘eCall only’ capable UE enters eCall inactivity state after T3243 expires and leaves this state only when specified events occur.

13.3.1.10.4	Method of test

Initial conditions

The ‘eCall only’ capable UE is equipped with ‘eCall only’ enabled USIM. If  an eCall is to be established using emergency short codes, as specified in 24.008,  then EFecc shall contain emergency short codes and  associated Emergency Service Category bit 6 (MIeC) and bit 7 (AIeC) set to 1. Alternatively, AT+CECALL, as specified in TS27.007, commands may be used for test eCall initiation.

UE is in eCALL INACTIVE state

System Simulator

- 1 cell, default parameters,

- the T3212 time-out value is 252 minutes.

The UE is equipped with a USIM containing default values except for those listed below.

| USIM field | Value/Remark |
| --- | --- |
| EFUST | Service n°2 Service Dialling Numbers (FDN) and Service n°89 eCall Data available |
| EFFDN | Last two entries of FDNs, eCall Test Number (123456) and eCall reconfiguration number (345678) |
| EFEST | Enabled Services Table |

Related ICS/IXIT statement(s)

UE supporting ‘eCall only’ support on the USIM yes/no

UE providing a means to trigger a test eCall yes/no

Test Procedure

- The UE is powered on. SS monitors to verify that UE does not attempt to register for a period of 120 seconds.

- The UE is made to initiate a test eCall  in accordance with manufacturer’s instructions. SS checks that UE performs registration before starting test eCall.

- The UE, having  reached  the ACTIVE state, call is cleared by the SS. SS monitors to verify that UE remains registered for the duration of T3243 (12 Hours) and sends periodic LAU messages as T3212 timer is set to 252 minutes.

- Upon expiry of timer T3243, the UE shall perform IMSI detach.

Expected sequence

| Step | Direction |  | Message | Comments |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | UE |  |  | The UE contains test USIM with data specified under initial conditions. The UE is switched on. |
| 2 | SS |  |  | The SS verifies for 120 sec that the UE does not send RACH or any other message. |
| 3 | UE |  |  | UE is made to initiate a test eCall in accordance with manufacturer’s instructions. |
| 4 |  |  |  | UE performs RRC procedures for network registration and Location Updating. |
| 5 |  |  | LOCATION UPDATING REQUEST | “LOCATION UPDATING TYPE”= Normal Location Updating |
| 6 |  |  | AUTHENTICATION REQUEST | IE Authentication Parameter AUTN shall be present in the message. |
| 7 |  |  | AUTHENTICATION RESPONSE | SRES specifies correct value. |
| 8 |  |  |  | SS starts security procedure. |
| 9 |  |  | LOCATION UPDATING ACCEPT |  |
| 10 |  |  | TMSI REALLOCATION COMPLETE |  |
| 11 |  |  |  | SS releases the RRC Connection. |
| 12 |  |  |  | UE establishes RRC procedure for conversational call. Establishment cause: originating conversational call. |
| 13 |  |  | CM SERVICE REQUEST | The CM service type IE indicates " originating conversational call". |
| 14 |  |  | AUTHENTICATION REQUEST | IE Authentication Parameter AUTN shall be present in the message. |
| 15 |  |  | AUTHENTICATION RESPONSE | SRES specifies correct value. |
| 16 |  |  |  | SS starts security procedure. |
| 17 |  |  | SETUP | If the Bearer capability IE is not included the default UMTS AMR speech version shall be assumed. SS verifies that the test number is sent in the SETUP message. |
| 18 |  |  | CALL PROCEEDING |  |
| 19 |  |  | ALERTING |  |
| 20 |  |  |  | SS sets up the radio bearer with the rate indicated by the SETUP message. |
| 21 |  |  | CONNECT |  |
| 22 |  |  | CONNECT ACKNOWLEDGE |  |
| 23 | UE |  |  | The DTCH is connected in both directions. Traffic channel is kept active for at least 5 seconds. |
| 24 |  |  | DISCONNECT |  |
| 25 |  |  | RELEASE |  |
| 26 |  |  | RELEASE COMPLETE |  |
| 27 | SS |  |  | The SS releases the RRC Connection |
| 28 | SS |  |  | For next 12 hours SS verifies that UE sends Location Update Request periodically every 252 minutes and remains registered until T3243 expires UE performs IMSI Detach after timer T3243 expires |
| 29 |  |  | IMSI DETACH INDICATION | "mobile identity" contains IMSI of UE. |
| 30 | SS |  |  | The SS releases the RRC connection |

13.3.1.10.5	Test requirements

a. In step 3 UE is made to initiate a test eCall in accordance with manufacturer’s instructions.

b. In step 4 UE shall perform RRC procedure for network registration and Location Updating.

c. In step 5 UE shall send a LOCATION UPDATING REQUEST message.

d. In step 17 UE shall send a SETUP message.

e. In step 23 UE has through connected the DTCH in both directions.

f. In step 25 UE shall send a RELEASE message.

g. In step 28 UE shall remain registered on the network for a duration of T3243 following completion of test eCall. UE shall detach from network upon expiry of T3243.

h. In step 29 UE shall send a IMSI DETACH INDICATION message.

### <End of modifed section>