3GPP TSG-RAN WG5 Meeting # 95-e	R5-223234

# Electronic meeting, 9 – 20 May, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2411 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial correction to 4.3.1.2.2 on test frequencies for NR inter-band CA configurations in FR2 for CA_n260-n261 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To reduce redundancy, editorial corrections to the test frequencies for NR inter-band CA configurations for CA_n260-n261 are needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the test frequencies for CA_ n260-n261. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test frequencies for CA_n260-n261 are not clear and redundant. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

<< Unchanged sections omitted >>

4.3.1.2.2	NR inter-band CA configurations in FR2

Table 4.3.1.2.2-1: NR inter-band CA configurations in FR2

| NR CA configuration | Uplink NR CA configuration | NR CA downlink configuration band 1 | NR CA downlink configuration band 2 | NR CA uplink configuration band 1 | NR CA uplink configuration band 2 | Applicable for protocol testing (Note 1) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n260A-n261A |  | - |  | n260A |  | n261A |  | - |  | - |  | Yes |  |
| CA_n260A-n261M |  | - |  | n260A |  | CA_n261M |  | - |  | - |  | No |  |
|  |  | CA_n261M |  | n260A |  | CA_n261M |  | - |  | CA_n261M |  | No |  |
| CA_n260G-n261M |  | - |  | CA_n260G |  | CA_n261M |  | - |  | - |  | No |  |
|  |  | CA_n260G |  | CA_n260G |  | CA_n261M |  | CA_n260G |  | - |  | No |  |
|  |  | CA_n261M |  | CA_n260G |  | CA_n261M |  | - |  | CA_n261M |  | No |  |
| CA_n260H-n261M |  | - |  | CA_n260H |  | CA_n261M |  | - |  | - |  | No |  |
|  |  | CA_n260H |  | CA_n260H |  | CA_n261M |  | CA_n260H |  | - |  | No |  |
|  |  | CA_n261M |  | CA_n260H |  | CA_n261M |  | - |  | CA_n261M |  | No |  |
| CA_n260I-n261M |  | - |  | CA_n260I |  | CA_n261M |  | - |  | - |  | No |  |
|  |  | CA_n260I |  | CA_n260I |  | CA_n261M |  | CA_n260I |  | - |  | No |  |
|  |  | CA_n261M |  | CA_n260I |  | CA_n261M |  | - |  | CA_n261M |  | No |  |
| CA_n260J-n261M |  | - |  | CA_n260J |  | CA_n261M |  | - |  | - |  | No |  |
|  |  | CA_n260J |  | CA_n260J |  | CA_n261M |  | CA_n260J |  | - |  | No |  |
|  |  | CA_n261M |  | CA_n260J |  | CA_n261M |  | - |  | CA_n261M |  | No |  |
| CA_n260K-n261M |  | - |  | CA_n260K |  | CA_n261M |  | - |  | - |  | No |  |
|  |  | CA_n260K |  | CA_n260K |  | CA_n261M |  | CA_n260K |  | - |  | No |  |
|  |  | CA_n261M |  | CA_n260K |  | CA_n261M |  | - |  | CA_n261M |  | No |  |
| CA_n260L-n261M |  | - |  | CA_n260L |  | CA_n261M |  | - |  | - |  | No |  |
|  |  | CA_n260L |  | CA_n260L |  | CA_n261M |  | CA_n260L |  | - |  | No |  |
|  |  | CA_n261M |  | CA_n260L |  | CA_n261M |  | - |  | CA_n261M |  | No |  |
| CA_n260M-n261M |  | - |  | CA_n260M |  | CA_n261M |  | - |  | - |  | No |  |
|  |  | CA_n260M |  | CA_n260M |  | CA_n261M |  | CA_n260M |  | - |  | No |  |
|  |  | CA_n261M |  | CA_n260M |  | CA_n261M |  | - |  | CA_n261M |  | No |  |
| Note 1: Protocol testing is limited to NR CA configurations with 2CC. |  |  |  |  |  |  |

==============================================================

<< End of changes >>