3GPP TSG-RAN5 Meeting #95-e	R5-223253

# Electronic Meeting, 9th May– 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0348 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction pc_dynamicPowerSharing to align with 38.306 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Google, Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - From the table A.4.3.2-1: UE Physical Layer Baseline Implementation Capabilities of 38.508-2, the Mnemonic of item 17 is pc_dynamicPowerSharing. Table A.4.3.2-1: UE Physical Layer Baseline Implementation Capabilities Item; UE Physical Layer Baseline Implementation Capabilities; Ref.; Release; Mnemonic; M; If indicated “Yes” the feature shall be implemented and successfully tested for the corresponding release; Comments. 17; Support dynamic EN-DC power sharing for at least one EN-DC band combination; 38.306, 4.2.7.9; Rel-15; pc_dynamicPowerSharing; Yes; If the UE supports this capability it will dynamically share the power between NR and LTE if P_LTE + P_NR > Pcmax.. - In the 4.2.7.9 of 38.306, the Mnemonic of definition should be dynamicPowerSharingENDC, and it is for FR1 only. #### 4.2.7.9 MRDC-Parameters Definitions for parameters; Per; M; FDD-TDD DIFF; FR1-FR2 DIFF. dynamicPowerSharingENDC Indicates whether the UE supports dynamic (NG)EN-DC power sharing between NR FR1 carriers and the LTE carriers. If the UE supports this capability the UE supports the dynamic power sharing behaviour as specified in clause 7 of TS 38.213 [11]. In this release of the specification, the UE supporting (NG)EN-DC shall set this field to supported.; BC; Yes; N/A; FR1 only. |
| Summary of change: |  | - Additional FR1 only in the description. - Correction the Mnemonic to pc_dynamicPowerSharingENDC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | dynamic EN-DC power sharing would be referenced incorrectly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.2-1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

### A.4.3.2	Physical Layer Baseline Implementation Capabilities

Table A.4.3.2-1: UE Physical Layer Baseline Implementation Capabilities

| Item | UE Physical Layer Baseline Implementation Capabilities | Ref. | Release | Mnemonic | M | If indicated “Yes” the feature shall be implemented and successfully tested for the corresponding release | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ⋮ | (Unchanged Sections Skipped) |  |  |  |  |  |  |
| 17 | Support dynamic EN-DC power sharing for at least one EN-DC band combination_FR1 only | 38.306, 4.2.7.9 | Rel-15 | pc_dynamicPowerSharingENDC | Yes |  | If the UE supports this capability it will dynamically share the power between NR and LTE if P_LTE + P_NR > Pcmax. |
| ⋮ | (Unchanged Sections Skipped) |  |  |  |  |  |  |

<< End of changes >>