3GPP TSG-RAN WG5 Meeting #95-e	R5-223276

# Online, , 9th May 2022 - 20th May 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2413 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to test procedure 4.9.11 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In IMS emergency call related test procedure 4.9.11, at step 6 the SS configures SRB2 and DRB and sets the defaultDRB IE in SDAP-Config to TRUE. Later at Step 9 when SS sends RRC Reconfiguration message, defaultDRB in SDAP-Config is set to true again. This results in more than one DRB having the IE defaultDRB set to TRUE in the SDAP-Config, which is an invalid configuration as per TS 38.331 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated Table 4.9.11.2.3-8 and added Table 4.9.11.2.3-8A in Specific Message Contents. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may unfairly fail the test case up on receiving SDAP configuration with default DRB set to true for multiple DRBs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.9.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <Start of modified section>

### 4.9.11	Test procedure for IMS Emergency call or eCall over IMS establishment in 5GC with IMS emergency registration

4.9.11.1	Scope

This procedure aims at verifying the UE establishment of IMS Emergency call or an eCall in 5GC when the UE is in 5GMM-IDLE and when IMS emergency registration is required e.g. under Normal Service conditions.

The procedure provides different emergency or eCall establishment options based on the condition parameters defined in Table 4.9.11.1-1.

Table 4.9.11.1-1: Condition parameters

| Condition | Explanation |
| --- | --- |
| emergency call | When this parameter is present the TC has triggered the UE to initiate an emergency call. (NOTE) |
| eCall | When this parameter is present the TC has triggered the UE to initiate an eCall. |
| NOTE: If this generic procedure is referred in test case without any condition, emergency call shall be used as default. |  |

The trigger to initiate MO call will be part of test case from where the generic procedure is called.

4.9.11.2	Procedure description

4.9.11.2.1	Initial conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

User Equipment:

-	The Test UICC shall be inserted. It shall provide Emergency Numbers.

The procedure assumes that the UE is in test state 1N-A, subclause 4.4A.2 on the NR Cell. All necessary details required shall be explicitly specified in the TC which calls the procedure in its entirety or refers to parts of it.

4.9.11.2.2	Procedure sequence

Table 4.9.11.2.2-1: Test procedure sequence UE IMS Emergency call or eCall over IMS establishment in 5GC with IMS emergency registration

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| - | EXCEPTION: Unless otherwise stated all the messages below are transmitted on the cell specified in the test case. | - | - | - | - |
| 1 | The UE transmits an RRCSetupRequest message with 'establishmentCause' set to 'emergency'. | --> | NR RRC: RRCSetupRequest | - | P |
| 2 | The SS transmits an RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| 3 | The UE transmits an RRCSetupComplete message and a SERVICE REQUEST message with 'Service type' set to 'emergency services'. | --> | NR RRC: RRCSetupComplete 5GMM: SERVICE REQUEST | - | P |
| 4 | The SS transmits a SecurityModeCommand message. | <-- | NR RRC: SecurityModeCommand | - | - |
| 5 | The UE transmits a SecurityModeComplete message. | --> | NR RRC: SecurityModeComplete | - | - |
| 6 | The SS transmits an RRCReconfiguration message and a SERVICE ACCEPT message to establish SRB2 and DRB. | <-- | NR RRC: RRCReconfiguration 5GMM: SERVICE ACCEPT | - | - |
| - | EXCEPTION: Depending upon UE implementation, steps 7 and 8 can occur in any order | - | - | - | - |
| 7 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 8 | The UE transmits an UL NAS TRANSPORT message with 'Request type' set to 'initial emergency request', and, a PDU SESSION ESTABLISHMENT REQUEST. | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT REQUEST | - | P |
| - | EXCEPTION: In parallel to the events described in steps 9-10 below the events specified in steps 1a1 to 2 of Table 4.9.11.2.2-2 take place. | - | - | - | - |
| 9 | The SS transmits an RRCReconfiguration message and an PDU SESSION ESTABLISHMENT ACCEPT | <-- | NR RRC: RRCReconfiguration 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT ACCEPT | - | - |
| 10 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| - | EXCEPTION: In parallel to the events described in steps 11-13 below the events specified in steps 3 of Table 4.9.11.2.2-2 take place. | - | - | - | - |
| 11 | The SS transmits an RRCReconfiguration message and an PDU SESSION MODIFICATION COMMAND. | <-- | NR RRC: RRCReconfiguration 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMMAND | - | - |
| - | EXCEPTION: Depending upon UE implementation, steps 12 and 13 can occur in any order. | - | - | - | - |
| 12 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 13 | The UE transmits an ULInformationTransfer message and an PDU SESSION MODIFICATION COMPLETE message. | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMPLETE | - | P |

Table 4.9.11.2.2-2: IMS signalling and Emergency call establishment

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour depending UE implementation; the "lower case letter" identifies a step sequence that take place if the UE performs a specific action. | - | - | - | - |
| 1a1 | The generic procedure for IP address allocation in the user plane specified in subclause 4.5A.3 takes place. | - | - | - | - |
| 2 | Generic Test Procedure for IMS Emergency registration / 5GS as defined in TS 34.229-5 [47], annex A.3 is performed. | - | - | - | - |
| - | EXCEPTION: Steps 3a1-3b1 describe a step sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value | - | - | - | - |
| 3a1 | IF ‘emergency call’ THEN Generic test procedure for setting up IMS Emergency Voice Call / 5G as defined in TS 34.229-5 [47], annex A.6 is performed. | - | - | - | - |
| 3b1 | IF ‘eCall’ THEN Generic test procedure for NR eCall Setup and MSD Update, steps 1-3, as defined in Annex A.23 of TS 34.229-5 [47] is performed. | - | - | - | - |

4.9.11.2.3	Specific Message content

All specific message contents shall be according subclause 4.6 and 4.7 with the exceptions below.

Table 4.9.11.2.3-1: SIB1 (at any time prior and during the procedure, Table 4.9.11.2.2-1)

| Derivation Path: Table 4.6.1-28. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| ims-EmergencySupport |  | Present | true |  |
| } |  |  |  |  |

Table 4.9.11.2.3-2: RRCSetupRequest (step 1, Table 4.9.11.2.2-1)

| Derivation Path: Table 4.6.1-23. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCSetupRequest ::= SEQUENCE { |  |  |  |  |
| rrcSetupRequest SEQUENCE { |  |  |  |  |
| establishmentCause |  | emergency |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 4.9.11.2.3-3: SERVICE REQUEST (step 3, Table 4.9.11.2.2-1)

| Derivation Path: Table 4.7.1-16. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Service type |  | ‘0011’B | emergency services |  |

Table 4.9.11.2.3-4: UL NAS TRANSPORT (step 8, Table 4.9.11.2.2-1)

| Derivation Path: Table 4.7.1-10, condition INITIAL_PDU_REQUEST. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Request type |  | ‘011’B | initial emergency request |  |
| S-NSSAI |  | Not Present |  |  |
| DNN |  | Not Present |  |  |

Table 4.9.11.2.3-5: PDU SESSION ESTABLISHMENT REQUEST (step 8, Table 4.9.11.2.2-1)

| Derivation Path: Table 4.7.2-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PDU session ID |  | A value that is not currently being used by another PDU session |  |  |
| PTI |  | A value currently not used |  |  |
| SSC mode |  | ‘001’B | SSC mode 1 |  |

Table 4.9.11.2.3-6: DL NAS TRANSPORT (step 9, Table 4.9.11.2.2-1)

Derivation Path: Table 4.7.1-11, condition 5GSM_MESSAGE.

Table 4.9.11.2.3-7: PDU SESSION ESTABLISHMENT ACCEPT (step 9, Table 4.9.11.2.2-1)

| Derivation Path: Table 4.7.2-2. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Selected SSC mode |  | ‘001’B | SSC mode 1 |  |
| Authorized QoS rules |  | Reference QoS rule #2 as defined in Table 4.8.2.1-2. |  |  |
| Authorized QoS flow descriptions |  | Reference QoS flow #2 as defined in Table 4.8.2.3-2. |  |  |

Table 4.9.11.2.3-8: RRCReconfiguration (step 9, Table 4.9.11.2.2-1)

| Derivation Path: Table 4.6.1-13 with condition NR. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| radioBearerConfig | RadioBearerConfig-DRB-NR(1, 0) |  |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup | CellGroupConfig-DRB(1, 0) | OCTET STRING (CONTAINING CellGroupConfig) |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 4.9.11.2.3-8A: RadioBearerConfig-DRB-NR(1, 0) (Table 4.9.11.2.3-8)

| Derivation Path: Table 4.6.3-132 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  |  |  |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config | SDAP-Config with conditionNo-defaultDRB |  |  |
| } |  |  |  |
| drb-Identity | j | j is allocated according to internal TTCN mapping |  |
| reestablishPDCP | Not present |  |  |
| recoverPDCP | Not present |  |  |
| pdcp-Config | PDCP-Config |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 4.9.11.2.3-9: PDU SESSION MODIFICATION COMMAND (step 11, Table 4.9.11.2.2-1)

| Derivation Path: Table 4.7.2-2. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PDU session ID |  | Same value as sent in PDU SESSION ESTABLISHMENT REQUEST message. |  |  |
| Authorized QoS rules |  | Reference QoS rule #7 as defined in Table 4.8.2.1-7 using condition IMS_VOICE. |  |  |
| Authorized QoS flow descriptions |  | Reference QoS flow #5 as defined in Table 4.8.2.3-5. |  |  |

Table 4.9.11.2.3-10: RRCReconfiguration (step 11, Table 4.9.11.2.2-1)

Derivation Path: 4.8.1-1C RRCReconfiguration-Speech

## <End of modifed section>