3GPP TSG-RAN5 Meeting #95-e	R5-223278
Electronic Meeting, 9th May – 20th May 2022

Title: 	Discussion on testability aspects for new test function to limit Pcell power

Source:	        Keysight Technologies UK Ltd

Agenda Item:	5.4.17

Document for:	Discussion and Endorsement

# 1.	Introduction

There have been extensive discussions over the course of several meetings regarding the SCell drop in FR2 UL-CA RF test cases (a summary of the discussions status prior to RAN5#94e can be found in [1]).

In last RAN5#94e meeting, three draft CRs were endorsed introducing a new conformance test function, for Release 16 onward testing only, to apply a backoff Xmax,f,Pcell that the UE can apply during the UL-CA conformance tests that are configured to test at max transmit power ([2], [3] and [4]).

The main reason to endorse draft CRs instead of regular CRs was that there were some testability concerns raised in terms of impact of the power backoff into dynamic ranges and hence testability of UL-CA when using the new conformance test function. Such discussion was postponed to RAN5#95e.

Additionally, the new test function was the option chosen among all the ones discussed as the RAN4 preferred option was testing UL CA requirements assuming equal PSD among different carriers ([5]).

This document performs some testability analysis of the new test function limiting the PCell power and exposes some gaps found.

# 2.	Discussion

# 2.1 Testability analysis for equal PSD

[3] defined the new test function to limit the PCell power and the mapping og Xmax,i,PCell to number of NR FR2 Component Carriers as shown below:

### 6.11.1 ACTIVATE POWER LIMIT REQUEST The values sent the SS as part of this test function maps to the values defined in Table 6.11.1-1. For intra-band contiguous FR2 UL-CA scenarios where it is critical to prevent drop of the secondary component carrier(s) due to priorization rules in section 7.5 of TS 38.213, a parameter Xmax,f,PCell is defined for the Pcell such that Xmax,i,PCell = 10 log10(i) for i=1…n component carriers Xmax,i,PCell is the back-off in transmit power applied on the primary component carrier fom PCMAX,f,c. This ensure sufficient power allocation is possible on the secondary component carrier(s) Table 6.11.1-1: Mapping of Xmax,i,PCell to number of NR FR2 Component Carriers Number of FR2 component carriers; POWER LIMIT REQUEST message value; Xmax,i,PCell (dB). 2; 1 0 1 0 0 0 0 0; 3.0103. 3; 1 0 1 0 0 0 0 1; 4.771213. 4; 1 0 1 0 0 0 1 0; 6.0206. 5; 1 0 1 0 0 0 1 1; 6.9897. 6; 1 0 1 0 0 1 0 0; 7.781513. 7; 1 0 1 0 0 1 0 1; 8.45098. 8; 1 0 1 0 0 1 1 0; 9.0309.

In the attached spreadsheet, testability for MPR and ACLR tests has been analyzed for the different values of Xmax,i,PCell , depending on the number of component carriers aggregated:

- Column I shows highlighted in orange background, the cases that would become not testable for the MPR test case (CBW=400 MHz), assuming that minimum SNR to allow accurate measurements is equal to 6 dB.

- Columns L to 0 show highlighted in orange background, the cases that would become not testable for the ACLR test case (one column per channel bandwidth), assuming that minimum SNR to allow accurate measurements is equal to 6 dB.

Observation 1: Table 2.1 shows additional limitations in terms of maximum testable MPR [dB] for different number of CCs and channel bandwidth per CC (all CCs use the same channel BW)

Table 2.1-1 Additional limitations in terms of maximum testable MPR [dB] for equal CBWs for all CCs

|  |  | 2CC | 3CC | 4CC | 5CC | 6CC | 7CC | 8CC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MPR test: additional testability restriction for CBW=400 MHz | FR2a | N/A | N/A | N/A | N/A | N/A | N/A | 9 |
|  | FR2b | N/A | N/A | 8 | 7 | 6.5 | 5.5 | 5 |
| ACLR test: additional testability restriction for CBW=50 MHz | FR2a | 7 | 5.5 | 5 | 4 | 4 | 3.5 | 3 |
|  | FR2b | 5 | 4 | 3.5 | 3 | 2.5 | 2 | 2 |
| ACLR test: additional testability restriction for CBW=100 MHz | FR2a | 5 | 4 | 3 | 3 | 2.5 | 2 | 1.5 |
|  | FR2b | 3 | 2.5 | 2 | 1 | 0 | 0 | 0 |
| ACLR test: additional testability restriction for CBW=200 MHz | FR2a | 3 | 2.5 | 1.5 | 0.5 | 0 | 0 | 0 |
|  | FR2b | 1.5 | 0 | 0 | None | None | None | None |
| ACLR test: additional testability restriction for CBW=400 MHz | FR2a | 1.5 | 0 | 0 | None | None | None | None |
|  | FR2b | 0 | None | None | None | None | None | None |

Cells in green show no additional testability issues.

Cells in orange show additional testability issues but probably still worthy to test

Cells in light red/red show additional testability issues not worthy to test.

Observation 2: MPR suffers a medium testability impact with the new test function limiting PCell power. Probably it is still worthy to test for any number of UL CCs, , assuming all CCs have the same channel bandwidth. New additional testability limitations are required for number of UL CCs ≥ 4.

Observation 3: ACLR suffers a high testability impact with the new test function limiting PCell power, making it almost not testable for any channel bandwidth even for 2CCs, assuming all CCs have the same channel bandwidth.

# 2.2 Testability analysis for unequal PSD per CC

Current endorsed draft CRs ([2], [3], [4]) specify a power backoff for the PCell depending on the number of component carriers.

In case of 2CC, this will allow the device to transmit in the SCell a power level equal to the PCell one. The approach behind this was having equal PSD but this is only achievable if CBW for both CCs is the same:

If equal PSD is the final goal, then the power backoff should be defined as:

Xmax,PCell  = 10 log10(total aggregated BW/PCell BW)

This would imply changes in the endorsed draft CRs as parameter for the new test function would take different values.

Proposal 1: RAN5 to confirm whether different channel bandwidths per carrier are in scope for the new test function.

Proposal 2: In case different channel bandwidths per carrier are in scope for the new test function, RAN5 to agree on required changes on existing draft CRs ([2], [3], [4])

In case of more than 1 SCC, it is unclear how the device distributes the power among all the secondary component carriers. Currently up power commands are sent once PCell power backoff has been configured.

How can it be ensured that device is not setting first SCC up to maximum and not letting any power left for secondaries SCCs?

Observation 4: It is unclear how the device splits the power among different SCCs.

Proposal 5: In case different channel bandwidth per carrier are in scope for the new test function, RAN5 should clarify the power split among different SCCs.

If the case of 2CCs is analyzed for the equal PSD approach considering the power backoff as:

Xmax,PCell  = 10 log10(total aggregated BW/PCell BW)

It can be see that the potential power backoff needed could be much higher compared to the one considered if Xmax,i,PCell  = 10 log10(i) for i=1…n component carriers (9.54 vs 3.01) which would lead to even more blocking testability issues (refer to table 2.2-1)

Table 2.2-1 Required power backoff for 2CCs in case of different CBWs for both CCs

| #CCs | CBW PCC | CBW SCC1 | Total Aggregated BW | Total aggregated BW/Pcell BW | Xmax,PCell = 10 log10(total aggregated BW/PCell BW) | Example of band combo using this CBWs |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | 50 | 50 | 100 | 2 | 3.010299957 | CA_n258O |
| 2 | 50 | 100 | 150 | 3 | 4.771212547 | CA_n257G |
| 2 | 50 | 200 | 250 | 5 | 6.989700043 | CA_n257D |
| 2 | 50 | 400 | 450 | 9 | 9.542425094 | CA_n257B |
| 2 | 100 | 50 | 150 | 1.5 | 1.760912591 | CA_n258O |
| 2 | 100 | 100 | 200 | 2 | 3.010299957 | CA_n257G |
| 2 | 100 | 200 | 300 | 3 | 4.771212547 | CA_n257D |
| 2 | 100 | 400 | 500 | 5 | 6.989700043 | CA_n257B |
| 2 | 200 | 50 | 250 | 1.25 | 0.96910013 | FFS |
| 2 | 200 | 100 | 300 | 1.5 | 1.760912591 | FFS |
| 2 | 200 | 200 | 400 | 2 | 3.010299957 | CA_n257D |
| 2 | 200 | 400 | 600 | 3 | 4.771212547 | CA_n257B |
| 2 | 400 | 50 | 450 | 1.125 | 0.511525224 | FFS |
| 2 | 400 | 100 | 500 | 1.25 | 0.96910013 | FFS |
| 2 | 400 | 200 | 600 | 1.5 | 1.760912591 | FFS |

Observation 5: Additional testability issues are expected if different channel bandwidths per CC are considered.

Proposal 3: In case different channel bandwidth per carrier are in scope for the new test function, RAN5 should carry out additional testability analysis before endorsing regular CRs for the new test function.

# 3. 	Conclusion

This document performs some testability analysis of the new test function limiting the PCell power and exposes some gaps found.

The following observation and proposals were done:

Observation 1: Table 2.1 shows additional limitations in terms of maximum testable MPR [dB] for different number of CCs and channel bandwidth per CC (all CCs use the same channel BW)

Table 2.1 Additional limitations in terms of maximum testable MPR [dB]

|  |  | 2CC | 3CC | 4CC | 5CC | 6CC | 7CC | 8CC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MPR test: additional testability restriction for CBW=400 MHz | FR2a | N/A | N/A | N/A | N/A | N/A | N/A | 9 |
|  | FR2b | N/A | N/A | 8 | 7 | 6.5 | 5.5 | 5 |
| ACLR test: additional testability restriction for CBW=50 MHz | FR2a | 7 | 5.5 | 5 | 4 | 4 | 3.5 | 3 |
|  | FR2b | 5 | 4 | 3.5 | 3 | 2.5 | 2 | 2 |
| ACLR test: additional testability restriction for CBW=100 MHz | FR2a | 5 | 4 | 3 | 3 | 2.5 | 2 | 1.5 |
|  | FR2b | 3 | 2.5 | 2 | 1 | 0 | 0 | 0 |
| ACLR test: additional testability restriction for CBW=200 MHz | FR2a | 3 | 2.5 | 1.5 | 0.5 | 0 | 0 | 0 |
|  | FR2b | 1.5 | 0 | 0 | None | None | None | None |
| ACLR test: additional testability restriction for CBW=400 MHz | FR2a | 1.5 | 0 | 0 | None | None | None | None |
|  | FR2b | 0 | None | None | None | None | None | None |

Observation 2: MPR suffers a medium testability impact with the new test function limiting PCell power. Probably it is still worthy to test for any number of UL CCs, , assuming all CCs have the same channel bandwidth. New additional testability limitations are required for number of UL CCs ≥ 4.

Observation 3: ACLR suffers a high testability impact with the new test function limiting PCell power, making it almost not testable for any channel bandwidth even for 2CCs, assuming all CCs have the same channel bandwidth.

Proposal 1: RAN5 to confirm whether different channel bandwidths per carrier are in scope for the new test function.

Proposal 2: In case different channel bandwidths per carrier are in scope for the new test function, RAN5 to agree on required changes on existing draft CRs ([2], [3], [4])

Observation 4: It is unclear how the device splits the power among different SCCs.

Proposal 5: In case different channel bandwidth per carrier are in scope for the new test function, RAN5 should clarify the power split among different SCCs.

Observation 5: Additional testability issues are expected if different channel bandwidths per CC are considered.

Proposal 3: In case different channel bandwidth per carrier are in scope for the new test function, RAN5 should carry out additional testability analysis before endorsing regular CRs for the new test function.

# 4.	References

- R5- 221630, “Avoiding Scell Drop in FR2 RF UL-CA tests”, Apple Portugal, RAN5#94-e, Feb 2022

- R5-221920, “Add new messages and procedure for test function to limit Pcell Power (38.508-1)”, Apple Portugal, RAN5#94-e, Feb 2022

- R5-221921, “Addition of new test function to limit Pcell power (38.509)”, Apple Portugal, RAN5#94-e, Feb 2022

- R5-221922, “Implement test function approach to avoid Scell Drop in FR2 UL-CA (38.521-2)”, Apple Portugal, RAN5#94-e, Feb 2022

- R4-2103124, LS from RAN4 to RAN5 on SCell Dropping due to PCC Prioritization, 3GPP TSG RAN WG5 Meeting #91-e, February 2021