3GPP TSG-RAN WG5 Meeting #95-e	R5-223393

# Online, , 9th May 2022 - 20th May 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2989 | rev | 1 | Current version: | 16.11.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR EIEI test case 11.5.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | New NR EIEI test case 11.5.6 needs to be added as per workplan. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test case 11.5.6. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR EIEI WP will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.5.6 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.523-2 CR 0227 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-223393 is the final version of R5-223260 with below changes: 1. Moved the condition pc_UTRA from step 1a1 to an earlier step (Exception). 2. Editorial update at step 19 from performs to perform. 3. At step 20 referred emergency call release procedure rather than eCall release procedure. |  |  |  |  |  |  |  |  |

## <Start of modified section>

### 11.5.6	eCall capable / 5GS supports IMS voice over PS session / 5GS supports emergency service / eCall over IMS is not supported / eCall using the CS domain / emergency call over IMS if eCall using the CS domain is not available / UTRA

11.5.6.1	Test Purpose (TP)

(1)

with { UE in 5GMM-REGISTERED.NORMAL-SERVICE state and 5GMM-IDLE mode }

ensure that {

when { eCallOverIMS-Support is not indicated in SIB1 and UE is requested to make an automatic eCall and UTRA Cell is available }

then { UE establishes the eCall using the CS domain (UTRA) }

}

(2)

with { UE in 5GMM-REGISTERED.NORMAL-SERVICE state and 5GMM-IDLE mode }

ensure that {

when { eCallOverIMS-Support is not indicated in SIB1 and UE is requested to make an automatic eCall }

then { UE establishes an RRC connection for normal emergency call with the RRC establishmentCause set to "emergency", and, sends a SERVICE REQUEST message with Service type IE set to "emergency services" }

}

11.5.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331, clauses 5.2.2.4.2, 6.2.2 and TS 23.167, Annex H.6.

[TS 38.331, clause 5.2.2.4.2]

Upon receiving the SIB1 the UE shall:

1>	store the acquired SIB1;

…

4>	forward the eCallOverIMS-Support to upper layers, if present;

[TS 38.331, clause 6.2.2]

eCallOverIMS-Support

Indicates whether the cell supports eCall over IMS services as defined in TS 23.501 [32]. If absent, eCall over IMS is not supported by the network in the cell.

[TS 23.167, clause H.6]

This clause details the domain priority and selection (see clause 7.3) for a UE that attempts to make an eCall over IMS session using E-UTRAN or NG-RAN radio access networks based on the availability of the CS or PS domains and the network support for IMS emergency, eCall over IMS and IMS voice over PS.

The following table (Table H.2) defines these rules based on the UE (last 2 columns) for different initial conditions (first 4 columns) when an eCall over IMS session is initiated and when the UE is not in limited service state.

Table H.2: Domain Selection Rules for eCall over IMS session attempts for E-UTRAN or NG-RAN radio access networks

|  | PS Available | VoIMS | EMS | ECL | First eCall Attempt | Second eCall Attempt |
| --- | --- | --- | --- | --- | --- | --- |
| A | Y | Y | Y | Y | PS | PS on another PS RAT if available with EMS=Y and ECL=Y or CS if available |
| B | Y | Y | Y | N | CS if available | PS (UE establishes IMS emergency session) |
| C | Y | Y or N | N | N | CS if available | PS on another PS RAT if available with EMS=Y or EMS unknown |
| D | Y | N | Y | Y | PS or CS if available | CS if first attempt in PS PS if first attempt in CS |
| E | Y | N | Y | N | CS if available | PS (UE establishes IMS emergency session) |
| F | N |  | - | - | CS if available |  |
| VoIMS = Voice over IMS over PS sessions support as indicated by IMS Voice over PS session supported indication as defined in TS 23.401 [28] and TS 23.502 [49]. EMS = IMS Emergency Services supported as indicated by Emergency Service Support indicator as defined in TS 23.401 [28] and TS 23.501 [48] and TS 23.502 [49]. ECL = eCall Over IMS support as indicated by the eCall support indicator defined in TS 23.401 [28] and TS 23.501 [48]. NOTE 1: As an implementation option, when the first attempt uses PS and fails for reasons other than related to IMS, the second attempt may use PS with a different 3GPP RAT. In this case the UE, can make a third attempt using CS. |  |  |  |  |  |  |

NOTE:	If the E-UTRAN and NG-RAN cells available to the UE have different settings, the UE assumes "PS Available" and "ECL" apply to whichever cell is indicated (as defined in TS 23.401 [28] and TS 23.501 [48]) as providing eCall over IMS support. When support by more than one cell is indicated, a UE may select any cell to attempt eCall over IMS according to the UE implementation.

11.5.6.3	Test description

11.5.6.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 as defined in TS 38.508-1 [4] Table 4.4.2-3. System information combination NR-1 as defined in TS 38.508-1 [4], subclause 4.4.3.1.2.

-	UTRA Cell 5 is configured as Non-suitable "Off" cell as defined in TS 36.508 [7].

UE:

-	the eCall capable UE is equipped with USIM configured as per TS 38.508-1 [4] Table 6.4.1-23.

Preamble:

-	The UE is in test state 1N-A as defined in TS 38.508-1 [4], subclause 4.4A.2 on NR Cell 1.

11.5.6.3.2	Test procedure sequence

Table 11.5.6.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Steps 1a1 to 1a26 describes behaviour that depends on the UE capability pc_UTRA = TRUE; the "lower case letter" identifies a step sequence that takes place if a capability is supported. | - | - | - | - |
| 1a1 | The SS configures UTRA Cell 5 as ''Suitable Neighbour cell''. | - | - | - | - |
| 1a2 | Cause the UE to originate an automatic eCall. (Note 1) | - | - | - | - |
| 1a3 | Check: Does the UE transmit an RRC CONNECTION REQUEST message on Cell 5 with establishment cause set to Emergency Call? | --> | RRC CONNECTION REQUEST | 1 | P |
| 1a4 | The SS transmits an RRC CONNECTION SETUP message. | <-- | RRC CONNECTION SETUP | - | - |
| 1a5 | The UE transmits an RRC CONNECTION SETUP COMPLETE message. | --> | RRC CONNECTION SETUP COMPLETE | - | - |
| - | EXCEPTION: In parallel to the event described in steps 1a6 to 1a17, if initiated by the UE the steps 5 to 13 of the generic procedure for Registration on PS specified in TS 34.108 subclause 7.2.2.2.3 take place. | - | - | - | - |
| 1a6 | Check: Does the UE transmit a CM SERVICE REQUEST with CM service type IE indicating “Emergency call establishment”? | --> | CM SERVICE REQUEST | 1 | P |
| 1a7 | The SS transmits an AUTHENTICATION REQUEST. | <-- | AUTHENTICATION REQUEST | - | - |
| 1a8 | The UE transmits AUTHENTICATION RESPONSE. | --> | AUTHENTICATION RESPONSE | - | - |
| 1a9 | The SS transmits a SECURITY MODE COMMAND message for the CS domain. | <-- | SECURITY MODE COMMAND | - | - |
| 1a10 | The UE transmits a SECURITY MODE COMPLETE message. | --> | SECURITY MODE COMPLETE | - | - |
| 1a11 | Check: Does the UE transmit an EMERGENCY SETUP message with Emergency Service Category IE bit 7 set to 1 and all other bits are set to 0? | --> | EMERGENCY SETUP | 1 | P |
| 1a12-1a17 | Steps 11 to 16 of the generic test procedure in TS 34.108 subclause 7.2.3.2.3 are performed on Cell 5. NOTE: the CS call setup is completed. | - | - | - | - |
| 1a18 | Traffic channel is kept active for at least 5 seconds. | - | - | - | - |
| 1a19 | The SS transmits DISCONNECT. | <-- | DISCONNECT | - | - |
| 1a20 | The UE transmits RELEASE. | --> | RELEASE | - | - |
| 1a21 | The SS transmits RELEASE COMPLETE. | <-- | RELEASE COMPLETE | - | - |
| 1a22 | The SS transmits an RRC CONNECTION RELEASE message. | <-- | RRC CONNECTION RELEASE | - | - |
| 1a23 | The UE transmits RRC CONNECTION RELEASE COMPLETE. |  | RRC CONNECTION RELEASE COMPLETE |  |  |
| 1a24 | if possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| - | EXCEPTION: Step1a24Aa1 describes behaviour that depends on the UE capability. | - | - | - | - |
| 1a24Aa1 | If pc_SwitchOnOff or pc_USIM_Removal then UE sends DETACH REQUEST message | --> | DETACH REQUEST | - | - |
| 1a25 | Set the power level of Cell 5 to Non-suitable" Off" level. | - | - | - | - |
| 1a26 | The UE is brought back to operation or the USIM configured as per TS 38.508-1 [4] Table 6.4.1-23 is inserted. | - | - | - | - |
| 2-17a1 | Steps 5-20a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | - | - |
| 18 | Cause the UE to originate Automatic eCall. (see Note 1) | - | - | - | - |
| 19 | Check: Does the UE perform Generic Test Procedure for IMS Emergency call establishment with IMS Emergency registration as specified in TS 38.508-1 [4], subclause 4.9.11? (see Note 2) | - | - | 2 | - |
| 20 | Test procedure for IMS MT Emergency call release is performed as specified in TS 38.508-1 [4], subclause 4.9.12B. | - | - | - | - |
| 21 | The SS releases the RRC connection. | - | - | - | - |
| Note 1: The request to originate an automatic eCall may be performed by MMI or AT command. Note 2: The UE shall set the “Request URI and To header” to "urn:service:sos.ecall.automatic" and shall not include the initial MSD in the SIP INVITE message. |  |  |  |  |  |

11.5.6.3.3	Specific message contents

Table 11.5.6.3.3-1: RRC CONNECTION REQUEST (Step 1a3, Table 11.5.6.3.2-1)

| Derivation Path: TS 34.108 clause 9.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Establishment cause | Emergency Call |  |  |

Table 11.5.6.3.3-2: CM SERVICE REQUEST (Step 1a6, Table 11.5.6.3.2-1)

| Derivation Path: TS 24.008 Table 9.2.11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CM service type | 0010 | Emergency call establishment |  |

## <End of modifed section>