3GPP TSG-RAN5 Meeting #95-e	R5-223414

# Online, , 9th May 2022 - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2307 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial updates to SIBs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Some SIBs have a descriptive text of its purpose before the table, taken from TS 38.331 (SIB2 to SIB9), while others have not although it exists in TS 38.331 (SIB10 to SIB14). There are two ways to resolve this inconcistency: - Add the descriptive text everywhere, or - Remove the descriptive text everywhere. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to remove the descriptive text everywhere, because: - It is already available in TS 38.331, and maintenance effort will be regularly required to keep TS 38.331 and TS 38.508-1 aligned. - The descriptive text is also not available in other clauses of TS 38.508-1, e.g. clause 4.6.1, although it exists in TS 38.331. A few other minor editorial issues have also been corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TS 38.508-1 will remain inconsistent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the outcome of 1 revision of R5-222379. |  |  |  |  |  |  |  |  |

### 4.6.2	System information blocks

#### –	SIB2

Table 4.6.2-1: SIB2

| Derivation Path: TS 38.331 [6], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| cellReselectionInfoCommon SEQUENCE { |  |  |  |
| nrofSS-BlocksToAverage | 2 |  |  |
| absThreshSS-BlocksConsolidation SEQUENCE { |  |  |  |
| thresholdRSRP | RSRP-Range | Table 4.6.3-152 |  |
| thresholdRSRQ | Not present |  |  |
| thresholdSINR | Not present |  |  |
| } |  |  |  |
| rangeToBestCell | dB0 |  |  |
| q-Hyst | dB0 | To reduce interference between intra-frequency multiple cells |  |
| speedStateReselectionPars | Not present |  |  |
| } |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearchP | Not present |  |  |
| s-NonIntraSearchQ | Not present |  |  |
| threshServingLowP | 0 | Actual value of threshold = field value * 2 [dB] |  |
| threshServingLowQ | Not present |  |  |
|  | 3 | 3dB | QBASED |
| cellReselectionPriority | 4 | A middle value in the range has been selected |  |
| cellReselectionSubPriority | Not present |  |  |
| } |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -70 | -140dBm |  |
|  | -55 | -110dBm | SIG |
| q-RxLevMinSUL | Not Present |  |  |
|  | -70 | -140dBm | SUL |
|  | -55 | -110dBm | SUL AND SIG |
| q-QualMin | Not present |  |  |
|  | -20 | -20dB | QBASED |
| s-IntraSearchP | 31 | Actual value of threshold = field value * 2 [dB] |  |
| s-IntraSearchQ | Not present |  |  |
| t-ReselectionNR | 0 |  |  |
| frequencyBandList | Not present |  |  |
| frequencyBandListSUL | Not present |  |  |
| p-Max | Not present |  |  |
| smtc | SSB-MTC | Table 4.6.3-185 |  |
| ss-RSSI-Measurement | Not present |  |  |
| ssb-ToMeasure | SSB-ToMeasure |  |  |
| deriveSSB-IndexFromCell | false |  | FDD |
|  | true |  | TDD |
| } |  |  |  |
| t-ReselectionNR-SF | Not present |  |  |
| smtc2-LP-r16 | Not present |  |  |
| ssb-PositionQCL-Common-r16 | Not present |  |  |
| } |  |  |  |
| relaxedMeasurement-r16 | Not present |  |  |

| Condition | Explanation |
| --- | --- |
| SUL | For test cases using SUL frequency for the serving cell, Qrxlevmin is obtained from q-RxLevMin-sul. |
| QBASED | This condition applies to Quality based signalling test cases. |

#### –	SIB3

Table 4.6.2-2: SIB3

| Derivation Path: TS 38.331 [6], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB3 ::= SEQUENCE { |  |  |  |
| intraFreqNeighCellList | Not present | Not required unless Qoffset configuration is tested. When Qoffset configuration is tested, see table 6.3.1.1-1 |  |
| intraFreqBlackCellList | Not present | Not required unless Blacklisted cell list configuration is tested. When Blacklisted cell list configuration is tested, see table 6.3.1.1-1 |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

#### –	SIB4

Table 4.6.2-3: SIB4

| Derivation Path: TS 38.331 [6], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB4 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo { | The same number of entries as the configured inter-freq carriers defined in table 6.3.1.2-1 | n denotes the index of the entry |  |
| InterFreqCarrierFreqInfo[n] SEQUENCE { |  | entry n |  |
| dl-CarrierFreq | Downlink NR SSB ARFCN. See table 6.3.1.2-1 |  |  |
| frequencyBandList | MultiFrequencyBandListNR-SIB |  |  |
| frequencyBandListSUL | Not present |  |  |
| nrofSS-BlocksToAverage | 2 |  |  |
| absThreshSS-BlocksConsolidation SEQUENCE { |  |  |  |
| thresholdRSRP | RSRP-Range | Table 4.6.3-152 |  |
| thresholdRSRQ | Not present |  |  |
| thresholdSINR | Not present |  |  |
| } |  |  |  |
| smtc | SSB-MTC | Table 4.6.3-185 |  |
| ssbSubcarrierSpacing | SubcarrierSpacing | Table 4.6.3-188 |  |
| ssb-ToMeasure | SSB-ToMeasure |  |  |
| deriveSSB-IndexFromCell | false |  | FDD |
|  | true |  | TDD |
| ss-RSSI-Measurement | Not present |  |  |
| q-RxLevMin | -70 | -140dBm |  |
|  | -55 | -110dBm | SIG and FR1 |
|  | ROUND((-110+Delta(NRfn))/2) | NOTE1 and NOTE2. | SIG and FR2 |
| q-RxLevMinSUL | -70 | -140dBm, For RF/RRM test cases | SUL |
|  | Not present |  |  |
|  | -55 | -110dBm | SUL and SIG |
| q-QualMin | Not present |  |  |
|  | -20 | -20dB | QBASED |
| p-Max | Not present |  |  |
| t-ReselectionNR | 0 |  |  |
| t-ReselectionNR-SF | Not present | Not required unless speed-dependent cell re-selection is tested. |  |
| threshX-HighP | 2 | 4dB, this value should be higher than threshServingLow of the serving cell to avoid ping-pong with lower priority cells. |  |
| threshX-LowP | 1 | 2dB |  |
| threshX-Q | Not present |  |  |
| threshX-Q SEQUENCE { |  |  | QBASED |
| threshX-HighQ | 5 | 5dB |  |
| threshX-LowQ | 5 | 5dB |  |
| } |  |  |  |
| cellReselectionPriority | 4 | The same priority as the one used for serving cell in SIB 2. |  |
| cellReselectionSubPriority | Not present | The same subpriority as the one used for serving cell in SIB 2. |  |
| q-OffsetFreq | dBXY with XY = (FLOOR((Delta(NRfn) – Delta(NRfs))/2))*2 (NOTE 3) | This value is type of Q-OffsetRange in TS 38.331 [6] which must be even value when its absolute value is larger than dB5. | FR2 AND NOT AbsoluteValue_Within_dB5 |
|  | dBXY with XY = Delta(NRfn) – Delta(NRfs) |  | FR2 AND AbsoluteValue_Within_dB5 |
|  | dB0 |  |  |
| interFreqNeighCellList | Not present | Not required unless Qoffset configuration is tested. |  |
| interFreqBlackCellList | Not present | Not required unless Blacklisted cell list configuration is tested. |  |
| } |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |
| NOTE 1: Delta(NRfn) and Delta(NRfs) is derived based on calibration procedure defined in the clause 6.1.3.3. NRfn and NRfs are NR frequencies in dl-CarrierFreq[n] and serving cell frequency on which SIB4 is broadcasted. NOTE 2: ROUND is rounded off to the nearest integer. As an example, ‘1 to 1.49’ set to ‘1’ while ‘1.5 to 2’ to ‘2’ and ‘-2.0 to 1.5’ set to ‘-2’ while ‘-1.49 to -1’ set to ‘-1’. NOTE 3: FLOOR is rounded off to the smaller integer. As an example, '1.0 to 1.99' set to 1, '-1.01 to -2.00' set to -2. |  |  |  |

| Condition | Explanation |
| --- | --- |
| SUL | For test cases using SUL frequency for the serving cell, Qrxlevmin is obtained from q-RxLevMin-sul. |
| QBASED | This condition applies to Quality based signalling test cases. |
| AbsoluteValue_Within_dB5 | -5dB <= (Delta(NRfn) – Delta(NRfs)) <= 5dB. |

#### –	SIB5

Table 4.6.2-4: SIB5

| Derivation Path: TS 38.331 [6], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB5 ::= SEQUENCE { |  |  |  |
| carrierFreqListEUTRA SEQUENCE (SIZE (1..maxEUTRA-Carrier)) OF CarrierFreqEUTRA { | The same number of entries as the configured E-UTRA carriers. For Signalling test cases, see table 6.3.1.3-1. | n denotes the index of the entry |  |
| CarrierFreqEUTRA[n] SEQUENCE { |  | entry n |  |
| carrierFreq | Downlink E-UTRA ARFCN under test. For Signalling test cases, see table 6.3.1.3-1. |  |  |
| eutra-multiBandInfoList | Not present |  |  |
| eutra-FreqNeighCellList | Not present | Not required unless EUTRA Qoffset configuration is tested. |  |
| eutra-BlackCellList | Not present | Not required unless Blacklisted cell list configuration is tested. |  |
| allowedMeasBandwidth | EUTRA-AllowedMeasBandwidth | The value of EUTRA-AllowedMeasBandwidth in Table 4.6.5-1. |  |
| presenceAntennaPort1 | FALSE |  |  |
|  | TRUE | At least two cell-specific antenna ports are used in all neighbouring cells. | All neighCells with port1 |
| cellReselectionPriority | 3 |  |  |
| cellReselectionSubPriority | Not Present |  |  |
| threshX-High | 2 (4 dB) |  |  |
| threshX-Low | 1 (2 dB) |  |  |
| q-RxLevMin | -70 (-140 dBm) | For RF/RRM test cases |  |
|  | -55(-110dBm) | For signalling test cases |  |
| q-QualMin | -20 (-20dB) |  |  |
| p-MaxEUTRA | 23 |  |  |
| threshX-Q | Not present |  |  |
| threshX-Q SEQUENCE { |  |  | QBASED |
| threshX-HighQ | 9 (9dB) |  |  |
| threshX-LowQ | 9 (9dB) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| t-ReselectionEUTRA | 0 |  |  |
| t-ReselectionEUTRA-SF | Not present | Not required unless speed-dependent cell re-selection is tested. |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| QBASED | This condition applies to Quality based cell (re)selection signalling test cases. |
| All neighCells with port1 | Used for all neighbouring cells with at least two cell-specific antenna ports |

#### –	SIB6

Table 4.6.2-5: SIB6

| Derivation Path: TS 38.331 [6], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB6 ::= SEQUENCE { |  |  |  |
| messageIdentifier | '0001 0001 0000 0010'B | ETWS message identifier for earthquake and tsunami message (see TS 23.041 [25]) |  |
| serialNumber | '0011 0000 0000 0000'B | Note 1 |  |
| warningType | '0000 0101 1000 0000'B | Note 2 |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |
| Note 1: Geographical Scope (Octet 1 bit 7 ~ 6) set to 'Cell wide', Emergency User Alert (Octet 1 bit 5) set to 'Activate emergency user alert', Popup (Octet 1 bit 4) set to 'Activate popup', Update Number (Octet 2 bits 3~0) for each update, incremented by one, See TS 23.041 [25]. Note 2: Warning Type Value (Octet 1 bit 7 ~ 1) set to 'Earthquake and Tsunami', Emergency User Alert (Octet 1 bit 0) set to 'Activate emergency user alert', Popup (Octet 2 bit 7) set to 'Activate Popup', see TS 23.041 [25], Padding (Octet 2 bit 6 ~ 0) set to '000 0000'B. |  |  |  |

#### –	SIB7

Table 4.6.2-6: SIB7 (1st Segment)

| Derivation Path: TS 38.331 [6], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB7 ::= SEQUENCE { |  |  |  |
| messageIdentifier | '0001 0001 0000 0010'B | ETWS message identifier for earthquake and tsunami message (see TS 23.041 [25]) |  |
| serialNumber | '0011 0000 0000 0000'B | Note 1 |  |
| warningMessageSegmentType | notLastSegment |  |  |
| warningMessageSegmentNumber | 0 |  |  |
| warningMessageSegment | Octetstring of N | Where N ≥ 1 and less than 1246 (see TS 23.041 [25]) |  |
| dataCodingScheme | Bitstring (8) ID of the alphabet/coding and the applied language | see TS 23.041 [25] | Segment 1 |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |
| Note 1: Geographical Scope (Octet 1 bit 7 ~ 6) set to 'Cell wide', Emergency User Alert (Octet 1 bit 5) set to 'Activate emergency user alert', Popup (Octet 1 bit 4) set to 'Activate popup', Update Number (Octet 2 bits 3~0) for each update, incremented by one, See TS 23.041 [25]. |  |  |  |

| Condition | Explanation |
| --- | --- |
| Segment1 | The field is mandatory present in the first segment of SIB7, otherwise it is not present. |

Table 4.6.2-7: SIB7 (2nd Segment)

| Derivation Path: TS 38.331 [6], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB7 ::= SEQUENCE { |  |  |  |
| messageIdentifier | '0001 0001 0000 0010'B | ETWS message identifier for earthquake and tsunami message (see TS 23.041 [25]) |  |
| serialNumber | '0011 0000 0000 0000'B | Note 1 |  |
| warningMessageSegmentType | notLastSegment |  |  |
| warningMessageSegmentNumber | 1 |  |  |
| warningMessageSegment | Octetstring of N | Where N ≥ 1 and less than 1246 (see TS 23.041 [25]) |  |
| dataCodingScheme | Not present |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |
| Note 1: Geographical Scope (Octet 1 bit 7 ~ 6) set to 'Cell wide', Emergency User Alert (Octet 1 bit 5) set to 'Activate emergency user alert', Popup (Octet 1 bit 4) set to 'Activate popup', Update Number (Octet 2 bits 3~0) for each update, incremented by one, See TS 23.041 [25]. |  |  |  |

Table 4.6.2-8: SIB7 (3rd Segment)

| Derivation Path: TS 38.331 [6], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB7 ::= SEQUENCE { |  |  |  |
| messageIdentifier | '0001 0001 0000 0010'B | ETWS message identifier for earthquake and tsunami message (see TS 23.041 [25]) |  |
| serialNumber | '0011 0000 0000 0000'B | Note 1 |  |
| warningMessageSegmentType | LastSegment |  |  |
| warningMessageSegmentNumber | 2 |  |  |
| warningMessageSegment | Octetstring of N | Where N ≥ 1 and less than 1246 (see TS 23.041 [25]) |  |
| dataCodingScheme | Not present |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |
| Note 1: Geographical Scope (Octet 1 bit 7 ~ 6) set to 'Cell wide', Emergency User Alert (Octet 1 bit 5) set to 'Activate emergency user alert', Popup (Octet 1 bit 4) set to 'Activate popup', Update Number (Octet 2 bits 3~0) for each update, incremented by one, See TS 23.041 [25]. |  |  |  |

#### –	SIB8

Table 4.6.2-9: SIB8 (1st Segment)

| Derivation Path: TS 38.331 [6], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB8 ::= SEQUENCE { |  |  |  |
| messageIdentifier | '0001 0001 0001 0010'B | CMAS CBS Message Identifier for CMAS Presidential Level Alerts (see TS 23.041 [25]) |  |
| serialNumber | '0011 0000 0000 0000'B | Note 1 |  |
| warningMessageSegmentType | notLastSegment |  |  |
| warningMessageSegmentNumber | 0 |  |  |
| warningMessageSegment | Octetstring of N | Where N ≥ 1 and less than 1246 (see TS 23.041 [25]) |  |
| dataCodingScheme | Bitstring (8) ID of the alphabet/coding and the applied language | see TS 23.041 [25] | Segment 1 |
| warningAreaCoordinatesSegment | Not present |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |
| Note 1: Geographical Scope (Octet 1 bit 7 ~ 6) set to 'Cell wide', Emergency User Alert (Octet 1 bit 5) set to 'Activate emergency user alert', Popup (Octet 1 bit 4) set to 'Activate popup', Update Number (Octet 2 bits 3~0) for each update, incremented by one, See TS 23.041 [25]. |  |  |  |

| Condition | Explanation |
| --- | --- |
| Segment1 | The field is mandatory present in the first segment of SIB8, otherwise it is not present. |

Table 4.6.2-10: SIB8 (2nd Segment)

| Derivation Path: TS 38.331 [6], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB8 ::= SEQUENCE { |  |  |  |
| messageIdentifier | '0001 0001 0001 0010'B | CMAS CBS Message Identifier for CMAS Presidential Level Alerts (see TS 23.041 [25]) |  |
| serialNumber | '0011 0000 0000 0000'B | Note 1 |  |
| warningMessageSegmentType | notLastSegment |  |  |
| warningMessageSegmentNumber | 1 |  |  |
| warningMessageSegment | Octetstring of N | Where N ≥ 1 and less than 1246 (see TS 23.041 [25]) |  |
| dataCodingScheme | Not present |  |  |
| warningAreaCoordinatesSegment | Not present |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |
| Note 1: Geographical Scope (Octet 1 bit 7 ~ 6) set to 'Cell wide', Emergency User Alert (Octet 1 bit 5) set to 'Activate emergency user alert', Popup (Octet 1 bit 4) set to 'Activate popup', Update Number (Octet 2 bits 3~0) for each update, incremented by one, See TS 23.041 [25]. |  |  |  |

Table 4.6.2-11: SIB8 (3rd Segment)

| Derivation Path: TS 38.331 [6], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB8 ::= SEQUENCE { |  |  |  |
| messageIdentifier | '0001 0001 0001 0010'B | CMAS CBS Message Identifier for CMAS Presidential Level Alerts (see TS 23.041 [25]) |  |
| serialNumber | '0011 0000 0000 0000'B | Note 1 |  |
| warningMessageSegmentType | LastSegment |  |  |
| warningMessageSegmentNumber | 2 |  |  |
| warningMessageSegment | Octetstring of N | Where N ≥ 1 and less than 1246 (see TS 23.041 [25]) |  |
| dataCodingScheme | Not present |  |  |
| warningAreaCoordinatesSegment | Not present |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |
| Note 1: Geographical Scope (Octet 1 bit 7 ~ 6) set to 'Cell wide', Emergency User Alert (Octet 1 bit 5) set to 'Activate emergency user alert', Popup (Octet 1 bit 4) set to 'Activate popup', Update Number (Octet 2 bits 3~0) for each update, incremented by one, See TS 23.041 [25]. |  |  |  |

#### –	SIB9

Table 4.6.2-11A: SIB9

| Derivation Path: TS 38.331 [6], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB9 ::= SEQUENCE { |  |  |  |
| timeInfo ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| referenceTimeInfo-r16 | ReferenceTimeInfo |  | TSC |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| TSC | For test cases requiring TSC (Time Sensitive Communication) functions enabled. |

#### –	SIB10

Table 4.6.2-12: SIB10

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB10-r16 ::= SEQUENCE { |  |  |  |
| hrnn-List-r16 SEQUENCE (SIZE (1..maxNPN-r16)) OF HRNN-r16 { |  |  |  |
| HRNN-r16[1] SEQUENCE { |  | 1 entry |  |
| hrnn-r16 | “3gppTest” | charstring “3gppTest” converted to OCTETSTRING |  |
| } |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

#### –	SIB11

Table 4.6.2-13: SIB11

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB11-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	SIB12

Table 4.6.2-14: SIB12

| Derivation Path: TS 38.331 [6], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB12-r16 ::= SEQUENCE { |  |  |  |
| segmentNumber-r16 | index of the segment contained in segmentContainer-r16 |  |  |
| segmentType-r16 | notLastSegment |  | NOT_LAST_SEG |
|  | lastSegment |  | LAST_SEG |
| segmentContainer-r16 | OCTET STRING (CONTAINING SIB12-IEs-r16 or segment of SIB12-IEs-r16) | Note 1 |  |
| } |  |  |  |
| Note 1: Size of the SIB12-IEs-r16 or segment of SIB12-IEs-r16 contained in segmentContainer-r16 shall not exceed the maximum TBS size of the SI. For signalling test cases, the size is defined in TS 38.523-3[23] clause 7.3.3.2. |  |  |  |

| Condition | Explanation |
| --- | --- |
| NOT_LAST_SEG | Used when SIB12-IEs-r16 is segmented and the segment contained in segmentContainer-r16 is not the last segment |
| LAST_SEG | Used when SIB12-IEs-r16 is not segmented, or SIB12-IEs-r16 is segmented and the segment contained in segmentContainer-r16 is the last segment |

Table 4.6.2-14A: SIB12-IEs-r16

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB12-IEs-r16 ::= SEQUENCE { |  |  |  |
| sl-ConfigCommonNR-r16 SEQUENCE { |  |  |  |
| sl-FreqInfoList-r16 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfigCommon-r16 { | 1 entry |  |  |
| SL-FreqConfigCommon-r16[1] | SL-FreqConfigCommon-r16 | entry 1 |  |
| } |  |  |  |
| sl-UE-SelectedConfig-r16 | SL-UE-SelectedConfig-r16 |  |  |
| sl-NR-AnchorCarrierFreqList-r16 | Not present |  |  |
| sl-EUTRA-AnchorCarrierFreqList-r16 | Not present |  |  |
| sl-RadioBearerConfigList-r16 SEQUENCE (SIZE (1..maxNrofSLRB-r16)) OF SL-RadioBearerConfig-r16 { | 1 entry |  |  |
| SL-RadioBearerConfig-r16[1] | SL-RadioBearerConfig-r16 | entry 1 |  |
| } |  |  |  |
| sl-RLC-BearerConfigList-r16 SEQUENCE (SIZE (1..maxSL-LCID-r16)) OF SL-RLC-BearerConfig-r16 { | 1 entry |  |  |
| SL-RLC-BearerConfig-r16[1] | SL-RLC-BearerConfig-r16 | entry 1 |  |
| } |  |  |  |
| sl-MeasConfigCommon-r16 | SL-MeasConfigCommon-r16 |  |  |
| sl-CSI-Acquisition-r16 | Not present |  |  |
| sl-OffsetDFN-r16 | Not present |  |  |
| t400-r16 | ms1000 |  |  |
| sl-MaxNumConsecutiveDTX-r16 | Not present |  |  |
| sl-SSB-PriorityNR-r16 | 1 |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

#### –	SIB13

Table 4.6.2-15: SIB13

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB13-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	SIB14

Table 4.6.2-16: SIB14

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB14-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |