3GPP TSG-RAN5 Meeting #95-e	R5-223424

# Electronic Meeting, 09th – 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2946 | rev | 1 | Current version: | 16.11.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR CA TC 8.1.5.7.1-CA duplication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon,Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - UE, support UL_2CC and CA duplication but not support twopucch-group(i.e PUCCH Scell), could not test 8.1.5.7.1.X because the default configuration add pucch-config and pucch-configcommon to Scell. However, it is not necessary to support pucch scell for testing CA duplication because SS could send UL grant for Scell without receiving SR in pucch of Scell and check whether or not UE could loop back data in SCell. - Step3 is used to configure two RLC for an exist DRB not add new DRB |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Delete the configuration of pucch-config and pucch-configcommon for SCell. - Add Step8A to trigger SS send periodic UL Grant on SCell to allow the UE loop back PDCP PDU on SCell without PUCCH Scell configuration. - Delete the “add new DRB”in the comment part of table 8.1.5.7.1.1.3.3-3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE, support UL_2CC and CA duplication but not support twopucch-group(i.e PUCCH Scell), could not test 8.1.5.7.1.X |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  | Revised from R5-222816r1 |  |  |  |  |  |  |  |  |

<Start of modified section 1>

8.1.5.7.1	Failure information / RLC failure / MCG

8.1.5.7.1.1	Failure information / RLC failure / MCG / Intra-band Contiguous CA

8.1.5.7.1.1.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { an indication from MCG RLC that the maximum number of retransmissions has been reached and when CA duplication is configured and activated and for the corresponding logical channel allowedServingCells only includes SCell(s) }

then { UE initiates the failure information procedure to report RLC failure via SRB1 }

}

8.1.5.7.1.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clauses 5.7.5.2 and 5.7.5.3. Unless otherwise stated these are Rel-15 requirements.

[TS 38.331, clause 5.3.10.3]

The UE shall:

…

1>	upon indication from MCG RLC that the maximum number of retransmissions has been reached:

2>	if the indication is from MCG RLC and CA duplication is configured and activated, and for the corresponding logical channel allowedServingCells only includes SCell(s):

3>	initiate the failure information procedure as specified in 5.7.5 to report RLC failure.

…

[TS 38.331, clause 5.7.5.2]

A UE initiates the procedure when there is a need inform the network about a failure detected by the UE. In particular, the UE initiates the procedure when the following condition is met:

1>	upon detecting failure for an RLC bearer, in accordance with 5.3.10.3;

Upon initiating the procedure, the UE shall:

1>	initiate transmission of the FailureInformation message as specified in 5.7.5.3;

[TS 38.331, clause 5.7.5.3]

The UE shall:

1>	if initiated to provide RLC failure information, set FailureInfoRLC-Bearer as follows:

2>	set logicalChannelIdentity to the logical channel identity of the failing RLC bearer;

2>	set cellGroupId to the cell group identity of the failing RLC bearer;

2>	set the failureType as rlc-failure;

1>	if used to inform the network about a failure for an MCG RLC bearer:

2>	submit the FailureInformation message to lower layers for transmission via SRB1;

…

8.1.5.7.1.1.3	Test description

8.1.5.7.1.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the PCell, NR Cell 3 is the SCell.

UE:

-	None.

Preamble:

-	The UE is in state 3N-A, and with UE test loop mode A prepared, as defined in TS 38.508-1 [4], subclause 4.4A.

8.1.5.7.1.1.3.2	Test procedure sequence

Table 8.1.5.7.1.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message to add SCell (NR Cell 3) for the UE. | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 2A | The SS transmits a SCell Activation/Deactivation MAC-CE to activate SCell (NR Cell 3). | <-- | MAC PDU (SCell Activation/Deactivation MAC CE of one octet (C1=1)) | - | - |
| 3 | The SS transmits an RRCReconfiguration message to establish DRB #j for the UE | <-- | NR RRC: RRCReconfiguration | - | - |
| 4 | The UE transmits an RRCReconfigurationComplete message | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 5 | The SS transmits a CLOSE UE TEST LOOP message | <-- | CLOSE UE TEST LOOP | - | - |
| 6 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message. | --> | CLOSE UE TEST LOOP COMPLETE | - | - |
| 7 | The SS stops sending MCG RLC acknowledgments for the next RLC PDU on PCell (NR Cell 1) and SCell (NR Cell 3) | - | - | - | - |
| 8 | The SS transmits one PDCP PDU to the UE on DRB #j. | <-- | PDCP PDU | - | - |
| 8A | SS transmits a periodic UL Grant on SCell (as PUCCH is not configured in the SCell), allowing the UE to loop back the PDCP PDU on SCell. | - | - | - | - |
| - | EXCEPTION: Steps 9-9A occur in any sequence. | - | - | - | - |
| 9 | The UE loops back the PDCP PDU on PCell (NR Cell 1) on DRB #j | --> | PDCP PDU | - | - |
| 9A | The UE loops back the PDCP PDU on SCell (NR Cell 3) on DRB #j | --> | PDCP PDU | - | - |
| 10 | Check: Does the UE transmit a FailureInformation message with failureType set to ‘rlc-failure’? | --> | NR RRC: FailureInformation | 1 | P |
| 11 | The SS transmits an RRCReconfiguration message containing an sCellToReleaseList with SCell NR Cell 3. | <-- | NR RRC: RRCReconfiguration | - | - |
| 12 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |

8.1.5.7.1.1.3.3	Specific message contents

Table 8.1.5.7.1.1.3.3-1: RRCReconfiguration (Step 1, Table 8.1.5.7.1.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup SEQUENCE { |  |  |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig { |  | 1 entry |  |  |
| SCellConfig[1] SEQUENCE { |  |  | entry 1 |  |
| sCellIndex |  | SCellIndex for NR Cell 3 |  |  |
| sCellConfigCommon |  | ServingCellConfigCommon |  |  |
| sCellConfigDedicated |  | ServingCellConfig |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.1.5.7.1.1.3.3-1A: ServingCellConfig (Table 8.1.5.7.1.1.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-167. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkDedicated |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.7.1.1.3.3-1B: BWP-UplinkDedicated (Table 8.1.5.7.1.1.3.3-1A)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkDedicated ::= SEQUENCE { |  |  |  |
| pucch-Config | Not present |  |  |
| } |  |  |  |

Table 8.1.5.7.1.1.3.3-1C: ServingCellConfigCommon (Table 8.1.5.7.1.1.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-168. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| physCellId | Physical Cell Identity of NR Cell 3 |  |  |
| uplinkConfigCommon | UplinkConfigCommon |  |  |
| } |  |  |  |

Table 8.1.5.7.1.1.3.3-1D: UplinkConfigCommon (Table 8.1.5.7.1.1.3.3-1C)

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkConfigCommon ::= SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkCommon |  |  |
| } |  |  |  |

Table 8.1.5.7.1.1.3.3-1E: BWP-UplinkCommon (Table 8.1.5.7.1.1.3.3-1D)

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkCommon ::= SEQUENCE { |  |  |  |
| pucch-ConfigCommon | Not present |  |  |
| } |  |  |  |

Table 8.1.5.7.1.1.3.3-2: RRCReconfiguration (Step 3, Table 8.1.5.7.1.1.3.2-1)

| Derivation Path: TS 38.508-1[4], Table 4.6.1-13 with condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig-DRB | Table 8.1.5.7.1.1.3.3-3 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig-DRB | Table 8.1.5.7.1.1.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.7.1.1.3.3-3: RadioBearerConfig-DRB (Table 8.1.5.7.1.1.3.3-2)

| Derivation Path: TS 508-1 [4], Table 4.6.3-132 with condition DRB2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| drb-Identity | DRB-Identity with condition DRB j | j is the ID of the DRB established during the preamble which is allocated according to internal TTCN mapping |  |
| pdcp-Config | PDCP-Config |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.7.1.1.3.3-4: PDCP-Config (Table 8.1.5.7.1.1.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-99 with condition Split |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCP-Config ::= SEQUENCE { |  |  |  |
| moreThanOneRLC SEQUENCE { |  |  |  |
| primaryPath SEQUENCE { |  |  |  |
| logicalChannel | LogicalChannelIdentity with Condition DRB j |  |  |
| } |  |  |  |
| pdcp-Duplication | True |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.7.1.1.3.3-5: CellGroupConfig-DRB

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 with Condition DRB j |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLC-ID)) OF RLC-BearerConfig { | 2 entries |  |  |
| RLC-BearerConfig[1] | RLC-BearerConfig-1 | entry 1 Table 8.1.5.7.1.1.3.3-6 Primary RLC |  |
| RLC-BearerConfig[2] | RLC-BearerConfig-2 | entry 1 Table 8.1.5.7.1.1.3.3-7 Secondary RLC |  |
| } |  |  |  |
| spCellConfig | Not present |  |  |
| } |  |  |  |

Table 8.1.5.7.1.1.3.3-6: RLC-BearerConfig-1 (Table 8.1.5.7.1.1.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-148 with Condition AM |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLC-BearerConfig ::= SEQUENCE { |  |  |  |
| logicalChannelIdentity | LogicalChannelIdentity with condition DRB j | ID of primary logical channel |  |
| servedRadioBearer CHOICE { |  |  |  |
| drb-Identity | DRB-Identity with condition DRB j |  |  |
| } |  |  |  |
| rlc-Config CHOICE { |  |  |  |
| am SEQUENCE { |  |  |  |
| ul-AM-RLC SEQUENCE { |  |  |  |
| maxRetxThreshold | t32 | To ensure RLC failure happens before RLF |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| mac-LogicalChannelConfig SEQUENCE { |  |  |  |
| ul-SpecificParameters SEQUENCE { |  |  |  |
| allowedServingCells SEQUENCE (SIZE (1..maxNrofServingCells-1)) OF ServCellIndex { | 1 entry |  |  |
| ServCellIndex[1] | ServCellIndex of NR Cell 1 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.7.1.1.3.3-7: RLC-BearerConfig-2 (Table 8.1.5.7.1.1.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-148 with Condition AM |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLC-BearerConfig ::= SEQUENCE { |  |  |  |
| logicalChannelIdentity | LogicalChannelIdentity with condition DRB j+1 | To ensure ID of secondary logical channel ID is different with existing logical channel and the primary logical channel defined in Table 8.1.5.7.1.1.3.3-6 |  |
| servedRadioBearer CHOICE { |  |  |  |
| drb-Identity | DRB-Identity with condition DRB j |  |  |
| } |  |  |  |
| rlc-Config CHOICE { |  |  |  |
| am SEQUENCE { |  |  |  |
| ul-AM-RLC SEQUENCE { |  |  |  |
| maxRetxThreshold | t1 | To ensure RLC failure happens before RLF |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| mac-LogicalChannelConfig SEQUENCE { |  |  |  |
| ul-SpecificParameters SEQUENCE { |  |  |  |
| allowedServingCells SEQUENCE (SIZE (1..maxNrofServingCells-1)) OF ServCellIndex { | 1 entry |  |  |
| ServCellIndex[1] | ServCellIndex of NR Cell 3 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.7.1.1.3.3-8: FailureInformation (Step 10, Table 8.1.5.7.1.1.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| FailureInformation ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| failureInformation SEQUENCE { |  |  |  |  |
| failureInfoRLC-Bearer SEQUENCE { |  |  |  |  |
| cellGroupId |  | CellGroupId as per TS 38.508-1 [4] Table 4.6.3-20 |  |  |
| failureType |  | rlc-failure |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.1.5.7.1.1.3.3-9: RRCReconfiguration (step 11, Table 8.1.5.7.1.1.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| masterCellGroup | CellGroupConfig |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.7.1.1.3.3-10: CellGroupConfig (Table 8.1.5.7.1.1.3.3-9)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-19 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| sCellToAddModList | Not present |  |  |
| sCellToReleaseList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellIndex { | 1 entry |  |  |
| SCellIndex[1] | 1 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section 1>