3GPP TSG-RAN5 Meeting #95-e	R5-223620
Electronic Meeting, 9th May – 20th May 2022

Agenda item:	5.4.17

Source:	Keysight Technologies

Title:	Discussion on TT and testability proposal for FR2 EVM

Document for:	Discussion and Endorsement

# Introduction

The aim of this document is to progress in the TT definition for the FR2 EVM test case.

# Discussion

In [1], the following proposal to define TT and testability was made:

Proposal 3. If the increase of the measured EVM, as defined in [2] equation (1), is: - Below 7.5% of the minimum conformance requirement, no TT is applied. - Equal or greater than 7.5% of the minimum conformance requirement, set TT = 1*(the increase of the measured EVM). - If resulting TT is above 50% of the minimum conformance requirement, skip the test as not testable.

In that formula the factor between the influence of agreed MU and the proposed TT is k=1, what avoids any risk of failing a conformance UE applying the permitted MPR. There was a concern on the resulting testability limitations so it was considered that a relaxation in the factor k could be made in order to increase the test coverage.

In this document, previous proposal is reformulated to apply a factor k < 1.

Proposal 1. If the increase of the measured EVM, as defined in [2] equation (1), is:

- Below 7.5% of the minimum conformance requirement, no TT is applied.

- Else

  - Set TT = k*(the increase of the measured EVM) if resulting TT is equal or smaller than 50% of the minimum conformance requirement,

  - Otherwise, skip the test as not testable.

In Annex A it is analyzed the resulting TT and testability for different values of factor k along with the actual EVM requirement to the UE when k<1. For FR2a it is observed that no testability enhancements are achieved for k<0.8 while for FR2b testability enhancements are achieved with k=0.65.

The analysis is limited down to k=0.65 assuming the risk of failing a conformance UE can already be too high.

As it can be observed in the annex, for those test points where we increase testability for lower values of k, the actual requirement to the UE, taking into account the agreed MU, can be very low compared to the minimum requirement defined in the core specs. Based on the values provided, to use K=0.8 is proposed for both FR2a and FR2b

Proposal 2. In proposal 1, use k=0.8.

Associated change requests can be found in [3], [4].

# Conclusion

The following observations and conclusions were made in this contribution.

Proposal 1. If the increase of the measured EVM, as defined in [2] equation (1), is:

- Below 7.5% of the minimum conformance requirement, no TT is applied.

- Else

  - Set TT = k*(the increase of the measured EVM) if resulting TT is equal or smaller than 50% of the minimum conformance requirement,

  - Otherwise, skip the test as not testable.

Proposal 2. In proposal 1, use k=0.8.

# References

- R5-221625: "Discussion on MU and TT proposal for FR2 EVM". 3GPP TSG-RAN5 Meeting #94-e, Electronic Meeting, 21st February - 4th March 2022

- R5-215476: "On the MU of Tx Modulation Quality test cases". 3GPP TSG-RAN5 Meeting #92-e, Electronic Meeting, 16th Aug 2021 - 27th Aug 2021.

- R5-222341: “FR2 SA EVM test case update based on TT analysis”, 3GPP TSG-RAN5 Meeting #95-e, Electronic Meeting, 9th May – 20th May 2022

- R5-222343: “FR2 NSA test case editor notes update”, 3GPP TSG-RAN5 Meeting #95-e, Electronic Meeting, 9th May – 20th May 2022

# Annex A Resulting TT and testability vs factor K

Changes vs the case k=1 highlighted in orange. Case with k=1 equivalent to proposal 3 in [1].

## K=1

| Test ID | Modulation | RB alloc. | 50MHz | 100MHz | 200MHz | 400MHz |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner_Full | 0.00% | 0.00% | 0.00% | 0.00% |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer_Full | 0.00% | 0.00% | 0.00% | 0.00% |
| 3 | DFT-s-OFDM QPSK | Inner_Full | 0.00% | 0.00% | 0.00% | 1.61% |
| 4 | DFT-s-OFDM QPSK | Outer_Full | 0.00% | 0.00% | 0.00% | 2.18% |
| 5 | DFT-s-OFDM 16 QAM | Inner_Full | 0.00% | 0.00% | 1.53% | 4.29% |
| 6 | DFT-s-OFDM 16 QAM | Outer_Full | 0.00% | 0.00% | 1.67% | 4.29% |
| 7 | DFT-s-OFDM 64 QAM | Inner_Full | 1.06% | 1.97% | 3.61% | NA |
| 8 | DFT-s-OFDM 64 QAM | Outer_Full | 1.44% | 2.68% | NA | NA |
| 9 | CP-OFDM QPSK | Inner_Full | 0.00% | 0.00% | 0.00% | 3.66% |
| 10 | CP-OFDM QPSK | Outer_Full | 0.00% | 0.00% | 1.37% | 3.66% |
| 11 | CP-OFDM 16 QAM | Inner_Full | 0.00% | 1.35% | 2.57% | NA |
| 12 | CP-OFDM 16 QAM | Outer_Full | 0.00% | 1.35% | 2.57% | NA |
| 13 | CP-OFDM 64 QAM | Inner_Full | 2.19% | 3.97% | NA | NA |
| 14 | CP-OFDM 64 QAM | Outer_Full | 2.19% | 3.97% | NA | NA |

Table 1. Resulting TT for EVM in PUSCH, PC3, FR2a for k=1 in proposal 1.

| Test ID | Modulation | RB alloc. | 50MHz | 100MHz | 200MHz | 400MHz |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner_Full | 0.00% | 0.00% | 0.00% | 0.00% |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer_Full | 0.00% | 0.00% | 0.00% | 2.50% |
| 3 | DFT-s-OFDM QPSK | Inner_Full | 0.00% | 0.00% | 1.31% | 2.49% |
| 4 | DFT-s-OFDM QPSK | Outer_Full | 0.00% | 0.00% | 1.79% | 4.01% |
| 5 | DFT-s-OFDM 16 QAM | Inner_Full | 0.00% | 1.48% | 2.85% | NA |
| 6 | DFT-s-OFDM 16 QAM | Outer_Full | 1.00% | 1.92% | 3.60% | NA |
| 7 | DFT-s-OFDM 64 QAM | Inner_Full | 2.49% | NA | NA | NA |
| 8 | DFT-s-OFDM 64 QAM | Outer_Full | 3.35% | NA | NA | NA |
| 9 | CP-OFDM QPSK | Inner_Full | 0.00% | 1.42% | 2.73% | 8.42% |
| 10 | CP-OFDM QPSK | Outer_Full | 0.00% | 1.58% | 3.04% | 8.42% |
| 11 | CP-OFDM 16 QAM | Inner_Full | 1.72% | 3.25% | 5.92% | NA |
| 12 | CP-OFDM 16 QAM | Outer_Full | 1.72% | 3.25% | 5.92% | NA |
| 13 | CP-OFDM 64 QAM | Inner_Full | NA | NA | NA | NA |
| 14 | CP-OFDM 64 QAM | Outer_Full | NA | NA | NA | NA |

Table 2. Resulting TT for EVM in PUSCH, PC3, FR2b for k=1 in proposal 1.

## K=0.9

No testability enhancements versus k=1.

## K=0.8

| Test ID | Modulation | RB alloc. | 50MHz | 100MHz | 200MHz | 400MHz |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner_Full | 0.00% | 0.00% | 0.00% | 0.00% |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer_Full | 0.00% | 0.00% | 0.00% | 0.00% |
| 3 | DFT-s-OFDM QPSK | Inner_Full | 0.00% | 0.00% | 0.00% | 1.29% |
| 4 | DFT-s-OFDM QPSK | Outer_Full | 0.00% | 0.00% | 0.00% | 1.74% |
| 5 | DFT-s-OFDM 16 QAM | Inner_Full | 0.00% | 0.00% | 1.22% | 3.43% |
| 6 | DFT-s-OFDM 16 QAM | Outer_Full | 0.00% | 0.00% | 1.34% | 3.43% |
| 7 | DFT-s-OFDM 64 QAM | Inner_Full | 0.85% | 1.58% | 2.89% | NA |
| 8 | DFT-s-OFDM 64 QAM | Outer_Full | 1.15% | 2.14% | 3.84% | NA |
| 9 | CP-OFDM QPSK | Inner_Full | 0.00% | 0.00% | 0.00% | 2.92% |
| 10 | CP-OFDM QPSK | Outer_Full | 0.00% | 0.00% | 1.10% | 2.92% |
| 11 | CP-OFDM 16 QAM | Inner_Full | 0.00% | 1.08% | 2.05% | 6.15% |
| 12 | CP-OFDM 16 QAM | Outer_Full | 0.00% | 1.08% | 2.05% | 6.15% |
| 13 | CP-OFDM 64 QAM | Inner_Full | 1.75% | 3.18% | NA | NA |
| 14 | CP-OFDM 64 QAM | Outer_Full | 1.75% | 3.18% | NA | NA |

Table 3. Resulting TT for EVM in PUSCH, PC3, FR2a for k=0.8 in proposal 1.

| Test ID | Modulation | EVM Min Req | Actual EVM Req to the UE |  |  |  | Actual EVM Req to the UE vs Min Req (%) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50MHz | 100MHz | 200MHz | 400MHz | 50MHz | 100MHz | 200MHz | 400MHz |
| 1 | DFT-s-OFDM PI/2 BPSK | 30.00% | 29.87% | 29.75% | 29.50% | 29.00% | 99.57% | 99.17% | 98.34% | 96.66% |
| 2 | DFT-s-OFDM PI/2 BPSK | 30.00% | 29.84% | 29.71% | 29.42% | 28.62% | 99.47% | 99.03% | 98.06% | 95.40% |
| 3 | DFT-s-OFDM QPSK | 17.50% | 17.28% | 17.07% | 16.63% | 17.15% | 98.73% | 97.55% | 95.05% | 97.98% |
| 4 | DFT-s-OFDM QPSK | 17.50% | 17.22% | 17.00% | 16.48% | 17.01% | 98.42% | 97.13% | 94.18% | 97.20% |
| 5 | DFT-s-OFDM 16 QAM | 12.50% | 12.05% | 11.66% | 12.16% | 11.32% | 96.42% | 93.29% | 97.25% | 90.56% |
| 6 | DFT-s-OFDM 16 QAM | 12.50% | 11.97% | 11.57% | 12.12% | 11.32% | 95.77% | 92.56% | 96.95% | 90.56% |
| 7 | DFT-s-OFDM 64 QAM | 8.00% | 7.76% | 7.50% | 6.91% | NA | 96.98% | 93.78% | 86.39% | NA |
| 8 | DFT-s-OFDM 64 QAM | 8.00% | 7.66% | 7.27% | 6.35% | NA | 95.73% | 90.86% | 79.38% | NA |
| 9 | CP-OFDM QPSK | 17.50% | 17.13% | 16.85% | 16.17% | 16.61% | 97.87% | 96.27% | 92.42% | 94.91% |
| 10 | CP-OFDM QPSK | 17.50% | 17.10% | 16.77% | 17.20% | 16.61% | 97.73% | 95.84% | 98.31% | 94.91% |
| 11 | CP-OFDM 16 QAM | 12.50% | 11.75% | 12.20% | 11.88% | 9.83% | 94.02% | 97.61% | 95.01% | 78.63% |
| 12 | CP-OFDM 16 QAM | 12.50% | 11.75% | 12.20% | 11.88% | 9.83% | 94.02% | 97.61% | 95.01% | 78.63% |
| 13 | CP-OFDM 64 QAM | 8.00% | 7.43% | 6.75% | NA | NA | 92.94% | 84.43% | NA | NA |
| 14 | CP-OFDM 64 QAM | 8.00% | 7.43% | 6.75% | NA | NA | 92.94% | 84.43% | NA | NA |

Table 3a. Actual EVM requirement to the UE in PUSCH, PC3, FR2a for k=0.8 in proposal 1.

| Test ID | Modulation | RB alloc. | 50MHz | 100MHz | 200MHz | 400MHz |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner_Full | 0.00% | 0.00% | 0.00% | 0.00% |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer_Full | 0.00% | 0.00% | 0.00% | 2.00% |
| 3 | DFT-s-OFDM QPSK | Inner_Full | 0.00% | 0.00% | 1.05% | 1.99% |
| 4 | DFT-s-OFDM QPSK | Outer_Full | 0.00% | 0.00% | 1.43% | 3.20% |
| 5 | DFT-s-OFDM 16 QAM | Inner_Full | 0.00% | 1.18% | 2.28% | NA |
| 6 | DFT-s-OFDM 16 QAM | Outer_Full | 0.80% | 1.54% | 2.88% | NA |
| 7 | DFT-s-OFDM 64 QAM | Inner_Full | 1.99% | 3.58% | NA | NA |
| 8 | DFT-s-OFDM 64 QAM | Outer_Full | 2.68% | NA | NA | NA |
| 9 | CP-OFDM QPSK | Inner_Full | 0.00% | 1.13% | 2.18% | 6.74% |
| 10 | CP-OFDM QPSK | Outer_Full | 0.00% | 1.27% | 2.43% | 6.74% |
| 11 | CP-OFDM 16 QAM | Inner_Full | 1.38% | 2.60% | 4.74% | NA |
| 12 | CP-OFDM 16 QAM | Outer_Full | 1.38% | 2.60% | 4.74% | NA |
| 13 | CP-OFDM 64 QAM | Inner_Full | 3.93% | NA | NA | NA |
| 14 | CP-OFDM 64 QAM | Outer_Full | 3.93% | NA | NA | NA |

Table 4. Resulting TT for EVM in PUSCH, PC3, FR2b for k=0.8 in proposal 1.

| Test ID | Modulation | EVM Min Req | Actual EVM Req to the UE |  |  |  | Actual EVM Req to the UE vs Min Req (%) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50MHz | 100MHz | 200MHz | 400MHz | 50MHz | 100MHz | 200MHz | 400MHz |
| 1 | DFT-s-OFDM PI/2 BPSK | 30.00% | 29.79% | 29.61% | 29.19% | 28.40% | 99.29% | 98.70% | 97.31% | 94.68% |
| 2 | DFT-s-OFDM PI/2 BPSK | 30.00% | 29.71% | 29.45% | 28.88% | 29.46% | 99.04% | 98.18% | 96.28% | 98.19% |
| 3 | DFT-s-OFDM QPSK | 17.50% | 17.13% | 16.82% | 17.22% | 16.93% | 97.91% | 96.12% | 98.38% | 96.74% |
| 4 | DFT-s-OFDM QPSK | 17.50% | 17.00% | 16.55% | 17.10% | 16.51% | 97.15% | 94.56% | 97.74% | 94.32% |
| 5 | DFT-s-OFDM 16 QAM | 12.50% | 11.65% | 12.17% | 11.79% | NA | 93.17% | 97.35% | 94.35% | NA |
| 6 | DFT-s-OFDM 16 QAM | 12.50% | 12.28% | 12.06% | 11.56% | NA | 98.28% | 96.44% | 92.46% | NA |
| 7 | DFT-s-OFDM 64 QAM | 8.00% | 7.34% | 6.51% | NA | NA | 91.70% | 81.44% | NA | NA |
| 8 | DFT-s-OFDM 64 QAM | 8.00% | 7.02% | NA | NA | NA | 87.69% | NA | NA | NA |
| 9 | CP-OFDM QPSK | 17.50% | 16.74% | 17.19% | 16.87% | 14.89% | 95.68% | 98.25% | 96.38% | 85.09% |
| 10 | CP-OFDM QPSK | 17.50% | 16.65% | 17.15% | 16.78% | 14.89% | 95.14% | 98.02% | 95.90% | 85.09% |
| 11 | CP-OFDM 16 QAM | 12.50% | 12.11% | 11.67% | 10.68% | NA | 96.86% | 93.37% | 85.41% | NA |
| 12 | CP-OFDM 16 QAM | 12.50% | 12.11% | 11.67% | 10.68% | NA | 96.86% | 93.37% | 85.41% | NA |
| 13 | CP-OFDM 64 QAM | 8.00% | 6.29% | NA | NA | NA | 78.64% | NA | NA | NA |
| 14 | CP-OFDM 64 QAM | 8.00% | 6.29% | NA | NA | NA | 78.64% | NA | NA | NA |

Table 4a. Actual EVM requirement to the UE in PUSCH, PC3, FR2b for k=0.8 in proposal 1.

## K=0.7

No testability enhancements versus k=0.8.

## K=0.65

| Test ID | Modulation | RB alloc. | 50MHz | 100MHz | 200MHz | 400MHz |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner_Full | 0.00% | 0.00% | 0.00% | 0.00% |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer_Full | 0.00% | 0.00% | 0.00% | 0.00% |
| 3 | DFT-s-OFDM QPSK | Inner_Full | 0.00% | 0.00% | 0.00% | 1.05% |
| 4 | DFT-s-OFDM QPSK | Outer_Full | 0.00% | 0.00% | 0.00% | 1.41% |
| 5 | DFT-s-OFDM 16 QAM | Inner_Full | 0.00% | 0.00% | 0.99% | 2.79% |
| 6 | DFT-s-OFDM 16 QAM | Outer_Full | 0.00% | 0.00% | 1.09% | 2.79% |
| 7 | DFT-s-OFDM 64 QAM | Inner_Full | 0.69% | 1.28% | 2.35% | NA |
| 8 | DFT-s-OFDM 64 QAM | Outer_Full | 0.93% | 1.74% | 3.12% | NA |
| 9 | CP-OFDM QPSK | Inner_Full | 0.00% | 0.00% | 0.00% | 2.38% |
| 10 | CP-OFDM QPSK | Outer_Full | 0.00% | 0.00% | 0.89% | 2.38% |
| 11 | CP-OFDM 16 QAM | Inner_Full | 0.00% | 0.88% | 1.67% | 4.99% |
| 12 | CP-OFDM 16 QAM | Outer_Full | 0.00% | 0.88% | 1.67% | 4.99% |
| 13 | CP-OFDM 64 QAM | Inner_Full | 1.42% | 2.58% | NA | NA |
| 14 | CP-OFDM 64 QAM | Outer_Full | 1.42% | 2.58% | NA | NA |

Table 5. Resulting TT for EVM in PUSCH, PC3, FR2a for k=0.6 in proposal 1.

| Test ID | Modulation | EVM Min Req | Actual EVM Req to the UE |  |  |  | Actual EVM Req to the UE vs Min Req (%) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50MHz | 100MHz | 200MHz | 400MHz | 50MHz | 100MHz | 200MHz | 400MHz |
| 1 | DFT-s-OFDM PI/2 BPSK | 30.00% | 29.87% | 29.75% | 29.50% | 29.00% | 99.57% | 99.17% | 98.34% | 96.66% |
| 2 | DFT-s-OFDM PI/2 BPSK | 30.00% | 29.84% | 29.71% | 29.42% | 28.62% | 99.47% | 99.03% | 98.06% | 95.40% |
| 3 | DFT-s-OFDM QPSK | 17.50% | 17.28% | 17.07% | 16.63% | 16.88% | 98.73% | 97.55% | 95.05% | 96.46% |
| 4 | DFT-s-OFDM QPSK | 17.50% | 17.22% | 17.00% | 16.48% | 16.64% | 98.42% | 97.13% | 94.18% | 95.08% |
| 5 | DFT-s-OFDM 16 QAM | 12.50% | 12.05% | 11.66% | 11.90% | 10.39% | 96.42% | 93.29% | 95.18% | 83.16% |
| 6 | DFT-s-OFDM 16 QAM | 12.50% | 11.97% | 11.57% | 11.83% | 10.39% | 95.77% | 92.56% | 94.65% | 83.16% |
| 7 | DFT-s-OFDM 64 QAM | 8.00% | 7.58% | 7.12% | 6.02% | NA | 94.70% | 89.00% | 75.29% | NA |
| 8 | DFT-s-OFDM 64 QAM | 8.00% | 7.40% | 6.70% | 4.88% | NA | 92.49% | 83.71% | 60.97% | NA |
| 9 | CP-OFDM QPSK | 17.50% | 17.13% | 16.85% | 16.17% | 15.93% | 97.87% | 96.27% | 92.42% | 91.02% |
| 10 | CP-OFDM QPSK | 17.50% | 17.10% | 16.77% | 16.98% | 15.93% | 97.73% | 95.84% | 97.03% | 91.02% |
| 11 | CP-OFDM 16 QAM | 12.50% | 11.75% | 11.98% | 11.40% | 7.41% | 94.02% | 95.81% | 91.21% | 59.31% |
| 12 | CP-OFDM 16 QAM | 12.50% | 11.75% | 11.98% | 11.40% | 7.41% | 94.02% | 95.81% | 91.21% | 59.31% |
| 13 | CP-OFDM 64 QAM | 8.00% | 7.00% | 5.71% | NA | NA | 87.49% | 71.43% | NA | NA |
| 14 | CP-OFDM 64 QAM | 8.00% | 7.00% | 5.71% | NA | NA | 87.49% | 71.43% | NA | NA |

Table 5a. Actual EVM requirement to the UE in PUSCH, PC3, FR2a for k=0.65 in proposal 1.

| Test ID | Modulation | RB alloc. | 50MHz | 100MHz | 200MHz | 400MHz |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner_Full | 0.00% | 0.00% | 0.00% | 0.00% |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer_Full | 0.00% | 0.00% | 0.00% | 1.63% |
| 3 | DFT-s-OFDM QPSK | Inner_Full | 0.00% | 0.00% | 0.85% | 1.62% |
| 4 | DFT-s-OFDM QPSK | Outer_Full | 0.00% | 0.00% | 1.16% | 2.60% |
| 5 | DFT-s-OFDM 16 QAM | Inner_Full | 0.00% | 0.96% | 1.85% | 6.15% |
| 6 | DFT-s-OFDM 16 QAM | Outer_Full | 0.65% | 1.25% | 2.34% | 6.15% |
| 7 | DFT-s-OFDM 64 QAM | Inner_Full | 1.62% | 2.91% | NA | NA |
| 8 | DFT-s-OFDM 64 QAM | Outer_Full | 2.18% | 3.84% | NA | NA |
| 9 | CP-OFDM QPSK | Inner_Full | 0.00% | 0.92% | 1.77% | 5.48% |
| 10 | CP-OFDM QPSK | Outer_Full | 0.00% | 1.03% | 1.97% | 5.48% |
| 11 | CP-OFDM 16 QAM | Inner_Full | 1.12% | 2.11% | 3.85% | NA |
| 12 | CP-OFDM 16 QAM | Outer_Full | 1.12% | 2.11% | 3.85% | NA |
| 13 | CP-OFDM 64 QAM | Inner_Full | 3.19% | NA | NA | NA |
| 14 | CP-OFDM 64 QAM | Outer_Full | 3.19% | NA | NA | NA |

Table 6. Resulting TT for EVM in PUSCH, PC3, FR2b for k=0.6 in proposal 1.

| Test ID | Modulation | EVM Min Req | Actual EVM Req to the UE |  |  |  | Actual EVM Req to the UE vs Min Req (%) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50MHz | 100MHz | 200MHz | 400MHz | 50MHz | 100MHz | 200MHz | 400MHz |
| 1 | DFT-s-OFDM PI/2 BPSK | 30.00% | 29.79% | 29.61% | 29.19% | 28.40% | 99.29% | 98.70% | 97.31% | 94.68% |
| 2 | DFT-s-OFDM PI/2 BPSK | 30.00% | 29.71% | 29.45% | 28.88% | 29.05% | 99.04% | 98.18% | 96.28% | 96.83% |
| 3 | DFT-s-OFDM QPSK | 17.50% | 17.13% | 16.82% | 17.00% | 16.50% | 97.91% | 96.12% | 97.17% | 94.28% |
| 4 | DFT-s-OFDM QPSK | 17.50% | 17.00% | 16.55% | 16.81% | 15.75% | 97.15% | 94.56% | 96.04% | 89.97% |
| 5 | DFT-s-OFDM 16 QAM | 12.50% | 11.65% | 11.92% | 11.25% | 4.67% | 93.17% | 95.34% | 90.03% | 37.34% |
| 6 | DFT-s-OFDM 16 QAM | 12.50% | 12.12% | 11.72% | 10.83% | 4.67% | 96.98% | 93.75% | 86.62% | 37.34% |
| 7 | DFT-s-OFDM 64 QAM | 8.00% | 6.82% | 5.23% | NA | NA | 85.24% | 65.35% | NA | NA |
| 8 | DFT-s-OFDM 64 QAM | 8.00% | 6.22% | 3.27% | NA | NA | 77.78% | 40.89% | NA | NA |
| 9 | CP-OFDM QPSK | 17.50% | 16.74% | 16.96% | 16.39% | 12.73% | 95.68% | 96.93% | 93.63% | 72.74% |
| 10 | CP-OFDM QPSK | 17.50% | 16.65% | 16.89% | 16.24% | 12.73% | 95.14% | 96.54% | 92.80% | 72.74% |
| 11 | CP-OFDM 16 QAM | 12.50% | 11.81% | 11.03% | 9.17% | NA | 94.48% | 88.27% | 73.38% | NA |
| 12 | CP-OFDM 16 QAM | 12.50% | 11.81% | 11.03% | 9.17% | NA | 94.48% | 88.27% | 73.38% | NA |
| 13 | CP-OFDM 64 QAM | 8.00% | 4.75% | NA | NA | NA | 59.34% | NA | NA | NA |
| 14 | CP-OFDM 64 QAM | 8.00% | 4.75% | NA | NA | NA | 59.34% | NA | NA | NA |

Table 6a. Actual EVM requirement to the UE in PUSCH, PC3, FR2b for k=0.65 in proposal 1.