3GPP TSG-RAN5 Meeting 95-e	R5-223628

# Electronic Meeting, 9th May – 20th  May 2022

Title:		Further discussion on large UE gain range

Source:			Ericsson

Agenda Item:			5.4.17

Document for:			Endorsement

# 1.	Introduction

During RAN5 #93e a preamble method for RRM was proposed in [1] in connection with issue #7 on the RRM issue list in [2] and outlined in [3]. The consensus was not reached, and more detailed further discussion was assumed.

In this document we further discuss the issue of large UE antenna gain range and propose a solution to make UE accuracy testing more meaningful.

# 2.	Discussion

What contributes to the total MU can be seen in below excerpt from 38.903 [6].

The biggest contributor to the total uncertainty is the UE gain variation ±15dB for PC3 UE.

As an example, for a UE power class 3 in band n257, with applied Io > -70dBm, the variation of SS-RSRP would be (±15dB ±8dB) = ±28.25dB (together with the Test system downlink power uncertainty of ±5.65dB) . It is possible that some test cases with very wide range of reported values may be considered irrelevant.

Example from TT analysis to TC 7.7.1.1 [7] where UE can report SS-RSRP between -106.03 dBm to -48.71 dBm:

Observation 1: RRM performance test cases with very wide range of reported values may be considered irrelevant.

Observation 2: UE gain variation of +/- 15 dB is largest contributor to the total measurement uncertainty.

Originally this issue was discussed in terms of testability of test cases with absolute thresholds comparison such as a4-threshold. As it was proven in TT analysis [4, 5, 6], no such issue exists. Moreover, during RAN5 #93_e the discussion paper on that topic were presented [1] with corresponding 38.508-1 CR [7]. The discussion outcome was that for test cases other than performance, where the purpose of the test is not strictly related to the SS-RSRP values measured and reported by the UE, rather the time requirements check – there is no need for the preamble procedure. Some concerns have been raised whether preamble method can be applied to the RRM performance test cases. Example comment from Qualcomm below:

Moreover, as it was pointed by R&S, the large UE gain G variation issue has been discussed in RAN4 and resulted in introducing additional requirements R7 and R8 comparing the absolute SS-RSRP reports in different time instances T1 and T2 and different SSB_RP conditions.

Observation 3: The influence of UE gain G can be removed by comparing results for the same cell in different time instances as already done in TC 7.7.1.1 for R7 and R8.

In this document we propose to extend the understanding applied in R7 and R8, namely removing the influence of UE gain variation G, into R1, R2, R4 and R5. This can be achieved without adding any new procedure but simply adding additional condition implying that the difference between max and min values measured respectively for R1, R2, R4 and R5 should not exceed certain value (in case 7.7.1.1 it is 16dB). The proposed change is provided in the accompanying 38.533 CR [8] as well as TT analysis correction in [9].

Proposal 1: Add new table 7.7.1.1.5-3a: evaluation limits for the ∆(Max-Min) reported values for each cell during each time periodto test case with additional condition that the difference between max and min reported absolute value shall not exceed the value calculated in the corresponding TT analysis.

Proposal 2: Correct the Test Tolerance calculation worksheet and word document in 38.903

# 3.	Conclusion

Observation 1: RRM performance test cases with very wide range of reported values may be considered irrelevant.

Observation 2: UE gain variation of +/- 15 dB is largest contributor to the total measurement uncertainty.

Observation 3: The influence of UE gain G can be removed by comparing results for the same cell in different time instances as already done in TC 7.7.1.1 for R7 and R8.

Proposal 1: Add note to test case with additional condition that the difference between max and min reported absolute value shall not exceed the value calculated in the corresponding TT analysis

Proposal 2: Correct the Test Tolerance calculation worksheet and word document in 38.903

# 4.	References

[1] R5-218413, RRM known issue #7 - Absolute thresholds / large UE gain range, Ericsson, RAN5#93e

[2] R5-215410, “FR2 RRM test cases: Known Issue List - after RAN5_92e”, Ericsson, RAN5#92e

[3] R5-212634, “FR2 RRM test cases: Known Issues ”, Anritsu, RAN5#89e

[4] R5-213943, Test Tolerance analysis for FR2 event triggered reporting test cases, Ericsson, RAN5#91e

[5] R5-213944, Test Tolerance analysis for FR2 event triggered reporting test cases, Ericsson, RAN5#91e

[6] R5-216363, Test Tolerance analysis for FR2 SSB-based L1-RSRP measurement for beam reporting test cases, Ericsson, RAN5#92e

[7] R5-217567, Procedure for UE preamble for RRM, Ericsson, RAN5#93e

[8] R5- R5-223169, Correction of RRM test case 7.7.1.1, Ericsson, RAN5#95e

[9] R5- R5-223170, Correction of Test Tolerance for RRM test case 7.7.1.1, Ericsson, RAN5#95e