3GPP TSG-RAN5 Meeting #95-e	R5-223688

# Electronic meeting, 9 – 20 May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.905 | CR | 0597 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of reference sensitivity test point analysis for DC_3A-7A_n5A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TP analysis needs to be done and documented in TR 38.905 before test requirements for a new EN-DC configuration can be introduced in TS 38.521-3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding reference sensitivity TP analysis for DC_3A-7A_n5A. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No reference sensitivity test point analysis defined for DC_3A-7A_n5A |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-3... CR 1348 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Correct configuration acronyms in the coversheet. |  |  |  |  |  |  |  |  |

-------- Start of changes --------

### 4.3.3	Test point analysis per EN-DC configuration

#### 4.3.3.1	Reference sensitivity test cases for EN-DC

This clause contains information on test point analysis and status for FR1 EN-DC test cases in TS 38.521-3 [4] clause 7. The analyses are performed per EN-DC configuration in Table 4.3.3.1-1, Table 4.3.3.1-2 and Table 4.3.3.1-3.

Table 4.3.3.1-1: Reference Sensitivity test cases per EN-DC configuration (2CC)

| Band or band configuration | Single UL | Refsens exception, victim band (NOTE 2) | Requirement coverage (NOTE 2) | Comments |
| --- | --- | --- | --- | --- |
| DC_(n)XAA |  |  |  |  |
| 5 | Only single UL requirements defined | 5 | Exception | Added at RAN5#92-e |
| 12 | Only single UL requirements defined | 12 | Exception | Added at RAN5#92-e |
| 38 | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| 41 | Yes | N/A | NE | Added at RAN5#88-e |
| 48 | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| 71 | Yes | 71 | Exception | Added at RAN5#88-e |
| DC_XA_nXA |  |  |  |  |
| DC_2A_n2A | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| DC_3A_n3A | Yes | 3 | Exception | Added at RAN5#88-e |
| DC_5A_n5A | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| DC_7A_n7A | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| DC_41A_n41A | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| DC_48A_n48A | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| DC_66A_n66A | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| DC_71A_n71A | Only single UL requirements defined | N/A | NE | Added at RAN5#92-e |
| DC_XA-nYA (NOTE 1) |  |  |  |  |
| DC_1A_n3A | Yes | 1 (IMD3) CBI | NE IMD3 CBI, CBI avoid | Added at RAN5#89-e |
| DC_1A-n5A | No | 1 (IMD4) | NE IMD4 | Added at RAN5#94-e |
| DC_1A-n7A | No | N/A | NE | Added at RAN5#94-e |
| DC_1A_n78A | No | 1 (IMD4) | NE IMD4 | Added at RAN5#88-e |
| DC_2A_n66A | No | 3 (IMD3) n66 (IMD5) | NE IMD3 IMD5 | Added at RAN5#94-e |
| DC_2A_n71A | No | 2 (HD3) n71 (HM) | NE HD HM | Added at RAN5#93-e |
| DC_2A_n77A | No | n77 (HD) 2 (HM) 2 (IMD2) 2 (IMD4) 2 (IMD5) | NE HD, HD avoid HM, HM avoid IMD2 PC2&PC3 IMD4 PC2&PC3 IMD5 PC3 | Added at RAN5#94-e |
| DC_2A_n78A | No | n78 (HD2) 2 (IMD2) 2 (IMD4) | NE HD IMD2 IMD4 | Added at RAN5#94-e |
| DC_3A_n1A | Yes | 1 (IMD3) CBI | NE IMD3 CBI, CBI avoid | Added at RAN5#89-e |
| DC_3A_n5A | Yes | 3 (IMD4) n5 (IMD2) | NE IMD2 IMD4 | Added at RAN5#94-e |
| DC_3A_n78A | Yes | 3 (IMD2) 3 (IMD4) 3 (HD) n78 (HM) | NE IMD2 PC2&PC3 IMD4 PC2&PC3 HD HM, HM avoid | Added at RAN5#89-e |
| DC_5A_n66A | Yes | 5 (IMD2) | NE (anchor agnostic) IMD2 | Added at RAN5#93-e |
| DC_5A_n77A | No | n77 (HD) 5 (IMD4) 5 (IMD5) | NE HD, HD avoid IMD4 PC2&PC3 IMD5 PC3 | Added at RAN5#94-e |
| DC_7A_n1A | No | N/A | NE | Added at RAN5#90-e |
| DC_7A_n3A | No | 7 (IMD4¦3*fB3-1*fB7¦) | NE, IMD4 | Added at RAN5#90-e |
| DC_7A_n5A | Yes | n5 (IMD3) | NE, IMD3 | Added at RAN5#94-e |
| DC_7A_n28A | No | N/A | NE | Added at RAN5#94-e |
| DC_7A_n78A | No | CBI | NE, CBI | Added at RAN5#94-e |
| DC_8A_n1A | No | n1 (IMD4 ¦2*fB1-2*fB8¦) | NE, IMD4 | Added at RAN5#90-e |
| DC_8A_n3A | No | 8 (IMD4 ¦3*fB8-1*fB3¦), n3 (IMD5 ¦4*fB8-1*fB3¦) | NE, IMD4, IMD5 | Added at RAN5#90-e |
| DC_8A_n20A | Yes | 8 (IMD3) n20 (IMD3) | NE IMD3 | Added at RAN5#94-e |
| DC_8A_n77A | No | 8 (HD4), 8 (IMD4) | NE HD, HD avoid IMD4 | Added at RAN5#92-e |
| DC_11A_n77A | No | N/A | NE | Added at RAN5#91-e |
| DC_11A_n78A | No | N/A | NE | Added at RAN5#91-e |
| DC_11A_n79A | No | 11 (HM) | NE HM, HM avoid | Added at RAN5#92-e |
| DC_13A_n77A | No | n77 (HD) 13 (HM) 13 (IMD5) | NE HD, HD avoid HM, HM avoid IMD5 PC2&PC3 | Added at RAN5#94-e |
| DC_19A_n79A | No | 19 (HM ¦5*fB19DL-1*fB79¦), | NE, HM, HM avoid | Added at RAN5#90-e |
| DC_20A_n1A | No | N/A | NE | Added at RAN5#90-e |
| DC_20A_n3A | No | n3 (IMD4), 20 (IMD4) | NE IMD4 | Added at RAN5#89-e |
| DC_20A_n7A | DC_20_n7 | 20 (IMD3) | NE IMD3 | Added at RAN5#94-e |
| DC_20A_n78A | No | 20 (IMD4), n78 (HD) | NE IMD4, HD, HD avoid | Added at RAN5#88-e |
| DC_25A_n41A | No | N/A | NE | Added at RAN5#91-e |
| DC_26A_n41A | No | n41 (HD3), 26 (IMD3) | NE HD, HD avoid IMD | Added at RAN5#92-e |
| DC_26A_n77A | No | n77 (HD4), 26 (IMD4) | NE HD, HD avoid IMD | Added at RAN5#92-e |
| DC_26A_n78A | No | n78 (HD4), 26 (IMD4) | NE HD, HD avoid IMD | Added at RAN5#92-e |
| DC_26A_n79A | No | 26 (HM ¦4*fB26DL-1*fB79¦) | NE HM, HM avoid | Added at RAN5#92-e |
| DC_28A_n5A | No | CBI | NE CBI, CBI avoid | Added at RAN5#94-e |
| DC_28A_n7A | No | N/A | NE | Added at RAN5#94-e |
| DC_28A_n77A | No | n77 (HD), n77 (HM), 28 (IMD5) | NE (HD avoid), HD, HM, IMD5 | Added at RAN5#92-e |
| DC_28A_n78A | No | n78 (HD), 28 (IMD5) | NE (HD avoid), HD, IMD5 | Added at RAN5#92-e |
| DC_40A_n1A | No | N/A | NE | Added at RAN5#89-e |
| DC_40A_n78A | No | 40 (HM) | NE HM, HM avoid | Added at RAN5#89-e |
| DC_40A_n79A | No | N/A | NE | Added at RAN5#94-e |
| DC_41A_n77A (NOTE 3) | No | 41 (HM ¦3*fB41DL-2*fB77¦), 41 (CBI), n77 (CBI) | NE HM CBI | Added at RAN5#92-e |
| DC_41A_n78A (NOTE 3) | No | 41 (HM ¦3*fB41DL-2*fB77¦), 41 (CBI), n78 (CBI) | NE HM CBI | Added at RAN5#92-e |
| DC_42A_n77A | Only single UL requirements defined | N/A | NE | Added at RAN5#91-e |
| DC_48A_n66A | No | 48 (HD2) 66 (IMD5) | NE (anchor agnostic) HD2 IMD5 | Added at RAN5#92-e |
| DC_66A_n2A | DC_66_n2 | n2 (IMD3) 66 (IMD5) | NE (anchor agnostic) IMD3 IMD5 | Added at RAN5#93-e |
| DC_66A_n71A | No | 66 (IMD4) | NE (anchor agnostic) IMD4 | Added at RAN5#93-e |
| DC_66A_n77A | No | n77 (HD) 66 (IMD2) 66 (IMD5) | NE HD, HD avoid IMD2 PC2&PC3 IMD5 PC2&PC3 | Added at RAN5#94-e |
| DC_66A_n78A | No | n78 (HD2) 66 (IMD5) | NE (anchor agnostic) HD2 IMD5 | Added at RAN5#93-e |
| NOTE 1: If single UL is allowed in a DC_XA_nYA configuration the IMD requirements does not apply for UEs supporting UE capability singleUL-Transmission. NOTE 2: Notations used NE: Non-exception as defined in TS 38.101-3 [7], meaning standalone LTE in TS 36.101 [8] and NR in 38.101-1 [5] requirements apply. HD: UL Harmonic Distortion, as defined in TS 38.101-3 [7], 7.3B.2.3.1 HM: RX Harmonic Mixing, as defined in TS 38.101-3 [7], 7.3B.2.3.2 IMD: Intermodulation Distortion, as defined in TS 38.101-3 [7], 7.3B.2.3.5 CBI: Cross Band Isolation, as defined in TS 38.101-3 [7], 7.3B.2.3.4 HD avoid: single carrier requirements apply with aggressor of UL Harmonic Distortion active. HM avoid: single carrier requirements apply with aggressor of RX Harmonic Mixing active. CBI avoid: single carrier requirements apply with aggressor of Cross Band Isolation active. NOTE 3: HM cannot be avoided with aggressor still active since CBI exception always exists. Therefore, no HM avoid test point is added. |  |  |  |  |

Table 4.3.3.1-2: Reference Sensitivity test cases per EN-DC configuration (3CC)

| Band or band configuration | Single UL | UL config | Refsens exception, victim band | Requirement coverage (NOTE 2) | Fallback DC configurations | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| DC_(n)XCA |  |  |  |  |  |  |
| 41 | Yes | DC_(n)41AA | N/A | NE | DC_(n)41AA DC_41A_n41A | Added at RAN5#89-e |
| 41 | Yes | DC_41A_n41A | N/A | NE | DC_(n)41AA DC_41A_n41A | Added at RAN5#89-e |
| 48 | Only single UL requirements defined | DC_(n)48AA | N/A | NE | DC_(n)48AA DC_48A_n48A | Added at RAN5#89-e |
| 48 | Only single UL requirements defined | DC_48A_n48A | N/A | NE | DC_(n)48AA DC_48A_n48A | Added at RAN5#89-e |
| DC_XA_nYC |  |  |  |  |  |  |
| DC_3A_n78C | Yes | DC_3A_n78A | N/A | NE |  | Added at RAN5#93-e |
| DC_XA_nY(2A) |  |  |  |  |  |  |
| FFS |  |  |  |  |  |  |
| DC_XA_nYA-nZA |  |  |  |  |  |  |
| DC_1A_n28A-n78A | No | DC_1A_n28A | n78 (3 band IMD3) | NE, IMD3 |  | Added at RAN5#92-e |
|  | No | DC_1A_n78A | n28 (3 band IMD5) | NE, IMD5 |  | Added at RAN5#92-e |
| DC_3A_n28A-n78A | No | DC_3A_n28A | n78 (3 band IMD5) | NE, IMD5 |  | Added at RAN5#92-e |
|  | Yes | DC_3A_n78A | No 3CC exception | No test required |  |  |
| DC_7A_n5A-n78A | Yes | DC_7A_n5A | No 3CC exception | No test required |  | Added at RAN5#94-e |
|  | No | DC_7A_n78A | No 3CC exception | No test required |  | Added at RAN5#94-e |
| DC_7A_n28A-n78A | No | DC_7A_n28A | n78 (3 band IMD2) | NE, IMD2 |  | Added at RAN5#92-e |
|  | No | DC_7A_n78A | n28 (3 band IMD2) | NE, IMD2 |  |  |
| DC_20A_n28A-n78A | No | DC_20A_n28A | n78 (3 band IMD4) | NE, IMD4 |  | Added at RAN5#93-e |
|  | No | DC_20A_n78A | n28 (3 band IMD4) | NE, IMD4 |  | Added at RAN5#93-e |
| DC_28A_n7A-n78A | No | DC_28A_n7A | n78 (3 band IMD2) | NE, IMD2 |  | Added at RAN5#94-e |
|  | No | DC_28A_n78A | n7 (3 band IMD2) | NE, IMD2 |  | Added at RAN5#94-e |
| DC_XA-YA_nZA |  |  |  |  |  |  |
| DC_1A-3A_n28A | No | DC_1A_n28A | 3 (3 band IMD5) | IMD5 |  | Added at RAN5#92-e |
|  | No | DC_3A_n28A | 1 (3 band IMD4) | IMD4 |  | Added at RAN5#92-e |
| DC_1A-7A_n3A | Yes | DC_1A_n3A | No 3CC exception | No test required |  |  |
|  | No | DC_7A_n3A |  |  |  |  |
| DC_1A-7A_n28A | No | DC_1A_n28A | 7 (3 band IMD2) | IMD2 |  | Added at RAN5#92-e |
|  | No | DC_7A_n28A | No 3 band exception | No test required |  | Added at RAN5#92-e |
| DC_1A-7A_n78A | No | DC_1A_n78A | 7 (IMD4 ¦3*fB1-1*fB78¦) | IMD4 |  |  |
|  | No | DC_7A_n78A | 1 (IMD4 ¦2*fB7-2*fB78¦) | IMD4 |  |  |
| DC_1A-8A_n3A | Yes | DC_1A_n3A | No 3CC exception | No test required |  |  |
|  | No | DC_8A_n3A |  |  |  |  |
| DC_1A-20A_n3A | Yes | DC_1A_n3A | No 3CC exception | No test required |  |  |
|  | No | DC_20A_n3A |  |  |  |  |
| DC_1A-20A_n28A | No | DC_1A_n28A | No 3CC exception | No test required |  | Added at RAN5#93-e |
|  | No | DC_20A_n28A | No 3CC exception | No test required |  | Added at RAN5#93-e |
| DC_1A-20A_n78A | No | DC_1A_n78A | 20 (IMD5 ¦ | IMD5 |  |  |
|  | No | DC_20A_n78A | 1 (IMD3 | IMD3 |  |  |
| DC_1A-28A_n3A | Yes | DC_1A_n3A | 1 (3 band IMD4) | IMD4 | Yes | Added at RAN5#90-e |
|  | No | DC_28A_n3A | No 3CC exception | - | No |  |
| DC_1A-28A_n78A | No | DC_1A_n78A | 28 (3 band IMD3) | IMD3 |  | Added at RAN5#92-e |
|  | No | DC_28A_n78A | 1 (3 band IMD5) | IMD5 | DC_28A_n78A |  |
| DC_3A-7A_n1A | Yes | DC_3A_n1A | No 3CC exception | No test required |  | Added at RAN5#90-e |
|  | No | DC_7A_n1A | No 3CC exception |  |  |  |
| DC_3A-7A_n5A | Yes | DC_3A_n5A | 7 (3 band IMD2) | IMD2 |  | Added at RAN5#95-e |
|  | Yes | DC_7A_n5A | No 3CC exception | - |  | Added at RAN5#95-e |
| DC_3A-7A_n28A | No | DC_3A_n28A | 7 (3 band IMD3) | IMD3 |  | Added at RAN5#92-e |
|  | No | DC_7A_n28A | 3 (3 band IMD2) | IMD2 |  | Added at RAN5#92-e |
| DC_3A-20A_n1 | Yes | DC_3A_n1A | No 3CC exception | No test required |  |  |
|  | No | DC_20A_n1A |  |  |  |  |
| DC_3A-20A_n28A | No | DC_3A_n28A | No 3CC exception | No test required |  | Added at RAN5#92-e |
|  | No | DC_20A_n28A | 3 (3 band IMD4) | IMD4 |  | Added at RAN5#92-e |
| DC_3A-20A_n78 | Yes | DC_3A_n78A | No 3CC exception |  |  |  |
|  | No | DC_20A_n78A | 3 (3 band IMD3) | IMD3 |  |  |
| DC_3A-28A_n78A | Yes | DC_3A_n78A | No 3CC exception |  | DC_3A_n78A | Added at RAN5#92-e |
|  | No | DC_28A_n78A | 3 (3 band IMD3) | IMD3 | DC_28A_n78A |  |
| DC_3A-40A_n1A | Yes | DC_3A_n1A | 40 (3 band IMD5) | IMD5 if UE supporting dual UL |  |  |
|  | No | DC_40A_n1A | No 3CC exception |  |  |  |
| DC_7A-20A_n1A | No | DC_20A_n1A | 20 (3 band IMD5) | IMD5 |  | Added at RAN5#90-e |
|  | No | DC_7A_n1A | No 3CC exception | _ |  |  |
| DC_7A-20A_n3A | No | DC_7A_n3A | 20 (IMD2 ¦1*fB7-1*fB3) | NE, IMD2 |  |  |
|  | No | DC_20A_n3A | 7 (IMD2 ¦1*fB3+1*fB20) | NE, IMD2 |  |  |
| DC_7A-20A_n28A | No | DC_7A_n28A | No 3CC exception | No test required |  | Added at RAN5#92-e |
|  | No | DC_20A_n28A | 7 (3 band IMD5) | IMD 5 |  |  |
| DC_7A-20A_n78A | No | DC_7A_n78A | 20 (3 band IMD2) 20 (3 band IMD5) | IMD2, IMD5 |  |  |
|  | No | 2 DC_0A_n78A | 7 (3 band IMD2) | IMD2 |  |  |
| DC_7A-28A_n3A | No | DC_7A_n3A | 28 (3 band IMD2) | IMD2 |  | Added at RAN5#90-e |
|  | No | DC_28A_n3A | 7 (3 band IMD3) | IMD3 |  |  |
| DC_7A-28A_n78A | No | DC_7A_n78A | 28 (3 band IMD2) 28 (3 band IMD5) | IMD2, IMD5 |  | Added at RAN5#92-e |
|  | No | DC_28A_n78A | 7 (3 band IMD2) | IMD2 | DC_28A_n78A |  |
| DC_(n)XAA-nYA |  |  |  |  |  |  |
| FFS |  |  |  |  |  |  |
| DC_XA_nXA_nYA |  |  |  |  |  |  |
| FFS |  |  |  |  |  |  |
| NOTE 1: If single UL is allowed in a DC_XA_nYA configuration the IMD requirements does not apply for UEs supporting UE capability singleUL-Transmission. NOTE 2: Notations used NE: Non-exception as defined in TS 38.101-3 [7], meaning standalone LTE in TS 36.101 [8] and NR in 38.101-1 [5] requirements apply. HD: UL Harmonic Distortion, as defined in TS 38.101-3 [7], 7.3B.2.3.1 HM: RX Harmonic Mixing, as defined in TS 38.101-3 [7], 7.3B.2.3.2 IMD: Intermodulation Distortion, as defined in TS 38.101-3 [7], 7.3B.2.3.5 CBI: Cross Band Isolation, as defined in TS 38.101-3 [7], 7.3B.2.3.4 |  |  |  |  |  |  |

-------- End of changes --------