3GPP TSG-RAN5 Meeting #95-e	R5-223762

# Electronic meeting, 9 – 20 May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1641 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of In-band emissions for NR-DC in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of In-band emissions for NR-DC in FR1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add NR-DC for clause 6.4B.2.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In-band emissions for NR-DC in FR1 will not be specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4B.2.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Add Test Channel Bandwidths separately for CA and NR-DC in Table 6.4A.2.3.1.4.1-1 r2: Add NRB to Test CBW, to be in line with R5-223140 adding NRB_agg to CA r3: Remove changes to CA clause. Add content and note to 6.4B.2.3 with reference to 6.4A.2.3. |  |  |  |  |  |  |  |  |

-------- Start of changes --------

6.4A.2.3.1.5	Test requirement

The averaged In-band emission result, derived in Annex E.4.3, shall not exceed the corresponding values in Table 6.4A.2.3.1.5-1. n is 10 for 15kHz SCS, 20 for 30kHz SCS and 40 for 60kHz SCS..

Table 6.4A.2.3.1.5-1: Test requirements for in-band emissions

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | + TT |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -28+TT | Image frequencies when output power > 10 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -25+TT | Image frequencies when output power ≤ 10 dBm |  |
| Carrier leakage | dBc | -28+TT | Output power > 10 dBm | Carrier leakage frequency (NOTES 4, 5) |
|  |  | -25+TT | 0 dBm ≤ Output power ≤10 dBm |  |
|  |  | -20+TT | -30 dBm ≤ Output power ≤ 0 dBm |  |
|  |  | -10+TT | -40 dBm Output power < -30 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB - 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRBis defined in NOTE 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the carrier leakage frequency, but excluding any allocated RBs. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit depend on the parameter txDirectCurrentLocation in UplinkTxDirectCurrent IE, and are those that are enclosed either in the RBs containing the carrier leakage frequency, or in the two RBs immediately adjacent to the carrier leakage frequency but excluding any allocated RB. NOTE 6: is the Transmission Bandwidth (see Section 5.3). NOTE 7: is the Transmission Bandwidth Configuration (see Section 5.3). NOTE 8: is the limit specified in Table 6.4.2.1.3-1 for the modulation format used in the allocated RBs. NOTE 9: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. or for the first adjacent RB outside of the allocated bandwidth. NOTE 10: is the transmitted power normalized by the number of allocated RBs, measured in dBm. |  |  |  |  |

Table 6.4A.2.3.1.5-2: Test Tolerance for In-band emission

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.8dB | 0.8dB |
| 40MHz < BW ≤ 100MHz | 0.8dB | 0.8dB |

#### 6.4B.2.3	In-band emissions for NR-DC

For inter-band dual connectivity, the in-band emissions for the corresponding inter-band CA configuration as specified in clause 6.4A.2.3 applies.

Note: For NR-DC testing, replace CA by NR-DC, PCC by PCell, and SCC by PSCell in clause 6.4A.2.3.

## 6.4C	Transmit signal quality for SUL

-------- End of changes --------