3GPP TSG-RAN5 Meeting #95-e	R5-223792

# Electronic Meeting, 9th – 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2322 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to test frequency for n53 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-05-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The mid test frequency for band n53 and an SCS of 60 kHz is incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The test frequency has been corrected in Table 4.3.1.1.1.53-3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Band n53 cannot be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1.1.53 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the final outcome of one revision of R5-222547. |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

4.3.1.1.1.53	Reference test frequencies for NR operating band n53

Table 4.3.1.1.1.53-1: Test frequencies for NR operating band n53 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 2486 | 497200 | 2483.75 | 496750 | 0 | 15 | 6215 | 497290 | 0 | 1 | 2 (4) | 5 |
|  |  | & | Mid | 2489.3 | 497860 | 2468.69 | 493738 | 102 |  | 6221 | 497770 | 0 | 0 | 0 (0) | 102 |
|  |  | Uplink | High | 2492.5 | 498500 | 2399.53 | 479906 | 504 |  | 6232 | 498530 | 4 | 1 | 1 (2) | 507 |
| 10 | 52 | Downlink | Low | 2488.5 | 497700 | 2483.82 | 496764 | 0 | 15 | 6216 | 497310 | 2 | 1 | 2 (4) | 5 |
|  |  | & | Mid | 2489.3 | 497860 | 2466.6 | 493252 | 102 |  | 6215 | 497290 | 2 | 0 | 0 (0) | 102 |
|  |  | Uplink | High | 2490 | 498000 | 2394.6 | 478920 | 504 |  | 6219 | 497550 | 6 | 1 | 1 (2) | 507 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.53-2: Test frequencies for NR operating band n53 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 2488.5 | 497700 | 2484.18 | 496836 | 0 | 15 | 6222 | 497790 | 6 | 0 | 3 (8) | 16 |
|  |  | & | Mid | 2489.3 | 497860 | 2448.26 | 489652 | 102 |  | 6221 | 497770 | 18 | 0 | 0 (5) | 214 |
|  |  | Uplink | High | 2490 | 498000 | 2304.24 | 460848 | 504 |  | 6225 | 498030 | 10 | 0 | 2 (7) | 1022 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.53-3: Test frequencies for NR operating band n53 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 2488.5 | 497700 | 2484.54 | 496908 | 0 | 15 | - | 497268 |
|  |  | & | Mid | 2489.3 | 497860 | 2411.9 | 482380 | 102 |  | - | 497428 |
|  |  | Uplink | High | 2490 | 498000 | 2123.16 | 424632 | 504 |  | - | 497568 |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.54 to 4.3.1.1.1.64	FFS

< End of changes >