3GPP TSG-RAN5 Meeting #95-e	R5-223793

# Electronic Meeting, May 9 – 20, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2369 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of PCC and SCC configuration for CA test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The PCC and SCC mapping is clearly indicated for intra-band contigous CA configurations, but still missing for intra-band non-contiguous CA and inter-band CA. The information is needed so that the test case execution could be aligned among different test equipment and labs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding clarification for PCC and SCC mapping for intra-band non-contiguous CA and inter-band CA. For intra-band non-contiguous CA, the PCC is configured on the lower sub-block. For inter-band CA, the PCC is configured on the band with lower band index. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PCC configuration is not aligned among test equipments and labs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1.2.1, 4.3.1.1.2.2, 4.3.1.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

4.3.1.1.2	NR inter-band CA configurations in FR1

4.3.1.1.2.1	NR inter-band CA configurations in FR1 (two bands)

Table 4.3.1.1.2.1-1: Inter-band NR CA configurations (FR1, two bands)

| NR CA configuration | Uplink NR CA configuration | NR CA downlink configuration band 1 (Note X) | NR CA downlink configuration band 2 | NR CA uplink configuration band 1 | NR CA uplink configuration band 2 | Applicable for protocol testing (Note 2) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n3A | CA_n1A-n3A | n1A | n3A | n1A | n3A | Yes |
| CA_n1(2A)-n3A | - | n1(2A) | n3A | n1A | n3A | No |
| CA_n1(2A)-n5A | - | n1(2A) | n5A | - | - | No |
| CA_n1(2A)-n8A | - | n1(2A) | n8A | - | - | No |
| CA_n1A-n77A | - | n1A | n77A | - | - | Yes |
| CA_n1A-n78A | CA_n1A-n78A | n1A | n78A | n1A | n78A | Yes |
| CA_n1A-n78(2A) | CA_n1A-n78A | n1A | n78(2A) | n1A | n78A | No |
| CA_n1A-n78C | CA_n1A-n78A | n1A | n78C | n1A | n78A | No |
| CA_n1(2A)-n78A | - | n1(2A) | n78A | - | - | No |
| CA_n3A-n5A | CA_n3A-n5A | n3A | n5A | n3A | n5A | Yes |
| CA_n3(2A)-n5A | - | n3(2A) | n5A | - | - | No |
| CA_n3(2A)-n8A | - | n3(2A) | n8A | - | - | No |
| CA_n3A-n41A | CA_n3A-n41A | n3A | n41A | n3A | n41A | Yes |
| CA_n3A-n77A | - | n3A | n77A | - | - | Yes |
| CA_n3A-n78A | CA_n3A-n78A | n3A | n78A | n3A | n78A | Yes |
| CA_n3A-n78(2A) | CA_n3A-n78A CA_n78(2A) | n3A | n78(2A) | n3A - | n78A n78(2A) | No |
| CA_n3(2A)-n78A | - | n3(2A) | n78A | - | - | No |
| CA_n3A-n79A | CA_n3A-n79A | n3A | n79A | n3A | n79A | Yes |
| CA_n5A-n7A | - | n5A | n7A | - | - | Yes |
| CA_n5A-n78A | - | n5A | n78A | - | - | Yes |
| CA_n5A-n78(2A) | CA_n5A-n78A | n5A | n78(2A) | n5A | n78A | No |
| CA_n7A-n78A | - | n7A | n78A | - | - | Yes |
| CA_n8A-n75A | - | n8A | n75A | - | - | Yes |
| CA_n8A-n78A | CA_n8A-n78A | n8A | n78A | n8A | n78A | Yes |
| CA_n8A-n78(2A) | CA_n8A-n78A | n8A | n78(2A) | n8A | n78(2A) | No |
| CA_n8A-n79A | CA_n8A-n79A | n8A | n79A | n8A | n79A | Yes |
| CA_n24A-n41A | CA_n24A-n41A | n24A | n41A | n24A | n41A | Yes |
| CA_n24A-n41(2A) | CA_n24A-n41A | n24A | n41(2A) | n24A | n41A | No |
| CA_n24A-n48A | CA_n24A-n48A | n24A | n48A | n24A | n48A | Yes |
| CA_n24A-n48B | CA_n24A-n48A | n24A | CA_n48B | n24A | n48A | No |
| CA_n24A-n48(2A) | CA_n24A-n48A | n24A | CA_n48(2A) | n24A | n48A | No |
| CA_n24A-n77A | CA_n24A-n77A | n24A | n77A | n24A | n77A | Yes |
| CA_n24A-n77C | CA_n24A-n77A | n24A | CA_n77C | n24A | n77A | No |
| CA_n26A-n66A | CA_n26A-n66A | n26A | n66A | n26A | n66A | Yes |
| CA_n26A-n66(2A) | CA_n26A-n66A | n26A | n66(2A) | n26A | n66A | No |
| CA_n26A-n70A | CA_n26A-n70A | n26A | n70A | n26A | n70A | Yes |
| CA_n28A-n41A | CA_n28A-n41A | n28A | n41A | n28A | n41A | Yes |
| CA_n28A-n75A | - | n28A | n75A | - | - | Yes |
| CA_n28A-n78A | CA_n28A-n78A | n28A | n78A | n28A | n78A | Yes |
| CA_n28A-n79A | CA_n28A-n79A | n28A | n79A | n28A | n79A | Yes |
| CA_n29A-n66A | - | n29A | n66A (Note 1) | - | - | Yes |
| CA_n29B-n66B | - | n29A | n66B (Note 1) | - | - | No |
| CA_n29A-n66(2A) | - | n29A | n66(2A) (Note 1) | - | - | No |
| CA_n29A-n70A | - | n29A | n70A (Note 1) | - | - | Yes |
| CA_n41A-n78A | - | n41A | n78A | - | - | Yes |
| CA_n41A-n79A | CA_n41A-n79A | n41A | n79A | n41A | n79A | Yes |
| CA_n41C-n79A | CA_n41A-n79A | CA_n41C | n79A | n41A | n79A | No |
|  | CA_n41C | CA_n41C | n79A | CA_n41C | - | No |
| CA_n48A-n66A | CA_n48A-n66A | n48A | n66A | n48A | n66A | Yes |
| CA_n48A-n66(2A) | CA_n48A-n66A | n48A | n66(2A) | n48A | n66A | No |
| CA_n48A-n70A | CA_n48A-n70A | n48A | n70A | n48A | n70A | Yes |
| CA_n48A-n71A | CA_n48A-n71A | n48A | n71A | n48A | n71A | Yes |
| CA_n48A-n71(2A) | CA_n48A-n71A | n48A | n71(2A) | n48A | n71A | No |
| CA_n48B-n66A | CA_n48A-n66A | n48B | n66 | n48A | n66A | No |
| CA_n48B-n70A | CA_n48A-n70A | n48B | n70A | n48A | n70A | No |
| CA_n48B-n71A | CA_n48A-n71A | n48B | n71A | n48A | n71A | No |
| CA_n48(2A)-n66A | CA_n48A-n66A | n48(2A) | n66 | n48A | n66A | No |
| CA_n48(2A)-n70A | CA_n48A-n70A | n48(2A) | n70A | n48A | n70A | No |
| CA_n48(2A-n71A | CA_n48A-n71A | n48(2A) | n71A | n48A | n71A | No |
| CA_n66A-n70A | - | n66A | n70A | - | - | Yes |
| CA_n66B-n70A | - | n66B | n70A | - | - | No |
| CA_n66(2A)-n70A | - | n66(2A) | n70A | - | - | No |
| CA_n66A-n71A | CA_n66A-n71A | n66A | n71A | n66A | n71A | Yes |
| CA_n66A-n71(2A) | CA_n66A-n71A | n66A | n71(2A) | n66A | n71A | No |
| CA_n66B-n71A | CA_n66A-n71A | n66B | n71A | n66A | n71A | No |
| CA_n66(2A)-n71A | CA_n66A-n71A | n66(2A) | n71A | n66A | n71A | No |
| CA_n66(2A)-n71(2A) | CA_n66A-n71A | n66(2A) | n71(2A) | n66A | n71A | No |
| CA_n70A-n71A | CA_n70A-n71A | n70A | n71A | n70A | n71A | Yes |
| CA_n70A-n71(2A) | CA_n70A-n71A | n70A | n71(2A) | n70A | n71A | No |
| CA_n75A-n78A | - | n75A | n78A (Note 1) | - | - | Yes |
| CA_n76A-n78A | - | n76A | n78A (Note 1) | - | - | Yes |
| CA_n77A-n79A | - | n77A | n79A | - | - | Yes |
| CA_n78A-n79A | - | n78A | n79A | - | - | Yes |
| Note 1: This band is used as PCell. Note 2: Protocol testing is limited to NR CA configurations with 2CC. Note X: PCell is configured on this band unless otherwise stated. |  |  |  |  |  |  |

4.3.1.1.2.2	NR inter-band CA configurations in FR1 (three bands)

Table 4.3.1.1.2.2-1: Inter-band NR CA configurations within FR1 (three bands)

| NR CA configuration (Note 3) | Uplink NR CA configuration | NR CA downlink configuration band 1 (Note X) | NR CA downlink configuration band 2 | NR CA downlink configuration band 3 | NR CA uplink configuration band 1 | NR CA uplink configuration band 2 | NR CA uplink configuration band 3 | Applicable for protocol testing (Note 2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n26A-n66A-n70A | CA_n26A-n66A | n26A | n66A | n70A | n26A | n66A | - | No |
|  | CA_n26A-n70A | n26A | n66A | n70A | n26A | - | n70A | No |
| CA_n29A-n66A-n70A | - | n29A (Note1) | n66A | n70A | - | - | - | No |
| CA_n29A-n66B-n70A | - | n29A (Note1) | CA_n66B | n70A | - | - | - | No |
| CA_n29A-n66(2A)-n70A | - | n29A (Note1) | CA_n66(2A) | n70A | - | - | - | No |
| CA_n48A-n66A-n70A | CA_n48A-n66A | n48A | n66A | n70A | n48A | n66A | - | No |
|  | CA_n48A-n70A | n48A | n66A | n70A | n48A | - | n70A | No |
| CA_n48A-n66A-n71A | CA_n48A-n66A | n48A | n66A | n71A | n48A | n66A | - | No |
|  | CA_n48A-n71A | n48A | n66A | n71A | n48A | - | n71A | No |
|  | CA_n66A-n71A | n48A | n66A | n71A | - | n66A | n71A | No |
| CA_n48A-n70A-n71A | CA_n48A-n70A | n48A | n70A | n71A | n48A | n70A | - | No |
|  | CA_n48A-n71A | n48A | n70A | n71A | n48A | - | n71A | No |
|  | CA_n70A-n71A | n48A | n70A | n71A | - | n70A | n71A | No |
| CA_n66A-n70A-n71A | CA_n66A-n71A | n66A | n70A | n71A | n66A | - | n71A | No |
|  | CA_n70A-n71A | n66A | n70A | n71A | - | n70A | n71A |  |
| CA_n66A-n70A-n71(2A) | CA_n66A-n71A | n66A | n70A | n71(2A) | n66A | - | n71A | No |
|  | CA_n70A-n71A | n66A | n70A | n71(2A) | - | n70A | n71A |  |
| CA_n66B-n70A-n71A | CA_n66A-n71A | CA_n66B | n70A | n71A | n66A | - | n71A |  |
|  | CA_n70A-n71A | CA_n66B | n70A | n71A | - | n70A | n71A |  |
| CA_n66(2A)-n70A-n71A | CA_n66A-n71A | CA_n66(2A) | n70A | n71A | n66A | - | n71A | No |
|  | CA_n70A-n71A | CA_n66(2A) | n70A | n71A | - | n70A | n71A |  |
| Note 1: This band cannot be used as PCell. Note 2: Protocol testing is limited to NR CA configurations with 3CC. Note 3: The band with the lowest UL frequency is used as PCell if nothing else is specified for in the table or in the test case for the specific configuration. Note X: PCell is configured on this band unless otherwise stated. |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

4.3.1.1.3.40	NR Intra-band contiguous configurations CA_n40

4.3.1.1.3.40.1	CA_n40B

Editor's note:	Test frequencies for CA_n40B with mixed numerology with SCS CC1=15 kHz and SCS CC2=30 kHz or 60 kHz; and SCS CC1=30 kHz and SCS CC2= 15 kHz or 60kHz is FFS.

Table 4.3.1.1.3.40.1-1: NR Intra-Band contiguous CA configuration CA_n40B (PCC=CC1 and SCC=CC2), CC1 SCS = 15kHz, CC2 SCS = 15kHz

| CBW combination [MHz] | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50+50 | CC1 | 50 | 15 | 270 | Downlink | Low | 2325 | 465000 | 2300.7 | 460140 | 0 | 30 |  | 5763 | 461070 |  | 10 |  | 3 | 0 (2) | 5 |
|  |  |  |  |  | & | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Uplink | High | 2325.2 | 465040 | 2210.18 | 442036 | 504 |  |  | 5762 | 461050 |  | 2 |  | 2 | 0 (2) | 508 |
|  | Channel spacing CC1-CC2=49.8 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 15 | 270 | Downlink | Low | 2374.8 | 474960 | 2350.5 | 470100 | 0 | 30 |  | 5886 | 470910 |  | 6 |  | 0 | 0 (2) | 2 |
|  |  |  |  |  | & | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Uplink | High | 2375 | 475000 | 2259.98 | 451996 | 504 |  |  | 5888 | 471130 |  | 6 |  | 1 | 1 (6) | 511 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-1 [7], clause 5.4A.1 for the CC1 and CC2 channel bandwidth combination. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

4.3.1.1.5	NR intra-band non-contiguous CA configurations in FR1

4.3.1.1.5.1 – 4.3.1.1.5.47	FFS

4.3.1.1.5.48	CA_n48(2A)

Editor’s note:	Test frequencies for CA_n48(2A) with mixed numerology with SCS CC1=15kHz and SCS CC2=30 kHz or 60kHz; and SCS CC1=30kHz and SCS CC2=15 kHz or 60 kHz is FFS.

Table 4.3.1.1.5.48-1: NR Intra-Band non-contiguous CA configuration CA_n48(2A) without UL CA, SCS=15 kHz, Max Wgap

| SB CBW combination | SB | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range (Note 1) | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n48(2A); n48A (10MHz) + n48A (10-50MHz) |  |  |  |  |  |  |
| 10+10 | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=10 MHz |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=10 MHz |
| 10+15 | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=10 MHz |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=15 MHz |
| 10+20 | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=10 MHz |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=20 MHz |
| 10+40 | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=10 MHz |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=40 MHz |
| 10+50 | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=10 MHz |
|  | SB2 | 50 | 270 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=50 MHz |
| CA_n48(2A); n48A (15MHz) + n48A (10-50MHz) |  |  |  |  |  |  |
| 15+10 | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=15 MHz |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=10 MHz |
| 15+15 | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=15 MHz |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=15 MHz |
| 15+20 | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=15 MHz |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=20 MHz |
| 15+40 | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=15 MHz |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=40 MHz |
| 15+50 | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=15 MHz |
|  | SB2 | 50 | 270 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=50 MHz |
| CA_n48(2A); n48A (20MHz) + n48A (10-50MHz) |  |  |  |  |  |  |
| 20+10 | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=20 MHz |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=10 MHz |
| 20+15 | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=20 MHz |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=15 MHz |
| 20+20 | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=20 MHz |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=20 MHz |
| 20+40 | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=20 MHz |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=40 MHz |
| 20+50 | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=20 MHz |
|  | SB2 | 50 | 270 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=50 MHz |
| CA_n48(2A); n48A (40MHz) + n48A (10-50MHz) |  |  |  |  |  |  |
| 40+10 | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=40 MHz |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=10 MHz |
| 40+15 | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=40 MHz |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=15 MHz |
| 40+20 | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=40 MHz |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=20 MHz |
| 40+40 | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=40 MHz |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=40 MHz |
| 40+50 | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=40 MHz |
|  | SB2 | 50 | 270 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=50 MHz |
| CA_n48(2A); n48A (50MHz) + n48A (10-50MHz) |  |  |  |  |  |  |
| 50+10 | SB1 | 50 | 270 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=50 MHz |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=10 MHz |
| 50+15 | SB1 | 50 | 270 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=50 MHz |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=15 MHz |
| 50+20 | SB1 | 50 | 270 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=50 MHz |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=20 MHz |
| 50+40 | SB1 | 50 | 270 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=50 MHz |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=40 MHz |
| 50+50 | SB1 | 50 | 270 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=50 MHz |
|  | SB2 | 50 | 270 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=50 MHz |
| Note 1: CA_n48(2A) is specified in [7] 38.101-1 without uplink CA. PCC is configured on SB1 unless otherwise stated. The test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. |  |  |  |  |  |  |

Table 4.3.1.1.5.48-2: NR Intra-Band non-contiguous CA configuration CA_n48(2A) without UL CA, SCS=30 kHz, Max Wgap

| CBW combination | SB | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range (Note 1) | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n48(2A); n48A (10MHz) + n48A (10-100MHz) |  |  |  |  |  |  |
| 10+10 | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=10 MHz |
|  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=10 MHz |
| 10+15 | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=10 MHz |
|  | SB2 | 15 | 38 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=15 MHz |
| 10+20 | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=10 MHz |
|  | SB2 | 20 | 51 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=20 MHz |
| 10+40 | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=10 MHz |
|  | SB2 | 40 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=40 MHz |
| 10+50 | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=10 MHz |
|  | SB2 | 50 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=50 MHz |
| 10+60 | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=10 MHz |
|  | SB2 | 60 | 162 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=60 MHz |
| 10+80 | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=10 MHz |
|  | SB2 | 80 | 217 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=80 MHz |
| 10+90 | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=10 MHz |
|  | SB2 | 90 | 245 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=90 MHz |
| 10+100 | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=10 MHz |
|  | SB2 | 100 | 273 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=100 MHz |
| CA_n48(2A); n48A (15MHz) + n48A (10-100MHz) |  |  |  |  |  |  |
| 15+10 | SB1 | 15 | 38 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=15 MHz |
|  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=10 MHz |
| 15+15 | SB1 | 15 | 38 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=15 MHz |
|  | SB2 | 15 | 38 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=15 MHz |
| 15+20 | SB1 | 15 | 38 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=15 MHz |
|  | SB2 | 20 | 51 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=20 MHz |
| 15+40 | SB1 | 15 | 38 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=15 MHz |
|  | SB2 | 40 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=40 MHz |
| 15+50 | SB1 | 15 | 38 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=15 MHz |
|  | SB2 | 50 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=50 MHz |
| 15+60 | SB1 | 15 | 38 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=15 MHz |
|  | SB2 | 60 | 162 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=60 MHz |
| 15+80 | SB1 | 15 | 38 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=15 MHz |
|  | SB2 | 80 | 217 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=80 MHz |
| 15+90 | SB1 | 15 | 38 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=15 MHz |
|  | SB2 | 90 | 245 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=90 MHz |
| 15+100 | SB1 | 15 | 38 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=15 MHz |
|  | SB2 | 100 | 273 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=100 MHz |
| CA_n48(2A); n48A (20MHz) + n48A (10-100MHz) |  |  |  |  |  |  |
| 20+10 | SB1 | 20 | 51 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=20 MHz |
|  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=10 MHz |
| 20+15 | SB1 | 20 | 51 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=20 MHz |
|  | SB2 | 15 | 38 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=15 MHz |
| 20+20 | SB1 | 20 | 51 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=20 MHz |
|  | SB2 | 20 | 51 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=20 MHz |
| 20+40 | SB1 | 20 | 51 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=20 MHz |
|  | SB2 | 40 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=40 MHz |
| 20+50 | SB1 | 20 | 51 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=20 MHz |
|  | SB2 | 50 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=50 MHz |
| 20+60 | SB1 | 20 | 51 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=20 MHz |
|  | SB2 | 60 | 162 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=60 MHz |
| 20+80 | SB1 | 20 | 51 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=20 MHz |
|  | SB2 | 80 | 217 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=80 MHz |
| 20+90 | SB1 | 20 | 51 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=20 MHz |
|  | SB2 | 90 | 245 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=90 MHz |
| 20+100 | SB1 | 20 | 51 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=20 MHz |
|  | SB2 | 100 | 273 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=100 MHz |
| CA_n48(2A); n48A (40MHz) + n48A (10-100MHz) |  |  |  |  |  |  |
| 40+10 | SB1 | 40 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=40 MHz |
|  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=10 MHz |
| 40+15 | SB1 | 40 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=40 MHz |
|  | SB2 | 15 | 38 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=15 MHz |
| 40+20 | SB1 | 40 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=40 MHz |
|  | SB2 | 20 | 51 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=20 MHz |
| 40+40 | SB1 | 40 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=40 MHz |
|  | SB2 | 40 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=40 MHz |
| 40+50 | SB1 | 40 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=40 MHz |
|  | SB2 | 50 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=50 MHz |
| 40+60 | SB1 | 40 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=40 MHz |
|  | SB2 | 60 | 162 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=60 MHz |
| 40+80 | SB1 | 40 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=40 MHz |
|  | SB2 | 80 | 217 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=80 MHz |
| 40+90 | SB1 | 40 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=40 MHz |
|  | SB2 | 90 | 245 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=90 MHz |
| 40+100 | SB1 | 40 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=40 MHz |
|  | SB2 | 100 | 273 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=100 MHz |
| CA_n48(2A); n48A (50MHz) + n48A (10-90MHz) |  |  |  |  |  |  |
| 50+10 | SB1 | 50 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=50 MHz |
|  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=10 MHz |
| 50+15 | SB1 | 50 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=50 MHz |
|  | SB2 | 15 | 38 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=15 MHz |
| 50+20 | SB1 | 50 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=50 MHz |
|  | SB2 | 20 | 51 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=20 MHz |
| 50+40 | SB1 | 50 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=50 MHz |
|  | SB2 | 40 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=40 MHz |
| 50+50 | SB1 | 50 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=50 MHz |
|  | SB2 | 50 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=50 MHz |
| 50+60 | SB1 | 50 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=50 MHz |
|  | SB2 | 60 | 162 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=60 MHz |
| 50+80 | SB1 | 50 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=50 MHz |
|  | SB2 | 80 | 217 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=80 MHz |
| 50+90 | SB1 | 50 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=50 MHz |
|  | SB2 | 90 | 245 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=90 MHz |
| CA_n48(2A); n48A (60MHz) + n48A (10-80MHz) |  |  |  |  |  |  |
| 60+10 | SB1 | 60 | 162 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=60 MHz |
|  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=10 MHz |
| 60+15 | SB1 | 60 | 162 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=60 MHz |
|  | SB2 | 15 | 38 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=15 MHz |
| 60+20 | SB1 | 60 | 162 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=60 MHz |
|  | SB2 | 20 | 51 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=20 MHz |
| 60+40 | SB1 | 60 | 162 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=60 MHz |
|  | SB2 | 40 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=40 MHz |
| 60+50 | SB1 | 60 | 162 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=60 MHz |
|  | SB2 | 50 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=50 MHz |
| 60+60 | SB1 | 60 | 162 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=60 MHz |
|  | SB2 | 60 | 162 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=60 MHz |
| 60+80 | SB1 | 60 | 162 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=60 MHz |
|  | SB2 | 80 | 217 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=80 MHz |
| CA_n48(2A); n48A (80MHz) + n48A (10-60MHz) |  |  |  |  |  |  |
| 80+10 | SB1 | 80 | 217 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=80 MHz |
|  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=10 MHz |
| 80+15 | SB1 | 80 | 217 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=80 MHz |
|  | SB2 | 15 | 38 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=15 MHz |
| 80+20 | SB1 | 80 | 217 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=80 MHz |
|  | SB2 | 20 | 51 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=20 MHz |
| 80+40 | SB1 | 80 | 217 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=80 MHz |
|  | SB2 | 40 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=40 MHz |
| 80+50 | SB1 | 80 | 217 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=80 MHz |
|  | SB2 | 50 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=50 MHz |
| 80+60 | SB1 | 80 | 217 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=80 MHz |
|  | SB2 | 60 | 162 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=60 MHz |
| CA_n48(2A); n48A (90MHz) + n48A (10-50MHz) |  |  |  |  |  |  |
| 90+10 | SB1 | 90 | 245 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=90 MHz |
|  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=10 MHz |
| 90+15 | SB1 | 90 | 245 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=90 MHz |
|  | SB2 | 15 | 38 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=15 MHz |
| 90+20 | SB1 | 90 | 245 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=90 MHz |
|  | SB2 | 20 | 51 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=20 MHz |
| 90+40 | SB1 | 90 | 245 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=90 MHz |
|  | SB2 | 40 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=40 MHz |
| 90+50 | SB1 | 90 | 245 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=90 MHz |
|  | SB2 | 50 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=50 MHz |
| CA_n48(2A); n48A (100MHz) + n48A (10-40MHz) |  |  |  |  |  |  |
| 100+10 | SB1 | 100 | 273 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=100 MHz |
|  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=10 MHz |
| 100+15 | SB1 | 100 | 273 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=100 MHz |
|  | SB2 | 15 | 38 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=15 MHz |
| 100+20 | SB1 | 100 | 273 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=100 MHz |
|  | SB2 | 20 | 51 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=20 MHz |
| 100+40 | SB1 | 100 | 273 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=100 MHz |
|  | SB2 | 40 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=40 MHz |
| Note 1: CA_n48(2A) is specified in [7] 38.101-1 without uplink CA. PCC is configured on SB1 unless otherwise stated. The test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. |  |  |  |  |  |  |

4.3.1.1.5.49 – 4.3.1.1.5.65	FFS

4.3.1.1.5.66	CA_n66(2A)

Editor’s note:	Test frequencies for CA_n66(2A) with mixed numerology with SCS CC1=15kHz and SCS CC2=30 kHz or 60kHz; and SCS CC1=30kHz and SCS CC2=15 kHz or 60 kHz is FFS.

Table 4.3.1.1.5.66-1: NR Intra-Band non-contiguous CA configuration CA_n66(2A) without UL CA, SCS=15 kHz, Max Wgap

| CBW combination (BCS) | SB | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range (Note 1) | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n66(2A); A (5MHz) - A(5-40MHz) |  |  |  |  |  |  |
| 5+5 (0,1,2) | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/5 |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/5 |
| 5+10 (0,1,2) | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/5 |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 10/10 |
| 5+15 (0,1,2) | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/5 |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 15/15 |
| 5+20 (0,1,2) | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/5 |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
| 5+25 (1) | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/5 |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 5+30 (1) | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/5 |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 5+40 (0,1,2) | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/5 |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); A (10MHz) - A(5-40MHz) |  |  |  |  |  |  |
| 10+5 (0,1,2) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/5 |
| 10+10 (0,1,2) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 10/10 |
| 10+15 (0,1,2) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 15/15 |
| 10+20 (0,1,2) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
| 10+25 (1) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 10+30 (1) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 10+40 (0,1,2) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); A (15MHz) - A(5-40MHz) |  |  |  |  |  |  |
| 15+5 (0,1,2) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/5 |
| 15+10 (0,1,2) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 10/10 |
| 15+15 (0,1,2) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 15/15 |
| 15+20 (0,1,2) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
| 15+25 (1) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 15+30 (1) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 15+40 (0,1,2) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); A (20MHz) - A(5-40MHz) |  |  |  |  |  |  |
| 20+5 (0,1,2) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 5/5 |
| 20+10 (0,1,2) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 10/10 |
| 20+15 (0,1,2) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 15/15 |
| 20+20 (0,1,2) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
| 20+25 (1) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 20+30 (1) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 20+40 (0,1,2) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); A (25MHz) - A(5-40MHz) |  |  |  |  |  |  |
| 25+5 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 5/5 |
| 25+10 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 10/10 |
| 25+15 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 15/15 |
| 25+20 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
| 25+25 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 25+30 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 25+40 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); A (30MHz) - A(5-40MHz) |  |  |  |  |  |  |
| 30+5 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 5/5 |
| 30+10 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 10/10 |
| 30+15 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 15/15 |
| 30+20 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
| 30+30 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 30+30 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 30+40 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); A (40MHz) - A(5-40MHz) |  |  |  |  |  |  |
| 40+5 (1,2) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 5/5 |
| 40+10 (1,2) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 10/10 |
| 40+15 (1,2) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 15/15 |
| 40+20 (1,2) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | able 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
| 40+25 (1) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 40+30 (1) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 40+40 (1,2) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
| Note 1: CA_n66(2A) is specified in [7] 38.101-1 without uplink CA. PCC is configured on SB1 unless otherwise stated. The test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. |  |  |  |  |  |  |

Table 4.3.1.1.5.66-2: NR Intra-Band non-contiguous CA configuration CA_n66(2A) without UL CA, SCS=30 kHz, Max Wgap

| CBW combination (BCS) | SB | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n66(2A); A (10MHz) - A(10-40MHz) |  |  |  |  |  |  |
| 10+10 (0,1,2) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 10/10 |
| 10+15 (0,1,2) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 15/15 |
| 10+20 (0,1,2) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 10/20 or 20/20 depending on required UL bandwidth |
| 10+25 (1) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 10+30 (1) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 10+40 (0,1,2) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); A (15MHz) - A(10-40MHz) |  |  |  |  |  |  |
| 15+10 (0,1,2) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 10/10 |
| 15+15 (0,1,2) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 15/15 |
| 15+20 (0,1,2) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 20/20 |
| 15+25 (1) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 15+30 (1) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 15+40 (0,1,2) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); A (20MHz) - A(10-40MHz) |  |  |  |  |  |  |
| 20+10 (0,1,2) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination 10/10 |
| 20+15 (0,1,2) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination 15/15 |
| 20+20 (0,1,2) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 10/20 or 20/20 depending on required UL bandwidth |
| 20+25 (1) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 20+30 (1) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 20+40 (0,1,2) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); A (25MHz) - A(10-40MHz) |  |  |  |  |  |  |
| 25+10 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination 10/10 |
| 25+15 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination 15/15 |
| 25+20 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
| 25+25 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 25+30 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 25+40 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); A (30MHz) - A(10-40MHz) |  |  |  |  |  |  |
| 30+10 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination 10/10 |
| 30+15 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination 15/15 |
| 30+20 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
| 30+30 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 30+30 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 30+40 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); A (40MHz) - A(10-40MHz) |  |  |  |  |  |  |
| 40+10 (1,2) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination 10/10 |
| 40+15 (1,2) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/40 or 20/40 depending on required UL bandwidth |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination 15/15 |
| 40+20 (1,2) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 10/20 or 20/20 depending on required UL bandwidth |
| 40+25 (1) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 40+30 (1) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 40+40 (1,2) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
| Note 1: CA_n66(2A) is specified in [7] 38.101-1 without uplink CA. PCC is configured on SB1 unless otherwise stated. The test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. |  |  |  |  |  |  |

4.3.1.1.5.67 – 4.3.1.1.5.70	FFS

4.3.1.1.5.71	CA_n71(2A)

Editor’s note:	Test frequencies for CA_n71(2A) with mixed numerology with SCS CC1=15kHz and SCS CC2=30 kHz; and SCS CC1=30kHz and SCS CC2=15 kHz is FFS.

Table 4.3.1.1.5.71-1: NR Intra-Band non-contiguous CA configuration CA_n71(2A) without UL CA, SCS=15 kHz, Max Wgap

| SB CBW combination | SB | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n71(2A); n71A (5MHz) + n71A (5-20MHz) |  |  |  |  |  |  |
| 5+5 | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=5 MHz |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=5 MHz |
| 5+10 | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=5 MHz |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=10 MHz |
| 5+15 | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=5 MHz |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=15 MHz |
| 5+20 | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=5 MHz |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=20 MHz |
| CA_n71(2A); n71A (10MHz) + n71A (5-20MHz) |  |  |  |  |  |  |
| 10+5 | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=10 MHz |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=5 MHz |
| 10+10 | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=10 MHz |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=10 MHz |
| 10+15 | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=10 MHz |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=15 MHz |
| 10+20 | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=10 MHz |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=20 MHz |
| CA_n71(2A); n71A (15MHz) + n71A (5-15MHz) |  |  |  |  |  |  |
| 15+5 | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=15 MHz |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=5 MHz |
| 15+10 | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=15 MHz |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=10 MHz |
| 15+15 | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=15 MHz |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=15 MHz |
| CA_n71(2A); n71A (20MHz) + n71A (5-10MHz) |  |  |  |  |  |  |
| 20+5 | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=20 MHz |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=5 MHz |
| 20+10 | SB1 | 10 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=20 MHz |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=10 MHz |
| Note 1: CA_n71(2A) is specified in [7] 38.101-1 without uplink CA. PCC is configured on SB1 unless otherwise stated. The test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. |  |  |  |  |  |  |

Table 4.3.1.1.5.71-2: NR Intra-Band non-contiguous CA configuration CA_n71(2A) without UL CA, SCS=30 kHz, Max Wgap

| SB CBW combination | SB | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n71(2A); n71A (10MHz) + n71A (10-20MHz) |  |  |  |  |  |  |
| 10+10 | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-2: Low range for CBW=10 MHz |
|  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.71-2: High range for CBW=10 MHz |
| 10+15 | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-2: Low range for CBW=10 MHz |
|  | SB2 | 15 | 38 | Downlink & Uplink |  | Table 4.3.1.1.1.71-2: High range for CBW=15 MHz |
| 10+20 | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-2: Low range for CBW=10 MHz |
|  | SB2 | 20 | 51 | Downlink & Uplink |  | Table 4.3.1.1.1.71-2: High range for CBW=20 MHz |
| CA_n71(2A); n71A (15MHz) + n71A (10-15MHz) |  |  |  |  |  |  |
| 15+10 | SB1 | 15 | 38 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-2: Low range for CBW=15 MHz |
|  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.71-2: High range for CBW=10 MHz |
| 15+15 | SB1 | 15 | 38 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-2: Low range for CBW=15 MHz |
|  | SB2 | 15 | 38 | Downlink & Uplink |  | Table 4.3.1.1.1.71-2: High range for CBW=15 MHz |
| CA_n71(2A); n71A (20MHz) + n71A (10MHz) |  |  |  |  |  |  |
| 20+10 | SB1 | 10 | 51 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-2: Low range for CBW=20 MHz |
|  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.71-2: High range for CBW=10 MHz |
| Note 1: CA_n71(2A) is specified in [7] 38.101-1 without uplink CA. PCC is configured on SB1 unless otherwise stated. The test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. |  |  |  |  |  |  |

4.3.1.1.5.72 – 4.3.1.1.5.76	FFS

4.3.1.1.5.77	CA_n77(2A)

Editor’s note:	Test frequencies for CA_n77(2A) with mixed numerology with SCS CC1=15kHz and SCS CC2=30 kHz or 60kHz; and SCS CC1=30kHz and SCS CC2=15 kHz or 60 kHz is FFS.

Table 4.3.1.1.5.77-1: NR Intra-Band non-contiguous CA configuration CA_n77(2A) without UL CA, SCS=15 kHz, Max Wgap

| CBW combination | SB | Bandwidth [MHz] | Range (Note 1) | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- |
| CA_n77(2A); A(20-40MHz) - A(20-40MHz) |  |  |  |  |  |
| CBW1+CBW2 | SB1 | CBW1 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.77-1: Low range for CBW = 20 OR 40 MHz |
|  |  | SB2 | CBW2 | Downlink & Uplink |  | Table 4.3.1.1.1.77-1: High range for CBW = 20 OR 40 MHz |
| Note 1: CA_n77(2A) BCS0 is specified in [7] 38.101-1 without uplink CA. PCC is configured on SB1 unless otherwise stated. The test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. |  |  |  |  |  |  |

Table 4.3.1.1.5.77-2: NR Intra-Band non-contiguous CA configuration CA_n77(2A) without UL CA, SCS=30 kHz, Max Wgap

| CBW combination | SB | Bandwidth [MHz] | Range (Note 1) | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- |
| CA_n77(2A); A(20-100MHz) - A(20-100MHz) |  |  |  |  |  |
| CBW1+CBW2 | SB1 | CBW1 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.77-2: Low range for CBW = 20, 40, 80 OR 100 MHz |
|  | SB2 | CBW2 | Downlink |  | Table 4.3.1.1.1.77-2: High range for CBW = 20, 40, 80 OR 100 MHz |
| Note 1: CA_n77(2A) BCS0 is specified in [7] 38.101-1 without uplink CA. PCC is configured on SB1 unless otherwise stated. The test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. |  |  |  |  |  |

4.3.1.1.5.78	CA_n78(2A)

Editor’s note:	Test frequencies for CA_n78(2A) with mixed numerology with SCS CC1=15kHz and SCS CC2=30 kHz or 60kHz; and SCS CC1=30kHz and SCS CC2=15 kHz or 60 kHz is FFS.

Table 4.3.1.1.5.78-1: NR Intra-Band non-contiguous CA configuration CA_n78(2A) without UL CA, SCS=15 kHz, Max Wgap

| CBW combination | SB | Bandwidth [MHz] | Range | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- |
| CA_n78(2A); A(10-50MHz) - A(10-50MHz) |  |  |  |  |  |
| CBW1+CBW2 | SB1 | CBW1 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.78-1: Low range for CBW = 10, 20, 25, 30, 40 OR 50 MHz |
|  | SB2 | CBW2 | Downlink & Uplink |  | Table 4.3.1.1.1.78-1: High range for CBW = 10, 20, 25, 30, 40 OR 50 MHz |
| Note 1: CA_n78(2A) is specified in [7] 38.101-1 with and without uplink CA. PCC is configured on SB1 unless otherwise stated. This table is applicable for the case of without UL CA but test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. |  |  |  |  |  |

Table 4.3.1.1.5.78-1A: NR Intra-Band non-contiguous CA configuration CA_n78(2A) with UL CA, SCS=15 kHz, Max Wgap

FFS

Table 4.3.1.1.5.78-2: NR Intra-Band non-contiguous CA configuration CA_n78(2A) without UL CA, SCS=30 kHz, Max Wgap

| CBW combination | SB | Bandwidth [MHz] | Range | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- |
| CA_n78(2A); A(10-100MHz) - A(10-100MHz) |  |  |  |  |  |
| CBW1+CBW2 | SB1 | CBW1 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.78-2: Low range for CBW = 10, 20, 25, 30, 40, 50, 60, 70, 80, 90 OR 100 MHz |
|  | SB2 | CBW2 | Downlink & Uplink |  | Table 4.3.1.1.1.78-2: High range for CBW = 10, 20, 25, 30, 40, 50, 60, 70, 80, 90 OR 100 MHz |
| Note 1: CA_n78(2A) is specified in [7] 38.101-1 with and without uplink CA. PCC is configured on SB1 unless otherwise stated. This table is applicable for the case of without UL CA but test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. |  |  |  |  |  |

Table 4.3.1.1.5.78-2A: NR Intra-Band non-contiguous CA configuration CA_n78(2A) with UL CA, SCS=30 kHz, Max Wgap

FFS

<End of Changes >