3GPP TSG-RAN5 Meeting #95-e	R5-222263r3

# Electronic Meeting, 9th May 2022 - 20th May 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2896 | rev | - | Current version: | 16.11.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new NR5G NPN TC 6.5.2.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm CDMA Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RAN_PRN_Vertical_LAN-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add NPN TC 6.5.2.3 CAG / Limited Service / No Suitable cell. |  |  |  |  |  |  |  |  |
| Summary of change: |  | NPN TC 6.5.2.3 CAG / Limited Service / No Suitable cell is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NPN WP implementation will not be complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 222263 222263r1: - Deleted the conformance requirements referring to TS 38.304 5.2.4.4 as not applicable for cell selection. - Updated the FFS in Table 6.5.2.3.3.3-3. - Updated Table 6.5.2.3.3.1-1 to specify PLMN on NR cell 11. - In order to ensure that the testcase is run by a UE that supports CAG, introduced special message content for REGISTRATION REQ in preamble (Table 6.5.2.3.3.3-4)which checks for support of CAG. 222263r1 222263r2: - Updated the CAG information list in te REG Accept message of Step 15 to correctly test TP 1. - To test TP2 , the CAG information list of NR cell 11 must be part of the CAG nformation list , hence the REG accept message for Step 40 is updated in the specific message contents. 222263r2 222263r3: - Fixed Step numbers in the main behaviour and updated Table 6.5.2.3.3.3-3 and 6.5.2.3.3.3-4. - Removed conformance requirement of 38.304 Cl 5.2.4.4 as not applicable. - CAG-specific combination NR-13 as specified in TS 38.508-1 [4] clause 4.4.3.1.2 is used in system simulator section. - PLMN identifiers updated in Table 6.5.2.3.3.1-1 to use the HPLMN of the test USIM to avoid UE caming issues on the NR cells because of unknown/unallowed PLMN. - Updated Length of CAG information list contents Table 6.5.2.3.3.3-3 and Table 6.5.2.3.3.3-4 to 2 Bytes. - SIB1 specific message content defined for NR Cell 4 to add CAG ID=2. |  |  |  |  |  |  |  |  |

#### 6.5.2.3	CAG / Limited Service / No Suitable cell

6.5.2.3.1	Test Purpose (TP)

(1)

with { UE in Automatic network selection mode provisioned with a "CAG information list” }

ensure that {

when { UE is switched on or returned to coverage, and a CAG cell with a CAG-ID not present in the "Allowed CAG list" becomes available }

then { UE does not select the CAG cell }

}

(2)

with { UE in Automatic network selection mode configured with "indication that the MS is only allowed to access 5GS via CAG cells" }

ensure that {

when { UE is switched on and a cell which is not a CAG cell becomes available, and there is an entry with the PLMN ID of the PLMN in the "CAG information list" but the "indication that the MS is only allowed to access 5GS via CAG cells" is included in the entry }

then { UE does not select the CAG cell }

}

(3)

with { UE in Automatic network selection mode provisioned with an empty "CAG information list” }

ensure that {

when { UE is switched on or returned to coverage, and a CAG cell with a PLMN ID becomes available }

then { UE does not select the CAG cell }

}

6.5.2.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 23.122 clause 3.5. Unless otherwise stated these are Rel-16 requirements.

[TS 23.122, clause 3.5]

There are a number of situations in which the MS is unable to obtain normal service from a PLMN or SNPN. These include:

…

i)	MS supporting CAG is camped on a CAG cell belonging to a PLMN, the CAG-ID of the CAG cell is not manually selected by the user and none of the CAG-ID(s) of the CAG cell are present in the "Allowed CAG list" associated with that PLMN in the "CAG information list";

j)	MS supporting CAG is camped on a non-CAG cell belonging to a PLMN, the PLMN ID of the non-CAG cell without a CAG-ID is not manually selected by the user and the UE is configured with "indication that the MS is only allowed to access 5GS via CAG cells" for that PLMN in the "CAG information list"; and

k)	MS supporting CAG is camped on a CAG cell belonging to a PLMN, the CAG-ID of the CAG cell is not manually selected by the user and the "CAG information list" does not contain an entry for the PLMN (e.g. because the UE is not (pre-)configured with a "CAG information list").

6.5.2.3.3	Test description

6.5.2.3.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1, NR Cell 2, NR Cell 4 and NR Cell 11.

-	NR Cell 2 and NR Cell 4 are CAG cells.

-	System information combination NR-2 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used for NR Cell 1, NR Cell 2, NR Cell 4 and NR Cell 11.

-	The PLMNs are identified in the test by the identifiers in Table 6.5.2.3.3.1-1 and the PLMN settings are defined in TS 36.523-1 [13] table 6.0.1-1.

Table 6.5.2.3.3.1-1: PLMN identifiers

| NR Cell | PLMN name | CAG IDs |
| --- | --- | --- |
| 1 | PLMN1 | - |
| 2 | PLMN1 | 1 |
| 4 | PLMN1 | 2 |
| 11 | PLMN1 | - |

UE:

-	The UE is in Automatic PLMN selection mode.

-	There is an empty “CAG information list” provisioned in the UE.

Preamble:

-	The UE is registered on NR Cell 1 using the procedure described in TS 38.508-1 [4] clause 4.5.2.2 except that the REGISTRATION ACCEPT message includes an empty CAG information list before it is in state Switched OFF (State 0N-B).

6.5.2.3.3.2	Test procedure sequence

Table 6.5.2.3.3.2-1 for FR1 and 6.5.2.3.3.2-2 for FR2 illustrate the downlink power levels to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1", "T2", "T3" and "T3" are to be applied subsequently in the Main behaviour. The exact instants on which these values shall be applied are described in the texts in this clause. Cell powers are chosen for a serving cell and a non-suitable “Off” cell as defined in TS 38.508-1 [4] Table 6.2.2.1-3 for FR1 and Table 6.2.2.2-2 for FR2.

Table 6.5.2.3.3.2-1: Cell configuration changes over time for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | NR Cell 4 | NR Cell 11 | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | "Off" | -88 | "Off" |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -88 | "Off" | "Off" | "Off" |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | "Off" | -88 | "Off" |  |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | -88 | "Off" | "Off" |  |
| T4 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | "Off" | "Off" | -88 |  |

Table 6.5.2.3.3.2-2: Cell configuration changes over time for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | NR Cell 4 | NR Cell 11 | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | "Off" | FFS | "Off" |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | FFS | "Off" | "Off" | "Off" |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | "Off" | FFS | "Off" |  |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | FFS | "Off" | "Off" |  |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | "Off" | "Off" | FFS |  |

Table 6.5.2.3.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Power on the UE. | - | - | - | - |
| 2 | Check: Does the UE send a RRCSetupRequest on NR Cell 4 within the next 60 s? | --> | NR RRC: RRCSetupRequest | 3 | F |
| 3 | SS adjusts cell levels according to row T1 of table 6.5.2.3.3.2-1 for FR1 and table 6.5.2.3.3.2-2 for FR2. | - | - | - | - |
| 4-16 | Steps 1 to 13 of the registration procedure described in TS 38.508-1 [4] subclause 4.5.2.2 are performed on NR Cell 1. NOTE: The UE starts registration. | - | - | - | - |
| 17 | The SS transmits a DLInformationTransfer message and a REGISTRATION ACCEPT message containing the CAG information list with the CAG ID of NR Cell 2. | <-- | NR RRC: DLInformationTransfer 5G MM: REGISTRATION ACCEPT | - | - |
| 18-23a1 | Steps 15 to 20a1 of the registration procedure described in TS 38.508-1 [4] subclause 4.5.2.2 are performed on NR Cell 1. NOTE: The UE completes registration and the RRC connection is released. | - | - | - | - |
| 24 | The UE is switched off by executing generic procedure in Table 4.9.6.1-1 in TS 38.508-1 [4] | - | - | - | - |
| 25 | SS adjusts cell levels according to row T2 of table 6.5.2.3.3.2-1 for FR1 and table 6.5.2.3.3.2-2 for FR2. | - | - | - | - |
| 26 | Power on the UE. | - | - | - | - |
| 27 | Check: Does the UE send a RRCSetupRequest on NR Cell 4 within the next 60 s? | --> | NR RRC: RRCSetupRequest | 1 | F |
| 28 | SS adjusts cell levels according to row T3 of table 6.5.2.3.3.2-1 for FR1 and table 6.5.2.3.3.2-2 for FR2. | - | - | - | - |
| 29-47a1 | Steps 1 to 20a1 of the registration procedure described in TS 38.508-1 [4] subclause 4.5.2.2 are performed on NR Cell 2. NOTE: The UE completes registration and the RRC connection is released. | - | - | - | - |
| 48 | The UE is switched off by executing generic procedure in Table 4.9.6.1-1 in TS 38.508-1 [4] | - | - | - | - |
| 49 | SS adjusts cell levels according to row T4 of table 6.5.2.3.3.2-1 for FR1 and table 6.5.2.3.3.2-2 for FR2. | - | - | - | - |
| 50 | Power on the UE. | - | - | - | - |
| 51 | Check: Does the UE send a RRCSetupRequest on NR Cell 11 within the next 60 s? | --> | NR RRC: RRCSetupRequest | 2 | F |

6.5.2.3.3.3	Specific message contents

Table 6.5.2.3.3.3-1: REGISTRATION REQUEST (Preamble)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| 5GS registration type | '0000 0001'B | Initial registration |  |
| 5GMM capability |  |  |  |
| CAG | ‘1’B | CAG supported |  |

Table 6.5.2.3.3.3-2: REGISTRATION ACCEPT (Preamble)

| Derivation path: TS 38.508-1 [4], table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| CAG information list |  |  |  |
| Length of CAG information list contents | ‘0000 0000 0000 0000’B |  |  |
| Entry 1 | Not present |  |  |

Table 6.5.2.3.3.3-3: REGISTRATION ACCEPT (step 17, Table 6.5.2.3.3.2-3)

| Derivation path: TS 38.508-1 [4], table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| CAG information list |  | MCC : 002; MNC : 11; CAG only : 0; CAG-ID 1 : 1 |  |
| Length of CAG information list contents | ‘0000 0000 0000 1001’B |  |  |
| Entry 1 |  |  |  |
| Length of entry contents | ‘0000 1000’B |  |  |
| MCC | MCC of NR Cell 2 | See Table 6.5.2.3.3.1-1 |  |
| MNC | MNC of NR Cell 2 | See Table 6.5.2.3.3.1-1 |  |
| CAG only | ‘0’B |  |  |
| CAG_ID 1 | 1 | CAG ID is coded as a 32 bit BITSTRING |  |

Table 6.5.2.3.3.3-4: REGISTRATION ACCEPT (step 42, Table 6.5.2.3.3.2-3)

| Derivation path: TS 38.508-1 [4], table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| CAG information list |  | MCC : 002; MNC : 11; CAG only : 0; CAG-ID 1 : 1 |  |
| Length of CAG information list contents | ‘0000 0000 0000 1001’B |  |  |
| Entry 1 |  |  |  |
| Length of entry contents | ‘0000 1000’B |  |  |
| MCC | MCC of NR Cell 11 | See Table 6.5.2.3.3.1-1 |  |
| MNC | MNC of NR Cell 11 | See Table 6.5.2.3.3.1-1 |  |
| CAG only | ‘1’B | Indication that the UE is only allowed to access 5GS via CAG cells (CAGonly) |  |
| CAG_ID 1 | 1 | CAG ID is coded as a 32 bit BITSTRING |  |

Table 6.5.2.3.3.3-5: SIB1 for NR Cell 4 (all steps in Table 6.5.2.3.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-28 with condition SNPN and CAG |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| CellAccessRelatedInfo SEQUENCE { |  |  |  |
| npn-IdentityInfoList-r16 SEQUENCE (SIZE (1..maxPLMN)) OF NPN-IdentityInfo-r16 { | 1 entry |  |  |
| NPN-IdentityInfo-r16[1] SEQUENCE { |  | entry 1 |  |
| npn-IdentityList-r16 SEQUENCE (SIZE (1..maxPLMN)) OF NPN-Identity-r16 { | 1 entry |  |  |
| NPN-Identity-r16[1] CHOICE { |  | entry 1 |  |
| pni-npn-r16 SEQUENCE { |  |  |  |
| plmn-Identity-r16 SEQUENCE { |  |  |  |
| mcc | 001 | PLMN1 MCC |  |
| mnc | 01 | PLMN1 MNC |  |
| } |  |  |  |
| cag-IdentityList-r16 SEQUENCE (SIZE (1..maxNPN-r16)) OF CAG-IdentityInfo-r16 { | 1 entry |  |  |
| CAG-IdentityInfo-r16[1] SEQUENCE { |  | entry 1 |  |
| cag-Identity-r16 | 2 | CAG ID is coded as a 32 bit BITSTRING |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| trackingAreaCode-r16 | TAC of NR Cell 4 |  |  |
| cellIdentity-r16 | Cell Identity of NR Cell 4 |  |  |
| cellReservedForOperatorUse-r16 | notReserved |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |