3GPP TSG-RAN5 Meeting #95-e	R5-222406r1

# Online, , 9th - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1495 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to A.2.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2022-04-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Contact and Record-Route header in 183 Session Progress responses is not specified yet for mid-call scenarios. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Expanded specification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing specification details. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: expanded Reason for change. Rephrased entry for Record-Route. |  |  |  |  |  |  |  |  |

# *** 1st Change ***

## A.2.3	183 Session Progress for INVITE

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Status-Line |  |  |  | RFC 3261 [15] |
| SIP-Version |  | SIP/2.0 |  |  |
| Status-Code |  | 183 |  |  |
| Reason-Phrase |  | Session Progress |  |  |
| Record-Route |  | order of the parameters in this header must be like in the respective rows |  | RFC 3261 [15] |
| rec-route | A1 | <sip:pcscf.other.com;lr>, <sip:scscf.other.com;lr>, <sip:orig@scscf.3gpp.org;lr>, <sip:SS P-CSCF address: protected server port of SS;lr> |  |  |
|  | A3 | <sip:pcscf.other.com;lr>, <sip:scscf.other.com;lr>, <sip:orig@scscf.3gpp.org;lr>, <sip:SS P-CSCF address: unprotected server port of SS (optional);lr> |  |  |
|  | A2 | same value as received in INVITE |  |  |
|  | A4 | same value as received in INVITE |  |  |
|  |  | A5 |  | <sip:orig@ecscf.other.com;lr>, <sip:SS P-CSCF address:protected server port of SS;lr> |  |  |  |  |  |
|  |  | A1 AND A15 |  | not present |  |  |  |  |  |
|  | A2 AND A15 | any value if present |  |  |
| Via |  |  |  | RFC 3261 [15] |
| via-parm |  | same value as received in INVITE message |  |  |
| Require |  | The option tags defined below shall be included additionally to any option tags defined in any specific message content, unless specified otherwise in this specific message content. |  | RFC 3261 [15] |
| option-tag |  | 100rel |  |  |
| From |  |  |  | RFC 3261 [15] |
| addr-spec |  | same value as received in INVITE message |  |  |
| tag |  | same value as received in INVITE message |  |  |
| To |  |  |  | RFC 3261 [15] |
| addr-spec |  | same value as received in INVITE message |  |  |
| tag | A1,A3, A5,A6 | common to-tag (invite) |  |  |
|  | A2,A4 | any value |  |  |
| P-Asserted-Identity | A5 |  |  | RFC 3325 [89] |
| addr-spec |  | A tel URI that can be recognized as valid emergency numbers if dialled by the user are specified in 3GPP TS 22.101 [39]. The emergency numbers 112 and 911 are stored on the ME, in accordance with 3GPP TS 22.101 [39] |  |  |
| uri-parameter |  | lr |  |  |
| Contact |  |  |  | RFC 3261 [15] |
| addr-spec | A1,A3 | px_IMS_CalleeContactUri |  |  |
|  | A2 AND NOT A9 | SIP URI with IP address or FQDN and protected server port of UE |  |  |
|  | A4 AND NOT A9 | SIP URI with IP address or FQDN and unprotected server port of UE |  |  |
|  | A2 AND A9 | Public GRUU as obtained during registration as pub-gruu contact parameter of the 200 OK for REGISTER response |  | RFC 5627 [61] |
|  | A4 AND A9 | Public GRUU as obtained during registration as pub-gruu contact parameter of the 200 OK for REGISTER response |  | RFC 5627 [61] |
|  | A1 AND A15 | MO call has been established: px_IMS_CalleeContactUri MT call has been established: contact address sent by SS in dialog creating INVITE |  |  |
|  | A2 AND A15 | MO call has been established: contact address sent by UE in dialog creating INVITE MT call has been established: contact address sent by UE in response to dialog creating INVITE |  |  |
| feature-param |  | +g.3gpp.icsi-ref="urn%3Aurn-7%3A3gpp-service.ims.icsi.mmtel"(see NOTE 2, 3) |  |  |
|  | A6 | video |  |  |
|  | A10 | audio |  |  |
|  | A11 AND A12 AND (A13 OR A14) | audio |  |  |
| RSeq |  |  |  | RFC 3262 [33] |
| response-num | A2,A4 | any value |  |  |
|  | A1,A3 | 121 (arbitrarily selected) |  |  |
| Call-ID |  |  |  | RFC 3261 [15] |
| callid |  | same value as received in INVITE message |  |  |
| CSeq |  |  |  | RFC 3261 [15] |
| value |  | same value as received in INVITE message |  |  |
| Feature-Caps |  |  |  | RFC 6809 [125] |
| feature-param | A8 | +g.3gpp.ps2cs-srvcc-orig-pre-alerting |  | TS 24.237 [110] |
| Content-Type |  |  |  | RFC 3261 [15] |
| media-type |  | application/sdp |  |  |
| Content-Length | A1,A3 |  |  | RFC 3261 [15] |
| value |  | length of message-body |  |  |

| Condition | Explanation |
| --- | --- |
| A1 | 183 sent by the SS (IMS security, A.6a/2 3GPP TS 34.229-2 [5]) |
| A2 | 183 sent by the UE (IMS security, A.6a/2 3GPP TS 34.229-2 [5]) |
| A3 | 183 sent by the SS (GIBA, A.6a/1 3GPP TS 34.229-2 [5]) |
| A4 | 183 sent by the UE (GIBA, A.6a/1 3GPP TS 34.229-2 [5]) |
| A5 | 183 sent by the SS for INVITE for a non-UE detectable emergency call |
| A6 | UE supports video media feature tag (A.12/32 3GPP TS 34.229-2 [5]) |
| A7 | Void |
| A8 | 183 sent by the SS for a voice call and UE supports pre-alerting media feature tag (A.12/36 3GPP TS 34.229-2 [5]) |
| A9 | obtaining and using GRUUs in the Session Initiation Protocol (SIP) (A.4/53 3GPP TS 34.229-2 [5]) |
| A10 | Void |
| A11 | Void |
| A12 | UE supports audio media feature tag (A.12/56 3GPP TS 34.229-2 [5]) |
| A13 | UE uses E-UTRAN access and has received IMS voice over PS Session Supported Indication in the NAS ATTACH ACCEPT message as described in TS 24.301 [150], clauses 8.2.1 and 9.9.3.12A. |
| A14 | UE uses UTRAN/GERAN access and has received IMS voice over PS Session Supported Indication in the NAS ATTACH ACCEPT message as described in TS 24.008 [12], clauses 9.4.2 and 10.5.5.23. |
| A15 | 183 sent for re-INVITE within an established dialog |

NOTE1:	All choices for applicable conditions are described for each header.

NOTE 2:	The “=” may include optional linear white spaces according to the EQUAL definition in chapter 25.1, RFC 3261 [15].

NOTE 3:	URN is the outcome of the URL encoding (“Percent-Encoding” according to RFC 3986 [129]) of urn:urn-7:3gpp-service.ims.icsi.mmtel.

# *** End of Changes ***