3GPP TSG-RAN5 Meeting #95-e	R5-222440r2
Electronic Meeting, 9th May – 20th May 2022

Title: 	Discussion on Rel-16 Common Uplink Configuration for PC2, PC3 and PC4

Source:	        Keysight Technologies UK Ltd

Agenda Item:	5.3.19.8

Document for:	Discussion and Endorsement

# 1.	Introduction

Table 6.1-1 in 38.521-2 ([1]) defines common uplink configuration for PC2, PC3 and PC4. RB allocations columns in that table are defined based on FR2 MPR minimum conformance requirements in [2], based on Inner RB allocations and Edge RB allocations.

However, FR2 MPR requirements have evolved from Rel-15 to Rel-16 to include an extension of Inner Region 1 and to include more granularity in MPRnarrow requirements if a PC3 device supports modifiedMPRbehaviour bit 0 capability.

This document analyses differences in FR2 MPR minimum conformance requirements between Rel-15 and Rel-16 and proposes updates to Table 6.1-1 in 38.521-2 ([1]) accordingly.

# 2.	Discussion

# 2.1 FR2 MPR minimum conformance requirements overview

FR2 MPR minimum conformance requirement for Rel-15 according to [3] are defined as:

### 6.2.2 UE maximum output power reduction #### 6.2.2.0 General The requirements in clause 6.2.2 only apply when both UL and DL of a UE are configured for single CC operation, and they are of the same bandwidth. A UE may reduce its maximum output power due to modulation orders, transmit bandwidth configurations, waveform types and narrow allocations. This Maximum Power Reduction (MPR) is defined in clauses below. The allowed MPR for SRS, PUCCH formats 0, 1, 3 and 4, and PRACH shall be as specified for QPSK modulated DFT-s-OFDM of equivalent RB allocation. The allowed MPR for PUCCH format 2 shall be as specified for QPSK modulated CP-OFDM of equivalent RB allocation. When the maximum output power of a UE is modified by MPR, the power limits specified in clause 6.2.4 apply. For a UE that is configured for single CC operation with different channel bandwidths in UL and DL, the requirements in clause 6.2A.2 apply. For all power classes, the waveform defined by BW = 100 MHz, SCS = 120 kHz, DFT-S-OFDM QPSK, 20RB23 is the reference waveform with 0 dB MPR and is used for the power class definition. #### 6.2.2.1 UE maximum output power reduction for power class 1 For power class 1, MPR for contiguous allocations is defined as: MPR = max(MPRWT, MPRnarrow) Where, MPRnarrow = 14.4 dB, when BWalloc,RB ≤ 1.44 MHz, MPRnarrow = 10 dB, when 1.44 MHz < BWalloc,RB ≤ 10.8 MHz, where BWalloc,RB is the bandwidth of the RB allocation size. MPRWT is the maximum power reduction due to modulation orders, transmission bandwidth configurations listed in table 5.3.2-1, and waveform types. MPRWT is defined in Tables 6.2.2.1-1 and 6.2.2.1-2. Table 6.2.2.1-1 MPRWT for power class 1, BWchannel ≤ 200 MHz Modulation; MPRWT (dB), BWchannel ≤ 200 MHz. Outer RB allocations; Inner RB allocations. Region 1; Region 2. DFT-s-OFDM; Pi/2 BPSK; ≤ 5.5; 0.0; ≤ 3.0. QPSK; ≤ 6.5; 0.0; ≤ 3.0. 16 QAM; ≤ 6.5; ≤ 4.0; ≤ 4.0. 64 QAM; ≤ 6.5; ≤ 5.0; ≤ 5.0. CP-OFDM; QPSK; ≤ 7.0; ≤ 4.5; ≤ 4.5. 16 QAM; ≤ 7.0; ≤ 5.5; ≤ 5.5. 64 QAM; ≤ 7.5; ≤ 7.5; ≤ 7.5. Table 6.2.2.1-2 MPRWT for power class 1, BWchannel = 400 MHz Modulation; MPRWT (dB), BWchannel = 400 MHz. Outer RB allocations; Inner RB allocations. Region 1; Region 2. DFT-s-OFDM; Pi/2 BPSK; ≤ 5.5; 0.0; ≤ 3.0. QPSK; ≤ 6.5; 0.0; ≤ 3.5. 16 QAM; ≤ 6.5; ≤ 4.5; ≤ 4.5. 64 QAM; ≤ 6.5; ≤ 6.5; ≤ 6.5. CP-OFDM; QPSK; ≤ 7.0; ≤ 5.0; ≤ 5.0. 16 QAM; ≤ 7.0; ≤ 6.5; ≤ 6.5. 64 QAM; ≤ 9.0; ≤ 9.0; ≤ 9.0. Where the following parameters are defined to specify valid RB allocation ranges for the RB allocations regions in Tables 6.2.2.1-1 and 6.2.2.1-2: NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1. RBend = RBStart + LCRB - 1 RBStart,Low = Max(1, Floor(LCRB/2)) RBStart,High = NRB – RBStart,Low – LCRB An RB allocation is an Outer RB allocation if RBStart < RBStart,Low OR RBStart > RBStart,High OR LCRB > Ceil(NRB/2) An RB allocation belonging to table 6.2.2.1-1 is a Region 1 inner RB allocation if RBstart ≥ Ceil(1/3 NRB) AND RBend < Ceil(2/3 NRB) An RB allocation belonging to table 6.2.2.1-2 is a Region 1 inner RB allocation if RBstart ≥ Ceil(1/4 NRB) AND RBend < Ceil(3/4 NRB) AND LCRB ≤ Ceil(1/4 NRB) An RB allocation is a Region 2 inner allocation if it is NOT an Outer allocation AND NOT a Region 1 inner allocation For the UE maximum output power modified by MPR, the power limits specified in clause 6.2.4 apply. #### 6.2.2.2 UE maximum output power reduction for power class 2 For power class 2, MPR specified in clause 6.2.2.3 applies. Table 6.2.2.2-1: Void #### 6.2.2.3 UE maximum output power reduction for power class 3 For power class 3, MPR for contiguous allocations is defined as: MPR = max(MPRWT, MPRnarrow) Where, MPRnarrow = 2.5 dB, BWalloc,RB≤ 1.44 MHz, and 0 ≤ RBstart < Ceil(1/3 NRB) or Ceil(2/3NRB) ≤ RBstart ≤ NRB-LCRB, where BWalloc,RB is the bandwidth of the RB allocation size. MPRWT is the maximum power reduction due to modulation orders, transmission bandwidth configurations listed in Table 5.3.2-1, and waveform types. MPRWT is defined in Table 6.2.2.3-1 and Table 6.2.2.3-2. Table 6.2.2.3-1 MPRWT for power class 3, BWchannel ≤ 200 MHz Modulation; MPRWT, BWchannel ≤ 200 MHz. Inner RB allocations, Region 1; Edge RB allocations. DFT-s-OFDM; Pi/2 BPSK; 0.0; ≤ 2.0. QPSK; 0.0; ≤ 2.0. 16 QAM; ≤ 3.0; ≤ 3.5. 64 QAM; ≤ 5.0; ≤ 5.5. CP-OFDM; QPSK; ≤ 3.5; ≤ 4.0. 16 QAM; ≤ 5.0; ≤ 5.0. 64 QAM; ≤ 7.5; ≤ 7.5. Table 6.2.2.3-2 MPRWT for power class 3, BWchannel = 400 MHz Modulation; MPRWT, BWchannel = 400 MHz. Inner RB allocations, Region 1; Edge RB allocations. DFT-s-OFDM; Pi/2 BPSK; 0.0; ≤ 3.0. QPSK; 0.0; ≤ 3.0. 16 QAM; ≤ 4.5; ≤ 4.5. 64 QAM; ≤ 6.5; ≤ 6.5. CP-OFDM; QPSK; ≤ 5.0; ≤ 5.0. 16 QAM; ≤ 6.5; ≤ 6.5. 64 QAM; ≤ 9.0; ≤ 9.0. Where the following parameters are defined to specify valid RB allocation ranges for RB allocations in Tables 6.2.2.3-1 and 6.2.2.3-2: NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1. RBend = RBStart + LCRB - 1 An RB allocation belonging to table 6.2.2.3-1 is a Region 1 inner RB allocation if RBstart ≥ Ceil(1/3 NRB) AND RBend < Ceil(2/3 NRB) An RB allocation belonging to table 6.2.2.3-2 is a Region 1 inner RB allocation if RBstart ≥ Ceil(1/4 NRB) AND RBend < Ceil(3/4 NRB) AND LCRB ≤ Ceil(1/4 NRB) An RB allocation is an Edge allocation if it is NOT a Region 1 inner allocation #### 6.2.2.4 UE maximum output power reduction for power class 4 For power class 4, MPR specified in clause 6.2.2.3 applies. Table 6.2.2.4-1: Void

While, according to [2], for Rel-16, FR2 MPR minimum conformance requirements are defined as:

### 6.2.2 UE maximum output power reduction #### 6.2.2.0 General The requirements in clause 6.2.2 only apply when both UL and DL of a UE are configured for single CC operation, and they are of the same bandwidth. A UE may reduce its maximum output power due to modulation orders, transmit bandwidth configurations, waveform types and narrow allocations. This Maximum Power Reduction (MPR) is defined in clauses below. The allowed MPR for SRS, PUCCH formats 0, 1, 3 and 4, and PRACH shall be as specified for QPSK modulated DFT-s-OFDM of equivalent RB allocation. The allowed MPR for PUCCH format 2 shall be as specified for QPSK modulated CP-OFDM of equivalent RB allocation. When the maximum output power of a UE is modified by MPR, the power limits specified in clause 6.2.4 apply. For a UE that is configured for single CC operation with different channel bandwidths in UL and DL, the requirements in clause 6.2A.2 apply. For all power classes, the waveform defined by BW = 100 MHz, SCS = 120 kHz, DFT-S-OFDM QPSK, 20RB23 is the reference waveform with 0 dB MPR and is used for the power class definition. #### 6.2.2.1 UE maximum output power reduction for power class 1 For power class 1, MPR for contiguous allocations is defined as: MPR = max(MPRWT, MPRnarrow) Where, MPRnarrow = 14.4 dB, when BWalloc,RB ≤ 1.44 MHz, MPRnarrow = 10 dB, when 1.44 MHz < BWalloc,RB ≤ 10.8 MHz, where BWalloc,RB is the bandwidth of the RB allocation size. MPRWT is the maximum power reduction due to modulation orders, transmission bandwidth configurations listed in table 5.3.2-1, and waveform types. MPRWT is defined in Tables 6.2.2.1-1 and 6.2.2.1-2. Table 6.2.2.1-1 MPRWT for power class 1, BWchannel ≤ 200 MHz Modulation; MPRWT (dB), BWchannel ≤ 200 MHz. Outer RB allocations; Inner RB allocations. Region 1; Region 2. DFT-s-OFDM; Pi/2 BPSK; ≤ 5.5; 0.0; ≤ 3.0. QPSK; ≤ 6.5; 0.0; ≤ 3.0. 16 QAM; ≤ 6.5; ≤ 4.0; ≤ 4.0. 64 QAM; ≤ 6.5; ≤ 5.0; ≤ 5.0. CP-OFDM; QPSK; ≤ 7.0; ≤ 4.5; ≤ 4.5. 16 QAM; ≤ 7.0; ≤ 5.5; ≤ 5.5. 64 QAM; ≤ 7.5; ≤ 7.5; ≤ 7.5. Table 6.2.2.1-2 MPRWT for power class 1, BWchannel = 400 MHz Modulation; MPRWT (dB), BWchannel = 400 MHz. Outer RB allocations; Inner RB allocations. Region 1; Region 2. DFT-s-OFDM; Pi/2 BPSK; ≤ 5.5; 0.0; ≤ 3.0. QPSK; ≤ 6.5; 0.0; ≤ 3.5. 16 QAM; ≤ 6.5; ≤ 4.5; ≤ 4.5. 64 QAM; ≤ 6.5; ≤ 6.5; ≤ 6.5. CP-OFDM; QPSK; ≤ 7.0; ≤ 5.0; ≤ 5.0. 16 QAM; ≤ 7.0; ≤ 6.5; ≤ 6.5. 64 QAM; ≤ 9.0; ≤ 9.0; ≤ 9.0. Where the following parameters are defined to specify valid RB allocation ranges for the RB allocations regions in Tables 6.2.2.1-1 and 6.2.2.1-2: NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1. RBend = RBStart + LCRB - 1 RBStart,Low = Max(1, Floor(LCRB/2)) RBStart,High = NRB – RBStart,Low – LCRB An RB allocation is an Outer RB allocation if RBStart < RBStart,Low OR RBStart > RBStart,High OR LCRB > Ceil(NRB/2) An RB allocation belonging to table 6.2.2.1-1 is a Region 1 inner RB allocation if RBstart ≥ Ceil(1/3 NRB) AND RBend < Ceil(2/3 NRB) An RB allocation belonging to table 6.2.2.1-2 is a Region 1 inner RB allocation if RBstart ≥ Ceil(1/4 NRB) AND RBend < Ceil(3/4 NRB) AND LCRB ≤ Ceil(1/4 NRB) An RB allocation is a Region 2 inner allocation if it is NOT an Outer allocation AND NOT a Region 1 inner allocation For the UE maximum output power modified by MPR, the power limits specified in clause 6.2.4 apply. #### 6.2.2.2 UE maximum output power reduction for power class 2 For power class 2, MPR specified in clause 6.2.2.3 applies. Table 6.2.2.2-1: Void #### 6.2.2.3 UE maximum output power reduction for power class 3 For power class 3, MPR for contiguous allocations is defined as: MPR = max(MPRWT, MPRnarrow) For transmission bandwidth configuration less than or equal to 200MHz, and 0 ≤ RBstart < Ceil(1/3 NRB) or Ceil(2/3NRB) ≤ RBstart ≤ NRB-LCRB: - MPRnarrow = 2.5 dB, when BWalloc,RB is less than or equal to 1.44 MHz, - MPRnarrow = 2.0 dB, when 1.44 MHz < BWalloc,RB <= 4.32 MHz, - otherwise MPRnarrow = 0 dB. MPRWT is the maximum power reduction due to modulation orders, transmission bandwidth configurations listed in Table 5.3.2-1, and waveform types. MPRWT is defined in Table 6.2.2.3-1. Table 6.2.2.3-1 MPRWT for power class 3, BWchannel ≤ 200 MHz Modulation; MPRWT, BWchannel ≤ 200 MHz. Inner RB allocations, Region 1; Edge RB allocations. DFT-s-OFDM; Pi/2 BPSK; 0.0; ≤ 2.0. QPSK; 0.0; ≤ 2.0. 16 QAM; ≤ 3.0; ≤ 3.5. 64 QAM; ≤ 5.0; ≤ 5.5. CP-OFDM; QPSK; ≤ 3.5; ≤ 4.0. 16 QAM; ≤ 5.0; ≤ 5.0. 64 QAM; ≤ 7.5; ≤ 7.5. Where the following parameters are defined to specify valid RB allocation ranges for RB allocations in Table 6.2.2.3-1: - RBStart,Low = max(1, LCRB), where max() indicates the largest value of all arguments. - RBStart,High = NRB – RBStart,Low – LCRB, An RB allocation belonging to table 6.2.2.3-1 is a Region 1 inner RB allocation if: - RBStart,Low ≤ RBStart ≤ RBStart,High, and LCRB ≤ ceil(NRB/3), where ceil(x) is the smallest integer greater than or equal to x. For transmission bandwidth configuration equal to 400MHz, MPRnarrow = 2.5 dB, when BWalloc,RB is less than or equal to 1.44 MHz, and 0 ≤ RBstart < Ceil(1/3 NRB) or Ceil(2/3NRB) ≤ RBstart ≤ NRB-LCRB, where BWalloc,RB is the bandwidth of the RB allocation size. MPRWT is the maximum power reduction due to modulation orders, transmission bandwidth configurations listed in Table 5.3.2-1, and waveform types. MPRWT is defined in Table 6.2.2.3-2. Table 6.2.2.3-2 MPRWT for power class 3, BWchannel = 400 MHz Modulation; MPRWT, BWchannel = 400 MHz. Inner RB allocations, Region 1; Edge RB allocations. DFT-s-OFDM; Pi/2 BPSK; 0.0; ≤ 3.0. QPSK; 0.0; ≤ 3.0. 16 QAM; ≤ 4.5; ≤ 4.5. 64 QAM; ≤ 6.5; ≤ 6.5. CP-OFDM; QPSK; ≤ 5.0; ≤ 5.0. 16 QAM; ≤ 6.5; ≤ 6.5. 64 QAM; ≤ 9.0; ≤ 9.0. Where the following parameters are defined to specify valid RB allocation ranges for RB allocations in Table 6.2.2.3-2: NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1. RBend = RBStart + LCRB - 1 An RB allocation belonging to table 6.2.2.3-2 is a Region 1 inner RB allocation if RBstart ≥ Ceil(1/4 NRB) AND RBend < Ceil(3/4 NRB) AND LCRB ≤ Ceil(1/4 NRB) For all transmission bandwidth configurations, an RB allocation is an Edge allocation if it is NOT a Region 1 inner allocation. #### 6.2.2.4 UE maximum output power reduction for power class 4 For power class 4, MPR specified in sub-clause 6.2.2.3 applies. Table 6.2.2.4-1: Void #### 6.2.2.5 UE maximum output power reduction for power class 5 For power class 5, MPR specified in sub-clause 6.2.2.3 applies.

Main differences are highlighted in yellow above.

Observation 1: Main differences in FR2 MPR minimum conformance requirement (excluding Power class 5 requirements) are related to to CBW≤200 MHz:

- include an extension of Inner Region 1

- include more granularity in MPRnarrow requirements

However, differences included in Observation 1 are only applicable to PC3 devices supporting modifiedMPRbehaviour bit 0 capability. The background for this statement can be found in [4], whose cover page indicated:

and where Annex H.1 was added to 38.101-2 as shown below:

To be noticed that:

- modifiedMPRbehaviour is defined per band

- modifiedMPRbehaviour is used either in FR1 and in FR2

- modifiedMPRbehaviour bits have different meaning in FR1 (Annex L.1 in 38.101-1 [10]) and in FR2 (Annex H.1 in 38.101-2 [2])

- The only reference to modifiedMPRbehaviour in 38.307 [12] in Annex B.4.1 refers to FR1.

- modifiedMPRbehaviour is defined in Rel-15 signalling as BIT STRING (SIZE (8)) in 38.331 [11] but its use for FR2 MPR enhancements under discussion applies from Rel-16 and onwards.

Observation 2: Differences in FR2 MPR requirement described in Observation 1 are applicable to:

- Rel-15 PC3 devices supporting modifiedMPRbehaviour bit 0 capability.

- Rel-16 and onwards PC3 devices (which shallsupport modifiedMPRbehaviour bit 0 capability).

As it can be seen in [2] and [3], FR2 MPR requirements for PC2, PC4 and PC5 devices point towards FR2 MPR requirements for PC3. However, modifiedMPRbehaviour bit 0 capability according to Annex P.1 seems to be only applicable to PC3. Hence it is not clear whether MPR enhancements subject to modifiedMPRbehaviour bit 0 capability are applicable to PC2, PC4 and PC5 devices.

Proposal 1: Clarify that NOTE 4 in Table 6.1-1 in 38.521-1 ([1]) applies to Rel-15 PC3 devices supporting modifiedMPRbehaviour bit 0 capability  and to Rel-16 and onwards PC3 devices (which shall support modifiedMPRbehaviour bit 0 capability according to Annex P.1). Clarify that NOTE 3 applies to PC3  devices which do not support modifiedMPRbehaviour bit 0 capability and to Rel-15 and forward PC2 and PC4 devices.

Proposal 2: Send an LS to RAN4 to ask for clarification on:

- whether MPR enhancements subject to modifiedMPRbehaviour bit 0 capability are also applicable to PC2, PC4 and PC5 devices

- whether the MPR enhancements as defined in 38.101-2 v16.2.0 are mandatory or optional for a Rel-16 UE.

- Whether the Rel-16 UE is expected to signal modifiedMPRbehaviour bit 0 = true

# 2.2 FR2 MPR enhancements: additional granularity for MPRnarrow requirements

As described in Observation 1, one of the differences between Rel-15 and Rel-16 minimum conformance requirements is the inclusion of additional granularity in MPRnarrow requirements, defining MPRnarrow=2.0 dB when 1.44 MHz < BWalloc,RB <= 4.32 MHz. However, current Inner_Partial_Left or Inner_partial_Right are defined using the criterion of no impact of MPRnarrow. Hence new columns need to be added to table 6.1-1 as Inner_Partial2_left and Inner_Partial2_Right in such a way that they allow to test MPRnarrow=2.0 dB. These 2 new columns will be only applicable as per NOTE 4.

Proposal 3: Add 2 columns to Table 6.1-1 in 38.521-2 ([1]) as Inner_Partial2_left and Inner_Partial2_Right to enable testing of MPRnarrow=2.0 dB and add clarification to Note 2 on the purpose for these 2 new columns. These 2 new columns will be only applicable as per NOTE 4.

Proposal 4: Choose for both waveforms LCRB=6 for SCS=60kHz and LCRB=3 for SCS=120 kHz for Inner_Partial2 allocations.

# 2.3 FR2 MPR enhancements: Inner Region 1 extension.

In Rel-16, Inner Region 1 has been extended for CBW≤200MHz according to the following statement:

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and LCRB  ≤  ceil(NRB/3), where

RBStart,Low = max(1, LCRB), where max() indicates the largest value of all arguments.

RBStart,High = NRB – RBStart,Low – LCRB,

Hence:

- For CBW≤200MHz, the left boundary of Inner Region 1 is given by RBStart,Low = max(1, LCRB) for minimal LCRB, i.e. 1.

- As Inner_Partial for CBW≤200MHz shall have no impact from MPRnarrow according to NOTE2 in Table 6.1-1 in 38.521-2 ([1]), the minimum values of LCRB values complying with this condition can be chosen as:

Table 2.3-1 LCRB and RBStart for Inner_Partial for CBW≤200MHz

|  | # CP-OFDM | # DFT-s OFDM |
| --- | --- | --- |
| # SCS=60 kHz | # 7 | # 8 |
| # SCS=120 kHz | # 4 | # 4 |

- RBStart for Inner_Partial_Left will be determined by RBStart,Low = max(1, LCRB) for CBW≤200MHz. Hence values for RBStart for Inner_Partial_Left will be the same as LCRB, i.e. values shown in Table 2.3-1.

- Taking maximum NRB allocations, SCS and CBW as described in [5] and LCRB for Inner_Partial as defined in table 2.3-1, as RBStart,High = NRB – RBStart,Low – LCRB,

  - RBstart for Inner_Partial_Right will be:

Table 2.3-2 RBStart for Inner_Partial_Right for CBW≤200MHz and CP-OFDM

| SCS [kHz] | BS / UE Channel bandwidths [MHz] |  |  |
| --- | --- | --- | --- |
|  | 50 | 100 | 200 |
| 60 | 52 | 118 | 250 |
| 120 | 24 | 58 | 124 |

Table 2.3-3 RBStart for Inner_Partial_Right for CBW≤200MHz and DFTs-OFDM

| SCS [kHz] | BS / UE Channel bandwidths [MHz] |  |  |
| --- | --- | --- | --- |
|  | 50 | 100 | 200 |
| 60 | 50 | 116 | 248 |
| 120 | 24 | 58 | 124 |

  - RBStart for Inner_1RB_Right will be:

Table 2.3-4 RBStart for Inner_1RB_Right for CBW≤200MHz and CP-OFDM and DFTs-OFDM

| SCS [kHz] | BS / UE Channel bandwidths [MHz] |  |  |
| --- | --- | --- | --- |
|  | 50 | 100 | 200 |
| 60 | 64 | 130 | 262 |
| 120 | 30 | 64 | 130 |

- RBStart for Inner_Partial_Left will be determined by RBStart,Low = max(1, LCRB) for CBW≤200MHz. Hence values for RBStart for Inner_Partial_Left will be the same as LCRB, i.e. values shown in Table 2.3-1.

- Regarding LCRB and RBStart for Inner_Full allocation, it is important to identify which of the 3 conditions:

  - Condition 1: RBStart,Low  ≤  RBStart   max(1, LCRB)= LCRB  ≤ RBStart

  - Condition 2: RBStart ≤  RBStart,High

  - Condition 3: LCRB  ≤  ceil(NRB/3)

will limit the maximum RB allocation.

For Inner Full max RB allocation:

- As RBStart,Low = max(1, LCRB), condition 1 can be written as:

Condition 1’:  LCRB ≤ RBStart

- Using condition 1’, condition 2 can be written as:

Condition 2’: LCRB ≤  RBStart,High

- If Condition 2’ and condition 3 are combined:

LCRB = MIN (RBStart,High, ceil(NRB/3))

- As according to NOTE 1 in Table 6.1-1 in 38.521-2 ([1]), RB allocation is left aligned within inner region:

RBStart = RBStart,Low = max(1, LCRB)= LCRB

To be noticed that final LCRB  for DFT-s waveform needs to comply with the condition:

Values for LCRB and RBStart for Inner_Full allocation are shown in Table 2.3-6:

Table 2.3-6 LCRB and RBStart for Inner_Full for CBW≤200MHz

| Waveform | SCS [kHz] | CBW [MHz] | NRB | max RBStart_High= NRB-RBStart_low-LCRB | max LCRB= Ceil(NRB/3) | LCRB = RBStart_Low= MIN (max LCRB, max RBStart_High) |
| --- | --- | --- | --- | --- | --- | --- |
| CP-OFDM | 60 | 50 | 66 | 22 | 22 | 22 |
|  |  | 100 | 132 | 44 | 44 | 44 |
|  |  | 200 | 264 | 88 | 88 | 88 |
| CP-OFDM | 120 | 50 | 32 | 10 | 11 | 10 |
|  |  | 100 | 66 | 22 | 22 | 22 |
|  |  | 200 | 132 | 44 | 44 | 44 |
| DFTs-OFDM | 60 | 50 | 66 | 22 | 22 | 20 |
|  |  | 100 | 132 | 44 | 44 | 40 |
|  |  | 200 | 264 | 88 | 88 | 81 |
| DFTs-OFDM | 120 | 50 | 32 | 10 | 11 | 10 |
|  |  | 100 | 66 | 22 | 22 | 20 |
|  |  | 200 | 132 | 44 | 44 | 40 |

Proposal 5: Set RBstart for Inner_1RB_Left to 1 for all waveforms, channel bandwidth and subcarrier spacing if CBW≤200 MHz.

Proposal 6: Define LCRB for Inner_Partial as described in Table 2.3-1 if CBW≤200 MHz.

Proposal 7: Define RBstart for Inner_Partial_Left with same values described in Table 2.3-1 if CBW≤200 MHz.

Proposal 8: Define RBstart for Inner_Partial_Right as defined in Tables 2.3-2 and 2.3-3 if CBW≤200 MHz.

Proposal 9: Define RBstart for Inner_1RB_Right as defined in Tables 2.3-4 and 2.3-5 if CBW≤200 MHz.

Proposal 10: Define LCRB and RBstart for Inner_Full as described in Table 2.3-6 if CBW≤200 MHz.

# 3. 	Conclusion

This document analyses differences in FR2 MPR minimum conformance requirements between Rel-15 and Rel-16 and makes the following observations and proposals:

Observation 1: Main differences in FR2 MPR minimum conformance requirement (excluding Power class 5 requirements) are related to CBW≤200 MHz:

- include an extension of Inner Region 1

- include more granularity in MPRnarrow requirements

Observation 2: Differences in FR2 MPR requirement described in Observation 1 are only applicable to Rel-16 and onwards PC3 devices supporting modifiedMPRbehaviour bit 0 capability.

Proposal 1: Clarify that NOTE 4 in Table 6.1-1 in 38.521-1 ([1]) applies to Rel-16 and onwards PC3 devices that support modifiedMPRbehaviour bit 0 capability according to Annex P.1.

Proposal 2: Send an LS to RAN4 to ask for clarification on whether MPR enhancements subject to modifiedMPRbehaviour bit 0 capability are also applicable to PC2, PC4 and PC5 devices.

Proposal 3: Add 2 columns to Table 6.1-1 in 38.521-2 ([1]) as Inner_Partial2_left and Inner_Partial2_Right to enable testing of MPRnarrow=2.0 dB and add clarification to Note 2 on the purpose for these 2 new columns. These 2 new columns will be only applicable to Rel-16 and onwards PC3 devices supporting modifiedMPRbehaviour bit 0 capability.

Proposal 4: Choose for both waveforms LCRB=6 for SCS=60kHz and LCRB=3 for SCS=120 kHz for Inner_Partial2 allocations.

Proposal 5: Set RBstart for Inner_1RB_Left to 1 for all waveforms, channel bandwidth and subcarrier spacing if CBW≤200 MHz.

Proposal 6: Define LCRB for Inner_Partial as described in Table 2.3-1 if CBW≤200 MHz.

Table 2.3-1 LCRB and RBStart for Inner_Partial for CBW≤200MHz

|  | # CP-OFDM | # DFT-s OFDM |
| --- | --- | --- |
| # SCS=60 kHz | # 7 | # 8 |
| # SCS=120 kHz | # 4 | # 4 |

Proposal 7: Define RBstart for Inner_Partial_Left with same values described in Table 2.3-1 if CBW≤200 MHz.

Proposal 8: Define RBstart for Inner_Partial_Right as defined in Tables 2.3-2 and 2.3-3 if CBW≤200 MHz.

Table 2.3-2 RBStart for Inner_Partial_Right for CBW≤200MHz and CP-OFDM

| SCS [kHz] | BS / UE Channel bandwidths [MHz] |  |  |
| --- | --- | --- | --- |
|  | 50 | 100 | 200 |
| 60 | 52 | 118 | 250 |
| 120 | 24 | 58 | 124 |

Table 2.3-3 RBStart for Inner_Partial_Right for CBW≤200MHz and DFTs-OFDM

| SCS [kHz] | BS / UE Channel bandwidths [MHz] |  |  |
| --- | --- | --- | --- |
|  | 50 | 100 | 200 |
| 60 | 48 | 112 | 240 |
| 120 | 24 | 56 | 120 |

Proposal 9: Define RBstart for Inner_1RB_Right as defined in Tables 2.3-4 and 2.3-5 if CBW≤200 MHz.

Table 2.3-4 RBStart for Inner_1RB_Right for CBW≤200MHz and CP-OFDM

| SCS [kHz] | BS / UE Channel bandwidths [MHz] |  |  |
| --- | --- | --- | --- |
|  | 50 | 100 | 200 |
| 60 | 64 | 130 | 262 |
| 120 | 30 | 64 | 130 |

Table 2.3-5 RBStart for Inner_1RB_Right for CBW≤200MHz and DFTs-OFDM

| SCS [kHz] | BS / UE Channel bandwidths [MHz] |  |  |
| --- | --- | --- | --- |
|  | 50 | 100 | 200 |
| 60 | 62 | 126 | 254 |
| 120 | 30 | 62 | 126 |

Proposal 10: Define LCRB and RBstart for Inner_Full as described in Table 2.3-6 if CBW≤200 MHz.

Table 2.3-6 LCRB and RBStart for Inner_Full for CBW≤200MHz

| Waveform | SCS [kHz] | CBW [MHz] | NRB | max RBStart_High= NRB-RBStart_low-LCRB | max LCRB= Ceil(NRB/3) | LCRB = RBStart_Low= MIN (max LCRB, max RBStart_High) |
| --- | --- | --- | --- | --- | --- | --- |
| CP-OFDM | 60 | 50 | 66 | 22 | 22 | 22 |
|  |  | 100 | 132 | 44 | 44 | 44 |
|  |  | 200 | 264 | 88 | 88 | 88 |
| CP-OFDM | 120 | 50 | 32 | 10 | 11 | 10 |
|  |  | 100 | 66 | 22 | 22 | 22 |
|  |  | 200 | 132 | 44 | 44 | 44 |
| DFTs-OFDM | 60 | 50 | 64 | 24 | 20 | 20 |
|  |  | 100 | 128 | 48 | 40 | 40 |
|  |  | 200 | 256 | 94 | 81 | 81 |
| DFTs-OFDM | 120 | 50 | 32 | 12 | 10 | 10 |
|  |  | 100 | 64 | 24 | 20 | 20 |
|  |  | 200 | 128 | 48 | 40 | 40 |

Proposals 1 and 3 to 10 are implemented in [5]. A draft LS implementing proposal 2 can be found in [6].

# 4.	References

[1] 3GPP TS 38.521-2 v16.11.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone

[2] 3GPP TS 38.101-2 v16.11.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 16)

[3] 3GPP TS 38.101-2 v15.17.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 15)

[4] R4-2009169, “Indication of modified MPR behaviour for FR2 MPR enhancement, Nokia, Nokia Shanghai Bell, RAN4#95e

[5] R5-22kkkk, “Discussion on Rel-15 Common Uplink Configuration for PC2, PC3 and PC4”, Keysight Technologies UK Ltd, RAN5#95e

[6] R5-222444, “Common Uplink Configuration updates for NR RF requirement enhancements for FR2”, Keysight Technologies UK Ltd, RAN5#95e, May 2022

[7] R5-222441 “Draft LS on ModifiedMPRbehaviour clarification for different power classes”, Keysight Technologies UK Ltd, RAN5#95e, May 2022