3GPP TSG-RAN5 Meeting #95-e	R5-222550r1

# Online, , 9th May 2022 - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2924 | rev | - | Current version: | 16.11.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC CAG testcase 6.5.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ, MediaTek |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RAN_PRN_Vertical_LAN-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current Prose at Steps 3, 24 and 47 the user is prompted to manually choose a cell, and as a result in Steps 4, 25 and 48 respectively the UE is expected to camp within 60s on the target NR cell. Some UEs may take longer depending on number of bands enabled, RATs enabled etc. Hence it is proposed to not put an upper limit on the time taken to camp on a cell at Steps 4, 25 and 48. This will not have any impact on TPs, but will make testcase more stable. According to 23.122v16.12.0 cl 4.4.3.1, Quote EXCEPTION: At switch on, if the MS is in manual mode and neither registered PLMN nor PLMN that is equivalent to it is available but EHPLMN is available, then instead of performing the manual network selection mode procedure of clause 4.4.3.1.2 the MS may select and attempt registration on the highest priority EHPLMN. If the EHPLMN list is not available or is empty and the HPLMN is available, then the MS may select and attempt registration on the HPLMN. The MS shall remain in manual mode. Unquote According to preamble, the UE is registered on NR Cell 1, so the HPLMN in this case would PLMN1. NR Cell 2 also belongs to PLMN1. Thus at Step 3, the condition highlighted above occurs, as a result the UE should camp on NR cell 2 without the AT command. Thus, there is no need for Step 3 and it should be voided. At Step 45, the UE is made to select PLMN1 manually. NR Cell 1 and NR Cell 2 are both on this PLMN (at same power level), and NR Cell 2 CAG ID is in the allowed list already because of Steps 5-22. Thus, if a UE detects NR Cell 2 before NR Cell 1, it may try to camp on the NR Cell 2 and thus fail the testcase incorrectly. This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Voided Steps 3, 4, 25 and 48, and reworked the Step numbers accordingly. Also updated the text for Steps 5-22, Steps 26-27 and Steps 49-53. Updated the T3 setting to make only NR Cell 1 selectable. Also introduced another power level setting T4, which sets NR Cell 1 to suitable neighbour cell power, and NR Cell 2 at serving cell power. Introduced a new Step 46A to apply T4 so that before the selection of NR Cell 2 at Step 47 the cell is at a serving cell power. This will not have any impact on the TP3 but will make the testcase mre stable. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TTCN based on existing Prose will fail a conformant UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-222550r1 : Introduced new Steps 4a1 and 4b1-4b3 to accommodate a situation where a UE may not select the HPLMN after Step 2, since the 23.122. cl 4.4.3.1, says “EXCEPTION: At switch on, if the MS is in manual mode and neither registered PLMN nor PLMN that is equivalent to it is available but EHPLMN is available, then instead of performing the manual network selection mode procedure of clause 4.4.3.1.2 the MS may select and attempt registration on the highest priority EHPLMN. If the EHPLMN list is not available or is empty and the HPLMN is available, then the MS may select and attempt registration on the HPLMN. The MS shall remain in manual mode” Also updated the length of IE Length of CAG information list contents in Table 6.5.2.1.3.3-2/3 from 1Byte 2bytes. |  |  |  |  |  |  |  |  |

#### 6.5.2.1	CAG Selection in Manual Mode

6.5.2.1.1	Test Purpose (TP)

(1)

with { UE in Manual network selection mode and provided with CAG information list }

ensure that {

when { there exists an entry with the PLMN ID of the PLMN in the "CAG information list" and the CAG-ID is included in the "Allowed CAG list" }

then { UE manually selects the CAG cell }

}

(2)

with { UE in Manual network selection mode and PLMN ID of the PLMN in the "CAG information list" and the CAG-ID are not included in the "Allowed CAG list" }

ensure that {

when { the available CAG cell broadcasts that the PLMN allows a user to manually select the CAG-ID }

then { UE manually selects the CAG cell that is outside the UE's Allowed CAG list }

}

(3)

with { UE in Manual network selection mode }

ensure that {

when { there exists an entry for the presented PLMN in the "CAG information list" and the entry includes an "indication that the MS is only allowed to access 5GS via CAG cells" }

then { UE manually selects the CAG cell }

}

6.5.2.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 23.122, clause 4.4.3.1.2. Unless otherwise stated these are Rel-16 requirements.

[TS 23.122, clause 4.4.3.1.2]

The MS indicates whether there are any PLMNs, which are available using all supported access technologies. This includes PLMNs in the "forbidden PLMNs" list, "forbidden PLMNs for GPRS service" list and PLMNs which only offer services not supported by the MS. An MS which supports GSM COMPACT shall also indicate GSM COMPACT PLMNs (which use PBCCH).

If displayed, PLMNs meeting the criteria above are presented in the following order:

i)-	either the HPLMN (if the EHPLMN list is not present or is empty) or, if one or more of the EHPLMNs are available then based on an optional data field on the SIM either only the highest priority available EHPLMN is to be presented to the user or all available EHPLMNs are presented to the user in priority order. If the data field is not present on the SIM, then only the highest priority available EHPLMN is presented;

ii)-	PLMN/access technology combinations contained in the " User Controlled PLMN Selector with Access Technology " data file in the SIM (in priority order);

iii)- PLMN/access technology combinations contained in the "Operator Controlled PLMN Selector with Access Technology" data file in the SIM (in priority order) or stored in the ME (in priority order);

iv)- other PLMN/access technology combinations with received high quality signal in random order;

v)-	other PLMN/access technology combinations in order of decreasing signal quality.

…

In i to v, if the MS supports CAG, for each PLMN/access technology combination of NG-RAN access technology, the MS shall present to the user:

a)	the PLMN/access technology combination and a list of CAG-IDs composed of one or more CAG-IDs such that for each CAG-ID:

1)	there is an available CAG cell which broadcasts the CAG-ID for the PLMN; and

2)	the following is true:

i)	there exists an entry with the PLMN ID of the PLMN in the "CAG information list" and the CAG-ID is included in the "Allowed CAG list" of the entry; or

ii)	the available CAG cell broadcasting the CAG-ID for the PLMN also broadcasts that the PLMN allows a user to manually select the CAG-ID.

For each of the presented CAG-ID, the MS may indicate to the user whether the CAG-ID is present in the "Allowed CAG list" stored in the UE; and

b)	the PLMN/access technology combination without a list of CAG-IDs, if there is an available NG-RAN cell which is not a CAG cell for the PLMN. If there exists an entry for the presented PLMN in the "CAG information list" and the entry includes an "indication that the MS is only allowed to access 5GS via CAG cells", the MS may indicate to the user that the MS is only allowed to access the PLMN via CAG cells.

6.5.2.1.3	Test description

6.5.2.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1, NR Cell 2 and NR Cell 4.

-	NR Cell 2 and NR Cell 4 are CAG cells.

-	System information combination NR-2 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used for NR Cell 1, NR Cell 2 and NR Cell 4.

-	The PLMNs are identified in the test by the identifiers in Table 6.5.2.1.3.1-1 and the PLMN settings are defined in TS 36.523-1 [13] table 6.0.1-1.

Table 6.5.2.1.3.1–1: PLMN and CAG identifiers

| NR Cell | PLMN name | CAG IDs |
| --- | --- | --- |
| 1 | PLMN 1 | - |
| 2 | PLMN 1 | 1 |
| 4 | PLMN 2 | 2 |

UE:

None.

Preamble:

-	The UE is registered on NR Cell 1 using the procedure described in TS 38.508-1 [4] clause 4.5.2.2 except that the REGISTRATION ACCEPT message includes CAG information list with CAG-ID 1.

-    The UE is set to Manual PLMN selection mode before it is in state Switched OFF (State 0N-B) as per TS 38.508-1 [4] Table 4.4A.2-0.

6.5.2.1.3.2	Test procedure sequence

Table 6.5.2.1.3.2-1 for FR1 and Table 6.5.2.1.3.2-2 for FR2 illustrate the downlink power to be applied for the cells at various time instants of the test execution. The configuration T0 indicates the initial conditions for preamble. Configurations marked "T1", "T2" and "T3"are applied at the points indicated in the Main behaviour description in Table 6.5.2.1.3.2-3.

Table 6.5.2.1.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | NR Cell 4 | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | "Off" | "Off" | Power level "Off" is defined in TS 38.508-1 [4] Table 6.2.2.1-3 |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | -88 | "Off" | Power level "Off" is defined in TS 38.508-1 [4] Table 6.2.2.1-3 |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | "Off" | -88 | Power level "Off" is defined in TS 38.508-1 [4] Table 6.2.2.1-3 |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | -88 | ”Off” | "Off" | Power level "Off" is defined in TS 38.508-1 [4] Table 6.2.2.1-3 |
| T4 | SS/PBCH SSS EPRE | dBm/SCS | -94 | -88 | "Off" | Power level "Off" is defined in TS 38.508-1 [4] Table 6.2.2.1-3 |

Table 6.5.2.1.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | NR Cell 4 | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS | FFS |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS | FFS |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS | FFS |  |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS | FFS |  |
| T4 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS | FFS |  |

Table 6.5.2.1.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes cell power levels according to row "T1" in Table 6.5.2.1.3.2-1 for FR1 and Table 6.5.2.1.3.2-2 for FR2. | - | - | - | - |
| 2 | The UE is switched on. | - | - | - | - |
| 3 | Void |  |  |  |  |
| 3A | SS waits for timer T1 = 120s |  |  |  |  |
| 4a1 | Check: Does the UE send a RRCSetupRequest on NR Cell 2 within 120s? | --> | NR RRC: RRCSetupRequest | 1 | P |
| 4b1 | Timer T1 expires | - | - | - | - |
| 4b2 | The UE is made to perform manual CAG selection and selects NR Cell 2 | - | - | - | - |
| 4b3 | Check: Does the UE send a RRCSetupRequest on NR Cell 2 | --> | NR RRC: RRCSetupRequest | 1 | P |
| 5-22 | Steps 3-20a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed on NR Cell 2. Note: The UE performs initial registration procedure with REGISTRATION ACCEPT message including CAG information list with PLMN ID 1 and CAG only indication and the RRC connection is released. | - | - |  |  |
| 23 | The SS changes cell power levels according to row "T2" in Table 6.5.2.1.3.2-1 for FR1 and Table 6.5.2.1.3.2-2 for FR2. | - | - | - | - |
| 24 | The UE is made to perform manual CAG selection and select NR Cell 4. | - | - | - | - |
| 25 | Void |  |  |  |  |
| 26-27A | Steps 1 to 3 of the test procedure to check that UE is camped on a new cell belonging to a new TA as specified in TS 38.508-1 [4] subclause 4.9.5 are performed on NR Cell 4. | - | - | 2 | P |
| 28-36 | Void | - | - | - | - |
| 37 | The SS transmits a DLInformationTransfer message and a REGISTRATION ACCEPT message. | <-- | NR RRC: DLInformationTransfer 5G MM: REGISTRATION ACCEPT | - | - |
| 38-39 | Steps 5 to 6a1 of the test procedure to check that UE is camped on a new cell belonging to a new TA as specified in TS 38.508-1 [4] subclause 4.9.5 are performed on NR Cell 4. | - | - | - | - |
| 40-43 | Void | - | - | - | - |
| 44 | The SS changes cell power levels according to row "T3" in Table 6.5.2.1.3.2-1 for FR1 and Table 6.5.2.1.3.2-2 for FR2. | - | - | - | - |
| 45 | The UE is made to perform manual PLMN selection and select NR Cell 1. | - | - | - | - |
| 46 | Check: Does the UE send a RRCSetupRequest on NR Cell 1 within 60s? | --> | NR RRC: RRCSetupRequest | 3 | F |
| 46A | The SS changes cell power levels according to row "T4" in Table 6.5.2.1.3.2-1 for FR1 and Table 6.5.2.1.3.2-2 for FR2. | - | - | - | - |
| 47 | The UE is made to perform manual CAG selection and select NR Cell 2. |  |  | - | - |
| 48 | Void |  |  |  |  |
| 49-53A | Steps 1-6a1 of the test procedure to check that UE is camped on a new cell belonging to a new TA as specified in TS 38.508-1 [4] subclause 4.9.5 are performed on NR Cell 2. | - | - | 3 | P |

6.5.2.1.3.3	Specific message contents

Table 6.5.2.1.3.3-1: REGISTRATION REQUEST (Preamble)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| 5GS registration type | '0000 0001'B | Initial registration |  |
| 5GMM capability |  |  |  |
| CAG | ‘1’B | CAG supported |  |

Table 6.5.2.1.3.3-2: REGISTRATION ACCEPT (Preamble)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| CAG information list |  | MCC : 001; MNC : 01; CAG only : 0; CAG-ID 1 : 1 |  |
| Length of CAG information list contents | ‘0000 0000 0000 1001’B |  |  |
| Entry 1 |  |  |  |
| Length of entry contents | ‘0000 1000’B |  |  |
| MCC | MCC of NR Cell 2 | See TS 38.508-1 [4] Table 4.4.2-3 |  |
| MNC | MNC of NR Cell 2 | See TS 38.508-1 [4] Table 4.4.2-3 |  |
| CAG only | ‘0’B |  |  |
| CAG_ID 1 | 1 | CAG ID is coded as a 32 bit BITSTRING |  |

Table 6.5.2.1.3.3-3: REGISTRATION ACCEPT (Step 16, Table 6.5.2.1.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| CAG information list |  | MCC : 001; MNC : 01; CAG only : 1; CAG-ID 1 : 1 |  |
| Length of CAG information list contents | ‘0000 0000 0000 1001’B |  |  |
| Entry 1 |  |  |  |
| Length of entry contents | ‘0000 1000’B |  |  |
| MCC | MCC of NR Cell 2 | See TS 38.508-1 [4] Table 4.4.2-3 |  |
| MNC | MNC of NR Cell 2 | See TS 38.508-1 [4] Table 4.4.2-3 |  |
| CAG only | ‘1’B |  |  |
| CAG_ID 1 | 1 | CAG ID is coded as a 32 bit BITSTRING |  |

Table 6.5.2.1.3.3-4: SIB1 for NR Cell 4 (all steps in T2, Table 6.5.2.1.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-28 with condition SNPN and CAG |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| CellAccessRelatedInfo SEQUENCE { |  |  |  |
| npn-IdentityInfoList-r16 SEQUENCE (SIZE (1..maxPLMN)) OF NPN-IdentityInfo-r16 { | 1 entry |  |  |
| NPN-IdentityInfo-r16[1] SEQUENCE { |  | entry 1 |  |
| npn-IdentityList-r16 SEQUENCE (SIZE (1..maxPLMN)) OF NPN-Identity-r16 { | 1 entry |  |  |
| NPN-Identity-r16[1] CHOICE { |  | entry 1 |  |
| pni-npn-r16 SEQUENCE { |  |  |  |
| plmn-Identity-r16 SEQUENCE { |  |  |  |
| mcc | 002 | PLMN2 MCC |  |
| mnc | 11 | PLMN2 MNC |  |
| } |  |  |  |
| cag-IdentityList-r16 SEQUENCE (SIZE (1..maxNPN-r16)) OF CAG-IdentityInfo-r16 { | 1 entry |  |  |
| CAG-IdentityInfo-r16[1] SEQUENCE { |  | entr 1 |  |
| cag-Identity-r16 | 2 | CAG ID is coded as a 32 bit BITSTRING |  |
| manualCAGselectionAllowed-r16 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| trackingAreaCode-r16 | TAC of NR Cell 4 |  |  |
| cellIdentity-r16 | Cell Identity of NR Cell 4 |  |  |
| cellReservedForOperatorUse-r16 | notReserved |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |