3GPP TSG-RAN WG5 Meeting #95-e	R5-222607r2

Electronic Meeting, 9th – 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-2 | CR | 0160 | rev | - | Current version: | 16.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of TC 7.5.2 PosSIB broadcasting followed by location information transfer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The feature of posSIBs broadcasting in EN-DC scenario need to be tested. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added TC 7.5.2 PosSIB broadcasting followed by location information transfer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The WI will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1, 7.5.2(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 37.571-3 CR 0153 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: - Added the test configuration for MBS and Sensor sub-tests. - Removed unnecessary posSIBs. R2: - Updated the systeminforation for different Sub-tests. |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

5.1	LCS Sub-Test Cases

Some test cases defined in clause 7 may include several sub-test cases dependent on the positioning method(s) supported by the UE. Each sub-test case is identified by a sub-test case number as defined in Table 5.1-1. The applicable sub-tests for each test case are specified in the test procedure sequence clause of each test case. If no sub-tests are defined for a specific test case it means that this particular test case is not dependent on a specific positioning method.

Table 5.1-1: Sub-Test Case Numbers for E-UTRA

| Sub-Test Case Number | Supported Positioning Methods |
| --- | --- |
| 1 | Void |
| 2 | Void |
| 3 | Void |
| 4 | Void |
| 5 | UE supporting OTDOA |
| 6 FDD | UE supporting ECID (FDD) |
| 6 TDD | UE supporting ECID (TDD) |
| 7 | UE supporting GNSS(1) and OTDOA |
| 8 | Void |
| 9 | Void |
| 10 | Void |
| 11 | UE supporting WLAN (Rel-13 only) |
| 12 | UE supporting MBS(2) (Rel-13 only) |
| 13 | UE supporting Bluetooth |
| 14 | UE supporting Sensor (Rel-13 only) |
| 15 | UE supporting GNSS(1) |
| 16 | UE supporting MBS(2) (Rel-14 onwards) |
| 17 | UE supporting WLAN (Rel-14 onwards) |
| 18 | UE supporting Sensor (Rel-14 onwards) |
| 23 | UE supporting MBS(2) (Rel-15 onwards) |
| 24 | UE supporting Sensor (Rel-15 onwards) |
| 25 | UE supporting GNSS(1) (Rel-15 onwards) |
| NOTE 1: The GNSS combination of GPS, GLONASS, Galileo, BDS supported by the UE NOTE 2: Metropolitan Beacon System (MBS) is a specific type of Terrestrial Beacon System (TBS) [29] |  |

# < Unchanged sections omitted >

7.5.2	PosSIB broadcasting followed by location information transfer

7.5.2.1	Test Purpose (TP)

(1)

with { a NAS signalling connection for EPC-NI-LR session existing }

ensure that {

when { UE has no assistance data stored and receives the positioning assistance data via posSIBs and then UE receives a location request from LMF}

then { the UE sends a PROVIDE LOCATION INFORMATION message containing a location estimate }

}

7.5.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 37.355 clauses 7.1, 7.2. Unless otherwise stated these are Rel-15 requirements.

[TS 37.355, clause 7.1]

Broadcast of positioning assistance data is supported via Positioning System Information Blocks (posSIBs) as specified in TS 36.331 [12] or TS 38.331 [35]. The posSIBs are carried in RRC System Information (SI) messages (TS 36.331 [12] or TS 38.331 [35]).

For LTE RRC System Information (SI), a single SystemInformationBlockPos IE is defined in TS 36.331 [12] which is carried in IE PosSystemInformation-r15-IEs specified in TS 36.331 [12]. The mapping of positioning SIB type (posSibType) to assistance data carried in SystemInformationBlockPos is specified in clause 7.2.

For NR RRC System Information (SI), a single SIBpos IE is defined in TS 38.331 [35] which is carried in IE PosSystemInformation-r16-IEs specified in TS 38.331 [35]. The mapping of positioning SIB type (posSibType) to assistance data carried in SIBpos is specified in clause 7.2.

[TS 37.355, clause 7.2]

The supported posSibType's are specified in Table 7.2-1. The GNSS Common and Generic Assistance Data IEs are defined in clause 6.5.2.2. The OTDOA Assistance Data IEs and NR DL-TDOA/DL-AoD Assistance Data IEs are defined in clause 7.4.2. The Barometric Assistance Data IEs are defined in clause 6.5.5.8. The TBS (based on MBS signals) Assistance Data IEs are defined in clause 6.5.4.8.

Table 7.2-1: Mapping of posSibType to assistanceDataElement

|  | posSibType | assistanceDataElement |
| --- | --- | --- |
| GNSS Common Assistance Data (clause 6.5.2.2) | posSibType1-1 | GNSS-ReferenceTime |
|  | posSibType1-2 | GNSS-ReferenceLocation |
|  | posSibType1-3 | GNSS-IonosphericModel |
|  | posSibType1-4 | GNSS-EarthOrientationParameters |
|  | posSibType1-5 | GNSS-RTK-ReferenceStationInfo |
|  | posSibType1-6 | GNSS-RTK-CommonObservationInfo |
|  | posSibType1-7 | GNSS-RTK-AuxiliaryStationData |
|  | posSibType1-8 | GNSS-SSR-CorrectionPoints |
| GNSS Generic Assistance Data (clause 6.5.2.2) | posSibType2-1 | GNSS-TimeModelList |
|  | posSibType2-2 | GNSS-DifferentialCorrections |
|  | posSibType2-3 | GNSS-NavigationModel |
|  | posSibType2-4 | GNSS-RealTimeIntegrity |
|  | posSibType2-5 | GNSS-DataBitAssistance |
|  | posSibType2-6 | GNSS-AcquisitionAssistance |
|  | posSibType2-7 | GNSS-Almanac |
|  | posSibType2-8 | GNSS-UTC-Model |
|  | posSibType2-9 | GNSS-AuxiliaryInformation |
|  | posSibType2-10 | BDS-DifferentialCorrections |
|  | posSibType2-11 | BDS-GridModelParameter |
|  | posSibType2-12 | GNSS-RTK-Observations |
|  | posSibType2-13 | GLO-RTK-BiasInformation |
|  | posSibType2-14 | GNSS-RTK-MAC-CorrectionDifferences |
|  | posSibType2-15 | GNSS-RTK-Residuals |
|  | posSibType2-16 | GNSS-RTK-FKP-Gradients |
|  | posSibType2-17 | GNSS-SSR-OrbitCorrections |
|  | posSibType2-18 | GNSS-SSR-ClockCorrections |
|  | posSibType2-19 | GNSS-SSR-CodeBias |
|  | posSibType2-20 | GNSS-SSR-URA |
|  | posSibType2-21 | GNSS-SSR-PhaseBias |
|  | posSibType2-22 | GNSS-SSR-STEC-Correction |
|  | posSibType2-23 | GNSS-SSR-GriddedCorrection |
|  | posSibType2-24 | NavIC-DifferentialCorrections |
|  | posSibType2-25 | NavIC-GridModelParameter |
| OTDOA Assistance Data (clause 7.4.2) | posSibType3-1 | OTDOA-UE-Assisted |
| Barometric Assistance Data (clause 6.5.5.8) | posSibType4-1 | Sensor-AssistanceDataList |
| TBS Assistance Data (clause 6.5.4.8) | posSibType5-1 | TBS-AssistanceDataList |
| NR DL-TDOA/DL-AoD Assistance Data (clauses 6.4.3, 7.4.2) | posSibType6-1 | NR-DL-PRS-AssistanceData |
|  | posSibType6-2 | NR-UEB-TRP-LocationData |
|  | posSibType6-3 | NR-UEB-TRP-RTD-Info |

7.5.2.3	Test description

7.5.2.3.1	Pre-test conditions

System Simulator:

-	Cell 1.

-	Satellite signals (Sub-test case 25): as specified in 5.2.1.

-	MBS signals (Sub-test 23): as specified in 5.2.4.

UE:

The UE shall begin the test with no assistance data stored.

Preamble:

-	The UE is in state Generic RB Established (state 3) according to 3GPP 36.508 [8].

-	Then the SS sends the RESET UE POSITIONING STORED INFORMATION message to the UE to clear the stored assistance data in the UE.

-	The UE is switched-off.

Related PICS/PIXIT Statements:

-

7.5.2.3.2	Test procedure sequence

Table 7.5.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. |  |  | - | - |
| 2 | System information that includes the needed posSystemInformation is broadcasted. | <-- | RRC: SYSTEM INFORMATION (BCCH) | - | - |
| 3-10 | Steps 2-9 of the generic procedure for UE registration specified in TS 36.508 [8] subclause 4.5.2.3 are performed. | - | - |  |  |
| 11 | The SS sends a LPP message of type Request Capabilities. | <-- | DLInformationTransfer (LPP REQUEST CAPABILITIES) | - | - |
| 12 | The UE sends a LPP message of type Provide Capabilities including the UE positioning capabilities. | --> | ULInformationTransfer (LPP PROVIDE CAPABILITIES) | - | - |
| 13 | IF the UE LPP message at step 12 includes an acknowledgment request THEN SS sends a LPP Acknowledgement response. | <-- | DLInformationTransfer (LPP ACKNOWLEDGEMENT) | - | - |
| 14 | The SS sends a LPP message of type Request Location Information including a request for location estimate and location measurements. | <-- | DLInformationTransfer (LPP REQUEST LOCATION INFORMATION) | - | - |
| 15 | The UE sends a LPP message of type Provide Location Information including location estimate. | --> | ULInformationTransfer (LPP PROVIDE LOCATION INFORMATION) | 1 | P |
| 16 | IF the UE LPP message at step 15 includes an acknowledgment request THEN SS sends a LPP Acknowledgement response. | <-- | DLInformationTransfer (LPP ACKNOWLEDGEMENT) | - | - |

7.5.2.3.3	Specific message contents

Table 7.5.2.3.3-1: RESET UE POSITIONING STORED INFORMATION (Preamble)

| Derivation Path: 36.509 clause 6.9 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE Positioning Technology |  | Sub-test 23: 0 0 0 0 0 0 1 0 Sub-test 24: 0 0 0 0 0 1 0 1 Sub-test 25: 0 0 0 0 0 0 0 0 | Sub-test 23: MBS Sub-test 24: Sensor Sub-test 25: GNSS |  |

Table 7.5.2.3.3-2: SystemInformationBlockType1 (step 2, Table 7.5.2.3.2-1)

| Derivation Path: TS 36.508, Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| posSchedulingInfoList-r15 |  | PosSchedulingInfoList-r15 as defined in Table 7.5.2.3.3-3 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.5.2.3.3-3: PosSchedulingInfoList-r15 (SystemInformationBlockType1, Table 7.5.2.3.3-2)

| Derivation Path: TS 36.331, clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PosSchedulingInfoList-r15 ::= SEQUENCE (SIZE (1..maxSI-Message)) OF PosSchedulingInfo-r15 { |  | 2 entries | The size of PosSchedulingInfo-r15 is 1 for Sub-tests 23 and 24. If Sub-test 25 and UE supporting X GNSS systems, the size of PosSchedulingInfo-r15 is (X+1) and the PosSchedulingInfo-r16 need to be broadcast X times for different GNSS. |  |
| PosSchedulingInfo-r15[1] SEQUENCE { |  |  | entry 1 |  |
| posSI-Periodicity-r15 |  | rf32 |  |  |
| posSIB-MappingInfo-r15 SEQUENCE (SIZE (1..maxSIB)) OF PosSIB-Type-r15 { |  | 3 entries for Sub-test 25, 1 entry for Sub-tests 23 and 24 | PosSibType-n = posSibType1-1, posSibType1-2, posSibType1-3 | Sub-test 25 |
|  |  |  | PosSibType-n = posSibType4-1, | Sub-test 24 |
|  |  |  | PosSibType-n = posSibType5-1, | Sub-test 23 |
| PosSIB-Type-r15[n] SEQUENCE { |  |  | entry n |  |
| encrypted-r15 |  | Not present |  |  |
| gnss-id-r15 |  | Set depending on the GNSS supported by the UE |  |  |
| sbas-id-r15 |  | Not present |  |  |
| posSibType-r15 |  | PosSibType-n, | The posSibType-r15 is depending on the sub–test cases and entry number n |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| PosSchedulingInfo-r15[2] SEQUENCE { |  |  | entry 2 If the UE supporting X GNSS systems, the PosSchedulingInfo-r15 need to be broadcast X times for different GNSS. | Sub-test 25 |
| posSI-Periodicity-r15 |  | rf32 |  |  |
| posSIB-MappingInfo-r15 SEQUENCE (SIZE (1..maxSIB)) OF PosSIB-Type-r15 { |  | 6 entries for Sub-test 25 | posSibType2-1, posSibType2-3, posSibType2-6, posSibType2-7, posSibType2-8, posSibType2-9 |  |
| PosSIB-Type-r15[m] SEQUENCE { |  |  | entry m |  |
| encrypted-r15 |  | Not present |  |  |
| gnss-id-r15 |  | Set depending on the GNSS supported by the UE |  |  |
| sbas-id-r15 |  | Not present |  |  |
| posSibType-r15 |  | PosSibType-m, | The posSibType-r15 is depending on the entry number m |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.5.2.3.3-4: SystemInformation (step 2, Table 7.5.2.3.2-1)

| Derivation Path: TS 36.331, clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SystemInformation ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| criticalExtensionsFuture-r15 CHOICE { |  |  |  |  |
| posSystemInformation-r15 SEQUENCE { |  |  |  |  |
| posSIB-TypeAndInfo-r15 SEQUENCE (SIZE (1..maxSIB)) OF CHOICE { |  | 3 entries |  | Sub-test 25 |
| posSib1-1-r15 |  | SystemInformationBlockPos-r15 as defined in 7.5.2.3.3-5 | entry 1 |  |
| posSib1-2-r15 |  | SystemInformationBlockPos-r15 as defined in 7.5.2.3.3-5 | entry 2 |  |
| posSib1-3-r15 |  | SystemInformationBlockPos-r15 as defined in 7.5.2.3.3-5 | entry 3 |  |
| posSIB-TypeAndInfo-r15 SEQUENCE (SIZE (1..maxSIB)) OF CHOICE { |  | 6 entries |  | Sub-test 25 |
| posSib2-1-r15 |  | SystemInformationBlockPos-r15 as defined in 7.5.2.3.3-5 | entry 1 |  |
| posSib2-3-r15 |  | SystemInformationBlockPos-r15 as defined in 7.5.2.3.3-5 | entry 2 |  |
| posSib2-6-r15 |  | SystemInformationBlockPos-r15 as defined in 7.5.2.3.3-5 | entry 3 |  |
| posSib2-7-r15 |  | SystemInformationBlockPos-r15 as defined in 7.5.2.3.3-5 | entry 4 |  |
| posSib2-8-r15 |  | SystemInformationBlockPos-r15 as defined in 7.5.2.3.3-5 | entry 5 |  |
| posSib2-9-r15 |  | SystemInformationBlockPos-r15 as defined in 7.5.2.3.3-5 | entry 6 |  |
| posSIB-TypeAndInfo-r15 SEQUENCE (SIZE (1..maxSIB)) OF CHOICE { |  | 1 entry |  | Sub-test 24 |
| posSib4-1-r15 |  | SystemInformationBlockPos-r15 as defined in 7.5.2.3.3-5 | entry 1 |  |
| posSIB-TypeAndInfo-r15 SEQUENCE (SIZE (1..maxSIB)) OF CHOICE { |  | 1 entry |  | Sub-test 23 |
| posSib5-1-r15 |  | SystemInformationBlockPos-r15 as defined in 7.5.2.3.3-5 | entry 1 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.5.2.3.3-5: SystemInformationBlockPos-r15 (SystemInformation, Table 7.5.2.3.3-4)

| Derivation Path: TS 36.331, clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SystemInformationBlockPos-r15 ::= SEQUENCE { |  |  |  |  |
| assistanceDataSIB-Element-r15 |  | OCTET STRING containing AssistanceDataSIBelement |  |  |
| } |  |  |  |  |

Table 7.5.2.3.3-6: AssistanceDataSIBelement

| Derivation Path: TS 37.355, clause 7.4.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| AssistanceDataSIBelement-r15 ::= SEQUENCE { |  |  |  |  |
| valueTag-r15 |  | Not present |  |  |
| expirationTime-r15 |  | Not present |  |  |
| cipheringKeyData-r15 |  | Not present |  |  |
| segmentationInfo-r15 |  | Not present |  | Not present when the posSIB is not segmented |
| segmentationInfo-r15 SEQUENCE { |  |  |  | Present when the posSIB is segmented |
| segmentationOption-r15 |  | octet-string-seg |  |  |
| assistanceDataSegmentType-r15 |  | notLastSegment |  | Used when posSIB is segmented and the segment is not the last segment |
|  |  | lastSegment |  | Used when posSIB is not segmented, or posSIB is segmented and the segment is the last segment |
| assistanceDataSegmentNumber-r15 |  | index of the segmented posSIB |  |  |
| } |  |  |  |  |
| assistanceDataElement-r15 |  | OCTET STRING containing LPP IEs (GNSS-ReferenceTime, GNSS-ReferenceLocation, GNSS-IonosphericModel, GNSS-TimeModelList, GNSS-NavigationModel, GNSS-AcquisitionAssistance, GNSS-Almanac, GNSS-UTC-Model, GNSS-AuxiliaryInformation) as defined in TS 37.571-5 [12] and mapped according to the content of posSibType-r15 in Table 7.5.2.3.3-3. |  |  |
|  |  | OCTET STRING containing sensor-AssistanceDataList as defined in Table 5.4-2 |  | Sub-test 24 |
|  |  | OCTET STRING containing tbs-AssistanceDataList as defined in Table 5e2 |  | Sub-test 23 |
| } |  |  |  |  |

Table 7.5.2.3.3-7: DLInformationTransfer (step 11, step 13, step 14 and step 16, Table 7.5.2.3.2-1)

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DLInformationTransfer ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| dlInformationTransfer-r8 SEQUENCE { |  |  |  |  |
| dedicatedInfoType CHOICE { |  |  |  |  |
| dedicatedInfoNAS OCTET STRING |  | Set according to Table 7.5.2.3.3-8 | DOWNLINK GENERIC NAS TRANSPORT |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.5.2.3.3-8: DOWNLINK GENERIC NAS TRANSPORT (DLInformationTransfer, Table 7.5.2.3.3-7)

| Derivation Path: 24.301 Table 8.2.31.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | 0111 | EPS mobility management messages |  |
| Security header type |  | 0000 | Plain NAS message |  |
| Downlink generic NAS transport message identity |  | 01101000 | Downlink generic NAS transport |  |
| Generic message container type |  | 00000001 | LTE Positioning Protocol (LPP) message container |  |
| Generic message container |  | Step 11: Set according to Table 5.4-1 | LPP Request Capabilities. |  |
|  |  | Step 14: Set according to Table 7.5.2.3.3-9 | LPP Request Location Information |  |
|  |  | Steps 13 and 16: Set according to Table 7.5.2.3.3-10 | LPP Acknowledgement |  |
| Additional information |  | Present | Routing Identifier/ Correlation ID |  |

Table 7.5.2.3.3-9: LPP Request Location Information (DOWNLINK GENERIC NAS TRANSPORT, Table 7.5.2.3.3-8)

| Derivation Path: Table 5.4-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| As defined in Table 5.4-3 with the following exceptions: |  |  |  |  |
| locationInformationType |  | locationMeasurementsPreferred |  |  |

Table 7.5.2.3.3-10: LPP Acknowledgement (DOWNLINK GENERIC NAS TRANSPORT, Table 7.5.2.3.3-8)

| Derivation Path: 36.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |  |
| transactionID |  | Not present |  |  |
| endTransaction |  | TRUE |  |  |
| sequenceNumber |  | Not present |  |  |
| acknowledgement SEQUENCE { |  |  |  |  |
| ackRequested |  | FALSE |  |  |
| ackIndicator |  | (0..255) | Contains the same value of the sequenceNumber field in step 12 or 15, Table 7.5.2.3.1-1 |  |
| } |  |  |  |  |
| lpp-MessageBody |  | Not present. |  |  |
| } |  |  |  |  |

Table 7.5.2.3.3-11: ULInformationTransfer (steps 12 and 15, Table 7.5.2.3.2-1)

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| ULInformationTransfer ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| ulInformationTransfer-r8 SEQUENCE { |  |  |  |  |
| dedicatedInfoType CHOICE { |  |  |  |  |
| dedicatedInfoNAS OCTET STRING |  | Set according to Table 7.5.2.3.3-12 | UPLINK GENERIC NAS TRANSPORT |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.5.2.3.3-12: UPLINK GENERIC NAS TRANSPORT (ULInformationTransfer, Table 7.5.2.3.3-11)

| Derivation Path: 24.301 Table 8.2.32.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | 0111 | EPS mobility management messages |  |
| Security header type |  | 0000 | Plain NAS message |  |
| Uplink generic NAS transport message identity |  | 01101001 | Uplink generic NAS transport |  |
| Generic message container type |  | 00000001 | LTE Positioning Protocol (LPP) message container |  |
| Generic message container |  | Step 12: Set according to Table 7.5.2.3.3-13 | LPP Provide Capabilities |  |
|  |  | Step 15: Set according to Table 7.5.2.3.3-14 | LPP Provide Location Information |  |
| Additional information |  | Present | The UE includes the Routing Identifier received in the Additional Information IE of the DOWNLINK GENERIC NAS TRANSPORT message (step 11 Table 7.5.2.3.2-1) |  |

Table 7.5.2.3.3-13: LPP Provide Capabilities (UL NAS TRANSPORT, Table 7.5.2.3.3-12)

| Derivation Path: 36.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |  |
| transactionID SEQUENCE { |  |  |  |  |
| initiator |  | locationServer |  |  |
| transactionNumber |  | (0..255) | Contains the same value as the corresponding field in the LPP Request Capabilities message in step 11, Table 7.5.2.3.2-1 |  |
| } |  |  |  |  |
| endTransaction |  | TRUE |  |  |
| sequenceNumber |  | (0..255) |  |  |
| acknowledgement SEQUENCE { |  | Present, or not present |  |  |
| ackRequested |  | TRUE |  |  |
| ackIndicator |  | Not present |  |  |
| } |  |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideCapabilities SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideCapabilities-r9 SEQUENCE { |  |  |  |  |
| commonIEsProvideCapabilities |  | Dependent on UE capabilities |  |  |
| a-gnss-ProvideCapabilities |  | Dependent on UE capabilities |  |  |
| otdoa-ProvideCapabilities |  | Not present |  |  |
| ecid-ProvideCapabilities |  | Not present |  |  |
| epdu-ProvideCapabilities |  | Not present |  |  |
| sensor-ProvideCapabilities-r13 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| tbs-ProvideCapabilities-r13 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| wlan-ProvideCapabilities-r13 |  | Not present |  |  |
| bt-ProvideCapabilities-r13 |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.5.2.3.3-14: LPP Provide Location Information (UL NAS TRANSPORT, Table 7.5.2.3.3-12)

| Derivation Path: 36.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |
| transactionID SEQUENCE { |  |  |  |
| initiator | locationServer |  |  |
| transactionNumber | (0..255) | Contains the same value as the corresponding field in LPP Request Location Information message in step 14, Table 7.5.2.3.2-1 |  |
| } |  |  |  |
| endTransaction | TRUE |  |  |
| sequenceNumber | (0..255) |  |  |
| acknowledgement SEQUENCE { | Present, or not present |  |  |
| ackRequested | TRUE |  |  |
| ackIndicator | Not present |  |  |
| } |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideLocationInformation SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideLocationInformation-r9 SEQUENCE { |  |  |  |
| commonIEsProvideLocationInformation | Not present |  |  |
| a-gnss-ProvideLocationInformation SEQUENCE { | Present for sub-test 25 |  |  |
| gnss-SignalMeasurementInformation SEQUENCE { | Present. |  |  |
| measurementReferenceTime | Any value acceptable |  |  |
| gnss-MeasurementList | Any value acceptable |  |  |
| } |  |  |  |
| gnss-LocationInformation SEQUENCE { | May be present |  |  |
| measurementReferenceTime | Any value acceptable |  |  |
| agnss-List | Any value acceptable |  |  |
| } |  |  |  |
| gnss-Error | Not present |  |  |
| } |  |  |  |
| ecid-ProvideLocationInformation | Not present |  |  |
| epdu-ProvideLocationInformation | Not present |  |  |
| sensor-ProvideLocationInformation-r13 SEQUENCE { | Present for sub-test 24 | Rel-16 onwards |  |
| sensor-MeasurementInformation-r13 | Present. Any value acceptable |  |  |
| sensor-Error-r13 | Not present |  |  |
| sensor-MotionInformation-r15 | May be present |  |  |
| } |  |  |  |
| tbs-ProvideLocationInformation-r13 | Present for sub-test 23 | Rel-16 onwards |  |
| tbs-MeasurementInformation-r13 | Present. Any value acceptable |  |  |
| tbs-Error-r13 | Not present |  |  |
| } |  |  |  |
| wlan-ProvideLocationInformation-r13 | Not present |  |  |
| bt-ProvideLocationInformation-r13 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |